3GPP TSG-RAN4 Meeting #112bis	R4-2416477

Hefei, China, 14th – 18th October, 2024

Agenda item:	6.17.4.2

Source:	Keysight Technologies UK Ltd

Title:	On Testability Aspects for FR2 AI/ML Beam Management

Document for:	Approval

# Introduction

This contribution addresses test system and testability considerations for AI/ML beam management use cases.

# Discussion

Beam Management is one objective of the AI/ML [1]

- Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]: - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”) - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”) - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2

While not specifically defined anywhere, we believe that the beam management topic is primarily applicable to FR2 and eventually possibly to the 7-24 GHz range but likely not to FR1. As RAN1 has only focused on FR2 simulations, we propose to limit the RAN4 work on FR2 only.

Observation 1: The AI/ML beam management objective seems primarily targeted for FR2.

Proposal 1: It is proposed to limit the testability discussions of AI/ML Beam Management test cases to FR2 only.

The following agreements were made in the May meeting regarding testability and test system topics [2]

Issue 2-5: Test setup needs Agreement: maximum number of set B Tx beams that test system should be able to emulate: [8-16] maximum number of set A Tx beams that test system should be able to emulate: [64-128] FFS on AoAs UE rotation during the test: FFS UE rotation/repositioning between different tests: Yes

and in the last August meeting, additional clarifications were requested

Issue 2-4: Test setup needs Issues listed below to be further investigated in order to determine the capabilities of the test equipment: - set B Tx beams; set A Tx beams - number of beams transmitted simultaneously - overall number of beams transmitted during the test - AoA - AoD - UE rotation during the test Other aspects can also be considered

In terms of channel models, no progress was made in the last two meetings, i.e., both CDL and TDL are kept as options [3]

Issue 2-3: Channel models Agreement: Further discuss and down-select the following options considering TE feasibility - Option 2: TDL - Option 5: simplified CDL - CDL channel model with a small number of clusters. CDL simplification needs to be further discussed

This contribution intends to provide additional views from our side how these agreements can be addressed in OTA test systems using channel emulation; this contribution is taking into account discussions in [4], [5].

# TDL vs CDL Models

An extensive review of TDL vs CDL Models was provided in [4],[5]. Additionally, two other TE vendors provided their views on this topic [6], [7] with overall favourable view towards CDL models while making sure that some simplifications are considered.

Observation 2: Three TE vendors provided overall favourable view towards CDL models while making sure that some simplifications are considered.

The following table shares our views of CDL and TDL models

Table 1: Overview of TDL vs CDL Models with respect to AI/ML Beam Management Use Cases

| Topic | Views |
| --- | --- |
| Realism to properly assess AI/ML algorithms | CDL: accurately presents the complexities of beam dependent fading characteristics and spatial characteristics of the multipath propagation environment TDL: cannot include the impact of 3D antenna beam patterns |
| L1-RSRP relations between the beams | CDL: The beam specific reference channel gains and respective L1-RSRPs for each beam of different beam sets can be calculated only by using a spatial channel model. TDL: Beam specific channel gains or L1-RSRPs cannot be determined based on TDL models |
| Implementation complexities in Channel Emulator | From a purely channel model perspective and the realization in state-of-the-art channel emulators, implementation of CDL models is not more complex than TDL models |
| Simplification | CDL: Applying beamforming at the gNB will effectively filter out many of the multipath propagation paths and simplifying the resulting channel model seen by the UE to a reasonable number of strong clusters for multiprobe OTA environment modelling TDL: simplified TDL models which cannot reflect critical beamforming effects at all |

# OTA Test Systems

The thorough review of FR2 OTA test systems is not repeated here but can be found in [4][5]. Table 2 summarizes those observations and findings.

Table 2: Applicability of existing OTA test systems to testability criteria for AI/ML beam management

| Applicability Criteria | IFF | Enhanced IFF | FR2 MIMO OTA |
| --- | --- | --- | --- |
| CDL Channel Models | No | No | Yes |
| TDL Channel Models | Yes | Yes | Yes |
| Multiple TRPs simultaneously | No | Yes | Yes |
| True FF Conditions | Yes | Yes | Not necessarily |
| Multiple TRPs with CDL channel models presented simultaneously | No | No | Requires upgrade whose cost & complexity might be not negligible |
| Multiple TRPs with TDL channel models presented simultaneously or sequentially at t0 and t0 + DT (without positioner movement) | No | Yes | Yes |
| Multiple TRPs with CDL channel models presented sequentially at t0 and t0 + DT (using positioner movement) | No | No | Yes, but DT≥1s might be excessive |
| Multiple TRPs with at most TDL channel models presented sequentially at t0 and t0 + DT (using positioner movement) | Yes, but DT≥1s might be too slow | Yes, but DT≥1s might be too slow | Yes, but DT≥1s might be too slow |

The IFF test system with just a single probe is certainly not able to implement a CDL channel model. While the Enhanced IFF test system, supporting RRM and Multi-RX test cases, has multiple probes, the large probe separations in excess of 30° and lack of absolute probe locations defined prevent it from being applicable to CDL channel models even when further simplified.

Proposal 2: Do not consider a single-probe IFF system for AI/ML Beam Management use cases due to lack of CDL support

Proposal 3: Do not consider the Enhanced IFF system supporting RRM and Multi-RX test cases for AI/ML Beam Management use cases due to large probe separation and lack of absolute probe locations.

As highlighted in [4][5], if the intention/requirement is to implement CDL channel models, only the FR2 MIMO OTA test system (or derivatives thereof) with DFF probes is applicable.

Proposal 4: Consider the FR2 MIMO OTA test system (or derivatives thereof) with DFF probes to properly and accurately implement CDL channel model.

Given the complexity of introducing multiple, simultaneous TRPs (with large angular separation between TRPs) beyond 2, e.g., lack of absolute probe locations for Enhanced IFF and lack of wide angular separation for FR2 MIMO OTA, it is advisable to limit the maximum number of spatially separated TRPs to 2 if more than 1 TRP is required.

Observation 3: It is advisable to limit the maximum number of spatially separated TRPs to 2 if more than 1 TRP is required

# Proposed Test System for AI/ML Beam Management Use Cases

Considering the BM-Case1, i.e., spatial-domain downlink beam prediction from [21], the requirement is to emulate the corresponding L1 RSRPs for Set B of beams in a lab environment as illustrated schematically in Figure 1.

Figure 1: Example Lab Environment for BM-Case1

In order to define requirements, the beam radiation patterns of the beams of Set A or B need to be defined for example by defining the antenna array model and beam directions of each beam index of specific beam set. The resulting average channel gain with the Tx- and Rx- antenna models embedded can be easily calculated for a known channel model using the principles defined in Clause 7.3 of [22]. Subsequently, the target L1 RSRP can be calculated by defining the Tx-power and NR numerology related frequency response sampling etc. parameters.

Based on our experience with developing beam management test cases, we believe a relatively simple and straightforward extension of an FR2 MIMO OTA system to 8 probes with a sample system visualized in Figure 2, is suitable to meet the following assumptions for a single TRP or two TRPs with close angular proximity:

- Emulation of Tx-beam powers and RX-PAS for 4 strongest or most relevant Tx-beams [5]

  - 4 Tx-beam RX-PAS profiles can be emulated for CDL-A, B, C, D and E UMi/UMa/InO scenarios utilizing 8 probes [5]

- Additionally, to emulate set B Tx beams beyond 4 [up to 16] while exercising the AI/ML algorithms, power offsets can be applied to the Tx-beams with the closest match of the 4 available RX-PAS profiles and exact power offset based on the 3D beam and CDL model characteristics

- More importantly, to emulate set A Tx beams [up to 128], power offsets can be applied to the Tx-beams with the closest match of the 4 available RX-PAS profiles and exact power offset based on the 3D beam and CDL model characteristics

- The RX-PAS AoA profile for beams emulated with power offsets cannot be guaranteed

- Support AoDs/ZoDs with a full range (0° to 360°)

- Support AoAs/ZoAs with a limited range of 110° in azimuth and 25° in elevation. for a single UE orientation.

- For accurate performance assessment of all or top-K Set A beams, testing will not require exercising the AI/ML algorithms on the UE-side and different UE orientations and/or sequential testing would be required when

More than 4 beams need to be assessed

N beams (with N≤4) have beam peaks with relative AoA/ZoA differences larger than the AoA/ZoA ranges supported by the system

Figure 2: Sample FR2 test system supporting AI/ML BM testing. Probes in blue correspond to the FR2 MIMO OTA probes.

The proposed OTA probe configuration with a range of 110° in azimuth for emulation of can cover 99.5% of cumulated power distribution over AoA sector for a set of 4 strongest Tx-beams as shown in Figure 3. Similarly, in the elevation domain, the 25° sector can cover 99.8% of the cumulated power distribution over ZoA sector for 4 beams as shown in Figure 4.

Figure 3: Envelope of power distribution over AoAs with 4 strongest Tx-beams of 128 beams (blue) and power distribution over AoAs for all 128 beams (grey).

Figure 4: Envelope of power distribution over EoAs with 4 strongest Tx-beams of 128 beams (blue) and power distribution over AoAs for all 128 beams (grey).

Observation 4: A relatively simple and straightforward extension of an FR2 MIMO OTA system to 8 probes is suitable to emulate 4 Tx-beam RX-PAS profiles, set B Tx beams up to [16] with power offsets, and set A Tx beams up to [192] with power offsets.

Observation 5: The extension of an FR2 MIMO OTA system to 8 probes is suitable to support unlimited range of AoDs/ZoDs and a range of AoAs/ZoAs of 110°/25°.

Proposal 5: UE/chipset/infra vendors to provide feedback on whether modelling 4 RX-PAS accurately with the remaining beams modelled with power offsets is sufficient

Some of these testing aspects are already supported by Keysight’s S8708A Advanced Performance Test (APT) toolset [5]. With just minor adjustments, it can address all the following challenges:

- Validate dynamic beam management performance for over the air 3D spatial fading and blocking conditions

- Verifying that gNodeB and UE beam management procedures operate seamlessly in highly dynamic fading channels at FR2 in all mobility scenarios

- Simulating dynamic channels with time-variant characteristics like Doppler shifts and temporal fading

- Testing AI models' robustness under various controlled yet realistic conditions to ensure reliability

Observation 6: Various beam management performance testing can readily be supported with a commercially available test system, i.e., complexity and time-to-market concerns should not be an issue.

Proposal 6: Consider an 8-probe OTA system with 2 additional probes when compared to the FR2 MIMO OTA system with a range of AoAs/ZoAs of 110°/25° as the starting point for AI/ML Beam management testing for 1 TRP and/or 2 TRPs with close angular proximity.

# AI/ML Training vs Conformance Testing

While the training of the AI/ML models/algorithms is not part of the 3GPP scope and conformance testing, the eventual assessment of the AI/ML models and the efficacy of these models related to beam management are scope of the WI [1].

The training exercise is left to OEMs and the systems and algorithms for training might be largely proprietary. However, it can be assumed that the AI/ML algorithms will be trained using highly realistic environmental conditions, i.e., channel models, AoAs, etc., to optimize the UE performance in the field. OTA test systems with a high degree of complexity to correspond to realistic, real-world conditions could be leveraged for such training. It is also possible that the AI models may need to be customized for each UE model/SKU and not just for each chipset. Training with anything other than highly realistic CDL channel models is likely to provide inferior performance. Training using synthetic data obtained from hyper-realistic digital twins of real-world environments is also a possibility.

Conformance testing certainly does not require test systems with the same degree of complexity and capabilities as the training and should rather be a subset of systems used for training. Performing conformance testing with TDL models that are far less realistic and that the ML model likely has not been trained on due to the lack of realism is likely to provide inconsistencies in test results and hence is not recommended.

Observation 7: Performing conformance testing with TDL models that are far less realistic and that the ML model likely has not been trained on due to the lack of realism is likely to provide inconsistencies in test results.

Proposal 7: Based on the overview of Table 1 and Observation 7, select CDL models for the testability aspects of the AI/ML Beam Management use cases

# Efficacy of AI/ML Models

While OEMs and chipset vendors have demonstrated that the use of AI/ML UE-side models adds gain, it is not clear whether the benefit of AI/ML outweighs the impact in performance, e.g., reduced battery life which is very critical for handhelds (PC3).

Observation 8: It is not clear whether the benefit of AI/ML UE-side models outweighs the impact in performance, e.g., reduced battery life which is very critical for handhelds (PC3)

# Conclusion

The following observations and conclusions were made in this contribution.

Observation 1: The AI/ML beam management objective seems primarily targeted for FR2.

Observation 2: Three TE vendors provided overall favourable view towards CDL models while making sure that some simplifications are considered.

Observation 3: It is advisable to limit the maximum number of spatially separated TRPs to 2 if more than 1 TRP is required

Observation 4: A relatively simple and straightforward extension of an FR2 MIMO OTA system to 8 probes is suitable to emulate 4 Tx-beam RX-PAS profiles, set B Tx beams up to [16] with power offsets, and set A Tx beams up to [192] with power offsets.

Observation 5: The extension of an FR2 MIMO OTA system to 8 probes is suitable to support unlimited range of AoDs/ZoDs and a range of AoAs/ZoAs of 110°/25°.

Observation 6: Various beam management performance testing can readily be supported with a commercially available test system, i.e., complexity and time-to-market concerns should not be an issue.

Observation 7: Performing conformance testing with TDL models that are far less realistic and that the ML model likely has not been trained on due to the lack of realism is likely to provide inconsistencies in test results.

Observation 8: It is not clear whether the benefit of AI/ML UE-side models outweighs the impact in performance, e.g., reduced battery life which is very critical for handhelds (PC3)

Proposal 1: It is proposed to limit the testability discussions of AI/ML Beam Management test cases to FR2 only.

Proposal 2: Do not consider a single-probe IFF system for AI/ML Beam Management use cases due to lack of CDL support

Proposal 3: Do not consider the Enhanced IFF system supporting RRM and Multi-RX test cases for AI/ML Beam Management use cases due to large probe separation and lack of absolute probe locations.

Proposal 4: Consider the FR2 MIMO OTA test system (or derivatives thereof) with DFF probes to properly and accurately implement CDL channel model.

Proposal 5: UE/chipset/infra vendors to provide feedback on whether modelling 4 RX-PAS accurately with the remaining beams modelled with power offsets is sufficient

Proposal 6: Consider an 8-probe OTA system with 2 additional probes when compared to the FR2 MIMO OTA system with a range of AoAs/ZoAs of 110°/25° as the starting point for AI/ML Beam management testing for 1 TRP and/or 2 TRPs with close angular proximity.

Proposal 7: Based on the overview of Table 1 and Observation 7, select CDL models for the testability aspects of the AI/ML Beam Management use cases

# References

- RP-240774, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, 3GPP TSG RAN Meeting #103, March 2024

- R4-2410570, WF on AI/ML, Qualcomm Incorporated, 3GPP TSG-RAN WG4 Meeting #111, May 2024

- R4-2414323, WF on AI/ML, Qualcomm Incorporated, 3GPP TSG-RAN WG1 Meeting #115, August 2024

- R4-2409770, OTA Test System/Testability Considerations for FR2 AI/ML Beam Management Use Cases, Keysight Technologies, 3GPP TSG-RAN4 Meeting #111, May 2024

- R4-2411254, OTA Test System/Testability Considerations for FR2 AI/ML Beam Management Use Cases, Keysight Technologies, 3GPP TSG-RAN4 Meeting #112, August 2024

- R4-2411587, Discussion on test setup for AI/ML based beam management, Rohde & Schwarz, 3GPP TSG-RAN4 Meeting #112, August 2024

- R4-2411279, Study on CDL channel model for AI/ML beam management in FR2, Anritsu Corporation, 3GPP TSG-RAN4 #112, August 2024

- TS 38.508-1, User Equipment (UE) conformance specification; Part 1: Common test environment

- TS 38.533, User Equipment (UE) conformance specification; Radio Resource Management (RRM)

- TR 38.871, Study on NR frequency range 2 (FR2) Over-the-Air (OTA) testing enhancements

- TR38.810, Study on test methods

- R4-2309469, On Absolute Probe Locations for Multi-Rx Testing, Keysight Technologies, 3GPP TSG-RAN4 Meeting #107, May 2023

- TR 38.827, Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR User Equipment (UE)

- TS 38.151, Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) performance requirements

- R4-2404215, On AI/ML RAN4 Use Cases: Beam Management, Qualcomm, 3GPP TSG-RAN WG4 Meeting # 110bis, April 2024

- R4-1711840, Channel model spatial filtering using 38.803 gNB Tx antenna assumptions, Keysight, 3GPP TSG-RAN WG4 Meeting #84bis, October 2017

- R4-1713774, Impact of fast fading on strongest beam index for static geometry channels, Keysight, 3GPP TSG-RAN WG4 Meeting #85, December 2017

- R4-2002478, System Implementation of FR2 3D MPAC Systems, Keysight Technologies, 3GPP TSG-RAN WG4 Meeting #94-e, March 2020

- R4-1705838, Simplified sectorized MPAC for RRM/Demodulation Setup, Keysight Technologies, 3GPP TSG-RAN WG4 Meeting #83, May 2017

- R4-1813602, Channel model and test method considerations for NR radiated requirements, Keysight Technologies, 3GPP TSG-RAN WG4 Meeting #88-bis, October 2018

- TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

- TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz