3GPP TSG-RAN WG4 Meeting #112bis										                 R4-2416848

Hefei, Anhui, China, 14th – 18th Oct 2024

Agenda item:			6.1.3

Source:	Huawei, HiSilicon

Title:	WF on RRM requirements for NR_ENDC_RF_Ph4

Document for:	Approval

Topic #1:	RRM requirements for R19 UE RF enhancements for NR FR1/FR2 and EN-DC

<Agreement> Issue 1-1: Interruption requirements at SRS antenna switching for 6Rx capable UE

Agreement:

- The existing interruption requirements at SRS antenna switching are applicable to 6RX capable UE, and no specification impact is observed.

  - This can be revisited if the existing requirement cannot be applied to any SRS antenna switching pattern agreed in RAN1.

<Agreement> Issue 1-2: SNR level for RLM and BFD testing for 6Rx capable UE

- Proposals

  - Option 1(Huawei): Reuse SNR levels specified for 4Rx capable UE.

  - Option 2 (Ericsson): Impact on SNR levels for RLM and BFD tests is discussed in the performance part of the WI.

- Online Agreement: Discuss the issue in performance part.

(For information: performance part will start from RAN4#114bis)

<Agreement> Issue 1-3: Antenna configurations for 6Rx capable UE

- Proposals

  - Option 1(Huawei): a new clause is to be introduced and the requirements of “A.3.6.1.2 Antenna connection for 8 Rx capable UEs” shall apply with “8RX” being replaced with “6RX”.

- Online Agreement: Discuss the issue in performance part.

(For information: performance part will start from RAN4#114bis)