# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2416853

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:			6.15.4

Source:	Apple

Title:	WF on RRM requirements for NR_RRM_Ph5_Part1

Document for:	Approval

# Topic #1: FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor (6.15.2.1)

Issue 1-1-1: Conditions to trigger UE to activate L3 measurement delay reduction by optimizing Rx BSF:

Agreement in online session:

For scenario 1/2/3/4:

  - RAN4 to consider conditions for triggering FBS for L3 measurement, including active condition and de-active conditions.

    - (De-)Activating multi-Rx for L3 measurements (intra/inter-frequency) is depending on the criteria/threshold(s) and UE current conditions.

  - The detailed criteria/condition(s) is FFS.

  - Meanwhile, RAN4 agree that the criteria/threshold(s) are configured by RRC signaling. FFS the further details.

For Scenario 5/7:

  - For UE supporting Rel-19 BSF reduction capability, faster beam sweeping always applies during handover (Tsearch).

  - For UE supporting Rel-19 BSF reduction capability, faster beam sweeping always applies during RRC re-establishment.

Issue 1-1-2: Conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF” :

Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2415656.

Issue 1-1-3: FFS: RRM measurement with two panels activated, two searchers are occupied by this single carrier

Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2415656.

Issue 1-1-4: FFS: SSB processing delay/time for processing multiple beams received in a SMTC

Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2415656.

Issue 1-1-5: FFS: L3 measurement delay reduction by optimizing Rx BSF with/without DRX

Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2415656.

Issue 1-2: Solutions to apply/specify L3 measurement delay reduction by optimizing Rx BSF

Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2415656.

Issue 1-3: feature capability of L3 measurement delay reduction by optimizing Rx BSF

Agreement:

- RAN4 is to introduce a new individual optional capability for L3 BSF reduction due to multi-Rx operation in R19.

  - The detailed definition can be postponed to the end of the core part discussion.

Issue 1-4-1: FFS: requirement at transition between the normal L3 measurement and FBS L3 measurement

Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2415656.

# Topic #2: FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC (6.15.2.2)

Issue 2-1: Clarification on the bullets in WID for this CSSF optimization

Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2415656.

Issue 2-2: UE measurement procedure to use L3 measurement delay reduction by optimizing CSSF

Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2415656.

Issue 2-3: Applicability requirement of L3 measurement delay reduction by optimizing CSSF

Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2415656.

Issue 2-4: Searcher assumption to apply L3 measurement delay reduction by optimizing CSSF

Agreement in online session:

On top of the previous agreement on the baseline assumption of 2 searchers, for 3 searchers:

- Discuss the applicable scenario(s) and/or solution(s) based on 3 searchers. Decide whether or not to define the requirement later.

Issue 2-5: Solutions to apply/specify L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC

Candidate options for further discussion:

- Option 1: UE can measure one serving CC per band if multiple serving CCs are in the same band. For the CCs to be measurement, further discuss option 1A and option 1B

  - Option 1A: Reuse the legacy principle for neighboring cell measurement.

  - Option 1B: Based on network indication

    - The other CCs (not measured based on network indication) need not to be measured and reported. Network assume the same measurement results on the measured CCs and unmeasured CCs in the same band.

- Option 2: UE can change the searcher occupancy ratio of measurement among the measured CCs

  - Option 2A: UE can change the searcher occupancy ratio of PCC or PSCC measurement to speed up SCC measurement for some conditions

For 3 searchers: In addition to option 1 and 2

- Option 3: consider to use 3 searchers for CSSF reduction.

  - Depending on issue 2-4.

Issue 2-6: feature capability of L3 measurement delay reduction by optimizing CSSF

Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2415656.

Issue 2-7: Other WID scope discussion

Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2415656.