3GPP TSG-RAN WG4 Meeting #112bis														R4-2416875

Hefei, Anhui, China, 14 – 18, 2024

Agenda item:			6.24.3

Source:	Apple

Title:	Ad-hoc minutes for NR_Mob_Ph4

Document for:	Approval

# Measurements related enhancements for LTM

## Event triggered L1 measurement reporting

#### Issue 1-1: RRM scope of Event triggered L1 measurement reporting

- Online Agreement:

  - RAN4 to define event-triggered L1 reporting requirement for SSB based and CSI-RS based measurements.

    - To start RAN4 discussion for SSB based L1 measurement firstly, and check how to apply/extend the agreements for SSB based L1 measurement to accommodate CSI-RS based L1 measurement.

  - From RAN4 RRM requirement perspective, for FR2-1, RRM requirements for multi-Rx based LTM L1 measurement is not considered in the WI scope.

  - RRM scope of Event triggered L1 measurement reporting

    - In addition to intra-frequency measurement and inter-frequency measurement without gap, support gap-based inter-frequency measurement for SSB-based L1 measurement. Further discuss for CSI-RS based L1 measurement.

#### Issue 1-2: supported frequency range in RRM requirements

- Candidate solutions:

  - Option 1: To investigate RRM requirement impacts for Event-triggered L1 reporting, RAN4 start from FR2-1. At a later stage, RAN4 to investigate whether FR2-1 conclusions achieved in the WI can also be applied to FR1 (vivo)

- Recommended WF

  - Continue discussion.

CMCC/Ericsson: not ok with option 1. Both FR1 and FR2-1 shall be considered.

- Ad-hoc Agreement:

  - Both FR1 and FR2-1 are in scope of RRM requirements for Event triggered L1 measurement reporting.

#### Issue 1-3: Requirements for reporting criteria per measurement category

- Candidate solutions:

  - Option 1: (MTK)

    - Define a separate capability for supported number of events triggered L1 measurement reporting.

    - Focus on SSB based L1-RSRP measurement for event triggered reporting at first.

    - For SSB based event triggered L1-RSRP measurement reporting, regard the events related to different candidate cells on different frequency layers as different events. Discuss how to differentiate intra-frequency and inter-frequency events. Further discuss the following options:

      - Option 1: if the LTM-CSI-ReportConfig is linked to multiple cells on different frequency layer, then the number of events related to this LTM-CSI-ReportConfig should be counted no less than the number of frequency layers.

      - Option 2: inform RAN2 that the events related to different candidate cells on different frequency layers are regarded as different events and ask RAN2 to take this into consideration when design the signaling.

  - Option 2: discuss and confirm in RAN4#112bis that whether requirements for reporting criteria per measurement category needs to be updated to cover L1-RSRP measurement on neighbor cell. If needed, it is proposed to use the following table as baseline. (Apple)

| Measurement category | Ecat | Note |
| --- | --- | --- |
| Intra-frequency Note 1,2,3,4,5 | 9 | Events for any one or a combination of intra-frequency SS-RSRP, SSB based L1-RSRP, CSI-RS based L1 RSRP, SS-RSRQ, SS-SINR, CSI-RSRP, CSI-RSRQ, and CSI-SINR for NG-RAN intra-frequency cells |
| Inter-frequency Note 2,3,4,5 | 10 | Events for any one or a combination of inter-frequency SS-RSRP, SSB based L1-RSRP, CSI-RS based L1 RSRP, SS-RSRQ, SS-SINR, CSI-RSRP, CSI-RSRQ, and CSI-SINR for NG-RAN inter-frequency cells |
| Inter-RAT (E-UTRA FDD, E-UTRA TDD) Note 2,4,5 | 10 | Only applicable for UE with this (inter-RAT) capability. These reporting criteria apply for any E-UTRA carrier frequencies other than the carrier frequency of the E-UTRA PSCell or E-UTRA SCell. |
| Inter-RAT (E-UTRA FDD, E-UTRA TDD) RSTD Note 2,4,5 | 1 | Inter-RAT RSTD measurement reporting for UE supporting OTDOA; 1 report capable of minimum 16 inter-RAT cell measurements. Only applicable for UE with this (inter-RAT RSTD via LPP [22]) capability. These reporting criteria apply for any E-UTRA carrier frequencies other than the carrier frequency of the E-UTRA PSCell or E-UTRA SCell. |
| Inter-RAT (E-UTRA FDD, E-UTRA TDD) RSRP and RSRQ measurements for E-CID Note 2,4,5 | 1 | Inter-RAT RSRP and RSRQ measurements for E-CID reported to E-SMLC via LPP [22]. One report capable of at least in total 10 inter-RAT RSRP and RSRQ measurements. Applicable to UE capable of reporting inter-RAT RSRP and RSRQ to E-SMLC via LPP. These reporting criteria apply for any E-UTRA carrier frequencies other than the carrier frequency of the E-UTRA PSCell or E-UTRA SCell. |
| Intra-frequency RSSI and channel occupancy measurements with CCA Note 1,2,3 | 1 | One report capable of one RSSI and one channel occupancy measurements over a channel [TS 37.213] with CCA. Applicable for UE capable of performing and reporting RSSI and channel occupancy on carrier frequencies under CCA. |
| Inter-frequency RSSI and channel occupancy measurements with CCA Note 2,3 | 1 | One report capable of one RSSI and one channel occupancy measurements over a channel [TS 37.213] with CCA. Applicable for UE capable of performing and reporting RSSI and channel occupancy on carrier frequencies under CCA. |
| Intra-frequency SSB-based measurements for NR E-CID Note 1,2,3,4,5 | 1 | Intra-frequency SS-RSRP and SS-RSRQ measurements for NR E-CID reported to LMF via LPP [34]. One report capable of at least in total 9 intra-frequency SS-RSRP and SS-RSRQ measurements. Applicable to UE capable of reporting at least one of SS-RSRP and SS-RSRQ to LMF via LPP. |
| Intra-frequency CSI-RS based measurements for NR E-CID Note 1,2,3,4,5 | 1 | Intra-frequency CSI-RSRP and CSI-RSRQ measurements for NR E-CID reported to LMF via LPP [22]. One report capable of at least in total 9 intra-frequency CSI-RSRP and/or CSI-RSRQ measurements. Applicable to UE capable of reporting any of CSI-RSRP and CSI-RSRQ to LMF via LPP, as indicated in nr-ECID-MeasSupported-r16. |
| Inter-frequency SSB-based measurements for NR E-CID Note 2,3,4,5 | 1 | Inter-frequency SS-RSRP and SS-RSRQ measurements for NR E-CID reported to LMF via LPP [34]. One report capable of at least in total 10 inter-frequency SS-RSRP and SS-RSRQ measurements. Applicable to UE capable of reporting at least one of SS-RSRP and SS-RSRQ to LMF via LPP. |
| Inter-frequency CSI-RS based measurements for NR E-CID Note 2,3,4,5 | 1 | Inter-frequency CSI-RSRP and CSI-RSRQ measurements for NR E-CID reported to LMF via LPP [22]. One report capable of at least in total 10 inter-frequency CSI-RSRP and CSI-RSRQ measurements. Applicable to UE capable of reporting any of CSI-RSRP and CSI-RSRQ to LMF via LPP, as indicated in nr-ECID-MeasSupported-r16. |
| DL RSTD Note 2,4,5 | 1 | DL RSTD measurement reporting; 1 report capable of multiple (within the UE PRS measurement capability, nr-DL-TDOA-MeasCapability, indicated via LPP [34]) DL RSTD measurements and if supported also multiple corresponding DL PRS-RSRP measurements configured for DL-TDOA. Only applicable for UE capable of reporting measurements for DL-TDOA to LMF via LPP [34]. |
| UE Rx-Tx Note 2,4,5 | 1 | UE Rx-Tx measurement reporting; 1 report capable of multiple (within the UE PRS measurement capability, nr-DL-PRS-MeasCapability, indicated via LPP [34] for multi-RTT) UE Rx-Tx measurements and if supported also multiple corresponding DL PRS-RSRP measurements configured for multi-RTT. Only applicable for UE capable of reporting measurements for multi-RTT to LMF via LPP [34]. |
| DL PRS-RSRP Note 2,4,5 | 1 | DL PRS-RSRP measurement reporting; 1 report capable of multiple (within the UE PRS measurement capability, nr-DL-PRS-MeasCapability, indicated via LPP [34] for AoD) DL PRS-RSRP measurements configured for DL-AoD. Only applicable for UE capable of reporting measurements for DL-AoD to LMF via LPP [34]. |
| DL PRS-RSRPP Note 2,4,5 | 1 | DL PRS-RSRPP measurement reporting; 1 report capable of multiple (within the UE PRS measurement capability, nr-DL-PRS-MeasCapability, indicated via LPP [34] for AoD) DL PRS-RSRPP measurements configured for DL-AoD. Only applicable for UE capable of reporting measurements for DL-AoD to LMF via LPP [34]. |
| SRS-RSRP Note 2,3,4,5 | 1 | SRS-RSRP measurement reporting for CLI; 1 report capable of up to 32 SRS resources measurements. Only applicable for UE supporting cli-SRS-RSRP-Meas-r16. |
| CLI-RSSI Note 2,3,4,5 | 1 | CLI-RSSI measurement reporting for CLI; 1 report capable of up to 64 CLi-RSSI resources measurements. Only applicable for UE supporting cli-RSSI-Meas-r16. |
| NOTE 1: When the UE is configured with PSCell and SCell carrier frequencies, Ecat for Intra-frequency is applied per corresponding NR serving frequency. NOTE 2: Applicable for UE configured with SA NR operation mode. NOTE 3: Applicable for UE configured with EN-DC operation mode. NOTE 4: Applicable for UE configured with NE-DC operation mode. NOTE 5: Applicable for UE configured with NR-DC operation mode. |  |  |

  - Option 3: RAN4 to introduce requirements of “Capabilities for Support of Event Triggering and Reporting Criteria” (OPPO)

- Recommended WF

  - Continue discussion.

MTK: new capability is proposed. For option 2, inter-frequency is open now. shall we define separate capability between LTM and MIMO?

E////Nokia: we think it is too early to make conclusion at current stage.

QC: both number of events and carriers UE can support need to be considered.

#### Issue 1-4: How to coordinate with R19 MIMO

- Candidate solutions:

  - Option 1: (MTK)

    - Events configured for mobility and MIMO should be considered as different events even when they are configured for the same beams and the same cell with the same threshold.

    - Define the capability for L1 events shared by mobility L1 events and MIMO L1 events.

- Recommended WF

  - Continue discussion.

#### Issue 1-5: filtering assumption in event evaluation

- Candidate solutions:

  - Option 1: Wait for more RAN1 progress to discuss how to handle L1 filtering in RAN4 for event triggered L1 report (MTK, Apple, HW, Nokia, E///, Qualcomm, CATT).

  - Option 1a: For whether to consider the RRM impact due to L1 filtering for event triggered L1 report, RAN4 needs to wait for more RAN1 input. (CATT)

    - If L1 filtering is introduced in RAN1, RAN4 needs to define additional SSB based L1 measurement delay requirements with filtering for event triggered L1 report.

    - Otherwise, there is no impact on RAN4.

  - Option 2: For filtering, we think it is better to wait the conclusion of RAN1. If it is up to UE implementation, it is unnecessary to define the related requirements. (CTC)

    - Event triggered reporting delay is up to whether the filtering is needed, when filtering is used an additional delay can be expected.

    - In case without filtering, we think current LTM L1-RSRP measurement delay can be as baseline.

  - Option 3: no need to define additional SSB based L1 measurement delay requirements with filtering, since the agreement about filtering in RAN2 is about filtering with filter coefficient as defined in Clause 5.5.3.2, TS38.331 or about Time-to-Trigger (TTT), neither of them have impact on L1 measurement delay requirements. (CMCC)

- Recommended WF

  - Continue discussion.

#### Issue 1-6: accuracy

- Candidate solutions:

  - Option 1: The accuracy requirement for event triggered L1 measurement reporting should also be defined. (CTC)

  - Option 1a: for SSB based event triggered L1 measurement reporting, it is proposed to define L1 measurement reporting accuracy requirements (Samsung)

    - The reported L1 measurement contained in event triggered measurement reports shall meet the accuracy requirements. And it is proposed to reuse Rel-18 LTM measurement accuracy

  - Option 1b: (Apple)

    - for SSB based L1 RSRP event triggered reporting, the reported L1-RSRP measurement result shall meet R18 LTM L1-RSRP accuracy requirements

    - for CSI-RS based L1-RSRP event triggered reporting, the reported L1-RSRP measurement result shall meet R19 CSI-RS based L1-RSRP accuracy requirements for LTM, which will be discussed under CSI-RS based L1-RSRP measurement agenda.

  - Option 1c: (CMCC)

    - for SSB based event triggered L1 measurement reporting, it is proposed to define L1 measurement reporting accuracy requirements

      - The reported L1 measurement contained in event triggered measurement reports shall meet the accuracy requirements. And it is proposed to reuse Rel-18 LTM measurement accuracy

    - for CSI-RS based event triggered L1 measurement reporting, it is proposed to define L1 measurement reporting accuracy requirements, the details are FFS

- Recommended WF

  - Continue discussion.

- Ad-hoc Agreement:

    - for SSB based L1 RSRP event triggered reporting, the reported L1-RSRP measurement result shall meet R18 LTM L1-RSRP accuracy requirements

    - for CSI-RS based L1-RSRP event triggered reporting, the reported L1-RSRP measurement result shall meet R19 CSI-RS based L1-RSRP accuracy requirements for LTM, which will be discussed under CSI-RS based L1-RSRP measurement agenda.

#### Issue 1-7: beam level or cell level measurement result for new LTM event evaluation

- Candidate solutions:

  - Option 1: For LTM event triggered L1 reporting, beam level measurement result will be used for evaluation. (Xiaomi, CMCC, Apple, Samsung)

- Recommended WF

  - Continue discussion.

#### Issue 1-8: starting point in event triggered L1 reporting requirements

- Candidate solutions:

  - Option 1: the definition for starting point for L3 event triggered reporting can be re-used for L1 as baseline. (Xiaomi)

  - Option 1a: starting point in event triggered L1 reporting requirements is the time when a condition exists at the UE which will trigger the reporting. (Apple)

  - Option 1b: an event that will trigger a measurement report (Samsung, Nokia)

- Recommended WF

  - Continue discussion.

- Ad-hoc Agreement:

  - Use the following as starting point. Wording can be discussed in CR.

    - Starting point in event triggered L1 reporting requirements is the time when a condition exists at the UE which will trigger the reporting.

#### Issue 1-9: L1 event triggered reporting delay

- Candidate solutions:

  - Option 1: The existing framework in L3 event triggered reporting RRM requirements can be as a starting point. (CTC, Samsung, ZTE)

  - Option 2: RAN4 to discuss how to consider the total measurement time if multiple cells are measured with different measurement period. (Xiaomi)

    - It’s possible that the periodicity of RS is different from each other and then the measurement period is different. How to define total measurement time for multiple cells is FFS.

  - Option 3: (CMCC)

    - for SSB based event triggered L1 measurement reporting, it is proposed to define L1 measurement reporting delay requirements.

      - The delay is less than TL1-RSRP_Measurement_Period or TL1-RSRP_Measurement_Perioa + TSSB_time_index, which are the L1-RSRP measurement period defined for Rel-18 LTM.

    - for CSI-RS based event triggered L1 measurement reporting, it is proposed to define L1 measurement reporting delay requirements, and L1 measurement reporting accuracy requirements

  - Option 4: Reporting delay is defined as one L1-RSRP measurement period on target cell. (Apple)

  - Option 4a: The event triggered measurement reporting delay, shall be less than LTM L1-RSRP measurement period based on SSB or CSI-RS. (Samsung)

  - Option 5: SSB-based LTM L1 event triggered reporting delay is TL1-RSRP_Measurement_Period_SSB_intra, TL1-RSRP_Measurement_Period_SSB_inter or TL1-RSRP_Measurement_Period_SSB_inter_withoutGap (Nokia)

  - Option 6: RAN4 to discuss and define the event evaluation time (i.e., the minimum time required to complete the event evaluation and be ready to send on the first reporting occasion from the measurement occasion). (Ericsson)

- Recommended WF

  - Continue discussion.

#### Issue 1-10: where to capture event triggered L1 report requirement

- Candidate solutions:

  - Option 1: new requirements for event triggered L1 measurement reporting shall be introduced in 9.14.3 and 9.15.3, respectively for intra-frequency and inter-frequency. (CATT, Apple, vivo, HW)

- Recommended WF

  - Continue discussion.

#### Issue 1-11: Impact of UL resource waiting time

- Candidate solutions:

  - Option 1: RAN4 to discuss whether to consider the impact of UL resource waiting time on L1 reporting requirement. (Xiaomi)

- Recommended WF

  - Continue discussion.

#### Issue 1-12: Impact on other LTM requirements

- Candidate solutions:

  - Option 1: RAN4 to discuss potential impact of event-triggered L1 reporting to other LTM requirements such as: (Nokia)

    - Known TCI state condition for early TCI state activation and cell switch

    - Applicability of cell switch delay requirements

  - Option 1a: RAN4 to revisit following requirements to accommodate event triggered LTM reporting (E///)

    - Definition of TCI state known condition

    - TCI state activation requirements

  - Option 2: RAN4 to consider the impact of L1 event triggered reporting to: (Nokia)

    - early DL sync (Tfirst-RS = 0 conditions)

    - early ASN.1 decoding and validity check

    - LTM L1 measurement requirements

- Recommended WF

  - Continue discussion.

#### Issue 1-13: others

- Candidate solutions:

  - Proposal 1: For the topic of ‘event triggered L1 measurement reporting,’ RAN4 to wait for further progress to be made in other working groups. (QC)

  - Proposal 2: For event triggered L1 reporting in FR2-1, regarding the neighbour cell which is prioritized to be measured, RAN4 re-use R18 solutions, i.e. if UE supports early TCI activation, UE prioritize the neighbour cell with pre-TCI activation in L1 measurement reporting. (vivo)

- Recommended WF

  - Continue discussion.

## CSI-RS based L1 measurement

#### Issue 2-1: CSI-RS based L1 measurement for known/unknown cell

- Candidate solutions

  - Option 1: CSI-RS based L1-RSRP measurement is performed only on known cells (MTK, CTC, Xiaomi, Apple, OPPO, ZTE)

- Recommended WF

  - Continue discussion.

- Ad-hoc Agreement:

  - RRM requirements for CSI-RS based L1-RSRP measurement only apply for known target cells

#### Issue 2-2: known cell definition in CSI-RS based L1 measurement

For CSI-RS based L1-RSRP measurement, the cell is considered as known if the following conditions are met:

- Online Agreement:

  - The UE has performed L3 measurement on the target cell during the last 5 seconds, and

    - Further check whether to consider SSB-based L3 measurement only

  - The SSB from the target cell configured for L3[/L1] measurement remains detectable according to the cell identification requirements in clause 9.2.

    - L1 measurement in the 2nd bullet is not needed for FR1.

    - Further discuss whether to keep or remove L1 measurement for FR2-1 in the 2nd bullet.

  - The CSI-RS from the target cell configured for L1 measurement remains [detectable/measurable].

    - Further discuss whether to use detectable or measurable.

    - Further discuss the side condition

#### Issue 2-3: CSI-RS based L1 measurement outside active BWP or with measurement gap

- Candidate solutions

  - Option 1: Deprioritize CSI-RS based L1-RSRP measurement outside active BWP (or with measurement gap) in R19. (MTK, QC, vivo, OPPO, Nokia)

  - Option 1a: RAN4 can first discuss measurement within active BWP for CSI-RS based L1 measurement and wait for progress in other WG about whether measurement gap will be supported. (Xiaomi)

  - Option 1b: RAN4 to define intra-frequency CSI-RS measurement requirements first. Support inter-frequency L1-RSRP CSI-RS measurements without gap. FFS for inter-frequency with gap. (Nokia)

  - Option 2: RAN4 at least support the following measurement types for CSI-RS based L1 measurement. (CATT)

    - Without gap

    - With gap

  - Option 2a: for CSI-RS based LTM, it is proposed to consider intra-frequency measurement, inter-frequency  measurement without gap and inter-frequency measurement with gap, which are aligned with Rel-18 SSB based LTM. (CMCC)

  - Option 2b: prioritize intra-frequency measurement without GAP, inter-frequency measurement with GAP. (Apple, Samsung, HW, OPPO, CATT)

  - Option 2c: CSI-RS based L1 measurements for candidate cells shall support

    - Measurement within active BWP, Measurements outside active BWP. (E///, ZTE)

- Recommended WF

  - Continue discussion.

#### Issue 2-4: whether SSB based L3 measurement is a pre-requisite of CSI-RS based L1 measurement

Tentative Agreement during online session: further discuss the two alternatives

Alternative 1:

    - RAN4 to consider the following measurement procedure for CSI-RS based LTM candidate cell measurements:

      - Step 1: SSB based L3 measurements [and reports]

      - Step 2: For FR2-1, SSB-based L1 measurements and reports on the same candidate cell

        - Further discuss whether Step 2 can be skipped under certain conditions. FFS the details of the conditions.

        - Note: Step 2 is removed for FR1.

      - Step 3: CSI-RS based L1 measurements on the same candidate cell

      - Note: The CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1 or 2.

        - Note: For FR1, the CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1.

Alternative 2:

    - RAN4 to consider the following measurement procedure for CSI-RS based LTM candidate cell measurements:

      - Step 1: SSB based L3 measurements [and reports]

      - Step 3: CSI-RS based L1 measurements on the same candidate cell

      - Note: The CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1.

#### Issue 2-5: supported frequency range

- Candidate solutions

  - Option 1: FR1 and FR2-1 (CATT, Apple, OPPO, Nokia, ZTE)

- Recommended WF

  - Continue discussion.

- Ad-hoc Agreement:

  - Both FR1 and FR2-1 are in scope of RRM requirements for CSI-RS based L1-RSRP measurement on LTM candidate cell(s).

#### Issue 2-6: other applicability

- Candidate solutions

  - Option 1: The applicability conditions are: all CSI-RS resources on candidate cells are configured with the same periodicity. (HW)

  - Option 2: FFS on whether to limit the CSI-RS resource to be measured is the same as active BWP bandwidth. (HW)

- Recommended WF

  - Continue discussion.

- Further discuss the following assumption:

  - RAN4 requirements apply all CSI-RS resources on candidate cells on the same layer are configured with the SCS and CP type.

  - FFS on other parameters, such as same center frequency, BW, periodicity and etc.

#### Issue 2-7: measurement gap related

- Candidate solutions

  - Proposal 1: In Rel-19 LTM, RAN4 needs to discuss whether to reuse or introduce the new gap for CSI-RS based L1 measurement. (CATT)

  - Proposal 2: RAN4 at least can follow the same approach as CSI-RS based L3 measurement for gap-based measurement, i.e., RAN4 to define requirements applicability for CSI-RS based L1 measurement. (CATT)

    - Motivation is to make sure “existing gap can cover the CSI-RS configuration in the same frequency layer”

  - Proposal 3: For CSI-RS based L1 with gap measurement, at least the type 1 MG needed to be supported in Rel-19 LTM. (CATT)

    - RAN4 to discuss whether to support other types of gap and wait for more RAN1 conclusions.

  - Proposal 4:For the decision on MG to use, RAN4 to wait for further RAN1 progress. (E///)

  - Proposal 5: defined a window to restrict the CSI-RS within gap which is used for CSI-RS based L3 measurement (ZTE).

- Recommended WF

  - Continue discussion.

#### Issue 2-8: L1 measurement on CSI-RS with repetition ON

- Candidate solutions

  - Option 1: Not to support CSI-RS resources with repetition ON in FR2 for L1 measurement on neighbor cell. (MTK)

  - Option 1a: RAN4 to not consider P3-like early beam management in CSI-RS based L1-RSRP measurement requirement definition, i.e. no Rx beam sweeping factor, assuming the CSI-RS is not configured with ‘repetition = on.’ (QC)

    - P1 (wide gNB Tx & wide UE Rx): SSB based BM (implicit/explicit SSB-ID report to gNB)

    - P2 (narrow gNB Tx & wide UE Rx): On CSI-RS resources with ‘repetition = off’, UE applies a fixed Rx beam obtained from P1 and measures/reports CRI and L1-RSRP.

    - P3(narrow gNB Tx & narrow UE Rx): On CSI-RS resources with ‘repetition = on’, UE sweeps its Rx beam with a finer beamwidth, desirably confined within the P1/P2 Rx beamwidth. No report is required.

  - Option 2: If the repetition is ON, UE can receive the configured CSI-RS resources and perform CSI-RS measurement with Rx beam sweeping directly. (CTC, CATT)

- Recommended WF

  - Continue discussion.

#### Issue 2-9: L1 measurement on CSI-RS with repetition OFF

- Candidate solutions

  - Option 1: For CSI-RS resources with repetition OFF in FR2, L1-RSRP measurement is performed only after UE has performed L1-RSRP measurement on the associated SSB. (MTK)

  - Option 1a: RAN4 to define the L1 measurement requirements for CSI-RS with repetition OFF in FR2 for the following procedure (CATT)

    - UE performs SSB based L3 measurements and acquires SSB index information of the candidate cells. And then UE performs SSB based L1 measurement to refine the RX beam (with RX beam sweeping) which is the same as R18 LTM. Afterwards UE perform measurements without RX beam sweeping on the configured CSI-RS resources of candidate cells where each CSI-RS resource is QCL-typeD with SSB for L1-RSRP measurement

      - UE is capable of supporting R18 LTM.

  - Option 2: If the repetition is OFF, UE needs to perform measurement on the associated SSB firstly, but we do not capture the significant benefit of second situation. (CTC)

- Recommended WF

  - Continue discussion.

#### Issue 2-10: CSI-RS resource type

- Candidate solutions:

  - Option 1: RAN4 prioritize CSI-RS based L1 measurement requirements for periodic CSI-RS. (vivo)

- Recommended WF

  - Continue discussion.

#### Issue 2-11: CSI-RS resource density and BW

- Candidate solutions:

  - Option 1: For CSI-RS based L1-RSRP measurement on neighbor cell, define the requirements for Density=3 with (≥) 48 PRBs and not to define tighter requirements for more 48 PRBs. (MTK, Xiaomi, Apple, HW, OPPO, E///)

- Recommended WF

  - Continue discussion.

- Ad-hoc Agreement:

  - For CSI-RS based L1-RSRP measurement on neighbor cell, define the measurement period and accuracy requirements based on Density=3 with (≥) 48 PRBs.

#### Issue 2-12: RTD assumption

- Candidate solutions:

  - Option 1: start from RTD<CP (MTK, QC, Apple, OPPO, ZTE, vivo)

    - Option 1a: Not to define CSI-RS based L1-RSRP measurement requirements on neighbor cell for RTD>CP case. (MTK, QC)

    - Option 1b: UE capability shall be considered if RAN4 agrees to consider the case where RTD is larger than CP between serving cell and neighbour cell on the same carrier. (Apple)

  - Option 2: Support both RTD<CP and RTD>CP for CSI-RS based L1-RSRP measurement. (CTC, CMCC, Nokia, E///)

    - Support of RTD>CP can be based on UE capability (CMCC)

- Recommended WF

  - Continue discussion.

QC: to address the concern that network may not know RTD level at UE side, we can consider let UE report RTD status.

- Further discuss based on the following assumption:

  - RTD is the received time difference among CSI-RS and SSB from different cells.

  - Alt 1: define requirements for both RTD<CP and RTD>CP. Introduce an optional UE capability for RTD>CP.

  - Alt 2: only define requirements for RTD<CP.

    - NW shall only enable this when RTD<CP is guaranteed.

#### Issue 2-13: category of CSI-RS based L1 RSRP measurement (already discussed during online session)

- Candidate solutions:

  - Option 1: Categorize CSI-RS based L1-RSRP measurement into CSI-RS based L1-RSRP measurement within active BWP and outside active BWP for further discussion (MTK, CATT, E///, vivo)

  - Option 2: Categorize CSI-RS based L1-RSRP measurement into intra-frequency and inter-frequency (CATT, CTC, CMCC, Apple, Samsung, HW, OPPO, Nokia, ZTE)

- Recommended WF

  - Continue discussion.

#### Issue 2-14: definition of intra-frequency and inter-frequency for CSI-RS based L1 measurement (already discussed during online session)

- Candidate solutions:

  - Option 1: Following CSI-RS based L3 measurement (CTC, CMCC, Apple, Samsung, OPPO, [Nokia?])

    - A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that:

      - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of the CSI-RS resource on the serving cell indicated for L1 measurement, and

      - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of the CSI-RS resource of the serving cell indicated for L1 measurement, and

        - It is applied for SCS = 60KHz

      - the centre frequency of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the centre frequency of the CSI-RS resource of the serving cell indicated for L1 measurement

  - Option 2: For definition of intra-frequency and inter-frequency, suggest to wait for more progress in other WGs. (Xiaomi)

  - Option 3: For backward compatibility, RAN4 better to have a definition for intra- and inter-frequency CSI-RS based L1-RSRP measurement as legacy. (CATT)

    - One possible way is to revise the legacy definition and clarify that the reference centre frequency can be any of centre frequencies of the CSI-RS resources of the serving cell.

  - Option 4: Considering the introduction of CSI-RS based LTM L1 measurement, whether an LTM cell switch is intra-frequency or inter-frequency may depend on whether the associated SSB reported in the CSI-RS-based measurement result is intra-frequency or inter-frequency. (vivo)

    - Whether the associated SSB is intra-frequency or inter-frequency follows the R18 rules

    - No additional definition of intra-F/inter-F for CSI-RS based L1 measurements.

  - Option 5: Intra-frequency CSI-RS based L1 measurement can be defined that the NZP-CSI-RS resource(s) of a candidate cell is within the DL active BWP of the serving cell and the SCS and CP of NZP-CSI-RS resource(s) of a candidate cell is same as those of the DL active BP of the serving cell. (HW)

    - It should be further discussed when the DL active BWP of the serving cell is within the bandwidth of NZP-CSI-RS resource(s) of a candidate cell.

- Recommended WF

  - Continue discussion.

#### Issue 2-15: definition of frequency layer in CSI-RS based L1-RSRP measurement

- Candidate solutions:

  - Option 1: Following CSI-RS based L3 measurement, for CSI-RS resources of the same L1 frequency layer, SCS, CP type, center frequency, BW and periodicity are the same. (MTK)

  - Option 2: Two CSI Resource are of same frequency layer if they have same SCS, CP type, BW, center frequency, and has overlapping periodicity. (E///)

- Recommended WF

  - Continue discussion.

Alt 1: define RRM requirement per layer

Support: MTK, E///

Alt 2: define RRM requirement per cell

Support: vivo

#### Issue 2-16: number of candidate cells and beams UE is expected to measure

- Candidate solutions:

  - Option 1: Discuss the number of candidate cells and beams UE is expected to measure for intra-frequency L1-RSRP CSI-RS measurements. (Nokia)

- Recommended WF

  - Continue discussion.

#### Issue 2-17: CSI acquisition on candidate cell(s) before or during LTM cell switch

- Candidate solutions:

  - Option 1: Wait for more RAN1 progress to start the discussion on CSI acquisition in RAN4. (MTK, QC)

- Recommended WF

  - Continue discussion.

#### Issue 2-18: accuracy and side condition for CSI-RS based L1-RSRP measurement

- Candidate solutions:

  - Option 1: The existing side condition for CSI-RS based L1-RSRP measurement for serving cell is used as the starting point. (Apple)

  - Option 1a: CSI-RS based L1-RSRP accuracy of the serving cell can be applied for CSI-RS based L1-RSRP of the candidate neighbour cells. (HW)

  - Option 2: RAN4 to discuss the CSI-RS accuracy requirements during performance part. (E///)

- Recommended WF

  - Continue discussion.

#### Issue 2-19: CSI-RS based L1-SINR measurement

- Candidate solutions:

  - Option 1: RAN4 need to discuss whether to define the requirements for CSI-RS based L1-SINR measurement. (CATT)

    - RAN4 to wait for more RAN1 input.

  - Option 2: RAN4 focus on CSI-RS based L1-RSRP measurement and FFS on L1-SINR measurement (Apple, Nokia)

- Recommended WF

  - Continue discussion.

#### Issue 2-20: other CSI-RS based L1 measurement issues

- Candidate solutions:

  - Option 1; Define early DL sync requirements for using CSI-RS once RAN1 approval, and SSB based DL sync in LTM can be used as baseline. (ZTE)

- Recommended WF

  - Continue discussion.

## Others

#### Issue 3-1: other issues

- Candidate solutions:

  - Proposal 1: it is proposed to define dedicated requirements, e.g.timing requirements, for UE based TA adjustment. (CMCC)

  - Proposal 2: For subsequent LTM, cell switch delay requirements for R18 LTM in clause 6.3 could be reused. (CMCC, ZTE)

  - Proposal 3: RAN4 to discuss whether to define requirements for subsequent LTM cell switching delay in R19. The requirements would be the same for both intra-CU and inter-CU. (HW)

- Recommended WF

  - Continue discussion.

- Agreements:

  - It is RAN4 common understanding that R18 LTM cell switch requirements also apply for R18 subsequent LTM.

# Conditional Intra-CU LTM

## Conditional LTM

#### Issue 1-1-1: RAN4 scope of Conditional LTM

Online Agreement

- RAN4 to define cell switch delay requirements for conditional LTM

- RAN4 to define subsequent conditional LTM requirement unless there is any technical issue identified.

- Wait for other WGs input to further discuss detailed RRM requirements.

#### Issue 1-1-2: The applicable scenarios for conditional LTM

Online Agreement:

Rel-19 CLTM requirements are applicable for NR SA scenarios defined in Rel-18 LTM

- PCell switch to a neighboring LTM candidate cell

- FR1 cell to FR1 cell

- FR1 cell to FR2 cell

- FR2 cell to FR2 cell

- FR2 cell to FR1 cell

- PCell switch to an LTM candidate cell that is a serving SCell in MCG

- FR1 cell to FR1 cell

- FR2 cell to FR2 cell

Further discuss the DC case if it will be included in the WID based on further RAN plenary discussion.

#### Issue 1-1-3: Measurement requirements on conditional LTM

Online Agreement:

On whether L3 measurement based conditional LTM in FR1 can be supported

- RAN4 can further discuss this issue in FR1 and make recommendation to RAN2 if RAN4 consensus can be reached.

As generic procedure for the objective led by RAN2, if any conclusion will be made in RAN2, RAN4 will follow the RAN2 conclusion unless any technical issue is identified.

#### Issue 1-1-4: Subsequent conditional LTM

- Candidate solutions:

  - Option 1: RAN4 to define delay requirements for conditional LTM and subsequent conditional LTM. (vivo, Nokia, ZTE, CATT)

    - Option 1a: FFS whether there is any difference in the delay for initial and subsequent CLTM cell switch. (Nokia)

    - Option 1b: RAN4 should further clarify the scenario for conditional LTM including subsequent LTM. (CATT)

  - Option 2: The starting point of the subsequent conditional LTM cell switch delay is the time UE transmits RRC Reconfigurationcomplete message for the previous PCell. (HW)

  - Option 3: Apart from the starting point, the subsequent conditional LTM cell switch delay can be discussed based on the conclusion of conditional LTM delay.(HW)

- Recommended WF

  - Discussion is needed.

CMCC: whether we need to define one set of requirements covering both CLTM and subsequent CLTM, or separate requirements?

Nokia: it is a bit early to conclude this.

- Ad-hoc Agreement:

  - RAN4 to define delay requirements for conditional LTM and subsequent conditional LTM.

#### Issue 1-1-5: The specification structure of the requirements for conditional LTM

- Candidate solutions:

  - Option 1: For specification structure, the requirements for conditional LTM can be defined in the clauses of Rel-18 LTM. (CATT)

- Recommended WF

  - Discussion is needed.

## Conditional LTM delay

#### Issue 1-2-1: General aspects on Conditional LTM delay

- Candidate solutions:

  - Option 1: CHO requirements framework can be used as baseline. (Apple, Samsung, CTC, CATT)

    - Option 1a: Rel-18 LTM requirements framework can be used as references. (Samsung, CTC)

  - Option 2: Revisit conditional LTM delay when other WGs achieve further conclusions. (CTC, CATT)

  - Option 3: Wait for RAN2 progress for defining cell switch delay requirements for conditional LTM. (E///)

  - Option 4: RAN4 to define cell switch delay requirements for conditional PCell switch in intra-CU scenario. (Nokia)

  - Option 5: Rel-18 UE capability for early ASN.1 decoding or fast RRC processing shall be supported for Rel-19 Conditional LTM. (E///)

- Recommended WF

  - Discussion is needed.

#### Issue 1-2-2: The starting point of conditional LTM delay (already discussed during online session)

- Candidate solutions:

  - Option 1: The starting point of conditional LTM delay is the end of the last TTI containing the RRC command for conditional LTM. (CTC, Samsung, HW)

  - Option 2: The starting point of conditional LTM delay is the time when UE transmits an RRCReconfigurationComplete message. (CMCC)

  - Option 3: The starting point of CLTM cell switch delay depends on when the condition evaluation begins, which is up to RAN1/RAN2. (Nokia)

- Recommended WF

  - Discussion is needed.

#### Issue 1-2-3: The ending point of conditional LTM delay

- Candidate options after online discussion:

  - Option 1:

    - The ending point of conditional LTM delay is the time when the UE transmits the first UL message on the target cell.

    - FFS the support of RACH-based and/or RACH-less, which is up to RAN1/2 agreement.

    - It can be revisited if any technical issue will be identified considering the progress in other WGs.

  - Other option is not precluded.

#### Issue 1-2-4: The components of conditional LTM delay

- Candidate solutions:

  - Option 1: DCLTM = TRRC + TEvent_DU + TTCI-state activate + Tmeasure + TCLTM_execution + Tinterrupt. (CTC)

  - Option 2: DLTM_Conditional = TEvent_DU + Tmeasure + TLTM_execution + TLTM-interrupt. (CMCC)

  - Option 3: DCLTM = TRRC + TEvent_DU + Tmeasure + TLTM-interrupt + TCLTM_execution. (Apple)

  - Option 4: DCLTM = TLTM_event_DU + TLTM_measure + TLTM-processing + TLTM-IU. (Nokia)

    - need of TLTM-RRC-processing, Tfirst-RS, TRS-proc and any additional components in the cell switch delay depends on when DL sync and ASN.1 decoding are to happen, which may depend on RAN1/2 CLTM design. (Nokia)

  - Option 5: At least the delay uncertainty (TEvent_DU), the measurements time (Tmeasure) and the conditional execution preparation time (TCLTM_execution) also need to be considered for Rel-19 conditional LTM Cell Switch delay. (CATT)

- Recommended WF

  - Discussion is needed.

#### Issue 1-2-5: TRRC

- Candidate solutions:

  - Option 1: TRRC is the RRC procedure delay defined in clause 12 in TS 38.331. (CTC, Apple)

- Recommended WF

  - Discussion is needed.

#### Issue 1-2-6: TEvent_DU

- Candidate solutions:

  - Option 1: TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional LTM command until a condition exists at the measurement reference point which will trigger the conditional LTM. (CTC, Apple)

  - Option 2: TEvent_DU is the delay uncertainty which is the time from when UE transmits RRCReconfigurationComplete message confirming the LTM configuration or the RRCReconfigurationcomplete message for the previous cell switch until a condition exists at the measurement reference point which will trigger the conditional LTM. (CMCC)

  - Option 3: Wait for more progress in RAN2. (HW)

- Recommended WF

  - Discussion is needed.

#### Issue 1-2-7: Tmeasure

- Candidate solutions:

  - Option 1: Tmeasure is measurement time. (CMCC)

    - Option 1a: Tmeasure is defined from the end of TTCI-state activate until UE executes LTM to a target cell and interruption time starts. (CTC)

    - Option 1b: Tmeasure is the measurements time. It is defined from the end of TEvent_DU until UE executes  LTM cell switch to a target cell and interruption time starts. (Apple)

- If SSB based L1-RSRP measurement is used in the event, Tmeasure is same as one measurement period of SSB based L1-RSRP measurement as introduced in R18.

- If CSI-RS based L1-RSRP measurement is used in the event, Tmeasure is same as one measurement period of CSI-RS based L1-RSRP measurement which will be introduced in R19.

- If L3 measurement is used in the event, Tmeasure is same as one measurement period of legacy L3 measurement.

  - Option 2: Wait for more progress in RAN2. (HW)

- Recommended WF

  - Discussion is needed.

#### Issue 1-2-8: TCLTM_execution / TLTM_execution

- Candidate solutions:

  - Option 1: TCLTM_execution / TCLTM_execution is the UE execution preparation time for conditional LTM and starts after UE realizes the condition of LTM is met and identity of the target cell is determined. (CTC, CMCC, Apple)

    - Option 1a: TCLTM_execution can be up to [10ms]. (Apple)

  - Option 2: TCHO_execution may need update in conditional LTM. (HW)

- Recommended WF

  - Discussion is needed.

#### Issue 1-2-9: Tinterrupt

- Candidate solutions:

  - Option 1: Tinterrupt is the time between when the UE starts to execute the conditional LTM to the target cell and the time the UE starts to transmit the first UL message on the target cell. (CTC)

  - Option 2: TLTM-interrupt is the interruption time as specified in Rel-18 LTM cell switch. (CMCC, Apple)

  - Option 3: Tinterrupt  may induce ambiguity when define conditional LTM delay, as interruptions may also happen in early PRACH procedure (if supported in conditional LTM). (HW)

  - Option 4: The interruption time defined in Rel-18 LTM Cell Switch delay can be a baseline. (CATT, CTC)

  - Option 5: RAN4 to discuss the starting point of the CLTM interruption, and the components included in TCLTM_interrupt. The goal shall be that CLTM interruption is shorter than CHO interruption. (Nokia)

- Recommended WF

  - Discussion is needed.

#### Issue 1-2-10: TTCI-state activate

- Candidate solutions:

  - Option 1: TTCI-state activate is the time for active the TCI state of candidate cell(s). (CTC)

- Recommended WF

  - Discussion is needed.

#### Issue 1-2-11: The components of Tinterrupt

- Candidate solutions:

  - Option 1: Tinterrupt = TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU. (CTC)

- TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change.

- Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell.

- TRS-proc is the time for SSB processing.

- TLTM-IU is the interruption uncertainty during LTM cell switch.

- Recommended WF

  - Discussion is needed.

## Early synchronization and TCI state activation

#### Issue 1-3-1: Early DL sync

- Candidate solutions:

  - Option 1: R18 LTM early DL-sync can be as baseline. (CTC)

  - Option 2: FFS whether new requirements due to CLTM specific DL/UL sync procedures are needed. (Nokia)

  - Option 3: Wait for RAN2 further agreement on the framework. (E///)

- Recommended WF

  - Discussion is needed.

#### Issue 1-3-2: Early UL sync

- Candidate solutions:

  - Option 1: For UL-sync, we can consider some situations as follows,(CTC)

- Do not need to acquire the TA, i.e., the NTA of candidate cell(s) is/are same as serving cell or NTA = 0.

- RAR-based TA acquisition.

- UE based TA measurement.

  - Option 2: How/whether to support early RACH on candidate cells during conditional LTM. (HW)

  - Option 3: Whether updates are needed to the Rel-18 PDCCH ordered RACH delay requirements. (Nokia)

  - Option 4:  If DL TODA-based UE autonomous TA estimation is found beneficial for the conditional LTM cell switch, RAN4 to define the requirements for DL TODA-based UE autonomous TA estimation. (QC)

  - Option 5: Wait for more progress from other WGs. (HW, E///)

- Recommended WF

  - Discussion is needed.

#### Issue 1-3-3: Early TCI state activation

- Candidate solutions:

  - Option 1: How/whether to support early TCI activation on candidate cells during conditional LTM. (HW)

  - Option 2: FFS whether updates are needed to the Rel-18 early TCI state activation delay requirements. (Nokia)

  - Option 3: R18 LTM early TCI state activation can be as baseline. (CTC)

  - Option 4: Wait for more progress from other WGs. (HW, Ericsson)

- Recommended WF

  - Discussion is needed.

## Others

#### Issue 1-4-1: Whether cover both RACH-based and RACH-less conditional LTM

- Candidate solutions:

  - Option 1: Cover both RACH-based and RACH-less conditional LTM. (CTC, HW)

  - Option 2: FFS whether both RACH-less and RACH-based CLTM are supported by RAN1/2. (Nokia)

- Recommended WF

  - Discussion is needed.

#### Issue 1-4-2: Whether to cover both known and unknown cell for conditional LTM

- Candidate solutions:

  - Option 1: The known cell condition for conditional LTM can be revised as, during the last 5 seconds before the reception of the RRC command that contain conditional LTM configuration……. (CTC)

  - Option 2: All the cell(s) configured as candidate cell(s) for conditional LTM is/are known. (CTC)

- Recommended WF

  - Discussion is needed.