3GPP TSG-RAN WG4 Meeting #112bis	R4-2416878

Hefei, Anhui, China, 14th – 18th October, 2024

Title: 	WF on UE report of incremental BWP switch delay for multi-carrier enhancements

Agenda Item:	7.3

Source: 	NTT DOCOMO, INC.

Document for:	Approval

Introduction

UE capability for switching between non-dormant and dormant BWPs triggered by the new DCI format 0-3 and 1-3 was introduced. The conclusion was informed to RAN4

RAN2 discussed the two options and concluded to down-select approach 2 because Approach 1 is a non-backward compatible change from RAN2 perspective. RAN2 introduced a field bwp-SwitchingMultiDormancyCC-DCI-0-3-And-1-3-r18 corresponding to RAN4 feature 38-9, and a field scellDormancyWithinActiveTime-DCI-0-3-And-1-3-r18 corresponding to its prerequisite feature (RAN1 feature 49-9). It is up to RAN4 whether the UE shall report the same value between RAN4 features 6-3 and 38-9.

After discussion, the original issue can be separated into two issues, the one is related to UE capability value (the original issue) and new issue about incremental delay for each additional CC involved in simultaneous BWP switch is raised. The current specification for simultaneous DCI based BWP switch delay on multiple CCs is copied as follows:

[…] UE shall finish BWP switch within the time duration TMultipleBWPswitchDelay + Y, which is defined as: TMultipleBWPswitchDelay = TBWPswitchDelay + D*(N-1) Where: - TBWPswitchDelay is the BWP switching delay on single CC defined in Table 8.6.2-1 depending on UE capability bwp-SwitchingDelay [2]. TBWPswitchDelay shall be based on the smallest SCS among SCS of all involved CCs before and after BWP switch. If the BWP switch on multiple CCs results in the change of the SCS on any CC among involved CCs, TBWPswitchDelay shall be based on the smallest SCS among all SCS values of all involved CCs. - D is the incremental delay for each additional CC involved in simultaneous BWP switch and depends on UE capability bwp-SwitchingMultiCCs-r16 [TS 38.306, 14] for switching between non-dormant BWPs, and bwp-SwitchingMultiDormancyCCs-r16 or bwp-SwitchingMultiDormancyCC-DCI-0-3-And-1-3-r18 for switching between non-dormant and dormant BWPs. […]

For BWP switch from one non-dormant BWP to another non-dormant BWP, the incremental value D depends on UE capability bwp-SwitchingMultiCCs-r16, and for BWP switch from dormant BWP to non-dormant BWP, the incremental value D depends on UE capability bwp-SwitchingMultiDormancyCC-DCI-0-3-And-1-3-r18.

If there are multiple types of BWP (non-dormant BWP and dormant BWP) to switch, which is allowed by using DCI 0-3/1-3, it is ambiguous which value shall be applied to calculate BWP switching delay.

Topic #1: UE capability value for multi-carrier enhancements

Issue 1-1: Report value for FG6-3 and FG38-9

<Way forward>

- Option 1: The same value D is assumed in the requirements between RAN4 features 6-3 and 38-9.

- Option 2: The different value D is assumed in the requirements between RAN4 features 6-3 and 38-9.

Topic #2: Incremental BWP switch delay definition

Issue 2-1: Requirements for DCI based BWP switch delay on multiple CCs

<Way forward>

- Option 1: Keep current requirements.

- Option 2: Update is needed, e.g., different increment value for different UE capabilities is used in the requirements, and will be discussed under Rel-18 maintenance.

- Option 3: Further discuss whether update on D is needed for simultaneous switching between non-dormant BWPs and switching between non-dormant and dormant BWPs by using DCI 0-1/1-1/2-6.