3GPP TSG-RAN WG4 Meeting #112bis 	R4-2416881

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:			4.2.2.1

Source:	Ericsson

Title:	Ad-hoc minutes #2 for NR_pos_enh2 WI

Document for:	Approval

# Introduction

The document contains discussions on the topics as indicated below.

Positioning RRM performance across the following 3 threads:

- Topic # 1: perf [112bis][202] NR_pos_enh2_part1 (- [discussed in Monday AH1/Oct14])

- Topic # 2: perf [112bis][203] NR_pos_enh2_part2 (only 2 CPP issues discussed in Monday AH2/Oct14)

Positioning RRM core requirements across the following 3 threads:

- Topic # 3: core [112bis][202] NR_pos_enh2_part1 (-)

- Topic # 4: core [112bis][203] NR_pos_enh2_part2 (-)

Topic # 5: Documents for approval and draft CRs:

- Core draft CRs for all threads (-)

- Perf draft CRs for all threads (CPP CR discussed in Monday AH2/Oct4)

- Other documents for approval (Updated summary of results for CCP discussed in Monday AH1/Oct14)

# Topic #1: performance [112bis][202] NR_pos_enh2_part1

## General (Agenda 4.2.2.1)

Issues recommended for discussion by moderator: -

Issues prioritized during the AH1: updated draft big CR R4-2416201

Issues prioritized during the AH2: -

## RedCap positioning (Agenda 4.2.2.4)

Issues recommended for discussion by moderator:

  - Issue 5-5: Accuracy requirements for RedCap positioning measurements

  - Issue 5-4: Test configuration for RedCap positioning TCs

  - Issue 5-3: PRS RMC for RedCap positioning TCs

  - Issue 5-1: Group delay margin for accuracy requirements

  - Issue 5-2: Frequency drift margin for accuracy requirements

Issues prioritized during the AH1: 5-5, 5-4, 5-3, 5-1, 5-2.

Issues prioritized during the AH2: -

## PRS/SRS bandwidth aggregation (Agenda 4.2.2.5)

Issues recommended for discussion by moderator:

- Issue 6-1: Accuracy requirement for Rx-Tx measurement in AWGN with reduced number of samples.

- Issue 6-2: Accuracy requirement for Rx-Tx measurement in AWGN with 4 samples.

- Issue 6-3: Accuracy requirement for Rx-Tx measurement in fading conditions with 4 samples.

- Issue 6-4: Accuracy requirement for RSTD measurement in AWGN conditions with reduced number of samples.

- Issue 6-5: Accuracy requirement for RSTD measurement in AWGN conditions with 4 samples.

- Issue 6-6: Accuracy requirement for RSTD measurement in fading conditions with 4 samples.

Issues prioritized during the AH1: 6-1, 6-2, 6-3, 6-4, 6-5, 6-6.

Issues prioritized during the AH2: -

# Topic #2: performance [112bis][203] NR_pos_enh2_part2

## SL positioning (Agenda 4.2.2.2)

Issues recommended for discussion by moderator:

- Sub topic 1-2: Issue 1-2-1/2/3/4/5

Issues prioritized during the AH1: -

Issues prioritized during the AH2: -

## Carrier phase positioning (Agenda 4.2.2.6)

Issues recommended for discussion by moderator:

- Sub topic 2-2: Issue 2-2-1/2/3

Issues prioritized during the AH1: 2-2-1

Issues prioritized during the AH2: 2-2-2, 2-2-3

### Issue 2-2-2: Margins for accuracy requirements due to carrier phase offset/RSTD uncertainty

Agreements from RAN4#112:

- RAN4 to clarify that the RSCPD accuracy requirement and the relative RSCP accuracy requirement apply given that the carrier phase measurements are performed on PRS resources within a slot/a set of symbols, which can be controlled at least by the window configuration.

- Companies can bring proposals on other scenarios in WI performance maintenance phase.

Agreements from RAN4#112bis, AH1:

RAN4 to only define accuracy for the scenarios where CPP measurements are performed on PRS resources within the same sets of slots and sets of symbols (i.e. the agreed scenario), considering PRS repetitions.

- Set of symbols is based on PRS configuration.

- Proposals

  - Option 1: (Huawei)

    - For defining CP measurement accuracy requirements, add additional margin on top of simulation results to account for the impact of carrier frequency offset and RSTD uncertainty.

      - 9.2 deg for FR1

      - 23 deg for FR2

  - Option 2: (Nokia)

    - RAN4 to further investigate how to reduce the impact of CFO to DL RSCPD measurement accuracy.

    - RAN4 to further investigate how to take into account the significant impact of CFO onto CPP accuracy (i.e. about 35 to 148 degrees in FR1) in the measurement accuracy requirements for RSCPD and DL RSCP, where the RF margin currently is set to TBD in following tables of the agreed (although not approved) big CR [4]:

Table 10.1.43.2-5: Margin for DL RSCPD measurement accuracy in FR1.

| PRS BW (RB number) |  |  | Margin (degree) |
| --- | --- | --- | --- |
| SCS=15kHz | SCS=30kHz | SCS=60kHz |  |
| ≥ 24 | N/A | N/A | [TBD] |
| ≥ 52 | ≥ 24 | N/A | [TBD] |
| ≥ 104 | ≥ 48 | ≥ 24 | [TBD] |
| N/A | ≥ 132 | ≥ 64 | [TBD] |
| N/A | N/A | ≥ 132 | [TBD] |

Table 10.1.43.2-6: Margin for DL RSCPD measurement accuracy in FR2.

| PRS BW (RB number) |  | Margin (degree) |
| --- | --- | --- |
| SCS=60kHz | SCS=120kHz |  |
| ≥ 24 | N/A | [TBD] |
| ≥ 64 | ≥ 32 | [TBD] |
| ≥ 132 | ≥ 64 | [TBD] |
| N/A | ≥ 128 | [TBD] |

Table 10.1.44.2-5: Margin for relative DL RSCP measurement accuracy in FR1

| PRS BW (RB number) |  |  | Margin (degree) |
| --- | --- | --- | --- |
| SCS=15kHz | SCS=30kHz | SCS=60kHz |  |
| ≥ 24 | N/A | N/A | [TBD] |
| ≥ 52 | ≥ 24 | N/A | [TBD] |
| ≥ 104 | ≥ 48 | ≥ 24 | [TBD] |
| N/A | ≥ 132 | ≥ 64 | [TBD] |
| N/A | N/A | ≥ 132 | [TBD] |

Table 10.1.44.2-6: Margin for relative DL RSCP measurement accuracy in FR2

| PRS BW (RB number) |  | Margin (degree) |
| --- | --- | --- |
| SCS=60kHz | SCS=120kHz |  |
| ≥ 24 | N/A | [TBD] |
| ≥ 64 | ≥ 32 | [TBD] |
| ≥ 132 | ≥ 64 | [TBD] |
| N/A | ≥ 128 | [TBD] |

- Recommended WF

  - Discuss options.

Discussion:

Option 1 clarification: repetitions would not impact the proposal, repetitions are only relevant for 24 PRBs.

Nokia: further discussion is needed because for some measurements we use 90%-ile and for other measurements we used 95%-ile.

Check offline percentiles in simulation results and how they are used, e.g., whether applied to the CDF of absolute errors or to the positive tail.

Temporary agreement:

- Define accuracy requirements based on the available simulation results for:

  - FR1:

    - BW>24 PRBs

    - FFS: 24 PRBs

  - FR2:

    - 60 kHz

      - BW>24 PRBs

      - FFS: 24 PRBs

    - 120 kHz

      - BW>32 PRBs

      - FFS: 32 PRBs

Come back on Wednesday (AH3) in the meeting.

### Issue 2-2-3: Accuracy requirements for additional PRS configurations

- Proposals

  - Option 1: (Nokia)

    - Take into account simulation results in R4-2416437 for the additional PRS configurations to determine remaining CP measurement accuracy requirements in the big CR.

- Recommended WF

  - Discuss whether or not to define accuracy requirements for additional PRS configurations, i.e., accuracy is set as TBD.

    - If yes, further discuss how to derive the specific values.

    - If no, delete the additional PRS configurations in bigCR.

Discussion:

Issue: TBDs e.g. in Table 10.1.43.2 (Measurement Accuracy Requirements for RSCPD) due to lack of simulation results. The latest simulation assumptions were agreed a year ago.

For RSCP/RSCPD accuracy requirements:

- CATT to update the simulation results summary, including Nokia’s results

- Discuss the average numbers and include the agreeable numbers and configurations into CR (revision of R4-2416329)

- The requirements may be updated in the next meeting (up to the discussion in the next meeting)

## LPHAP (Agenda 4.2.2.3)

Issues recommended for discussion by moderator:

- …

Issues prioritized during the AH1: -

Issues prioritized during the AH2: -

<no submitted documents>

# Topic #3: core [112bis][202] NR_pos_enh2_part1

## RedCap positioning (Agenda 4.2.1.4)

Issues recommended for discussion by moderator: -

  - Issue 2-1: Update to Nhops,effect,i,j

  - Issue 2-2: Update to Nhop, i,  j

Issues prioritized during the AH1: -

Issues prioritized during the AH2: -

## PRS/SRS bandwidth aggregation (Agenda 4.2.1.4)

Issues recommended for discussion by moderator: -

  - Issue 3-2: Discussion on Tnon-aggregate in core requirement for PRS aggregation

  - Issue 3-3: Update to core requirement for PRS aggregation

Issues prioritized during the AH1: -

Issues prioritized during the AH2: -

# Topic #4: core [112bis][203] NR_pos_enh2_part2

## SL positioning (Agenda 4.2.1.2)

Issues recommended for discussion by moderator:

- Sub topic 1-1: Issue 1-1-2

Issues prioritized during the AH1: -

Issues prioritized during the AH2: -

## Carrier phase positioning (Agenda 4.2.1.2)

Issues recommended for discussion by moderator:

- Sub topic 2-1: Issue 2-1-1

Issues prioritized during the AH1: -

Issues prioritized during the AH2: -

## LPHAP (Agenda 4.2.1.3)

Issues recommended for discussion by moderator: no discussion tdocs

Issues prioritized during the AH1: -

Issues prioritized during the AH2: -

<no submitted documents>

- Topic #5: CRs and other documents for approval

## Core draft CRs for all threads

### General (AI 4.2.1.1)

| Old TDoc | Title | Source | Comments | Recommendation/decision |
| --- | --- | --- | --- | --- |
|  | Draft Big CR to 38133 for RRM core part for expanded and improved NR positioning | Ericsson |  | New document: R4-241xxxx |

### RedCap positioning (AI 4.2.1.4)

| Old TDoc | Title | Source | Comments | Recommendation/decision |
| --- | --- | --- | --- | --- |
| R4-2415160 | Draft CR on core requirements for RedCap positioning | CATT |  | Return to |
| R4-2415851 | draftCR on requirements for RedCap positioning | Huawei, HiSilicon |  | Return to |
| R4-2416321 | draftCR to 38.133 on corrections to core requirements for RedCap positioning | Ericsson |  | Can be endorsed |

### PRS/SRS bandwidth aggregation (AI 4.2.1.4)

| Old TDoc | Title | Source | Comments | Recommendation/decision |
| --- | --- | --- | --- | --- |
| R4-2415161 | Draft CR on core requirements for SRS BW aggregation | CATT |  | Return to |
| R4-2415852 | draftCR on requirements for PRS BWA | Huawei, HiSilicon |  | Return to |
| R4-2416320 | draftCR to 38.133 on interruption requirement for SRS aggregation for positioning | Ericsson |  | Return to |

### SL positioning (AI 4.2.1.2)

| Old TDoc | Title | Source | Comments | Recommendation/decision |
| --- | --- | --- | --- | --- |
| R4-2415162 | Draft CR on core requirements for SL positioning | CATT | Keep capability names, remove other changes. | Revise? |
| R4-2415848 | draftCR on RRM requirements for SL positioning | Huawei, HiSilicon |  | Postpone? |
| R4-2416198 | Draft CR 38133 on remaining core issues for SL positioning | Ericsson |  | Revise? |

### Carrier Phase positioning (AI 4.2.1.2)

| Old TDoc | Title | Source | Comments | Recommendation/decision |
| --- | --- | --- | --- | --- |
| R4-2415163 | Draft CR on core requirements for CPP | CATT |  | Return to |
| R4-2415849 | draftCR on RRM requirements for CPP | Huawei, HiSilicon |  | Return to |
| R4-2416318 | draftCR to 38.133 on corrections to core requirements for CPP | Ericsson | Remove changes to clause 9.9.7.5 and clause 9.9.8.4 | Revise |

### LPHAP (AI 4.2.1.3)

| Old TDoc | Title | Source | Comments | Recommendation/decision |
| --- | --- | --- | --- | --- |

## Perf draft CRs for all threads

### General (AI 4.2.2.1)

| Old TDoc | Title | Source | Comments | Recommendation/decision |
| --- | --- | --- | --- | --- |
| R4-2416201 | Updated draft Big CR to 38133 for RRM performance part for expanded and improved NR positioning | Ericsson |  | Can be endorsed |
|  | Draft Big CR to 38133 for RRM performance part for expanded and improved NR positioning | Ericsson |  | New document: R4-241xxxx |

### RedCap positioning (AI 4.2.2.4)

| Old TDoc | Title | Source | Comments | Recommendation/decision |
| --- | --- | --- | --- | --- |
| R4-2415858 | draftCR on performance requirements for RedCap positioning | Huawei, HiSilicon | Consider the agreements reached in ad-hoc session(s). | Revise |
| R4-2416324 | draftCR to 38.133 on performance requirements for RedCap positioning | Ericsson | Consider the agreements reached in ad-hoc session(s). Remove overlapping clauses with R4-2415858. | Revise |

### PRS/SRS bandwidth aggregation (AI 4.2.2.5)

| Old TDoc | Title | Source | Comments | Recommendation/decision |
| --- | --- | --- | --- | --- |
| R4-2415859 | draftCR on performance requirements for PRS BWA | Huawei, HiSilicon |  | Merged with R4-2416326 |
| R4-2416326 | draftCR to 38.133 on accuracy requirement and test cases for BW aggregation for positioning measurements | Ericsson | Consider agreements reached in ad-hoc session(s) | Revise |

### SL positioning (AI 4.2.2.2)

| Old TDoc | Title | Source | Comments | Recommendation/decision |
| --- | --- | --- | --- | --- |
| R4-2415853 | draftCR on PRS RMC | Huawei, HiSilicon |  | Merged with R4-2415164 |
| R4-2415164 | Draft CR on SL-PRS configuration for TCs | CATT | Add Note 2 from R4-2415853. | Revise |
| R4-2415855 | draftCR on accuracy requirements for SL positioning | Huawei, HiSilicon |  | Can be technically endorsed? |
| R4-2416200 | Draft CR 38133 on remaining performance issues for SL positioning | Ericsson | Consider the agreements from the meeting | Revise |

### Carrier Phase positioning (AI 4.2.2.6)

| Old TDoc | Title | Source | Comments | Recommendation/decision |
| --- | --- | --- | --- | --- |
| R4-2415605 | Corrections on test cases for CPP | CATT | Consider capturing changes to overlapping clauses from R4-2416329. | Revise |
| R4-2415861 | draftCR on performance requirements for CPP | Huawei, HiSilicon |  | Return to |
| R4-2416329 | draftCR to 38.133 on performance requirement for CPP | Ericsson | Remove overlapping changes with R4-2415605. | Revise |
| R4-2416436 | CR 38.133 Introduction of TCs on CPP measurement accuracy | Nokia |  | Merged with R4-2416329 |

### LPHAP (AI 4.2.2.3)

| Old TDoc | Title | Source | Comments | Recommendation/decision |
| --- | --- | --- | --- | --- |
| R4-2415856 | draftCR on performance requirements for LPHAP | Huawei, HiSilicon | Cat B. Remove changes to clause A.7.8.2.3. Align changes to overlapping clauses with R4-2416322. | Revise |
| R4-2416322 | draftCR to 38.133 on test cases for LPHAP | Ericsson | Remove remaining [] in Tables A.16.8.3.3.1-3, A.16.11.2.2.1-3, A.17.8.3.3.1-2. Capture non-overlapping changes to A.7.8.2.3 from R4-2415856. Remove overlapping changes with R4-2415856. | Revise |

## Other documents for approval

| TDoc | Title | Source | Comments | Recommendation/decision |
| --- | --- | --- | --- | --- |
|  | WF on RedCap positioning and PRS/SRS bandwidth aggregation | Ericsson |  | New document: R4-241xxxx |
|  | WF on SL positioning and carrier phase positioning | CATT |  | New document: R4-241xxxx |
|  | Ad-hoc minutes #1 on RRM for NR_pos_enh2 | Ericsson |  | New document: R4-241xxxx |
|  | Ad-hoc minutes #2 on RRM for NR_pos_enh2 | Ericsson |  | New document: R4-241xxxx |
|  | Ad-hoc minutes #3 on RRM for NR_pos_enh2 | Ericsson |  | New document: R4-241xxxx |
| R4-2416325 | Updated summary of simulation results for RedCap positioning | Ericsson |  | Revise |
| R4-2416328 | Updated summary of simulation results for PRS aggregation | Ericsson |  | Revise |
|  | Updated summary of simulation results for SL positioning accuracy requirements | CATT |  | New document: R4-241xxxx |
|  | Updated summary of simulation results for CPP accuracy requirements | CATT |  | New document: R4-241xxxx |

# Annex: Remaining issues after ad hoc #1 which were not discussed in the meeting

## Core part

### RedCap positioning (Agenda 4.2.1.4)

### Issue 2-1: Update to Nhops,effect,i,j

Background:

The options were agreed FFS in the last meeting:

- Option 1: the parameter NrepPRS shall take into account the number of unmuted inter-slot PRS repetitions that overlap with the sampling window within each hop

- Option 2: Nhops,effect shall take into account the number of unmuted inter-slot PRS repetitions that overlap with the sampling windows of the hopping pattern with Nhopsslot hops per slot, within a single MG occasion, excluding the gap retuning times.

- Option 3: Other options are not precluded.

- Proposals

  - Option 1: CATT

    - Nhops,effect,i,j is updated as: Nhops,effect,i,j=hNrepmax,

Where

- h is the index of sampling window within a MG instance;

- Nrepmax=2 if Nhopsslot=2, otherwise, Nrepmax=1;

- When deciding the value of Nhopsslot, only the unmuted PRS resources that fully or partially overlap with this sampling window are considered. If all PRS resources within sampling window h are muted, Nrepmax=0.

  - Option 2: Huawei

    - Nhops,effect is defined as the number of unmuted inter-slot PRS repetitions that overlap with the sampling windows of the hopping patterns with length Thop, within a single MG occasion, excluding the gap retuning times.

  - Option 3: Qualcomm

    - For PRS measurements with Rx hopping with Nhopsslot≥1 or MrepPRS>1, the current formulas for Nhops,  effect apply with the clarification that NrepPRS is the number of unmuted inter-slot PRS repetitions within the MG occasion, excluding the gap retuning times.

    - For PRS measurements with Rx hopping with Nhopsslot=1/2 and MrepPRS=1,

      - Nhops, effect=maxNrep, evenPRS, Nrep, oddPRS

        - where Nrep, evenPRS and Nrep, oddPRS are the number of unmuted inter-slot PRS repetitions in even and odd slots, respectively, within the MG occasion, excluding the gap retuning times.

  - Option 4: Ericsson

    - RAN4 to clarify that NrepPRS denotes the number of PRS repetitions within a MG occasion that concerns PRS resources that are unmuted and fully or partially overlapped with the sampling duration within each hop.

  - Option 5: Nokia

    - RAN4 to define Nrep PRS by using Option 2 (‘Nhops,effect shall take into account the number of unmuted inter-slot PRS repetitions that overlap with the sampling windows of the hopping pattern with Nhopsslot hops per slot, within a single MG occasion, excluding the gap retuning times’).

- Recommended WF

  - Discuss option(s).

Discussion:

### Issue 2-2: Update to Nhop, i,  j

Background:

The following was agreed in RAN4#112.

- Update the requirements on Nhop by adding an upper bound for N3 capability

Nhop=minNhops,effect,N3Lper-hop,Nhop,max.

- Proposals

  - Option 1: CATT

Nhop, i,  j is updated for cases with MGL>T3 as follows:

- Nhop, i,  j=minNhops,effect,i, j,MGL*N3iT3iLper-hop,i,j,Nhop,max,i

Where

- MGL is the measurement gap length of the current MG occasion;

- T3i is indicated via UE capability processingDurationT3-r18 for positioning frequency layer i.

- Recommended WF

  - Discuss option(s).

Discussion:

### PRS/SRS bandwidth aggregation (Agenda 4.2.1.4)

### Issue 3-2: Discussion on Tnon-aggregate in core requirement for PRS aggregation

- Proposals

  - Option 1: CATT

    - When UE receives a request to perform aggregated measurements, UE is requested to measures resources only indicated by LMF as linked resources. RAN4 removes requirements on Tnon-aggregate from the existing core requirements for PRS aggregation.

  - Option 2: Ericsson, Huawei, Qualcomm

    - When UE is indicated by LMF in nr-DL-PRS-JointMeasurementRequest-r18 to perform measurements by aggregating resources from multiple PFLs, UE measures resources that are indicated by LMF as linked resources and resources that are not indicated by LMF as linked resources. i.e. the current requirements apply.

- Tentative agreement

  - When UE is indicated by LMF in nr-DL-PRS-JointMeasurementRequest-r18 to perform measurements by aggregating resources from multiple PFLs, UE measures resources that are indicated by LMF as linked resources and resources that are not indicated by LMF as linked resources. i.e. the current requirements apply.

- Recommended WF

  - Agree on tentative agreement.

Check for agreement:

When UE is indicated by LMF in nr-DL-PRS-JointMeasurementRequest-r18 to perform measurements by aggregating resources from multiple PFLs, UE measures resources that are indicated by LMF as linked resources and resources that are not indicated by LMF as linked resources. i.e. the current requirements apply.

Discussion:

### Issue 3-3: Update to core requirement for PRS aggregation

- Proposals

  - Option 1: Huawei

    - In PRS BWA requirements for Taggregate_RSTD, replace the margin (G-1)*maxTeffect_agg,i as (G-1)*Tmargin.

- Recommended WF

  - Discuss option(s).

Discussion:

### SL positioning (Agenda 4.2.1.2)

### Issue 1-1-1: RAN1 reply LS (R4-2414905/R1-2407401)

- Proposals

  - Option 1: (CATT)

    - RAN4 confirms that the defined measurement period requirements do not apply when the synchronization reference source changes at transmitting UE during measurements. No updates on spec are needed.

- Recommended WF

  - Confirm option 1. Neither reply nor updates on spec are needed.

Check:

RAN4 action on the RAN1 reply LS: Confirm option 1. Neither reply nor updates on spec are needed.

Discussion:

### Issue 1-1-2: End point of SL-PRS based RSTD measurement period requirements

- Proposals

  - Option 1a: (CATT)

    - RAN4 to revise the SL RSTD measurement delay requirements to explicitly define the start point and the end point which are the earliest start point and the last end point among all links respectively for each SL RSTD measurement, where the reference UE is selected by the measuring UE for each SL RSTD request.

  - Option 1b: (Huawei)

    - Update the SL RSTD requirements to reflect that measurement period ends after the UE has measured SL PRS resources from at least two different Tx UEs.

- Recommended WF

  - Discuss options.

Discussion:

### Carrier phase positioning (Agenda 4.2.1.2)

### Issue 2-1-1: CPP measurement period requirements with one-shot time window

- Proposals

  - Option 1: (CATT)

    - The measurement period requirements with one shot time window for DL RSCPD with RSTD can be defined as:

TRSCPD with RSTD=LoneShotTimeWindow+ Tprocessing_onoShot,i

Where

LoneShotTimeWindow is length of the indicated one shot time window;

Tprocessing_onoShot,i  is the processing time duration and is defined as

Tprocessing_onoShot,i= Lavailable_PRS_onoShot,iN*T

Where

- Lavailable_PRS_onoShot,i is the time duration of available PRS in the positioning frequency layer i to be measured during the indicated one shot time window, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lavailable_PRS_onoShot,i, only the PRS resources unmuted and fully or partially overlapped with one shot time window are considered.

- Where {N,T} is UE capability combination per band where N is a duration of DL PRS symbols in ms corresponding to durationOfPRS-ProcessingSysmbols in TS 37.355 [34] processed every T ms corresponding to durationOfPRS-ProcessingSymbolsInEveryTms in TS 37.355 [34] for a given maximum bandwidth supported by UE corresponding to supportedBandwidthPRS in TS 37.355 [34].

  - Option 2: (Huawei)

    - When one-shot time window is configured,

- the measurement period for CPP is Tuncertainty+Ti,

- the measurement period for RSTD/Rx-Tx is Tuncertainty+maxTeffective,i+Tlegacy,

- Tuncertainty is the time from the start of the measurement to the start of the one-shot time window,

- Ti is the processing time for PFL i,

- Tlegacy is the legacy requirements for RSTD and Rx-Tx.

- Recommended WF

  - Discuss options.

Discussion:

### LPHAP (Agenda 4.2.1.3)

No open issues

## Performance part

### RedCap positioning (Agenda 4.2.2.4)

### Issue 5-1: Group delay margin for accuracy requirements

Background:

- For RSTD measurement accuracy requirement for non-RedCap UEs, margins to account for group delay (depends on the PRS BW) and frequency drift (depends on the separation of PRS resources and whether the PRS resources from reference cell and target cell are in the same PFL or not) are considered.

- For UE Rx-Tx measurement accuracy requirement for non-RedCap UEs margin to only account for group delay that depends on the PRS BW is considered.

- For no Rx FH case, RAN4#112 agreed to reuse existing values defined for non-RedCap for the applicable BW.

- Proposals

  - Option 1: CATT

    - Clarify that margin values are limited by per-hop BW in performance requirements, and no additional margin is added on top of the legacy margin values.

  - Option 2: Huawei

    - On the group delay calibration margin for RedCap with FH, use existing values for non-RedCap for per-hop BW plus an extra margin of [4]ns.

  - Option 3: Ericsson

    - Group delay margin for positioning measurement by FH is defined for the total bandwidth of the PRS resources after all hops.

      - Values for non-RedCap UE are reused to define the group delay margin for RedCap UE where the group delay margin corresponding to a PRS BW for non-RedCap UE is applied to positioning measurement performed by RedCap UE with FH provided the BW of PRS resources for non-RedCap UE is equal to the BW of PRS resources after all hops for RedCap UE.

  - Option 4:  Nokia

    - RAN4 can initially define the margin as the same as the margin in the RSTD case.

- Recommended WF

  - Discuss option(s).

Discussion:

Adding margin:

- Option 1: yes, based on per-hop BW

- Option 2: no, based on total BW

- Option 3: no, based on per-hop BW

Discuss more offline. Come back later during the meeting.

### Issue 5-2: Frequency drift margin for accuracy requirements

Background:

- For RSTD measurement accuracy requirement for non-RedCap UEs, margins to account for group delay (depends on the PRS BW) and frequency drift (depends on the separation of PRS resources and whether the PRS resources from reference cell and target cell are in the same PFL or not) are considered.

- For UE Rx-Tx measurement accuracy requirement for non-RedCap UEs margin to only account for group delay that depends on the PRS BW is considered.

- For no Rx FH case, RAN4#112 agreed to reuse existing values defined for non-RedCap for the applicable BW.

- Proposals

  - Option 1: Ericsson

    - Reuse frequency drift margin values, Y values in existing specification, for single PFL and multiple PFLs defined for non-RedCap UE to specify impact of frequency drift margin value on RSTD measurement performed by RedCap UE with FH on single PFL and multiple PFLs.

- Recommended WF

  - Discuss option(s).

Discussion:

Further check offline:

- Reuse frequency drift margin values, Y values in existing specification, for single PFL and multiple PFLs defined for non-RedCap UE to specify impact of frequency drift margin value on RSTD measurement performed by RedCap UE with FH on single PFL and multiple PFLs.

Come back later during the meeting.

### PRS/SRS bandwidth aggregation (Agenda 4.2.2.5)

No remaining issues.

### SL positioning (Agenda 4.2.2.2)

### Issue 1-2-1: Update values of accuracy requirement

- Proposals

  - Option 1: (Qualcomm)

    - Exclude the lowest and highest outliers prior to averaging simulation results to derive measurement accuracy requirements for SL Rx-Tx and SL RSTD.

- Recommended WF

  - Discuss option 1.

Check:

Update the average numbers in R4-2410196 (CATT to update), exclude the lower outliers from averaging.

Discussion:

### Issue 1-2-2: SL-PRS signal level configurations

- Proposals

  - Option 1: (CATT)

    - Es/Iot for PSCCH with 3 dB can be used to avoid PSCCH decoding failure.

    - Muting and RE offset can be configured to achieve non-overlapping SL-PRS resources.

  - Option 2: (Huawei)

    - Es/Iot: 3dB (to ensure 100% PSCCH decoding).

    - Overlapping SL-PRS is not be used in SL TCs.

  - Option 3: (Qualcomm)

    - Do not use overlapping/colliding SL PRS allocations in the SL positioning test cases.

  - Option 4: (Ericsson)

    - In SL RSTD test cases, reference UE1’s and anchor UE2’s SL-PRS transmissions (but not necessarily their PSSCH) overlap with each other, but not with anchor UE3’s SL-PRS.

    - SL-PRS Es/Iot for the measured anchor (not reference) UE2 and UE3 is close to side conditions in the requirements.

    - For SL RSTD delay test case, the following SL-PRS signal level configurations are proposed:

|  | UE1 | UE2 | UE3 |
| --- | --- | --- | --- |
| SL-PRS Es/Noc | 5 dB | 3.2 dB | -3 dB |
| SL-PRS Es/Iot | 0.1 dB | -2.99 dB | -3 dB |

    - For SL Rx-Tx delay test case, the following SL-PRS signal level configurations are proposed:

|  | UE1 | UE2 |
| --- | --- | --- |
| SL-PRS Es/Noc | 5 dB | 3.2 dB |
| SL-PRS Es/Iot | 0.1 dB | -2.99 dB |

    - In SL AoA delay test case, the SL-PRS signal level configurations can be the same as in SL Rx-Tx test case.

    - In SL RTOA delay test case, it is proposed to add one interfering UE, so the total number of UEs in the test case is 3, including the target UE. The SL-PRS signal configuration for the three UEs can then be the same as for SL Rx-Tx:

|  | Interfering UE | Target UE |
| --- | --- | --- |
| SL-PRS Es/Noc | 5 dB | 3.2 dB |
| SL-PRS Es/Iot | 0.1 dB | -2.99 dB |

- Recommended WF

  - Discuss options.

Check:

What are the signal levels of SL-PRS? of PSSCH? can be different?

- SL PRS Es/Iot: -3 dB

- PSSCH Es/Iot: 0 dB

- Different Noc in SL PRS and PSSCH Res.

Discussion:

### Issue 1-2-3: Comb size configuration

- Proposals

  - Option 1: (CATT)

    - Both comb configurations are already randomly used in SL test cases. RAN4 does not discuss SL-PRS comb configuration for test cases anymore.

  - Option 2: (Huawei)

    - (symbol num, comb size): (4, 4) and (2, 4)

- Recommended WF

  - Discuss options.

Discussion:

### Issue 1-2-5: Multiplexing of TX UEs in TCs

- Proposals

  - Option 1: (Huawei)

    - MUX of multiple TX UEs: TX UE1: slot n, TX UE2: slot n + 1 and slot n + 160ms.

- Recommended WF

  - Discuss option 1.

Check:

Related also to issue 1-2-2?

Discussion:

### Issue 1-2-4: Type of resource pool

- Proposals

  - Option 1: (CATT)

    - RAN4 not to explicitly configure the type of resource pools in TCs.

  - Option 2: (Huawei)

    - Shared and dedicated resource pools are randomly used among different TCs.

  - Option 3: (Qualcomm)

    - If the effort is deemed acceptable, each test case would be defined so that it can be run with either shared or dedicated resource pool, and a UE would be tested in one configuration (by UE choice if the UE supports both).

    - Otherwise, RAN4 should split the test cases evenly between the two types of resource pool.

- Recommended WF

  - Discuss options.

Discussion:

### Carrier phase positioning (Agenda 4.2.2.6)

TBD

### LPHAP (Agenda 4.2.2.3)

No open issues.