3GPP TSG-RAN WG4 Meeting #112bis	R4-24170912

Hefei, China, October 14th – 18th, 2024

Agenda item:			6.4.4

Source:	Intel Corporation

Title:	WF on UE RF requirements for Rel-19 NR channel BW less than 5MHz for FR1 Phase 2

Document for:	Approval

# Topic #1: UE RF requirements for inter-band NR CA/DC with less than 5MHz CBW

## Sub-topic 1-1 Scell SSB position restrictions (Sync raster applicability)

### Issue 1-1-1: SCell SSB position restrictions

Agreement

- Update RAN4 #112 agreement as follows:

  - SCell with 15 PRB transmission bandwidth configuration within 3 MHz channel can be configured only with SS Block frequency position same as defined in TS 38.101-1 Table 5.4.3.1-2

  - For band n100

    - SCell with 12 PRB transmission bandwidth configuration within 3 MHz channel can be configured only with SS Block frequency positions same as defined in TS 38.101-1 Table 5.4.3.1-3

    - SCell with 20 PRB transmission bandwidth configuration within 5 MHz channel can be configured only with SS Block frequency positions same as defined in TS 38.101-1 Table 5.4.3.1-3

  - Capture corresponding restrictions as notes in TS 38.101-1 TS 38.101-1 Table 5.4.3.1-2 and Table 5.4.3.1-3

## Sub-topic 1-2 UE Capability signalling

RAN4 #112 agreements are provided below:

- Options for further discussion - Option 1: Request RAN2 to introduce UE capability signaling to enable less than 5MHz CBW operation for CA/DC - The following Rel-18 capabilities for NR less than 5MHz CBW operation need to be extended or new capabilities defined to enable CA/DC operation scenario with one or more component carriers with less than 5MHz CBW - [support12PRB-CORESET0-r18] - [support3MHz-ChannelBW-Asymmetric-r18] - [support3MHz-ChannelBW-Symmetric-r18] - [support5MHz-ChannelBW-20PRB-CORESET0-r18] - [support12PRB-CORESET0-GSCN-41637-r18] - UE capability signalling granularity is FFS - The respective UE capabilities shall be early-implementable from Rel-18 - The details including whether to update existing Rel-18 capabilities or introduce new Rel-19 capabilities are up to RAN2 - Option 2: Do not introduce new UE capability signaling to enable less than 5MHz CBW operation for CA/DC - RAN4 to define BCSs for band combination CA_n100A-n101A or combinations with other band where 3 MHz is supported as usual where 3MHz / 5MHz channel bandwidths are listed for each band. BCS5 with supportedMinBandwidthUL-r17 set to 5 MHz can be used to indicate that a UE supports CA channel BWs other than 3 MHz for a given band. - RAN2 specs need to remove “This feature is only applicable to single-carrier operation from Rel-19. RAN2 to confirm no NBC issues. - No new RAN2 UE capability signaling is required.” - Other options are not precluded

### Issue 1-2-1: UE capability signalling

Agreement

- UE needs to be able to indicate for each carrier in the CA/DC band combination whether it supports less than 5 MHz bandwidth operation for DL/UL

  - To declare the support of less than 5 MHz bandwidth operation in the CA/DC band combination UE needs to indicate UE capabilities defined for NR < 5MHz in Rel-18 and the existing applicability restriction to single carrier needs to be removed.

  - UE also indicates BCS including support of less than 5MHz channel BW

- UE should be capable to support CA/DC with less than 5MHz starting from Rel-18 (i.e., allow early implementation from Rel-18)

- RAN4 has not identified a need for new signaling element. The signalling details are up to RAN2.

- For BCS signalling the following framework is used:

  - Traditional BCSs (i.e., not BCS4 or BCS5): UE is expected to support 3MHz channel bandwidth in the band in the band combination if it indicates a corresponding BCS, which includes 3MHz channel bandwidth in RAN4 specifications. No signalling changes are expected by RAN4.

  - BCS4: If UE indicates support for BCS4 including a band that it also supports 3 MHz channel bandwidth for, then it shall support 3 MHz for CA for that band in the band combination. No signalling changes are expected by RAN4.

  - BCS5:

    - Option 1: if UE indicates support for BCS5 and minimum channel BW of 3MHz as a part of BCS signalling, then it shall support 3 MHz for CA for that band in the combination.  The existing BCS5 signalling framework needs to be modified to allow indication of 3 MHz minimum channel BW.

    - Option 2: if UE indicates support for BCS5 and does not indicate a minimum channel BW for the band combination, then it shall support 3 MHz for CA for that band in the band combination.  The existing BCS5 signalling framework does not need to be modified to allow indication of 3 MHz minimum channel BW.