3GPP TSG-RAN WG4 Meeting #112	R4-2417114

Hefei, Anhui, China, 14th – 18th October, 2024

Title: 	WF on Rel-19 non-collocated scenario

Agenda Item:	6.5.4

Source: 	KDDI

Document for:	Approval

Topic #3: Other aspects (incl. clarification of contiguous LTE CCs)

<Issue 3-1: UE Type definition for EN-DC>

Way Forward:

- The following table is a starting point, and continue further discussion the following table in the next meeting.

- The table will be added into TS 38.101-3: 7.10B	power imbalance for DC in FR1.

- Table 1: EN-DC UE Capability types

| [UE Capability Type] | Type description | Dedicated UE capability |
| --- | --- | --- |
| Type 1 | It supports intra-band RF requirements (with a maximum power spectral density imbalance between downlink carriers, within 6 dB) and an (NG)EN-DC/NE-DC MTTD/MRTD according to clause 7.5.3/7.6.3 in TS 38.133. | N/A |
| Type 2 | The UE supports inter-band RF requirements (with DL power imbalance as specified in caluse 7.10A in TS 38.101-3) and an (NG)EN-DC MTTD/MRTD according to clause 7.5.2/7.6.2 in TS 38.133 and NE-DC MTTD/MRTD according to clause 7.5.5/7.6.5 in TS 38.133 | interBandMRDC-WithOverlapDL-Bands-r16, requirementTypeIndication-r18 |
| Type 4 | The UE supports inter-band RF requirements (with DL power imbalance as specified in caluse 7.10A in TS 38.101-3) and an (NG)EN-DC MTTD/MRTD according to clause 7.5.2/7.6.2 in TS 38.133 and NE-DC MTTD/MRTD according to clause 7.5.5/7.6.5 in TS 38.133 | TBD |

<Issue 3-2: UE Type definition for NR-CA>

Way Forward:

- The following table is a starting point, and continue further discussion the following table in the next meeting.

- The table will be added into TS 38.101-1: 7.10A	Power imbalance for CA.

- Table 2: NR-CA UE Capability types

| [UE capability types] | Type description | dedicated UE capability |
| --- | --- | --- |
| Type 1 | It supports intra-band RF requirements (with a maximum power spectral density imbalance between downlink carriers, within 6 dB) and MRTD requirements according to clause 7.6.4-1 in 38.133 | N/A |
| Type 2 | The UE supports the inter-band RF requirements (with DL power imbalance as specified in clause 7.10A.2) and MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133 | intraBandNR-CA-non-collocated-r18 |
| Type 4 | The UE supports the inter-band RF requirements (with DL power imbalance as specified in clause 7.10A.2) and MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133 | TBD |