3GPP TSG-RAN WG4 Meeting #112-bis	R4-2417122

Hefei, Anhui, China, 14th ‒ 18th, 2024

Title: 	WF on co-existence study and HPUE for NR-NTN

Agenda Item:	6.8.5

Source: 	Samsung

Document for:	Approval

Co-existence studies

Issue 1-1: Isolation distance

Agreement:

- The baseline is to use 2 x ISD (ISD = 750 meters) as the isolation distance

Issue 1-2: Worst case scenario

Agreement:

- RAN4 to focus on scenario 4, and to skip scenario 5

Issue 1-3: Wrap around assumption for TN network

Agreement:

- Follow the previous agreement.

Issue 2-3-2 TN network topology - Agreement: Reuse existing method from TR 38.863 as baseline, and encourage companies to further evaluate any performance impact with and w/o wrap-around functionality.

Issue 1-4 ~ Issue 1-6:

Agreement:

- Take option 2 in Issue 1-4 as an optional parameters, and the options in Issue 1-5 and Issue 1-6 as a package under this option. It’s shown in the table below.

| Scenarios | NR NTN for LEO |  | NR NTN for GEO | NB IoT NTN for LEO | NB IoT NTN for GEO |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional | Baseline | Baseline | Baseline | Optional |
| Orbits | LEO-1200 prioritized LEO-600 not precluded | LEO-1200 (Option 1 in Issue 1-5) | GEO | LEO-1200 prioritized LEO-600 not precluded | GEO | GEO (Option 2 in Issue 1-5) |
| UL BW | 2RB | 5MHz (Option 2 in Issue 1-4) | 2RB | Single tone | Single tone | [0.18] MHz (Option 2 in Issue 1-4) |
| SNR target (dB) | 15 | 3 (Option 2 in Issue 1-4) | 15 | R4-2217473 | R4-2217473 | 0 (Option 2 in Issue 1-4) |
| ACLR model | TR 38.863 |  For the NTN UE1 which is closest to the victim TN channel, it’s assumed that flat ACLR1 will have impacts on all the Victim UE(s).  For the other NTN UEs except for NTN UE1, it’s assumed that only flat ACLR2 will have impacts on all the Victim UE(s). (Option 1 in Issue 1-6) | TR 38.863 | R4-2217473 | R4-2217473 | R4-2217473 |

Issue 1-7: HPUE ratio

Agreement:

- RAN4 to consider all NR NTN/NB-IoT NTN UEs as HPUEs, taking it as the worst-case scenario.

Issue 1-8: NR-NTN ACIR/ACLR

Agreement:

- Consider following observations can be used as a starting point for RF requirements analysis/studies:

  - Candidate ACLR value for NR-NTN PC2/PC1.5/PC1, based on initial co-ex study results received in #112bis meeting, is 30 dBc

  - The current TN requirements are more stringent than the observations from co-existence results. Overlapping geographic coverage between NTN and TN is discussion in RAN4 in other WIs.

- FFS for final ACLR requirements

Issue 1-9: IoT-NTN ACIR/ACLR

Agreement:

- Take same approach in issue 1-8

NR-NTN HPUE Tx requirements

Issue 2-1: General for Tx requirements

Agreement:

- Follow the previous agreements.

Issue 3-1-2: Consideration of band(s) Agreement: - Use 2GHz for the co-existence study - WI can be completed when the band specific requirements for one pair of {n256,256}and {n255, 255} are completed in this WI - Specify the band specific requirements for other potential NTN bands in the separate spectrum WI Issue 3-1-4: Consideration of regulation(s) - Agreement - Applicable regulations should be taken into account for requirements discussions. - If any identified corrections to be made for PC3, those should be considered in another work item, it is not in the current scope of this WI.

- And the following regulatory information contributed to this meeting are noted and can be further considered in proper places per the guidance from RAN4 leadership:

  - For bands 255/253, HPUE UL/TX requirements should not be driven by TN-NTN coexistence. (Inmarsat/Viasat)

  - ETSI EN 301 681 can be used to inform system design features for the protection of the radio astronomy bands at 1660-1660.5MHz and 1668-1670MHz. (Inmarsat/Viasat)

  - RAN4 to consider introducing protection limits below B255 for Iridium protection as specified in EN 301 681 and ITU-R M.1480 and evaluate how to meet these requirements with higher output power. (Qualcomm)

  - Based on the study outcome, necessary corrections should be made also for PC3 in 36.102 and 38.101-5 (Qualcomm)

  - In case ETSI TC SES identifies later that some requirements are not required for market access in Europe, those requirements should be removed from 3GPP. (Qualcomm)

Issue 2-2: UE architecture/type considerations for NR-NTN PC2 UE

Agreement:

- Follow previous agreement

Issue 3-1-5: Consideration of HPUE Architecture - Agreement: - For PC2 (NR NTN), 1 Tx and 2 Tx, are to be considered for requirements. - ...

Issue 2-3: UE architecture/type considerations for NR-NTN PC1.5 UE

Agreement:

- Both 1Tx and 2Tx for PC1.5 non-handheld UE can be considered as starting point for requirements discussions:

- FFS whether PC1.5 non-handheld UE need to be specified if PC1 non-handheld UE be specified

- FFS whether PC1.5 handheld UE can be specified or not

Issue 2-4: UE architecture/type considerations for NR-NTN PC1 UE

Agreement:

- Confirm feasibility of PC1 1Tx for non-handheld UE.

- For PC1, at least 1Tx to be considered for requirements.

- Further discuss consideration of PC1 1Tx, 2Tx, 3Tx for handheld UE.

- Further discuss consideration of PC1 2Tx, 3Tx for non-handheld UE.

Issue 2-5: ACLR

Agreement:

- Co-existence study results and conclusion is an important factor for deriving out-band emission requirements

  - Check the derived ACLR for A-MPR and additional spurious emission against regulatory information.

Issue 2-8: SEM

Agreement:

- To re-use existing SEM requirements from PC3 to other PCs as starting point.

Issue 2-9: MOP tolerance

Agreement:

- Re-use NR TN TS 38.101-1 nominal MOP and +2/[-3] tolerance for PC2 for NR-NTN Non-Redcap HPUE

NR-NTN HPUE Rx requirements

Issue 3-1: RSD form in spec

Agreement:

- Use similar way in TN spec to derive RSD, if needed.

Issue 3-2: RSD requirements for PC2

Agreement:

- For n256: [0dB] RSD for PC2 NR-NTN for 1Tx and 2Tx

  - FFS on the duplexer isolation and PA interference

Issue 3-4: REFSENS/RSD testing

Agreement:

- For PC2 NR NTN UE, the REFSENs test can adopt the same UL configuration with PC3 NR NTN UE as starting point.

Open issues

Issue 2-6: MPR

  - Option 1: RAN4 to consider applicable regulatory requirements in ECC/CEPT in MPR and A-MPR evaluations. (Qualcomm)

  - Option 2: Reusing NR-NTN PC3 MPR values (both single Tx and Tx diversity) for PC2 MPR as starting point (Mediatek, Samsung)

  - Option 3: MPR for NTN HPUE should be further evaluated according to the agreed ACLR value, PA model and other limiting factor for MPR evaluation. (ZTE)

Way forward:

  - Further discuss with ACLR candidate values, following agreement and above options.

Issue 3-3-2: MPR - Agreement: - FFS the MPR requirements. Following potential factors to be further discussed: - ACLR from co-existence study - Current TN MPR requirements - MPR simulation assumptions - Other factors not precluded if identified.

Issue 2-7: A-MPR

  - Option 1: RAN4 to consider applicable regulatory requirements in ECC/CEPT in MPR and A-MPR evaluations. (Qualcomm)

  - Option 2: To reduce number of power back-off simulations for the FR1 satellite bands, we suggest not considering scenarios/combinations that are not expected for the satellite bands: large RB allocations, CP-OFDM, higher-order modulations. (Apple)

  - Option 3: Regarding the A-MPR requirements, once HPUE e.g. PC2/PC1.5or PC1 are supported in any specific bands, then the A-MPR requirement should be revisited again to identify the appropriate the A-MPR requirements. (ZTE)

Way forward:

  - Further discuss with following agreement and above options.

Issue 3-3-3: A-MPR - Agreement - FFS A-MPR requirements. Following potential factors to be further discussed: - Simulation reduction by not considering large RB allocations, CP-OFDM, higher-order modulations - A-MPR simulation assumption including ACLR from co-existence study - Applicable bands - Regional requirements for A-MPR - Other factors

Issue 3-3: RSD requirements for other PC (PC1.5 and PC1)

For PC1.5:

  - Option 1-1: For power class 1.5, more study on RSD is needed for NR-NTN, depending on the conclusion on the feasibility and agreed architecture. (vivo)

  - Option 1-2: If PC1.5 2Tx RSD is studied, it should be based on handheld UE, and can also applied to non-handheld. (vivo)

For PC 1:

  - Option 2: For PC1 1Tx for NR-NTN, which is only considered feasible for non-handheld, the RSD is proposed to be 0dB. Whether and how to make it clear in the spec that this is for non-handheld only is FFS. (vivo)

Way forward:

  - FFS RSD for PC1/PC1.5 for applicable UE architectures.