3GPP TSG-RAN4 Meeting #112-bis	R4-2417124

# Hefei, China, 14th Oct 2024 – 18th Oct 2024

Source:	Samsung, TELUS, Bell Mobility

Title:	TP for TR 38.719-02-01 CA_n29-n77

Agenda item:			5.3.3

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.719-02-01 to include CA_n29-n77 according to the request in [1].

2. Reference

[1] RP-241833 Revised WID: Rel-19 NR Carrier Aggregation (CA)/Dual Connectivity (DC) for x bands DL with y bands UL (x<7, y<3) and Supplementary Uplink (SUL) band combinations/CA band combinations with a single SUL or two SUL cells

# 3. Text Proposal

<Start of Text Proposal>

## 5.x	CA_n29-n77

### 5.x.1	Common for 1 band UL and 2 bands UL CA

#### 5.x.1.1	Operating bands for CA

Table 5.x.1.1-1:  CA band combination of band n29+n77

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n29 | N/A | 717 MHz – 728 MHz | SDL |
| n77 | 3300 MHz – 4200 MHz | 3300 MHz – 4200 MHz | TDD |

#### 5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination of band n29+n77

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n29A-n77A | - | n29 | n29 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n29A-n77(2A) | CA_n77(2A) | n29 | n29 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77(2A)_BCS4 and 5 |  |
| CA_n29A-n77(3A) | CA_n77(2A) | n29 | n29 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77(3A)_BCS4 and 5 |  |

#### 5.x.1.3	UE co-existence studies for 1 band UL

Table 5.x.1.3-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n29-n77.

Table 5.x.1.3-1 UL/DL harmonics/harmonic mixing analysis

| UL/DL harmonics |  | n77 | UL12 | UL2 | UL33 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | fLow | 3300 | 6600 | 9900 | 13200 | 16500 |  |
| n29 | fLow | fHigh | 4200 | 8400 | 12600 | 16800 | 21000 |  |
| DL1 | 717 | 728 | N/A |  |  |  |  | UL harmonic |
| DL23 | 1434 | 1456 |  | N/A |  | N/A | N/A | Harmonic mixing |
| DL34 | 2151 | 2184 |  |  | N/A |  | N/A |  |
| DL4 | 2868 | 2912 |  | N/A | N/A | N/A | N/A |  |
| DL54 | 3585 | 3640 | D |  | N/A | N/A | N/A |  |
| Analysis |  |  | There is UL1/DL5 harmonic mixing into n29 |  |  |  |  |  |
| UL/DL harmonics |  | n29 | UL14 | UL2 | UL33 | UL4 | UL5 | MSD type |
|  |  | fLow |  |  |  |  |  |  |
| n77 | fLow | fHigh |  |  |  |  |  |  |
| DL1 |  |  | N/A |  |  |  |  | UL harmonic |
| DL23 |  |  |  | N/A |  | N/A | N/A | Harmonic mixing |
| DL34 |  |  |  |  | N/A |  | N/A |  |
| DL4 |  |  |  | N/A | N/A | N/A | N/A |  |
| DL54 |  |  |  |  | N/A | N/A | N/A |  |
| Analysis |  |  | There is no need to analyze harmonics/harmonic mixing for n29 UL/n7DL, since band n29 is SDL band and only has DL. |  |  |  |  |  |
| Note 1: ULx means UL xth harmonic frequency, and DLy means DL yth harmonic frequency range Note 2: When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the ULx/DLy cell is marked “D” for direct hit. When the gap between ULx and DLy frequency range is from 0Hz to x*MinULCBW, the ULx/DLy cell is marked “N” for Near miss. Note 3: UL3/DL2 harmonic mixing direct hit case for PC3/5 only apply for DL>3GHz Note 4: For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders. |  |  |  |  |  |  |  |  |

Table 5.x.1.3-2 summarizes frequency ranges where cross band isolation may occur for CA_n29-n77.

Table 5.x.1.3-2: Cross-band isolation analysis

| Bands | n29 |  | n77 |  |
| --- | --- | --- | --- | --- |
| Frequency limit | flow | fhigh | flow | fhigh |
| fUL (MHz) |  |  | 3300 | 4200 |
| fDL (MHz) | 717 | 728 | 3300 | 4200 |
| UL CBW (MHz)2 | Minimum CBW | Maximum CBW | Minimum CBW | Maximum CBW |
|  |  |  | 10 | 100 |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) |  |  | 3200 | 4300 |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) |  |  | 3100 | 4400 |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) |  |  | 3000 | 4500 |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) |  |  | 2900 | 4600 |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) |  |  | 2800 | 4700 |
| Analysis | There is no need to calculate the band n29 ACLR range, since band n29 is SDL band and only has DL. |  | There is no overlap of the band n77 ACLR range with the band n29 DL up to order 5. Moreover, n7 and n29 are not the same or adjacent band group as described in table A.1, so no cross-band MSD is needed. |  |
| NOTE 1: Even if there is no overlap up to ACLR5, MSD beyond the ACLR5 range should be evaluated further if: - The UL aggressor band and DL aggressor band are part of the same or adjacent band group as described in table A.1 - If the DL band is above the UL band, it’s lower frequency edge must be below the UL lowest 2nd harmonic frequency - As an indicative threshold, if >45dB UL rejection at the DL band frequency can be guaranteed, assuming a -130dBm/Hz TX noise floor level, the transmitter noise floor related MSD should be negligible NOTE 2: The maximum UL channel bandwidth of the BCS (noted maxULCBW) is used to calculate the band ACLR ranges while the minimum DL channel bandwidth of the BCS (noted minDLCBW) is used for the DL band victim channel bandwidth. |  |  |  |  |

Table 5.X.1.3-3 summarizes frequency ranges where IMD products caused by one UL band with 2CC intra-band UL CA may occur for CA_ n29-n77.

Table 5.x.1.3-3: Intra-band ULCA IMD overlap with the other DL band analysis.

| All in MHz | flow | fhigh | BB IMD range3 |  |  |
| --- | --- | --- | --- | --- | --- |
| n77 fUL | 3300 | 4200 | Order | flow | fhigh |
| n29 fDL | 717 | 728 |  |  |  |
| 2CCBW1 | Minimum | Maximum | IMD2 (1-1) | Min2CCBW | Max2CCBW |
|  | 0 | 600 |  | 0 | 600 |
| Close to UL IMD range2 |  |  | IMD4 (2-2) | 2*Min2CCBW | 2*Max2CCBW |
| Order | flow | fhigh |  | 0 | 1200 |
| IMD3 (2-1) | fULlow-Max2CCBW | fULhigh+Max2CCBW | IMD6 (3-3) | 3*Min2CCBW | 3*Max2CCBW |
|  | 2700 | 4800 |  | 0 | 1800 |
| IMD5 (3-2) | fULlow-2*Max2CCLBW | fULhigh+2*Max2CCBW | Close to H2 IMD range4 |  |  |
|  | 2100 | 5400 | Order | flow | fhigh |
| IMD7 (4-3) | fULlow-3*Max2CCBW | fULhigh+3*Max2CCBW | IMD4 (3-1) | 2*fULlow-Max2CCBW | 2*fULhigh+Max2CCBW |
|  | 1500 | 6000 |  |  |  |
| IMD9 (5-4) | fULlow-4*Max2CCBW | fULhigh+4*Max2CCBW | IMD6 (4-2) | 2*fULlow-2*Max2CCBW | 2*fULhigh+2*Max2CCBW |
|  | 900 | 6600 |  |  |  |
| IMD11 (6-5) | fULlow-5*Max2CCBW | fULhigh+5*Max2CCBW | Close to H3 IMD range4 |  |  |
|  | 300 | 7200 | Order | flow | fhigh |
| IMD13 (7-6) | fULlow-6*Max2CCBW | fULhigh+6*Max2CCBW | IMD5 (4-1) | 3*fULlow-Max2CCBW | 3*fULhigh+Max2CCBW |
|  | -300 | 7800 |  |  |  |
| Analysis | The analysis shows there’s IMD4 due to one UL band with 2CC intra-band UL CA. |  |  |  |  |
| NOTE 1: 2CCBW is the instantaneous transmit bandwidth of the two intra-band UL CCs: - The minimum 2CCBW for contiguous / non-contiguous intra-band ULCA is 0 / minimum UL channel bandwidth - The maximum 2CCBW for contiguous / non-contiguous ULCA is Min(maximum aggregated bandwidth / maximum separation bandwidth(600MHz),fULhigh-fULlow) NOTE 2: The close to UL IMD range is the most critical when the victim DL band in proximity to the UL band: - For contiguous/non-contiguous intra-band ULCA within a TDD band, IMD order up to 9/7 should be considered and MPR assumed - For intra-band ULCA within a FDD band, IMD order up to 13 should be considered for bands in the same band group and MPR is not assumed. If justified by poor filtering performance, higher order IMD may need to be specified. NOTE 3: The BB IMD range should only be considered if the DL band is below the UL band and for non-contiguous ULCA within a TDD band >3GHz (assuming CA with 450MHz bands is not considered) - IMD2 is not considered assuming CA with 450MHz bands is not considered - IMD4 is considered for FDD or SimRx/Tx TDD bands <1GHz - IMD6 is considered for FDD or SimRx/Tx TDD bands <1.68GHz NOTE 4: The harmonic 2 and 3 IMD ranges should only be considered if the DL band is above the UL band |  |  |  |  |  |

#### 5.x.1.4	∆TIB,c and ∆RIB,c values

For CA_n29-n77, the TIB,c and RIB,c have already been specified and this section does not need to be revisited.

#### 5.x.1.5	REFSENS requirements

-  Based on the co-existence studies on Table 5.x.1.3-1, the 5th harmonic mixing falls into Rx frequencies of band 29. And the MSD value has been defined in Table 7.3A.4-4 of TS38.101-1. There is no newer requirement needed.
-  Based on co-existence studies on Table 5.x.1.3-3, IMD4 of ULCA_n77(2A) falls into the n29DL, and the MSD requirements are defined as following.

Table 5.x.1.5-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n29-n77 | n29 | N/A | 5 | N/A | 720 | 8.6 | SDL | IMD415 |
|  | n7712 | 3540 | 10 | 1 (RBSTART=25) | 3410 | N/A | TDD | N/A |
|  |  | 3900 | 10 | 1 (RBSTART=25) | 3850 |  |  |  |
| NOTE 12: This band supports intra-band non-contiguous uplink configuration. NOTE 15: This band is subject to IMD6 also which MSD is not specified |  |  |  |  |  |  |  |  |

#### 5.x.1.6	OOB blocking exception requirements

There is no OOB exception for this CA combination.

Table 5.x.1.6-1: CA band combination with exceptions allowed

CA band combination

<End of Text Proposal>