3GPP TSG-RAN WG4 Meeting#112bis							         		R4-2417180

Hefei, China, 14 – 18 October 2024

Title: 	CA_n40A-n41C and CA_n41C-n79A MSD due to n41C UL

Source: 	Skyworks Solutions, Inc.

Agenda Item:	5.3.3	UE RF requirements for NR inter-band CA/DC configurations including inter band CA for 2 DL with up to 2UL (NR_CADC_R19_2BDL_xBUL)	[NR_CADC_SUL_R19-Core]

Document for:	Approval (not for block approval)

# 1	Background

In RAN#112, we presented the measured spectrum for single CC UL and contiguous ULCA spectrums with worst case and full allocation in [1] and showed that ULCA interference is significantly larger than single CC and has a different nature as it does not depend on transceiver impairments. A way forward collected the different options for MSD configurations in [2] and also captured our wideband spectrum measurements of the interference. In this contribution, we propose to capture the intra-band ULCA related MSD using the specification framework that we have used since Release 17 and as a compromise propose that a second optional test point captures MSD for fully allocated contiguous ULCA allocations.

# Discussion

## 2.1 Way forward inputs

The way forward [2] captured our input on single CC and contiguous intra-band ULCA spectrum for different allocations as background information and listed options for the MSD related to n41C UL into CA-n40A-n41C and CA_n41C-n79A DL. The way forward options for proposals is captured below in italics.

Proposal 1: In Rel-19, to keep as-is in spec as most of cases are covered and some history is built since Rel-17.

Proposal 2: For intra-band TDD ULCA within an inter-band DL CA, the proponent/operator can choose either one of the following test configurations for MSD analysis and potential definition of RF requirements.

•	Test case 1: Discrete UL RB allocation, e.g. 1RB + 1RB UL RB allocation in UL intra-band CA.

•	Test case 2: fully UL RB allocation with maximum aggregated BW.

Proposal 3: For the specific cases DL_n40A-n41C_UL_n41C and DL_n41C-n79A_UL_n41C, fully UL RB allocation with maximum aggregated BW will be used for MSD analysis and potential definition based on the evaluation. It’s encouraged for companies to provide the evaluation in next meeting.

Proposal 4: From RAN4 perspective, some reasonable UL RB allocations for intra-band UL CA can be recommended to benefit both UE and network and the corresponding condition can be reflected in the spec.

In the last meeting, we made a compromise proposal close to Proposal 2. In this contribution, we re-iterate our proposal in detail and then provide DL_n40A-n41C_UL_n41C and DL_n41C-n79A_UL_n41C MSDs according to this proposal.

## 2.2 Proposed intra-band ULCA related MSD test points.

In the previous meetings, we provided a lot of technical background in [1] as to why an Intra-band ULCA related MSD test is needed versus the single UL CC case and also showed that a Full+Full allocation did not dramatically change the MSD as the interfering spectrum around the UL signal and its second harmonic are very strong in both cases.

At the same time, if we are ready to listen to the fact that 1RB+1RB is not the most representative allocation for intra-band ULCA, it served us well since Release 17 as it allows an easy evaluation of the interference level and MSD test point frequency calculations.

Since we cannot make an exception in general and create discrepancies in the specification, we also cannot make an exception for only one band combination. If we are still open to discuss other options for this type of MSD, we can only agree to it when all existing similar MSDs for other band combinations are realigned with a potential new specification framework for Release 19.

In the meantime, to progress on the two requested band combination, we offer as a compromise that two test points are specified:

- One mandatory test point using the current specification framework with 1RB+1RB allocation.

- An optional test point using Full+Full contiguous allocation.

- This does not preclude changing the above in Release 19 if we potentially agree for a new specification framework and realign all existing MSDs accordingly.

Proposal on MSD test points related to contiguous intra-band ULCA:

- For Release19, on top of the mandatory test point based on current MSD specification framework based on 1RB+1RB allocation at MPR (IMD3 meets -13dBm/MHz and IMD5 meets -30dBm/MHz), an optional test point using fully allocated CCs in a contiguous allocation configuration at full power can be added.

- It is suggested that this second optional test point is only added when the MSD value is significantly different than for the mandatory test point.

- If further amendment to the above occurs during Release 19, the change will only be possible if all related MSDs test points for all band combinations with contiguous intra-band ULCA UL configuration are corrected accordingly.

## 2.3 CA_n40A-n41C DL n41C UL MSD.

Since the Band n40 and Band n41 are only present simultaneously in Japan and China, we have used the worst-case restricted spectrum for n41C (China) for spectrum calculations in Table 1.

Table 1: 1UL 2CC IMD calculation for CA_n40A-n41C DL n41C UL (China spectrum)

| all in MHz | flow/Min | fhigh/Max | BB IMD range3 |  |  |
| --- | --- | --- | --- | --- | --- |
| fUL5 | 2515 | 2675 | order | flow | fhigh |
| 2CCBW1 | 0 | 160 | IMD2 (1-1) | Min2CCBW | Max2CCBW |
| fDL | 2300 | 2400 |  | 0 | 160 |
| close to UL IMD range2 |  |  | IMD4 (2-2) | 2*Min2CCBW | 2*Max2CCBW |
| order | flow | fhigh |  | 0 | 320 |
| IMD3 (2-1) | fULlow-MaxULBW | fULhigh+MaxULBW | IMD6 (3-3) | 3*Min2CCBW | 3*Max2CCBW |
|  | 2355 | 2835 |  | 0 | 480 |
| IMD5 (3-2) | fULlow-2*MaxULBW | fULhigh+2*MaxULBW | close to H2 IMD range4 |  |  |
|  | 2195 | 2995 | order | flow | fhigh |
| IMD7 (4-3) | fULlow-3*MaxULBW | fULhigh+3*MaxULBW | IMD4 (3-1) | 2*fULlow-Max2CCBW | 2*fULhigh+Max2CCBW |
|  | 2035 | 3155 |  | 4870 | 5510 |
| IMD9 (5-4) | fULlow-4*MaxULBW | fULhigh+4*MaxULBW | IMD6 (4-2) | 2*fULlow-2*Max2CCBW | 2*fULhigh+2*Max2CCBW |
|  | 1875 | 3315 |  | 4710 | 5670 |
| IMD11 (6-5) | fULlow-5*MaxULBW | fULhigh+5*MaxULBW | close to H3 IMD range4 |  |  |
|  | 1715 | 3475 | order | flow | fhigh |
| IMD13 (7-6) | fULlow-6*MaxULBW | fULhigh+6*MaxULBW | IMD5 (4-1) | 3*fULlow-Max2CCBW | 3*fULhigh+Max2CCBW |
|  | 1555 | 3635 |  | 7385 | 8185 |
| analysis | The lower n41C IMD3 overlaps with band n40 DL (IMD5 also) |  |  |  |  |

For the analysis of the proposed two test points, we used our measurements from [1] where:

- For 1RB+1RB, 5dB MPR is used such that IMD3 meets -13dBm/MHz at antenna output and thus -9dBm/MHz at PA output

- For Full+full, no MPR is assumed and -13dBm/MHz is met in ACLR1 region. In that case, the measured IMD3 level at the appropriate offset and BW is -20.6dBm at antenna output and thus -16.6dBm at PA output

- .

- The post MRC MSD calculation is provided in Table 2 using the same filtering assumptions than for the single Cc n41UL cross-band MSD into n40 DL.

Table 2: Post MRC MSD analysis for the 1RB+1RB and Full+Full allocations in n40 DL

| 1+1RB at MPR Tx noise in band n40 10 MHz CBW | [dBm/MBW] | -9.0 |  |
| --- | --- | --- | --- |
| n41 SAW Tx to Ant. port rejection at n40 DL freq. | [dB] | 35 |  |
| Diplexer rejection of PA Tx noise at n40 DL freq. | [dB] | 0 |  |
| Main LNA to ant insertion loss / Post PA diplexer to ant IL | [dB] | 4 | 0 |  |
| Antenna-to-Antenna isolation | [dB] | 0 | 10 |  |
| In channel levels: primary/diversity (antenna) | [dBm/MBW] | -48.0 | -58.0 |  |
| n40 5MHz CBW ideal REFSENS | [dBm/MBW] | -100.0 |  |  |
| noise floor | [dBm/MBW] | -96.0 | -96.0 |  |
| IMD2 noise due to Tx leakage - referred to antenna connector | [dBm/MBW] | -999.0 | -999.0 |  |
| Total noise level at antenna connector | [dBm/MBW] | -48.0 | -58.0 |  |
| n40 5MHz degradation | [dB] | 48.0 | 38.0 |  |
| MSD afterMRC | uncor [dB] | 40.60 |  |  |
| MSD afterMRC | cor [dB] | 42.50 |  |  |
| Full+full at MPR0 Tx noise in band n40 10 MHz CBW | [dBm/MBW] | -16.6 |  |  |
| n41 SAW Tx to Ant. port rejection at n40 DL freq. | [dB] | 35 |  |  |
| Diplexer rejection of PA Tx noise at n40 DL freq. | [dB] | 0 |  |  |
| Main LNA to ant insertion loss / Post PA diplexer to ant IL | [dB] | 4 | 0 |  |
| Antenna-to-Antenna isolation | [dB] | 0 | 10 |  |
| In channel levels: primary/diversity (antenna) | [dBm/MBW] | -55.6 | -65.6 |  |
| n40 5MHz CBW ideal REFSENS | [dBm/MBW] | -100.0 |  |  |
| noise floor | [dBm/MBW] | -96.0 | -96.0 |  |
| IMD2 noise due to Tx leakage - referred to antenna connector | [dBm/MBW] | -999.0 | -999.0 |  |
| Total noise level at antenna connector | [dBm/MBW] | -55.6 | -65.6 |  |
| n40 5MHz degradation | [dB] | 40.4 | 30.4 |  |
| MSD afterMRC | uncor [dB] | 33.00 |  |  |
| MSD afterMRC | cor [dB] | 34.90 |  |  |

Observation: Even when the full+full contiguous allocation is used, the MSD is large and is of the same order of magnitude as the 1RB_1RB allocation case used in the current specification framework. Since this is related to an IMD3/ACLR1 range, the performance is anyhow bound by the -13dBm/MHz SEM requirement. With around 8dB difference at >30dB MSD, the MSD difference may not be considered significant.

Proposal on CA_n40A-n41C DL n41C UL MSD: The following Table is captured in TS 38.101-1 Table 7.3A.5-1.

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| CA_n40-n41 | n40 | N/A | 5 | N/A | 2397.5 | 42.5 | FDD | IMD3x |
|  | n41 | 2555 | 80 | 1 (RBSTART=0) | 2555 | N/A | TDD | N/A |
|  |  | 2635 | 80 | 1 (RBSTART=107) | 2635 | N/A | TDD | N/A |
|  | n40 | N/A | 5 | N/A | 2397.5 | 34.9 | FDD | IMD3xy |
|  | n41 | 2555 | 80 | 216 (RBSTART=1) | 2555 | N/A | TDD | N/A |
|  |  | 2635 | 80 | 216 (RBSTART=0) | 2635 | N/A | TDD | N/A |
| Note x: this DL band is also subject to IMD5 that is not specified Note y: this second test point is optional |  |  |  |  |  |  |  |  |

## 2.4 CA_n41C-n79A DL n41C UL MSD.

Since the Band n40 and Band n79 are only present simultaneously in Japan and China, we have used the worst-case restricted spectrum for n41C (China) for spectrum calculations in Table 3.

Table 3: 1UL 2CC IMD calculation for CA_n41C-n79A DL n41C UL (China spectrum)

| all in MHz | flow/Min | fhigh/Max | BB IMD range3 |  |  |
| --- | --- | --- | --- | --- | --- |
| fUL5 | 2515 | 2675 | order | flow | fhigh |
| 2CCBW1 | 0 | 160 | IMD2 (1-1) | Min2CCBW | Max2CCBW |
| fDL | 4800 | 5000 |  | 0 | 160 |
| close to UL IMD range2 |  |  | IMD4 (2-2) | 2*Min2CCBW | 2*Max2CCBW |
| order | flow | fhigh |  | 0 | 320 |
| IMD3 (2-1) | fULlow-MaxULBW | fULhigh+MaxULBW | IMD6 (3-3) | 3*Min2CCBW | 3*Max2CCBW |
|  | 2355 | 2835 |  | 0 | 480 |
| IMD5 (3-2) | fULlow-2*MaxULBW | fULhigh+2*MaxULBW | close to H2 IMD range4 |  |  |
|  | 2195 | 2995 | order | flow | fhigh |
| IMD7 (4-3) | fULlow-3*MaxULBW | fULhigh+3*MaxULBW | IMD4 (3-1) | 2*fULlow-Max2CCBW | 2*fULhigh+Max2CCBW |
|  | 2035 | 3155 |  | 4870 | 5510 |
| IMD9 (5-4) | fULlow-4*MaxULBW | fULhigh+4*MaxULBW | IMD6 (4-2) | 2*fULlow-2*Max2CCBW | 2*fULhigh+2*Max2CCBW |
|  | 1875 | 3315 |  | 4710 | 5670 |
| IMD11 (6-5) | fULlow-5*MaxULBW | fULhigh+5*MaxULBW | close to H3 IMD range4 |  |  |
|  | 1715 | 3475 | order | flow | fhigh |
| IMD13 (7-6) | fULlow-6*MaxULBW | fULhigh+6*MaxULBW | IMD5 (4-1) | 3*fULlow-Max2CCBW | 3*fULhigh+Max2CCBW |
|  | 1555 | 3635 |  | 7385 | 8185 |
| analysis | n41C upper IMD4 overlaps with n79 DL (IMD6 also) |  |  |  |  |

For the analysis of the proposed two test points, we used our measurements from [1] where:

- For 1RB+1RB, 5dB MPR is used such that IMD3 meets -13dBm/MHz. In that case the measured IMD4(3-1) level is -37.6dBm at antenna output and thus -33.6dBm/MHz at PA output

- For Full+full, no MPR is assumed and -13dBm/MHz is met in ACLR1 region. In that case the measured IMD4(3-1) level at the appropriate offset is -44.2dBm at antenna output and thus -40.2dBm/MHz at PA output.

- Since n79 is close to twice the n41 frequency, a Band n41 Tx filter rebound at 27dB rejection and 15dB <2.7GH/>3.3GHz diplexer isolation are assumed.

- The post MRC MSD calculation is provided in Table 4.

Table 4: Post MRC MSD analysis for the 1RB+1RB and Full+Full allocations in n79 DL

| 1+1RB at MPR Tx noise in band n79 10 MHz CBW | [dBm/MBW] | -33.6 |  |
| --- | --- | --- | --- |
| n41 SAW Tx to Ant. port rejection at n79 DL freq. | [dB] | 27 |  |
| Diplexer rejection of PA Tx noise at n79 DL freq. | [dB] | 15 |  |
| Main LNA to ant insertion loss / Post PA diplexer to ant IL | [dB] | 4 | 0 |
| Antenna-to-Antenna isolation | [dB] | 0 | 10 |
| In channel levels: primary/diversity (antenna) | [dBm/MBW] | -79.6 | -89.6 |
| n79 10MHz CBW ideal REFSENS | [dBm/MBW] | -95.8 |  |
| noise floor | [dBm/MBW] | -91.8 | -91.8 |
| IMD2 noise due to Tx leakage - referred to antenna connector | [dBm/MBW] | -999.0 | -999.0 |
| Total noise level at antenna connector | [dBm/MBW] | -79.6 | -89.6 |
| n79 10MHz degradation | [dB] | 12.5 | 4.2 |
| MSD afterMRC | uncor [dB] | 6.65 |  |
| MSD afterMRC | cor [dB] | 8.40 |  |
| Full+full at MPR0 Tx noise in band n79 10 MHz CBW | [dBm/MBW] | -40.2 |  |
| n41 SAW Tx to Ant. port rejection at n79 DL freq. | [dB] | 27 |  |
| Diplexer rejection of PA Tx noise at n79 DL freq. | [dB] | 15 |  |
| Main LNA to ant insertion loss / Post PA diplexer to ant IL | [dB] | 4 | 0 |
| Antenna-to-Antenna isolation | [dB] | 0 | 10 |
| In channel levels: primary/diversity (antenna) | [dBm/MBW] | -86.2 | -96.2 |
| n79 10MHz CBW ideal REFSENS | [dBm/MBW] | -95.8 |  |
| noise floor | [dBm/MBW] | -91.8 | -91.8 |
| IMD2 noise due to Tx leakage - referred to antenna connector | [dBm/MBW] | -999.0 | -999.0 |
| Total noise level at antenna connector | [dBm/MBW] | -86.2 | -96.2 |
| n79 10MHz degradation | [dB] | 6.7 | 1.3 |
| MSD afterMRC | uncor [dB] | 3.23 |  |
| MSD afterMRC | cor [dB] | 4.60 |  |

Observation: Even when the full+full contiguous allocation is used, the MSD is large and is of the same order of magnitude as the 1RB_1RB allocation case used in current specification framework. With around 3dB difference at 3dB MSD, the MSD difference may not be considered significant.

Proposal on CA_n41C-n79A DL n41C UL MSD: The following Table and notes is captured in TS 38.101-1 Table 7.3A.5-1.

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| CA_n41-n79 | n41 | 2555 | 80 | 1 (RBSTART=0) | 2555 | N/A | TDD | N/A |
|  |  | 2635 | 80 | 1 (RBSTART=0) | 2635 | N/A | TDD | N/A |
|  | n79 | N/A | 10 | N/A | 4950 | 8.4 | FDD | IMD4x |
|  | n41 | 2555 | 80 | 216 (RBSTART=1) | 2555 | N/A | TDD | N/A |
|  |  | 2635 | 80 | 216 (RBSTART=0) | 2635 | N/A | TDD | N/A |
|  | n79 | N/A | 10 | N/A | 4950 | 4.6 | FDD | IMD4xy |
| Note x: this DL band is also subject to IMD6 that is not specified Note y: this second test point is optional |  |  |  |  |  |  |  |  |

# Conclusion

In this contribution, based on our previous work on measurements of the intra-band contiguous ULCA spectrums, we make the following proposals.

Proposal on MSD test points related to contiguous intra-band ULCA:

- For Release19, on top of the mandatory test point based on current MSD specification framework based on 1RB+1RB allocation at MPR (IMD3 meets -13dBm/MHz and IMD5 meets -30dBm/MHz), an optional test point using fully allocated CCs in a contiguous allocation configuration at full power can be added.

- It is suggested that this second optional test point is only added when the MSD value is significantly different than for the mandatory test point.

- If further amendment to the above occurs during Release 19, the change will only be possible if all related MSDs test points for all band combinations with contiguous intra-band ULCA UL configuration are corrected accordingly.

Proposal on CA_n40A-n41C DL n41C UL MSD: The following Table is captured in TS 38.101-1 Table 7.3A.5-1.

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| CA_n40-n41 | n40 | N/A | 5 | N/A | 2397.5 | 42.5 | FDD | IMD3x |
|  | n41 | 2555 | 80 | 1 (RBSTART=0) | 2555 | N/A | TDD | N/A |
|  |  | 2635 | 80 | 1 (RBSTART=107) | 2635 | N/A | TDD | N/A |
|  | n40 | N/A | 5 | N/A | 2397.5 | 34.9 | FDD | IMD3xy |
|  | n41 | 2555 | 80 | 216 (RBSTART=1) | 2555 | N/A | TDD | N/A |
|  |  | 2635 | 80 | 216 (RBSTART=0) | 2635 | N/A | TDD | N/A |
| Note x: this DL band is also subject to IMD5 that is not specified Note y: this second test point is optional |  |  |  |  |  |  |  |  |

Proposal on CA_n41C-n79A DL n41C UL MSD: The following Table and notes is captured in TS 38.101-1 Table 7.3A.5-1.

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| CA_n41-n79 | n41 | 2555 | 80 | 1 (RBSTART=0) | 2555 | N/A | TDD | N/A |
|  |  | 2635 | 80 | 1 (RBSTART=0) | 2635 | N/A | TDD | N/A |
|  | n79 | N/A | 10 | N/A | 4950 | 8.4 | FDD | IMD4x |
|  | n41 | 2555 | 80 | 216 (RBSTART=1) | 2555 | N/A | TDD | N/A |
|  |  | 2635 | 80 | 216 (RBSTART=0) | 2635 | N/A | TDD | N/A |
|  | n79 | N/A | 10 | N/A | 4950 | 4.6 | FDD | IMD4xy |
| Note x: this DL band is also subject to IMD6 that is not specified Note y: this second test point is optional |  |  |  |  |  |  |  |  |

# References

[1] R4-2411551 On intra-band ULCA interference versus single ULCC, Skyworks Solutions Inc., RAN4#112

[2] R4-2414351 WF on intra-band UL CA interference and RF requirements, Huawei, RAN4#112