3GPP TSG-RAN WG4 Meeting #112bis	 R4-2417188

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	6.17.6

Source: 	Qualcomm Incorporated

Title: 	AI/ML ad-hoc meeting minutes

Document for:	Approval

# Introduction

The discussion on the AI/ML study is organized under a single thread [127] in RAN4#112-bis. The ad-hoc meeting will discuss some of the topics from the moderator summary in [1].

# Discussion

## Topic #5: Logistical issues for CSI compression

### Sub-topic 5-1

AI/ML model format

Issue 5-1: AI/ML model format

- Proposals

  - Company preferences:

| TensorFlow | PyTorch | ONNX |
| --- | --- | --- |
| 4 | 6 | 9 |

- Recommended WF

RAN4 to discuss whether ONNX can be agree and if any further clarifications are needed

Discussion:

MTK: there are different versions of tensorFlow and some are not backwards compatible. We would need to agree the exact version for the sharing to work

E///: we support ONNX as the most generic framework. The other are relatively specific.

Intel: we are ok with ONNX. It supports only inference, people might need to convert it to python. We would prefer for all companies to also share PyTorch source code.

Nokia: using ONNX, it has to be imported in the tools that companies use

QC: can we allow other companies to share other formats.

Oppo: what’s the benefit of having a single one?

Agreement:

All agreements for data sharing are only for the CSI compression use case feasibility study. These agreements can be revisited in an eventual WI phase.

Agreement for AI/ML formats:

Use at least ONNX format for sharing.

Companies can also share PyTorch model

The models in different formats from one company shall be the same

### Sub-topic 5-2

Dataset format

Issue 5-2: Dataset format

- Proposals

  - Company preferences:

| Numpy | Maltab | Parquet |
| --- | --- | --- |
| 11 | 4 | 1 |

- Recommended WF

To be discussed whether Numpy is agreeable and if any further clarifications are needed

Discussion:

Agreement:

Use NumPy for dataset sharing

Do not use pickled data(compression mechanism)

### Sub-topic 5-3

Training Data file format

Issue 5-3: Training data file format

- Proposals

  - Option 1: An example of detaset file description is proposed as following:

    - The number of TX ports is P. N samples are provided in total. Each sample include M complex numbers corrsponding to a eigenvector concatenated by K sub-bands: W=[w1, w2,..., wK] .  wq is the eigenvector for the q-th sub-band channel, 1<=q<=K. wq =[Re{wq,1}, Im{wq,1}, ..., Re{wq,P}, Im{wq,P}].

  - Option 2:

    - Use a single file for training and validation datasets, and a separate file for inference dataset

    - Use separate dataset files to represent different system-level scenarios and for different number of target MIMO layers.

    - Dataset file includes a single float32 array of size [Ns = 600000, NCH = 2, NSB = 13, NANT = 32]

    - The dataset includes eigenvectors calculated based on average covariance channel matrix for the given MIMO layer for each sub-band based on pre-processing assumptions in WF R4-2414447

  - Option 3: RAN4 should specify different contents of dataset along with their dimension in the following way:

    - N (samples) X nSB (number of subbands) X nLayers (number of layers) X nPorts (number of CSI-RS ports) X 2 (IQ)

      - In other words, dataset should content following info:

      - 1st dimension: Number of samples

      - 2nd dimension: Number of sub-bands

      - 3rd dimension: Number of layers

      - 4th dimension: Number of antenna ports

      - 5th dimension: Real and imaginary

      - Note: Each element of the dataset will be a floating-point real number

  - Option 4: RAN4 to agree on shared dataset structure for CSI compression use-case based on the encoder input, i.e., specify the dimensions of the dataset (a, b, c, d, e), where, a is the total number of sample; b = 2 – real and imaginary part; c – number of sub-band (13); d – number of ports (32); e – optionally, number of layers (1 or 2).

  - Option 5: other

- Recommended WF

To be discussed

Discussion:

### Sub-topic 5-4

File naming scheme

Issue 5-4: File naming scheme

- Proposals

  - Option 1: Each dataset file’s name should include:

    - a unique identifier for the company,

    - meeting number

    - part number

      - dataset can be split among different parts where each part contains data of a subset of samples.

    - One of the following two options can be selected:

      - Option 1 (Different files for training and testing)

        - Dataset file’s name should include info regarding the use case of the dataset (training vs testing)

        - Note1: An example filename can be “EncoderInput_Train_Company1_113_Part0”

      - Option 2 (Same file for training and testing)

        - RAN4 needs to specify how to split the samples between training and testing

        - e.g., the 1st 80% can be used for training and the last 20% can be used for testing

        - Note2: An example filename can be “EncoderInput _Company1_113_Part0”

        - In note1 and note2, #1 denotes the unique identifier of a company and #113 denotes meeting number.

  - Option 2: The RAN4 moderator could setup the necessary folder to collect data/model, under which different companies’ sub-folder can be set up, such as:

    - Folder: RAN4_113_AI_CSI_compression_CNN_model_sharing (Cat-1)

    - Sub-folder: Company-A, Company-B, ...

  - Option 3:

    - File(s) belonging to a single model/dataset to be packed into a single zip archive.

    - File naming template can be agreed in RAN4, e.g., Meeting#_AI_DataType_Company_Version.zip.

  - Option 4: others

- Recommended WF

To be discussed

Discussion:

- Folders for AI/ML data sharing and current WI/use case to be created under “RAN4 folder”

- Subfolder for each meeting

  - Separate folders for models and dataset in the meeting subfolder

File naming scheme( ML model file and dataset file)

- a unique identifier for the company (4 characters – list to be maintained by RAN4 secretary)

- meeting number

- differentiate model and dataset by identifier (2 characters ml and ds)

- additional identifier with 2 digits (TBD)

- dataset can be split in multiple files – 2 digits

- files to be compressed to zips and uploaded

### Sub-topic 5-5

Retention policies

Issue 5-5: Retention policies

- Proposals

  - Option 1: For different categories of lifetimes for data/model sharing, RAN4 can further study based on the following ones as baseline

|  | AI/ML Model |  |  | AI/ML Dataset |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Cat-1 | Cat-2 | Cat-3 | Cat-a | Cat-b | Cat-c |
| Lifetime (i.e., minimum storage time in 3GPP server) | 3 months (i.e., to be deleted 3 months after model submission) | 1 release (i.e., to be deleted after the current release ASN.1 is frozen) | Forever (i.e., to be kept in 3GPP server forever) | 3 months (i.e., to be deleted 3 months after model submission) | 1 release (i.e., to be deleted after the current release ASN.1 is frozen) | Forever (i.e., to be kept in 3GPP server forever) |
| Typical sharing purpose | e.g., temporarily shared model for cross-company evaluation purpose etc. | e.g., reference model (e.g., reference encoder) used to derive RAN4 requirements | e.g., Standardized reference model (e.g., test model used in RAN4 spec.) | e.g., temporarily shared dataset for cross-company evaluation purpose etc. | e.g., the common dataset to derive RAN4 requirement | FFS the necessity |

The folder (to collect shared AI/ML model and/or dataset) shall be set up, by marking different categories of model/dataset, based on the RAN4-agreed rule of “lifetime” for data/model sharing, e.g.,

- Folder: RAN4_113_AI_CSI_compression_CNN_model_sharing (Cat-1)

  - Option 2: Consider retention in the light of the following uses for dataset/models, including whether each usage type should have a different retention policy:

    - Datasets or models shared between companies on a meeting to meeting basis as part of developing a study or work item

    - Datasets or models submitted as reference models

    - Datasets or models that are formally agreed to be references for deriving requirements

    - Datasets or models that are formally captured as part of a TS

  - Option 3:

    - The retention period of the models/datasets shared only for the study purposes can be the length of the 3GPP release, i.e., 18-24 month.

    - The retention policy of the models/datasets agreed to be captured in the outcome of the study or as a part of the TR can be the same as for regular TDocs/TRs.

  - Option 4: others

- Recommended WF

To be discussed

Discussion:

MCC: there should be 1-2 categories and remove the extreme case

Retention policy to be further discussed

## Topic #2: Testability and interoperability issues for beam management

### Sub-topic 2-6

Simulations for error impact evaluation

In the previous RAN4 meeting it was agreed to conduct a study on the impact of measurement errors. More details about the study need to be discussed

Issue 2-6:	Measurement error impact evaluation

- Proposals

  - Option 1: Agree on what simulations to perform

    - SLS – which assumptions to use?

    - LLS – which assumptions to use?

  - Option 2: how to model the measurement errors?

    - Use L1-RSRP difference to model errors as the baseline, errors can be:

      - option 2a: BB error

      - option 2b: RF error

      - option 2c: BB error +RF error

      - option 2d: others

  - Option 3: Whether training/inference dataset contains error or not

    - is there a need to model errors for both training data or only for inference?

  - Option 4: SLS results should be considered first and then LLS simulations can be performed if needed.

  - Option 5: others

- Recommended WF

  - To be discussed how to move forward on this study, what set of assumptions are needed and what are the next steps

    - Next step can be, e.g.,

      - Use SLS assumptions in RAN1 for deriving ideal L1-RSRP dataset

      - Use ideal L1-RSRP dataset for training or model errors in the training dataset

      - Agree a model for L1-RSRP errors (BB and/or RF errors in the L1-RSRP)ompanies to provide simulation results to evaluate the impact of errors

Discussion:

Samsung: we have concern such sims. RAN1 did already some SLS, some companies already evaluated the impact, there are some observations in the TR. What would be the difference?we normally do not do SLS in RAN4 to evaluate performance.

Xiaomi: we are fine to simulate but we need to align the assumptions

Nokia: this is needed to evaluate the performance. How can we define performance requirements for prediction?

Apple: would this be training or just inference?

Moderator: could be both?

Apple: depends on how data is collected. If data is only for inner cell, the accuracy should be good. We cannot have a single error model. Most important thing would be the inference. We could start with ideal measurements for training. Vivo: we observe that RAN1 saw performance difference even assuming similar assumptions on the errors. In RAN4 we would need to define requirements so we need to consider measurement errors. Our simulations show that noise degrades the performance a lot. We support to do the simulations.

E///: similar concern with Apple. We can use ideal measurement for training but this will impact data consistency. We have different KPIs for prediction, there could be different requirements. If model can output both beam and RSRP, we would have 2 requirements.

Oppo: we agree we should do some sims but the requirements will depend on channel models also, LLS might be enough.

Samsung: when RAN1 did this study, the concern was that measurement uncertainty will make the performance very low. The question will be what is the impact from SLS to define requirements. We should consider the feasibility of the metric. MU will also play a role. If based on the measurement error that AI/ML does not have good performance. We are fine with LLS.

Oppo: in RAN1, most companies did not do SLS. But the channel model derived from SLS to use in LLS. The simulation itself is basically a link level simulation.

Apple: we do not understand what we are trying to achieve. Training and inference are separate. We won’t be able to tell which one is more important. If we decouple, then we have to do step by step. We can assume ideal measurement data for training.

Nokia: we should start with ideal measurements for training. If we have measurement errors in the training data, the errors could be minimized by the prediction model.

QC: RAN1 studied the impact of measurement error for network side models, here we would do it for UE side models.

Vivo: we could have some intermediate KPIs.

Huawei: ideal measurements are for training? Or for model input and output? To define the requirements we would need to consider that the training data also has measurement errors.

Vivo: SLS is used to generate the dataset to train the UE model

Nokia: we should be close to what RAN1 did, otherwise it would take more time to get results

Agreement:

RAN4 to perform study to assess the impact of measurement errors on prediction accuracy

Perform LLS to evaluate the RSRP prediction accuracy and other KPIs

      - UE Model trained using ideal measurements

Vivo to lead offline discussion on simulation assumptions

## Topic #4: Testability and interoperability issues for CSI compression and CSI prediction

### Sub-topic 4-1

Option 3 simulation results for 2nd round

Issue 4-1: Simulation results discussion

- Proposals

  - Option 1: See simulation results in the table below.

comparison results from RAN4#112 vs. results from RAN4#112bis (after further alignment)

- Recommended WF

Discuss simulation results, need for further alignment or other issues to be studied until the next meeting

Compare the following between companies to obtain insight into the degree of alignment of the training/testsets: Amplitude CDF, strongest Eigenvalue CDF, SGCS for 2 and/or 4 strongest beams ? See exmples below:

  - Option 1: Companies are encouraged to provide CDF data for SGCS

    - Even better if the CDF could be plotted together on the same axes

  - Option 2: Companies are requested to provide CDF for e.g. amplitude from their training dataset

  - Option 3: Each company to report training/testing results across training epochs. If overfitting is consistently observed, investigate the sources of overfitting. To better understand the relative gains of AI/ML compared to type-II SGCS in adapting optimized simulation parameters for feasibility analysis, we propose conducting a related study. This will help pave the way for simulation alignment and for the definition of performance requirements.

We have identified two parameters for optimization:

1.	Learning rate (it could be adaptive)

2.	Phase normalization of the eigenvectors

3.Increasing the data size remedies tendency to overfit   (See R4-2415498)

  - Option 4: Companies to consider cross-validation of results on mixed/aggregated datasets

  - Option 5: others

Discussion:

E///: it’s best to compare the SGCS. There is variation in Type II performance. It would be useful to compare some statistics like the amplitude CDF.

ZTE: we  believe that the results still have some gap, we aligned almost all parameters. For next meeting we need to check data set or other statistics.

QC: we agree with ZTE that parameters are aligned, the difference should come from the training dataset. We cannot align further with these datasets.

Nokia: agree with QC comments. Whether the statistics of the dataset will help, it’s not clear.

Apple: the results are better aligned. Is this good enough? Is the gain we are seeing sufficient. For better alignment and improved gain, we have identified.

E///: we are not aiming to get gain. It would be good to find a way to compare the training data set. There are some outliers, would be good to see if the statistics of the training data set are also different.

Xiaomi: we discuss about alignment. We need to define what is alignment? Or higher performance? Delta is smaller than a certain value. If we provide the Cdf curve, what do you want to compare? Which %-ile?

Nokia: we would like to derive a metric but it’s not yet clear. I don’t think we can easily understand a metric for alignment. We would have to compare the dataset for alignment.

Vivo:we agree with QC. The dataset is probably not well aligned, we can see from Type II performance. We could aggregate the dataset from different companies.

Intel: I believe we have reasonably good alignment. The discrepancies are probably from dataset and system level sim differences.

E///: companies to bring to the next meeting CDFs of the SGCS and amplitudes in training data.

CATT: we think the alignment is good enough, we might not need all those statistics

Apple: how to ensure there is no overfitting? Companies can ensure that overfitting is not done. There are some companies not showing gain at all.

Nokia: there might be a gap due to overfitting. Should be enough if we have the same assumptions.

Agreement:

Next step:

Interested companies to share decoder ML models and dataset in next meeting (RAN4#113)

Model and dataset used to generate the data submitted to RAN4#112-bis

Additional data to be submitted:

CDF of SGCS per subband

CDF of the largest eigen value of the channel matrix of training data for each subbands (preferably to be submitted in an xls format)

### Sub-topic 4-4

Option 4a for 2-sided model

Option 4 was further split into 4 sub-options in the previous meeting in R4-2414323. The next steps on feasibility study for these options should be discussed

Issue 4-4: Option 4a(Dataset based) for 2-sided model

- Proposals

  - Option 1: Next steps:

      - Discuss/decide how to generate the training dataset and other assumptions

- need to align the ML model, what should the dataset contain (channel/eigenvector realization, etc), how to generate the dataset (aggregate dataset based on inputs from multiple companies, etc),

      - Check on performance alignment see simulation results from contributing companies

- repeat simulations until good alignment is achieved

- move to next step after alignment

      - Share models (encoder and/or decoder)/datasets (training/testing/inference) (sharing framework is currently under discussion)

      - Select one or more datasets for further analysis

- Selection criteria is TBD

      - Companies bring results for “own encoder and decoder” with selected dataset

- performance alignment to be checked (

      - Conclude Option 4a feasibility

    - feasibility criteria to be discussed (e.g. Perform testing of all the UE encoders using all test decoders (replicating the process of testing UE’s against RAN4 requirements with different TE vendor decoders, others given in R4-2415376, etc ).

  - Option 2: Others

- Recommended WF

  - To be discussed

Discussion:

The steps should be discussed and if any changes and/or additions are needed

Apple: do we really need to discuss this?

E///: in a WI for Option 3, we would have to discuss what model to use. We should also study if Option 4 is feasible or not, we would have multiple choices. It could be a way out in a potential WI if we cannot agree on a decoder model to standardize

Intel: we think it would be helpful to study Option 4. On the steps, I would also agree with Nokia. If we go with option 4 with dataset flavor, we might need a different dataset, we need decoder output also.

ZTE: do we need all parameters for the model?

E///: in step 3, why do we need models? Shouldn’t we just exchange models

Apple: with these steps, we only know that the own encoder of a company works with its own decoder, not with a decoder of a test equipment.

Vivo: we discuss for Option 3 and 4, to specify how to generate the decoder.

Intel: it’s really important the concept of Option 4. The whole idea is that we have datasets, the task is whether the decoder of the TE will work with a UE vendor developed encoder.

Discussion:

      - Discuss/decide contents to generate the training dataset and other assumptions (based as Option 3)

- need to align the ML model, what should the dataset contain (channel/eigenvector realization, etc), how to generate the dataset (aggregate dataset based on inputs from multiple companies, etc)

- Check on performance alignment see simulation results from contributing companies (based as Option 3)

- repeat simulations until good alignment is achieved

- move to next step after alignment

- Share datasets (training/testing/inference)

- Select one or more datasets for further analysis or how to aggregate

- Selection criteria is TBD

- Companies bring results for “own encoder and decoder” with selected dataset

- performance alignment to be checked

- Conclude Option 4a feasibility

    - feasibility criteria to be discussed (e.g. Perform testing of all the UE encoders using all test decoders (replicating the process of testing UE’s against RAN4 requirements with different TE vendor decoders, others given in R4-2415376, etc ).

## Topic #3: Testability and interoperability issues for positioning accuracy enhancement

### Sub-topic 3-1

Requirements for case 1

Several companies proposed to define requirements for case 1

Issue 3-1: Requirements for case 1

- Proposals

  - Option 1: Introduce requirements for case 1

    - requirements:

      - accuracy requirements: can be defined as the difference in meters/sub-meters between reported position and ground truth labels in each dimension of x-axis and y-axis

      - ground truth: according to RAN1 discussion, LMF can derive the ground truth label, e.g.  by PRU or target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label

    - RAN4 to consider the straight-line distance (unit: m) between the real position and the estimated position as the metric for accuracy requirement for case 1

    - relative positioning accuracy metric (change in position due to a change in environment)

    - other requirement/definition

  - Option 2: do not introduce any requirement for case 1

- Recommended WF

Discussion:

QC: we support to have reqs for case 1. The details need to be discussed. Ground truth can come from GNSS, the accuracy can be defined for both axis.

Vivo: we support option 1, second subbulet for the reqs. UE reports to LMF but we would need to define a straight line distance for the requirement.

CMCC: support option 1. We are also open to discuss other solutions for requirement definition

Nokia: also supports option 1.

E///: we do not agree, there is no signaling support for this. Only LCM is discussed for case 1.

CATT: prefer option option 1 with requirements in straight line. We need to study how to acquire the ground truth.

Huawei: share similar view with E///. We do not agree that it is feasible to define requirements

CMCC: legacy LPP procedure is used for signaling. There is no issue with signaling.

E///:

Agreement:

Companies are invited to bring further analysis on feasibility of defining requirements for case 1 and how to define requirements, if feasible

### Sub-topic 3-2

Requirements for case 3/3b

In the previous meeting it was agreed to wait for further RAN1 agreements to decide whether any requirements will be defined or not

Issue 3-2: Requirements for case 3a/3b

- Proposals

  - Option 1: for case 3a/3b, it is proposed to define requirements for the measurement reported by gNB, and legacy requirements for positioning measurement performed by gNB, e.g.  gNB Rx-Tx time difference absolute accurac and gNB SRS-RSRP, can be conaisdered as baseline.

    - According to RAN1 agreements, for AI/ML based positioning case 3b, timing information and the paired timing information and power information are supported for UE reporting to LMF.

    - According to RAN1 agreements, for AI/ML based positioning case 3a, at least LOS/NLOS indicator and/or timing information (e.g. UL RTOA or gNB Rx-Tx time difference) are supported for reporting

  - Option 2: RAN4 to define performance accuracy requirements for use case 3a/3b for the measurements reported by gNB. The specifics and scope of the core and performance requirements for use case 3a/3b are contingent on the outcomes of discussions within RAN1 and RAN2

  - Option 3:RAN4 will not define any requirement for gNB reports to LMF

  - Option 4: others

- Recommended WF

  - To be discussed

Discussion:

CMCC: subbullet of option 1, RAN1 already agreed on what needs to be reported, we should discuss. What else should we wait for? We can define requirement for 3a/3b

Nokia: we need to wait for further RAN1 agreements, there are no formal agreements for what to report

E///: when you say gNB Rx-Tx, do we also need UE Rx-Tx?

CMCC: intuition of proposal 1, we already have reqs for Rx-Tx. At least the legacy reqs should apply.

E///: this doesn’t answer the question. The model would be at the UE side.

Huawei: the measurement is confirmed but whether there is signaling to differentiate if the procedure is based on AI or not is not agreed so this might not testable.

### Sub-topic 3-3

Reference applicability for existing metrics

Issue 3-3: Report applicability for existing reported metrics

- Proposals

  - Option 1: Extend applicability of existing report mappings for UL-RToA and UL SRS-RSRPP to report UL measurements for AI/ML based positioning use cases.

  - Option 2: others

- Recommended WF

  - Option 1

Agreement:

Extend applicability of existing report mappings for UL-RToA and UL SRS-RSRPP to report UL measurements for AI/ML based positioning use cases.

# References

[1] R4-2415231, “Topic summary for [112-Bis][127] NR_AIML_air”, Moderator(Qualcomm Incorporated), RAN4#112-bis