3GPP TSG-RAN WG4 Meeting #112-bis	R4-2417196

Hefei, China, 14th – 18th Oct. 2024

Agenda item:			6.21.4

Source:	CMCC

Title:	WF on co-existence study for A-IOT

Document for:	Approval

Introduction

This way forward captures the agreements for co-existence evaluation for Rel-19 ambient IOT study item.

The summary in RAN4#112bis is R4-2415233. The way forward agreed in previous RAN4 meetings are R4-2406714, R4-2410567 and R4-2410304.

Evaluation methodology and cases

- Topic 3-2: Performance metric and SINR definition

Issue 3-2-1: Performance metric for AIOT

Agreement in RAN4#110bis: - For NR system, use 5% throughput loss as performance metric as legacy. - For AIOT system, including reader, device, intermediate UE, further discuss the performance metric: - Option 1: [10%] BLER, [Rx power] - Option 2: SINR degradation - Other options are precluded Agreement in RAN4#111: - Use SINR for calibration purpose - FFS on performance metric for co-existence evaluation and requirements definition. Agreement in RAN4#112: For inter-system interference (between AIOT and NR): - If SINR degradation is smaller than or equal to [1]dB, it can be considered that inter-system interference is negligible. - If SINR degradation is lager than [1]dB, consider following criteria: - Outage percentage consider SINR level with [10%] BLER - FFS on the outage percentage - Note: For SINR degradation, SINR refers to the 5% and 50% CDF SINR For intra-system interference (between AIOT and AIOT): - Outage percentage consider SINR level with [10%] BLER - FFS on the outage percentage

Agreement:

- Consider 3 SINR values for D2R and R2D respectively and derive the outage percentage value/range.

- The 3 values can be selected according to following steps:

- Select one set SNR/CNR values @ 10% BLER from companies based on RAN1 LLS (refer to latest RAN1 LLS summary)

- Set the SNR/CNR range by removing the highest and lowest value from the set in step 1

- Pick 3 values from the range in step 2, e.g. highest, lowest, middle

Note: R2D CNR needs to be transformed to SINR

Issue 3-2-2: SINR definition for D2R

Agreement in RAN4#111: Do not consider CW interference for calibration purpose for D1T1-A2 and D2T2-A2 FFS on how to consider CW cancellation capability in formal simulation Agreement in RAN4#112: - SINR includes CW interference is used as the baseline reference for co-existence evaluation for CW reader. baseline SINR=received wanted signal powernoise + intra_system interferencewithin total receiver bandwidth+residual CW interference after cancellationin linear scale - SINR is calculated as total power ratio: SINR=received wanted signal powernoise + intra_system interference + inter_system interferencewithin total receiver bandwidth+residual CW interference after cancellationin linear scale

Agreement:

- For -B and -A1, consider following baseline SINR:

  - Option 1 (last meeting agreement):

baseline SINR for CW outside=received wanted signal powernoise + intrasysteminterferencewithin total receiver bandwidth+residual CW interference after cancellationin linear scale

  - Option 2 (new proposal):

baseline SINR for CW outside=received wanted signal powernoise + intrasysteminterference+phase noisewithin total receiver bandwidth+residual CW interference after cancellationin linear scale

Note: If option2 is used, companies needs to report phase noise modelling.

- Topic 3-3: CW considerations

Issue 3-3-1: CW cancellation capability

Agreement in RAN4#111: Do not consider CW interference for calibration purpose for D1T1-A2 and D2T2-A2 FFS on how to consider CW cancellation capability in formal simulation Agreement in RAN4#112: - Companies to report the CW cancellation capability used for co-existence evaluation, CW cancellation capability for inside topology and outside topology can be different. - total interference cancellation capability = spatial isolation + RF cancellation + digital cancellation - CW transmission impact on the received SINR, e.g. degradation

Agreement:

Following is optional for co-existence evaluation:

- Use SNR degradation of 1 dB, 3 dB and 10 dB in coexisting study to evaluate the CW cancellation impact for CW inside topology A2

  - Assume perfect CW cancellation in SINR calculation, add degradation afterwards in the simulation

Evaluation parameters and results

- Topic 4-1: Adjacent RB Tx and Rx characteristics

Issue 4-1-1:  A-IOT reader and NR BS

Agreement:

|  | In-band |  |
| --- | --- | --- |
|  | Tx | Rx |
| NR UE/A-IOT Intermediate UE | For NR UE: - Baseline: Legacy UE IBE For intermediate UE: - Baseline: legacy UE IBE - Optional: NB-IOT ACLR | In-band A-IOT signal interference to NR is not considered when calculating SINR. Companies to check the interference caused by non-orthogonal A-IOT signal |
| NR BS | Option 1: 30dBc Option 2: 17dBc Option 3: 12dBc with 1 blank RB | In-band A-IOT signal interference to NR is not considered when calculating SINR. Companies to check the interference caused by non-orthogonal A-IOT signal |
| A-IOT BS | ACLR of legacy NB -IOT gNB (i.e. ACLR1:40dBc，ACLR2:50dBc) | ACS of legacy gNB |

Issue 4-1-2: CW unwanted emissions

Agreement in RAN4#112: Discuss whether and how to model the CW unwanted emission.

Agreement:

The impact of CW node to adjacent NR carrier can be analysed and captured in TR.

Note: no impact to co-existence simulation

Issue 4-1-3: Rx for device 1 and 2a

Agreement in RAN4#111: Assume no frequency selectivity for co-existence evaluation for calibration purposes for device 1 and 2a. Agreement in RAN4#112: - Alt-1: Assume no frequency selectivity - Alt-2: SINR; R2D with LPF. Interference from NR; Frequency selectivity: [7.5 dB]. Noise bandwidth; [180kHz]. SINRR2D_with_LPF=Wanted_SR2D_180kHzNoiseFloor 180kHz+(InterferenceNR10MHz/LPF_Selectivitylinear). - Alt-3: For R2D receiver with LPF. RF BW; 10MHz. BB BW; [180kHz]. SINRi (linear, not dB); Input SINR at the antenna connector with noise/interferencde BW=RF BW. SINRo (linear, not dB); Output SINR after BB LPF assuming downconverted noise/interference BW=BB BW. SINRo=SINRi28*SINRi+4*BWRFBWBB. - Alt-4: other LLS assumptions are not excluded

Agreement:

- For R2D, SINR at antenna connector is used as the starting point, i.e interference and noise bandwidth is 10MHz.

- Topic 4-2: General parameters and layout

Issue 4-2-1: Active rate of reader for D1T1

Agreement in RAN4#112: - Minimum distance between active readers: 60m as baseline, other values can be reported by other companies. - 2 readers are activated in one drop as baseline

Agreement:

- Keep 2 readers activated in one drop as baseline.

- Topic 4-6: Evaluation Conclusion

| deployment scenario and topology |  | spectrum | Evaluation case No. | (Aggressor -> Victim) | Tentative Co-existence conclusion |
| --- | --- | --- | --- | --- | --- |
| 1-1 | D1T1-A2- legacy UE only outdoor | R2D: DL CW2D and D2R: DL | a | Device->NR DL | Negligible NR DL throughput degradation for both average throughput and cell edge throughput is observed |  |
|  |  |  | b | NR DL->reader | TBA |  |
|  |  |  | c | Reader->NR DL | Negligible NR DL throughput degradation for both average throughput and cell edge throughput is observed |  |
|  |  |  | d | NR DL->device | SINR degradation is less than 1dB. Inter-system interference is negligible. TBD on the outage percentage |  |
| 1-2 | D1T1-A2-legacy UE indoor | R2D: DL CW2D and D2R: DL | a | Device->NR DL | Negligible NR DL throughput degradation for both average throughput and cell edge throughput is observed |  |
|  |  |  | b | NR DL->reader | TBA |  |
|  |  |  | c | Reader->NR DL | FFS: NR DL throughput degradation for both average throughput and cell edge throughput is observed. Note: TBD on the throughput degradation range FFS on the UE blocking issue. Interference coordination/mitigation solution can be considered. |  |
|  |  |  | d | NR DL->device | SINR degradation is less than 1dB. Inter-system interference is negligible. TBD on the outage percentage. |  |
| 2-1 | D1T1-B-legacy UE only outdoor | R2D: DL CW2D and D2R: UL | e | Device->NR UL | Negligible NR UL throughput degradation for both average throughput and cell edge throughput is observed. |  |
|  |  |  | f | NR UL->reader | TBA |  |
|  |  |  | c | Reader->NR DL | Negligible NR DL throughput degradation for both average throughput and cell edge throughput is observed. |  |
|  |  |  | d | NR DL->device | SINR degradation is less than 1dB. Inter-system interference is negligible. TBD on the outage percentage. |  |
| 2-2 | D1T1-B-legacy UE indoor | R2D: DL CW2D and D2R: UL | e | Device->NR UL | Negligible NR UL throughput degradation for both average throughput and cell edge throughput is observed. |  |
|  |  |  | f | NR UL->reader | TBA |  |
|  |  |  | c | Reader->NR DL | FFS: NR DL throughput degradation for both average throughput and cell edge throughput is observed. Note: TBD on the throughput degradation range level FFS on the UE blocking issue. Interference coordination/mitigation solution can be considered. |  |
|  |  |  | d | NR DL->device | SINR degradation is less than 1dB. Inter-system interference is negligible. TBD on the outage percentage. |  |
| 3 (O) | D1T1-A2-legacy UE indoor | R2D: UL CW2D and D2R: DL | a | Device->NR DL | TBA |  |
|  |  |  | b | NR DL->reader | TBA |  |
|  |  |  | g | Reader->NR UL | TBA |  |
|  |  |  | h | NR UL->device | TBA |  |
| 4 (O) | D1T1-B-legacy UE indoor | R2D: UL CW2D and D2R: UL | e | Device->NR UL | TBA |  |
|  |  |  | f | NR UL->reader | TBA |  |
|  |  |  | g | Reader->NR UL | TBA |  |
|  |  |  | h | NR UL->device | TBA |  |
| 5-1 | D2T2-A2-legacy UE only outdoor | R2D: UL CW2D and D2R: UL | e | Device->NR UL | Negligible NR UL throughput degradation for both average throughput and cell edge throughput is observed |  |
|  |  |  | f | NR UL->reader | SINR degradation is less than 1dB. Inter-system interference is negligible. TBD on the outage percentage. |  |
|  |  |  | g | Reader->NR UL | TBA |  |
|  |  |  | h | NR UL->device | SINR degradation is less than 1dB. Inter-system interference is negligible. TBD on the outage percentage. |  |
| 5-2 | D2T2-A2-legacy UE indoor | R2D: UL CW2D and D2R: UL | e | Device->NR UL | Negligible NR UL throughput degradation for both average throughput and cell edge throughput is observed |  |
|  |  |  | f | NR UL->reader | TBA |  |
|  |  |  | g | Reader->NR UL | FFS: NR UL throughput degradation for both average throughput and cell edge throughput is observed. Note: TBD on the throughput degradation range Interference coordination/mitigation solution can be considered. |  |
|  |  |  | h | NR UL->device | TBA |  |
| 6-1(O) | D2T2-B-legacy UE only outdoor | R2D: UL CW2D and D2R: DL | a | Device->NR DL | TBA |  |
|  |  |  | b | NR DL->reader | TBA |  |
|  |  |  | g | Reader->NR UL | TBA |  |
|  |  |  | h | NR UL->device | TBA |  |
| 6-2(O) | D2T2-B-legacy UE indoor | R2D: UL CW2D and D2R: DL | a | Device->NR DL | TBA |  |
|  |  |  | b | NR DL->reader | TBA |  |
|  |  |  | g | Reader->NR UL | TBA |  |
|  |  |  | h | NR UL->device | TBA |  |