3GPP TSG-RAN WG4 Meeting #112bis	Rev R4-2414982
Hefei, Anhui, China, 14th – 18th October, 2024

Title:	R19 AMPR for n28 40MHz

Source:	OPPO

Agenda item:	5.20.2

Document for:	Approval

- Introduction

This paper discusses the NS_18 AMPR for n28 40MHz for both PC3 and PC2 per WF [1].

- Discussion

  - AMRP for PC3 with 1Tx

2.1.1 AMRP region definition

The AMPR simulation configurations are:

- power class PC3

- One PC3 low band PA

- 40MHz CBW with 30khz scs (figure 1)

- CP-OFDM / DFT-S-OFDM

- Modulation pi/2 bpsk, qpsk, 16qam, 64qam, 256qam

- AMPR-MPR results are shown in annex A

- AMPR results are shown in annex B

Figure 1 40MHz CBW location and NS_18 emission region

The AMPR-MPR results can be found in annex A.

Following similar region definition in 30MHz, below figure 2 shows AMPR-MPR results (where AMPR is larger than MPR) and gives the region definition for 40MHz PC3.

Figure 2 Shown of AMPR region definition (30khz scs, CP-OFDM 16QAM AMPR-MPR results)

The region definition is in table 1.

Table 1 AMPR region definition for 1T PC3

| CBW, MHz | Tx BW, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart *12*SCS (MHz) | LCRB *12*SCS (MHz) |  |
| 40MHz | 703-743 | >(LCRB*12*SCS)/2+8.46 | ≥Max (0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+8.46 | ≥5.4 | A4 |
|  |  | ≤11.16 | <5.4 | A5 |

2.1.2 AMRP requirements

Based on above AMPR region definition and the AMPR simulation results in Annex B, the AMPR requirements are summarized in table 2 for DFT-S-OFDM and CP-OFDM with 1dB implementation margin considered.

Table 2 AMPR for 1T PC3

|  | DFT-S-OFDM |  |  | CP-OFDM |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | A3 | A4 | A5 | A3 | A4 | A5 |
| Pi/2 BPSK | 1 | 8 | 5 |  |  |  |
| QPSK | 4 | 8 | 5 | 6 | 10 | 7 |
| 16QAM | 6 | 10 | 6 | 6 | 10 | 7 |
| 64QAM | 6 | 10 | 6 | 6 | 10 | 7 |
| 256QAM | 6 | 10 | 6 | 6 | 10 | 7 |

Proposal 1:         Consider Table 1 as the AMPR region definition and Table 2 as the AMPR requirements for 1T PC3.

  - AMRP for PC2

2.1.1 AMRP region definition

The AMPR simulation configurations are:

- Power class 2 with 1*PC2 PA or 2*PC3 PA

- 40MHz CBW with 30khz scs (figure 1)

- CP-OFDM / DFT-S-OFDM

- Modulation pi/2 bpsk, qpsk, 16qam, 64qam, 256qam

- AMPR-MPR results are shown in annex A

- AMPR results are shown in annex B

Below figure 3 and 4 gives the AMPR region status for 1T PC2. It can be seen most of PC3 region definition can be reused except for QPSK with DFT and CP-OFDM where MPR might be needed for the yellow highlighted region. And this new region is marked with A6.

Similar issue can be found also in 2T PC2 QPSK and 16QAM simulation results (Annex A).

Figure 3 Shown of AMPR region definition for PC2 1Tx (30khz scs, DFT QPSK AMPR-MPR results)

Figure 4 Shown of AMPR region definition for PC2 1Tx (30khz scs, CP-OFDM QPSK AMPR-MPR results)

Proposal 2:         Consider Table 3 as the AMPR region definition for PC2 with 1Tx or 2Tx.

Table 3 AMPR region definition for PC2 with 1Tx or 2Tx

| CBW, MHz | Tx BW, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart *12*SCS (MHz) | LCRB *12*SCS (MHz) |  |
| 40MHz | 703-743 | >(LCRB*12*SCS)/2+8.46 | ≥Max (0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+8.46 | ≥5.4 | A4 |
|  |  | ≤11.16 | <5.4 | A5 |
|  |  | >(LCRB*12*SCS)/2+8.46 ≤(LCRB*12*SCS)/2+11.7 | <Max (0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) ≥5.4 | A6 |

2.2.1 1T PC2 results

The AMPR simulation results for 1T PC2 can be found in annex B.

Below table 4 summarizes the AMPR results for 1T PC2 with 1dB margin based on the region definition in Table 3.

Table 4 AMPR for 1T PC2

|  | DFT-S-OFDM |  |  |  | CP-OFDM |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A3 | A4 | A5 | A6 | A3 | A4 | A5 | A6 |
| Pi/2 BPSK | 2 | 8 | 5 | 2 |  |  |  |  |
| QPSK | 4 | 10 | 5 | 4 | 6 | 13 | 8 | 6 |
| 16QAM | 5 | 10 | 6 | 5 | 6 | 13 | 8 | 6 |
| 64QAM | 5 | 10 | 6 | 5 | 6 | 14 | 8 | 6 |
| 256QAM | 5 | 10 | 6 | 5 | 6 | 14 | 8 | 6 |

2.2.2 2T PC2 results

Similar as 1T PC2, the AMPR results for 2T PC2 can be found in annex B.

Below table 5 summarizes the AMPR results for 2T PC2 with 1dB margin based on the region definition in Table 3.

Table 5 AMPR for 2T PC2

|  | DFT-S-OFDM |  |  |  | CP-OFDM |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A3 | A4 | A5 | A6 | A3 | A4 | A5 | A6 |
| Pi/2 BPSK | 3 | 10 | 5 | 3 |  |  |  |  |
| QPSK | 5 | 13 | 7 | 5 | 6 | 13 | 8 | 6 |
| 16QAM | 6 | 13 | 7 | 6 | 6 | 13 | 8 | 6 |
| 64QAM | 6 | 13 | 7 | 6 | 6 | 13 | 8 | 6 |
| 256QAM | 6 | 13 | 7 | 6 | 6 | 13 | 8 | 6 |

2.2.3 AMRP requirements

From table 4 and table 5, it can be seen the difference between 1T and 2T are around 0-2dB. If one AMPR table for 1Tx and 2Tx, then AMPR table 6 can be used.

Table 6 AMPR for both 1T and 2T PC2

|  | DFT-S-OFDM |  |  |  | CP-OFDM |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A3 | A4 | A5 | A6 | A3 | A4 | A5 | A6 |
| Pi/2 BPSK | 3 | 10 | 5 | 3 |  |  |  |  |
| QPSK | 5 | 13 | 7 | 5 | 6 | 13 | 8 | 6 |
| 16QAM | 6 | 13 | 7 | 6 | 6 | 13 | 8 | 6 |
| 64QAM | 6 | 13 | 7 | 6 | 6 | 14 | 8 | 6 |
| 256QAM | 6 | 13 | 7 | 6 | 6 | 14 | 8 | 6 |

Proposal 3:         If separate AMPR tables for PC2 with 1Tx and 2Tx, then AMPR table 4 and 5 can be used.

Proposal 4:         If one AMPR table for PC2 with 1Tx and 2Tx, then AMPR table 6 can be used.

- Conclusion

This paper shared the AMPR simulation results for NS_18 with 40MHz CBW in n28 and got below observations and proposals.

Proposal 1:         Consider Table 1 as the AMPR region definition and Table 2 as the AMPR requirements for 1T PC3.

Table 1 AMPR region definition for 1T PC3

| CBW, MHz | Tx BW, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart *12*SCS (MHz) | LCRB *12*SCS (MHz) |  |
| 40MHz | 703-743 | >(LCRB*12*SCS)/2+8.46 | ≥Max (0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+8.46 | ≥5.4 | A4 |
|  |  | ≤11.16 | <5.4 | A5 |

Table 2 AMPR for 1T PC3

|  | DFT-S-OFDM |  |  | CP-OFDM |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | A3 | A4 | A5 | A3 | A4 | A5 |
| Pi/2 BPSK | 1 | 8 | 5 |  |  |  |
| QPSK | 4 | 8 | 5 | 6 | 10 | 7 |
| 16QAM | 6 | 10 | 6 | 6 | 10 | 7 |
| 64QAM | 6 | 10 | 6 | 6 | 10 | 7 |
| 256QAM | 6 | 10 | 6 | 6 | 10 | 7 |

Proposal 2:         Consider Table 3 as the AMPR region definition for PC2 with 1Tx or 2Tx.

Table 3 AMPR region definition for PC2 with 1Tx or 2Tx

| CBW, MHz | Tx BW, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart *12*SCS (MHz) | LCRB *12*SCS (MHz) |  |
| 40MHz | 703-743 | >(LCRB*12*SCS)/2+8.46 | ≥Max (0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+8.46 | ≥5.4 | A4 |
|  |  | ≤11.16 | <5.4 | A5 |
|  |  | >(LCRB*12*SCS)/2+8.46 ≤(LCRB*12*SCS)/2+11.7 | <Max (0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) ≥5.4 | A6 |

Proposal 3:         If separate AMPR tables for PC2 with 1Tx and 2Tx, then AMPR table 4 and 5 can be used.

Table 4 AMPR for 1T PC2

|  | DFT-S-OFDM |  |  |  | CP-OFDM |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A3 | A4 | A5 | A6 | A3 | A4 | A5 | A6 |
| Pi/2 BPSK | 2 | 8 | 5 | 2 |  |  |  |  |
| QPSK | 4 | 10 | 5 | 4 | 6 | 13 | 8 | 6 |
| 16QAM | 5 | 10 | 6 | 5 | 6 | 13 | 8 | 6 |
| 64QAM | 5 | 10 | 6 | 5 | 6 | 14 | 8 | 6 |
| 256QAM | 5 | 10 | 6 | 5 | 6 | 14 | 8 | 6 |

Table 5 AMPR for 2T PC2

|  | DFT-S-OFDM |  |  |  | CP-OFDM |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A3 | A4 | A5 | A6 | A3 | A4 | A5 | A6 |
| Pi/2 BPSK | 3 | 10 | 5 | 3 |  |  |  |  |
| QPSK | 5 | 13 | 7 | 5 | 6 | 13 | 8 | 6 |
| 16QAM | 6 | 13 | 7 | 6 | 6 | 13 | 8 | 6 |
| 64QAM | 6 | 13 | 7 | 6 | 6 | 13 | 8 | 6 |
| 256QAM | 6 | 13 | 7 | 6 | 6 | 13 | 8 | 6 |

Proposal 4:         If one AMPR table for PC2 with 1Tx and 2Tx, then AMPR table 6 can be used.

Table 6 AMPR for both 1T and 2T PC2

|  | DFT-S-OFDM |  |  |  | CP-OFDM |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A3 | A4 | A5 | A6 | A3 | A4 | A5 | A6 |
| Pi/2 BPSK | 3 | 10 | 5 | 3 |  |  |  |  |
| QPSK | 5 | 13 | 7 | 5 | 6 | 13 | 8 | 6 |
| 16QAM | 6 | 13 | 7 | 6 | 6 | 13 | 8 | 6 |
| 64QAM | 6 | 13 | 7 | 6 | 6 | 14 | 8 | 6 |
| 256QAM | 6 | 13 | 7 | 6 | 6 | 14 | 8 | 6 |

- Reference

[1] R4-2414274 WF on introduction of PC2 and 40MHz CBW in NR band n28, CMCC, #112 2024-08

Annex A: AMPR-MPR simulation results

Annex A.1 1T PC3 DFT-S-OFDM

[pi/2 BPSK]													 [QPSK]

[16QAM]													 [64QAM]

[256QAM]

Annex A.2 1T PC3 CP-OFDM

[QPSK]													 [16QAM]

[64QAM]													 [256QAM]

Annex A.3 1T PC2 DFT-S-OFDM

[pi/2 BPSK]													 [QPSK]

[16QAM]													 [64QAM]

[256QAM]

Annex A.4 1T PC2 CP-OFDM

[QPSK]													 [16QAM]

[64QAM]													 [256QAM]

Annex A.5 2T PC2 DFT-S-OFDM

[pi/2 BPSK]													 [QPSK]

[16QAM]													 [64QAM]

[256QAM]

Annex A.6 2T PC2 CP-OFDM

[QPSK]													 [16QAM]

[64QAM]													 [256QAM]

Annex B: AMPR simulation results

Annex B.1 1T PC3 DFT-S-OFDM

[pi/2 BPSK]													 [QPSK]

[16QAM]													 [64QAM]

[256QAM]

Annex B.2 1T PC3 CP-OFDM

[QPSK]													 [16QAM]

[64QAM]												 [256QAM]

Annex B.3 1T PC2 DFT-S-OFDM

[pi/2 BPSK]													 [QPSK]

[16QAM]													 [64QAM]

[256QAM]

Annex B.4 1T PC2 CP-OFDM

[QPSK]													 [16QAM]

[64QAM]												 [256QAM]

Annex B.5 2T PC2 DFT-S-OFDM

[pi/2 BPSK]													 [QPSK]

[16QAM]													 [64QAM]

[256QAM]

Annex B.6 2T PC2 CP-OFDM

[QPSK]													 [16QAM]

[64QAM]												 [256QAM]