3GPP TSG-RAN5 Meeting #104	R5-244431

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3502 | rev | - | Current version: | 17.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Routine maintenance for TS 38.523-3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 1. The NR/5GC test model needs to be enhanced to enable the implementation of the NR MBS broadcast test cases. 2. The execution guidelines need to be updated to consider test specification updates and the new delivered test cases. 3. For UL messages received by the SS and sent up to the TTCN, race conditions may apply under certain conditions (and not under other conditions). Those principles exist since Rel-8 LTE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Added new clauses 5.2.1.8, 7.3.9. Added new MMI commands in clause 6.1. Added Frequency resources used for MCCH and MTCH in clause 7.1.2.2.3.1. 2. Updated clause 11. 3. An explicit reference to TS 36.523-3 clause 7.10 has been added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1. New MBS broadcast test cases will not be implementable in TTCN. 2. Execution guidelines will remain incomplete. 3. Lack of documentation may be perceived. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 5.2.1.8 (new clause), 6.1, 7.1.2.2.3.1, 7.1.8(new), 7.3.9 (new clause), 11.3.1.2, 11.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GC	5G Core Network

ASP	Abstract Service Primitive

ATS	Abstract Test Suite

AWGN	Additive White Gaussian Noise

BWP	Bandwidth Part

CA	Carrier Aggregation

CBRA	Contention Based Random Access

CC	Component Carriers

CCE	Control Channel Element

CFRA	Contention Free Random Access

CG-SDT	Configured Grant-based SDT

CORESET	Control Resource Set

CSS	Common Search Space

DAI	Downlink Assignment Index

DCI	Downlink Control Information

DL	Downlink

DL-SCH	Downlink Shared Channel

DMRS	Demodulation Reference Signal

DRX	Discontinuous Reception

EHC	Ethernet Header Compression

EN-DC	E-UTRA-NR Dual Connectivity

EPC	Evolved Packet Core

FDD	Frequency Division Duplex

FD-FDD	Full-Duplex FDD

FR1	Frequency Range 1

FR2	Frequency Range 2

GSO	Geosynchronous Orbit

HD-FDD	Half-Duplex FDD

HO	Handover

H-SFN	Hyper SFN

ICS	Implementation Conformance Statement

IUT	Implementation Under Test

IXIT	Implementation eXtra Information for Testing

LBT	Listen Before Talk

LSB	Least Significant Bit

MBS	Multicast/Broadcast Service

MCCH	MBS Control Channel

MCG	Master Cell Group

MN	Master Node

MRB	MBS Radio Bearer

MSB	Most Significant Bit

MTCH	MBS Traffic Channel

MUSIM	Multi-Universal Subscriber Identity Module

NCD-SSB	Non Cell Defining SSB

NE-DC	NR-E-UTRA Dual Connectivity

NGSO	Non-Geosynchronous Orbit

NR	NR Radio Access

NR-DC	NR-NR Dual Connectivity

NTN	Non-Terrestrial Network

PCell	Primary Cell

PDCCH	Physical Downlink Control Channel

PDSCH	Physical Downlink Shared Channel

PEI	Paging Early Indication

PEI-O	Paging Early Indication-Occasion

PF	Paging Frame

PH	Paging Hyperframe

PO	Paging Occasion

PRACH	Physical Random Access Channel

PRB	Physical Resource Block

PSCell	Primary SCG Cell

PSS	Primary Synchronisation Signal

PTC	Parallel Test Component

PTW	Paging Time Window

PUCCH	Physical Uplink Control Channel

PUSCH	Physical Uplink Shared Channel

RA	Random Access

RA-SDT	Random Access-based SDT

RACH	Random Access Channel

RAR	Random Access Response

RAT	Radio Access Technology

RMSI	Remaining Minimum SI

SCell	Secondary Cell

SCG	Secondary Cell Group

SDT	Small Data Transmission

SFN	System Frame Number

SN	Secondary Node

SRS	Sounding Reference Signal

SS	System Simulator

SSB	Synchronization Signal and PBCH block

SSS	Secondary Synchronisation Signal

TC	Test Case

TDD	Time Division Duplex

UDC	Uplink Data Compression

UL	Uplink

UL-SCH	Uplink Shared Channel

USS	UE-specific Search Space

UT	Upper Tester

VNG	Virtual Noise Generator

<NEXT MODIFIED SECTION>

#### 5.2.1.7	Multi-USIM

Figure 5.2.1.7-1: Test model for Multi-USIM testing

The test model consists of 2 instances of the NR PTC and the RRC/NAS Emulator PTC.  These PTCs operate as normal.

A Multiplex PTC receives all messages from the SS and decides, based on the cell id, to which instance of the NR PTCs to forward each message.

The existing IP PTC is extended to include an additional co-ordination port to the 2nd instance of the NR PTC.

2 instances of the IMS PTC are always created: IMS_A is connected to NR5GC_A and IMS_B to NR5GC_B.

The interface to the SS remains the same.

#### 5.2.1.8	Multicast/Broadcast Service

##### 5.2.1.8.1	MBS broadcast

The MBS NR/5GC test model on NR single carrier builds on top of the NR/5GC Layer 3 test model defined in clause 5.2.1.1, with the differences specified hereafter. The MBS NR/5GC NR carrier aggregation test model builds on top of the NR carrier aggregation test model defined in clause 5.2.1.2, with the differences specified hereafter.

Figure 5.2.1.8.1-1: Test model for MBS broadcast testing on NR single carrier

Figure 5.2.1.8.1-2: Test model for MBS broadcast testing on NR carrier aggregation

The UE is configured in Test Loop Mode C to count the successfully received MBS Packets. On the UE side, Ciphering/Integrity (PDCP and NAS) are enabled on SRBs/DRB. No security is configured on MCCH/MTCH and header compression is not configured.

On the SS side, the cell is configured as a normal cell with the additional MBS parameters. L1, MAC, RLC, PDCP and SDAP are configured for MTCH in normal mode; they shall perform all of their functions.

The MRB port is above SDAP.

<NEXT MODIFIED SECTION>

# 6	System interface

## 6.1	Upper tester interface

The Upper Tester (UT) interface is the same as specified in TS 36.523-3 [12] clause 5.

In addition, the following MMI commands are defined.

Table 6.1-1: MMI commands

| Command | Parameters |  |
| --- | --- | --- |
|  | Name | Value |
| "CAPABILITY_ENQUIRY" | "Freq. Band List" | <charstring> |
|  | "Two-Way" | "TRUE" / "FALSE" |
| "CHANGE_RAC_ID" | (none) |  |
| "CLEAR_RSNPN" | (none) |  |
| "NO_PRECONDITIONS" | (none) |  |
| "PERIODICAL_ MEASUREMENT_PSBCH_RSRP" | "Report Interval" | <charstring> |
| "SET_MUSIM_UAI" | "RRC_STATE" | "idle” / "inactive" / "outOfConnected" |
|  | "STARTING_SFN" NOTE 1 | <integer> |
|  | "STARTING_SUBFRAME" NOTE 1 | <integer> |
|  | "GAP_LENGTH" NOTE 1 | “ms3” / “ms4” / “ms6” / “ms10” / “ms20” |
|  | "GAP_REPETITION" NOTE 1 | “ms20” / “ms40” / “ms80” / “ms160” / “ms320” / “ms640” / “ms1280” / “ms2560” / “ms5120” |
|  | "GAP_OFFSET" NOTE 1 | <integer> |
| "SL_CSI_RS_CONFIGURATION" | "Freq. allocation One antenna Port" | <charstring> |
|  | “First symbol" | <charstring> |
|  | "Latency Bound" | <charstring> |
| "SL_CSI_REPORT" | (none) |  |
| "SNPN_AUTOMATIC" | (none) |  |
| "SNPN_MANUAL" | "PLMN" | <PLMN ID> |
|  | "N_ID" | <N ID> |
| "SNPN_SUBSCRIBER_DATA" | "PLMN" | <PLMN ID> |
|  | "N_ID" | <N ID> |
|  | "ACCESS_ID" | "ACCESS_ID" |
| "UEAI" | "SL-QoS Flow ID" | <charstring> |
| "UNICAST_SLDRB" | "Action" | "ESTABLISH" / "MODIFY" / "RELEASE" |
| "MBS_SERVICE_INTEREST" | "Service" | <MBS Service ID> |
|  | "Interest" | "ON" / "OFF" |
| “MBS_SERVICE_ACTIVE” | "Service" | <MBS Service ID> |
|  | “Active” | "ON" / "OFF" |
| NOTE 1: These 5 parameters make up one instance of musim-GapInfo. Up to 4 instances of musim-GapInfo make up the musim-GapPreferenceList, which is itself optional. Each of these parameters is therefore also optional. |  |  |

The following AT commands are also applied in the TTCN.

Table 6.1-2: AT Commands

Command

AT+C5GNSSAI

AT+C5GNSSAIRDP

AT+C5GQOS

AT+CGBRRREQ

AT+CLCCS

AT+CMICO

AT+CNASCREL

AT+CPAGRES

AT+CPAGTCC

AT+CPSDO

AT+CREJPAG

AT commands are referred to TS 27.007 [49].

<NEXT MODIFIED SECTION>

##### 7.1.2.2.3	DL scheduling scheme

###### 7.1.2.2.3.0	Introduction

Different kinds of PDSCH transmissions need to be scheduled:

-	System information (SI: SIB1 and other system information)

-	Paging

-	Random access response (RAR)

-	DCCH/DTCH transmissions

-	MCCH/MTCH transmissions

The scheduling needs to be done so that there is no overlapping in frequency or time domain.

In general there are different ways to do the scheduling:

a)	Multiplexing in frequency domain of a single BWP

b)	Multiplexing in time domain of a single BWP (at slot or at symbol level)

c)	Use of different BWPs: e.g. initial BWP for SI, RAR, Paging + dedicated BWP for DCCH/DTCH

d)	Combinations of the above

In frequency domain the different kinds of PDSCH transmissions may use different resource allocation types (TS 38.214 [22] clause 5.1.2.2):

-	Resource allocation (RA) type 0: Bitmap-based allocation of Resource Block Groups (RBGs):
Applicable only for DCI format 1_1  not applicable for scheduling of SI, Paging and RAR

-	Resource allocation (RA) type 1, non-interleaved: continuous allocation of RBs with one-by-one mapping of virtual resource blocks (VRBs) to physical resource blocks (PRBs).

-	Resource allocation (RA) type 1, interleaved: continuous allocation of RBs with interleaved VRB-to-PRB mapping according to TS 38.211 [19] clause 6.3.1.7.

###### 7.1.2.2.3.1	DL scheduling scheme: Frequency domain multiplexing, RA type1, non-interleaved

This scheduling scheme multiplexes the different kinds of PDSCH transmissions in the frequency domain of a single BWP by exclusively using resource allocation type 1 with non-interleaved VRB-to-PRB mapping. Assuming the resource blocks being numbered from 0 to NBWP-1 (with NBWP being the size of the BWP) the following allocation is done:

Table 7.1.2.2.3.1-1: Resource allocation for frequency domain multiplexing, RA type1, non-interleaved

| Kind of PDSCH transmission (Note 1) | CORESET# | SSB index of the cell (Note 4) | Resource block allocation (Note 2) |  |
| --- | --- | --- | --- | --- |
|  |  |  | RBstart (Note 2) | LRBs,max (Note 2) |
| System information | 0 | SSB#1 | 0 | NSI |
|  |  | SSB#0 | NBWP-NSI | 7NSI |
| Paging (Note 3) | 0 | SSB#1 | NSI | NBWP-NSI |
|  |  | SSB#0 | 0 | NBWP-NSI |
| Random access response (Note 3) | 0 | SSB#1 | NSI | NBWP-NSI |
|  |  | SSB#0 | 0 | NBWP-NSI |
| CCCH and DCCH transmission on CSS (Note 3) | 0 | SSB#1 | NSI | NBWP-NSI |
|  |  | SSB#0 | 0 | NBWP-NSI |
| DCCH/DTCH transmissions (Note 3) | 1 | SSB#1 | NSI+ Offset Carrier CORESET#0 [RBs] (Note 5) | NBWP-NSI |
|  |  | SSB#0 | 0 | NBWP-NSI |
| MCCH/MTCH | 0 | SSB#1 | NSI | NBWP-NSI |
|  |  | SSB#0 | 0 | NBWP-NSI |
| NOTE 1: In context of a generic 5G test model it is not relevant whether or not there is SI and Paging for a given deployment option (e.g. EN-DC). NOTE 2: LRBs ≤ LRBs,max with LRBs: number of resource blocks being eventually used for a particular transmission. LRBs and RBstart are as specified in TS 38.214 [22] clause 5.1.2.2.2. NOTE 3: In general Paging, Random access response and CCCH/DCCH/DTCH transmissions are mutual exclusive and therefore share the same allocation. NOTE 4: The SSB index used by an NR Cell is specified in TS 38.508-1 [5] Table 4.4.2-2. NOTE 5: Offset Carrier CORESET#0 [RBs] values are specified in the frequency tables in TS 38.508-1 [5]. |  |  |  |  |

In order to achieve a test case behaviour being independent from the frequency channel bandwidth NBWP is limited to the minimum value of 24 RBs in accordance with Table 5.3.2-1 of TS 38.101-1/2 [5, 6]. This implies that LRBs,max< 24 RBs for all configurations.

NSI is the number of RBs reserved for system information, it is set to 7 RBs by default. NSI is set to 9 RBs in test cases requiring an extended SIB1 TBS size.

In general PDCCH and corresponding PDSCH transmissions are in the same slot (K0 = 0).

<NEXT MODIFIED SECTION>

### 7.1.7	Random Access procedure

Two types of NR Random Access procedure are supported: 4-step RA type with Msg1 and 2-step RA type with MSGA.

By default, according to TS 38.508-1 [5], 4-step RA type is configured, unless specified otherwise in the test case prose.

### 7.1.8	Race conditions

The same principles and rules are applied as according to clause 7.10 of TS 36.523-3 [12].

<NEXT MODIFIED SECTION>

### 7.3.8	Void

7.3.9	Multicast/Broadcast Service

7.3.9.1		Schedule transmission of MCCH messages

The rules for the transmission of MCCH messages are specified in TS 38.331 [16], clause 5.9.1. The SS shall:

-	When segmentation is not applied, TTCN provides the MBSBroadcastConfiguration message to the SS as a structured ASN.1 type using a control ASP (NR_SYSTEM_CTRL_REQ). The MCCH information is sent to SS using the ASN.1 types, SS shall encode the MBSBroadcastConfiguration ASN.1 message as specified in Table 8.1-1.

-	When segmentation is applied, TTCN provides the MCCH information as octetstring, containing the RLC data SDUs with segments of the encoded MBSBroadcastConfiguration message using a control ASP (NR_SYSTEM_CTRL_REQ). All segments shall be broadcasted in the same MCCH period at the given offsets.

-	SS shall add the necessary padding bits as specified in TS 38.331 [16], clause 8.5.

-	SS shall transmit the encoded MCCH information periodically as specified in TS 38.331 [16].

-	The scheduling information sent to SS is the same as the scheduling information sent to the UE. For each MCCH message or segment, the slotOffset list in NR_SYSTEM_CTRL_REQ indicates the exact point in time in the MCCH period at which SS shall transmit the related message or segment.

In frequency domain, the resource blocks used to broadcast MCCH information are defined in Table 7.1.2.2.3.1-1.

The MCCH message is sent only once within the MCCH transmission window and redundancy version 0 is selected. The following slot offset values defined in Table 7.3.9.1-1 are used for MCCH messages when no segmentation is applied. The slotOffset depends on the SSB index the cell is configured with.

Table 7.3.9.1-1: SlotOffset values

| Configuration | slotOffset for SSB#0 | slotOffset for SSB#1 |
| --- | --- | --- |
| FR1 FDD SCS=15kHz | 3 | 4 |
| FR1 TDD SCS=15kHz | 10 | 11 |
| FR1 TDD SCS=30kHz | 3 | 4 |
| FR2 TDD SCS=120kHz | 10 | 11 |

7.3.9.2		MCCH change notification

The MCCH change notification is used to indicate the modification of MCCH information. The MCCH change notification are 2 bits included in the DCI format 4_0, scrambled with MCCH-RNTI, used to transmit the MCCH information.

The modified MCCH information with the DCI information including the MCCH change notification bits, indicating a change, are provided in the ASP NR_SYSTEM_CTRL_REQ at the start of the modification period. SS is provided with another ASP at the start of the next modification period with the MCCH change notification bits indicating no change and with the same MCCH information.

The SFN for the start of modification period is calculated by TTCN.

7.3.9.3	MTCH data scheduling

The SS is configured with SDAP/PDCP/RLC/MAC for MTCH in normal mode.

For data transmission on MRB, the timing information and slotOffset in NR_MRB_COMMON_REQ shall indicate the exact point in time, corresponding to MTCH PDCCH occasions, at which SS shall transmit the data.

<NEXT MODIFIED SECTION>

# 11	Guidelines on test execution

## 11.1	Introduction

Clause 11 provides the guidelines on test executions.

## 11.2	Void

## 11.3	Test execution restrictions

### 11.3.1	Bands / band combinations

#### 11.3.1.1	Introduction

The restriction on test case execution is due to the number of frequencies available for the specific band under test specified in TS 38.508-1[5] and the number of frequencies used by the test cases specified in TS 38.523-1[8].

#### 11.3.1.2	EN-DC, NR/5GC, Inter-RAT

The list of test cases, that cannot be executed due to restrictions on bands and/or band combinations, is specified in Table 11.3.1.2-1.

Table 11.3.1.2-1: Restriction on bands / band combinations for test case execution

| Category of test case | Category of restriction | Bands / band combinations with restrictions | TS 38.523-1 [8] test cases that cannot be executed |
| --- | --- | --- | --- |
| EN-DC |  |  |  |
| Single NR carrier | More than one NR radio frequency (i.e., using NRf2 or NRf3 or NRf4) | DC_2A_n2A, DC_21A_n28A, DC_41A_n41A, DC_66A_n66A | 8.2.3.6.1a, 8.2.3.7.1a, 8.2.3.8.1a, 8.2.3.10.1, 8.2.3.11.1, 8.2.3.18.4 |
| Single NR carrier | More than two NR radio frequencies (i.e., using NRf3 or NRf4) | DC_(n)41AA | 8.2.3.18.4 |
| NR-5GC |  |  |  |
| Single RAT / Single NR carrier | More than one NR radio frequency (i.e., using NRf2 or NRf3 or NRf4) | n51, n91, n93 | 6.1.1.1, 6.1.1.2, 6.1.1.3, 6.1.1.5, 6.1.1.6, 6.1.1.7, 6.1.1.8, 6.1.2.3a, 6.1.2.7, 6.1.2.11, 6.1.2.13, 6.1.2.14, 6.1.2.16, 6.1.2.18, 6.1.2.20, 6.1.2.21, 6.1.2.22, 6.1.2.24, 6.1.2.25, 6.1.2.27, 6.3.1.1, 6.3.1.2, 6.3.1.3, 6.3.1.4, 6.3.1.5, 6.3.1.8, 6.3.1.9, 6.3.1.10, 6.3.2.1, 6.3.2.2, 6.3.2.3, 6.3.2.4, 6.3.2.5, 6.3.2.6, 6.3.3.1, 6.3.3.2, 6.3.3.3, 6.3.3.4, 6.4.1.1, 6.4.1.2, 6.4.2.2, 6.4.2.3, 6.5.1.2, 6.5.2.4, 6.5.2.6, 6.5.3.1, 6.5.3.4, 6.5.3.5, 6.5.3.7, 6.5.3.8, 6.5.3.9, 7.1.3.4.4, 7.1.3.6.5, 8.1.1.2.1, 8.1.1.3.1, 8.1.1.3.3, 8.1.1.3.7, 8.1.1.3.7b, 8.1.1.3.8, 8.1.1.3.9, 8.1.3.1.3, 8.1.3.1.6, 8.1.3.1.9, 8.1.3.1.11, 8.1.3.1.12, 8.1.3.1.14A, 8.1.3.1.15A, 8.1.3.1.20, 8.1.3.21, 8.1.4.1.2, 8.1.4.1.10, 8.1.4.3.4, 8.1.4.3.5, 8.1.5.10.2, 8.1.5.10.3, 8.1.5.11.1, 8.1.5.11.3, 8.1.5.11.4, 8.1.6.1.2.2, 8.1.6.1.2.10, 8.1.6.1.3.2, 8.1.6.1.3.7, 8.1.6.1.4.3, 8.1.6.1.4.7, 8.1.6.1.4.9, 9.1.4.1, 9.1.5.1.1, 9.1.5.1.2, 9.1.5.1.8, 9.1.5.1.10, 9.1.5.1.12, 9.1.5.1.14, 9.1.5.1.16, 9.1.5.2.10, 9.1.7.4, 9.1.9.3, 9.1.12.3, 11.3.5, 11.3.6, 11.3.6a, 11.3.8, 11.3.9, 11.3.9a, 11.3.11, 11.4.7, 11.4.8 |
| Single RAT / Single NR carrier | More than two NR radio frequencies (i.e., using NRf3 or NRf4) | n13, n14, n24, n30, n53 | 6.1.1.1, 6.1.1.2, 6.1.1.3, 6.1.1.5, 6.1.1.6, 6.1.1.7, 6.1.1.8, 6.1.2.7, 6.1.2.11, 6.1.2.20, 6.1.2.24, 6.1.2.25, 6.3.1.1, 6.3.1.2, 6.3.1.3, 6.3.1.5, 6.3.1.8, 6.4.1.1, 6.4.1.2, 6.4.2.2, 6.4.2.3, 6.5.2.4, 6.5.2.6, 8.1.1.3.3, 8.1.1.3.9, 9.1.5.1.2, 9.1.5.1.8 |
| Single RAT / NR CA inter-band | Switched allocation of PCell and SCell if SCell frequency has no UL | CA_n29A_n66A, CA_n29A_n70A, CA_n29A_n71A | 8.1.4.1.7.2, 8.1.4.1.9.2 |
| Single RAT / NR CA intra-band | Switched allocation of PCell and SCell if SCell frequency has no UL | CA_n5B, CA_n66B | 8.1.4.1.7.1, 8.1.4.1.9.1 |
| Single RAT / NR additional BWP without CORESET#0 | No frequency specified | n29, n51, n75, n76, n91, n93, n100, n101 | 7.1.1.8.4 |
| NR/5GC Inter-RAT |  |  |  |
| NR / E-UTRA Inter-RAT | More than one E-UTRA radio frequency (i.e., using f2 or f3 or f4) | 13, 18, 31, 54, 72, 73, 87, 88 | 6.4.3.1, 8.1.1.3.4, 11.1.4, 11.1.5. |
| NR / E-UTRA Inter-RAT | More than one NR radio frequency (i.e., using NRf2 or NRf3 or NRf4) | n51, n91, n93, n94 | 6.2.1.1, 6.2.1.2, 6.2.1.3, 6.2.1.4, 8.1.1.3.7a. |
| NR / E-UTRA Inter-RAT | More than two NR radio frequencies (i.e., using NRf3 or NRf4) | n13, n14, n24, n30, n53 | 6.2.1.1 |
| NR / EN-DC Inter-RAT | More than one NR radio frequency (i.e., using NRf2 or NRf3 or NRf4) | DC_2A_n2A, DC_21A_n28A, DC_41A_n41A, DC_66A_n66A | 8.1.4.2.1.2 |

### 11.3.2	Void

### 11.3.3	NR MFBI

The following NR MFBI test case shall be executed using the combinations specified in Table 11.3.3-1 for px_NR_OverlappingNotSupportedBand_MFBI and px_NR_PrimaryBand:

6.1.2.23.

The following NR MFBI test case shall be executed using the combinations specified in Table 11.3.3-1 for px_NR_OverlappingSupportedBand_MFBI and px_NR_PrimaryBand:

6.1.2.30.

Table 11.3.3-1: NR MFBI bands combinations

| px_NR_OverlappingNotSupportedBand_MFBI or px_NR_OverlappingSupportedBand_MFBI | px_NR_PrimaryBand (Note) |
| --- | --- |
| n2 | n25 |
| n25 | n2 |
| n38 | n41 |
| n41 | n38 |
| n77 | n78 |
| n78 | n77 |
| n257 | n258, n261 |
| n258 | n257 |
| n261 | n257 |
| Note: When px_NR_OverlappingNotSupportedBand_MFBI applies, the UE supports one or more of the listed MFBI bands and does not support at least one overlapping band. If the UE supports all overlapping bands, these test cases are not applicable. When px_NR_OverlappingSupportedBand_MFBI applies, the UE supports one or more of the listed MFBI bands and supports at least one overlapping band. If the UE does not support any overlapping bands, these test cases are not applicable. |  |