TSG SA Meeting #SP-105	SP-24xxxx

10 – 13 September 2024, Melbourne, Australia

- Title:				chapter 5.2 AI/ML related activities in TSG SA & CT Working Groups

- Source:

- Document for:				Approval

Agenda Item:				7

Work Item / Release:	FS_AIML_CAL/ Rel-19

- 1. Introduction

Chapter 5.2 AI/ML related activities in TSG SA & CT Working Groups was addressed by following documents:

| SP-241325 | 22.850: Rel-19 pCR TR 22.850 AIML related activities in 3GPP | Nokia | Rel-18 SA1 WID (AIML_MT) Rel-19 SA1 SID (FS_AIML_MT_Ph2) Rel-19 SA1 WID (AIML_MT_Ph2) Rel-18 SA2 WID (eNA_Ph3) Rel-18 SA2 WID (AIMLsys) Rel-19 SA2 SID (FS_AIML_CN) Rel-19 SA2 WID (AIML_CN) Rel-18 SA3 WID (eNA_SEC_PH3) Rel-19 SA3 SID (FS_AIML_CN_SEC) Rel-19 SA4 SID (FS_AI4Media) Rel-18 SA5 WID (AIML_MGT) Rel-19 SA5 SID (FS_AIML_MGT_Ph2) Rel-19 SA6 SID (FS_AIMLAPP) Rel-19 SA6 WID (AIML_App) |
| --- | --- | --- | --- |
| SP-241336 | 22.850: Data collection, ML Model Storage Analytics and ML Model Accuracy Monitoring in Rel-18 eNA_Ph3 | vivo | R18 Enablers for Network Automation for the 5G System (5GS) Phase 3 (eNA_Ph3) |
| SP-241338 | 22.850: AI/ML related activities on FS_PAIDC_UPF | China Telecom | FS_PAIDC_UPF |
| SP-241339 | 22.850: AI/ML related activities on eNA_Ph3 | China Telecom | eNA_Ph3 |
| SP-241347 | 22.850: AI/ML related activities in SA & CT Working Groups | Rakuten Mobile, Inc | Table introduced, but more fits under 5.1 |

- 2. Proposal

Incorporate the following change in TR 22.850.

*** Begin of Change ***

## 5.2	AI/ML related activities in TSG SA & CT Working Groups

### 5.2.X	AI/ML related activities on WID/SID#X

### 5.2.X.1	Description

Editor's Note:	This clause will investigate and identify AI/ML related activities of all TSG SA&CT working groups of Rel-18 features and Rel-19.

### 5.2.X.2	Terminology

Editor's Note:	This clause describes AI/ML related terminology (i.e. set of definitions, acronyms)

### 5.2.X.3	Activities Summary

Editor's note:	This clause describes high-level AI/ML activities e.g. LCM for AI/ML, data collection/storage/exposure, model training/delivery/ (de)-activation/inference emulation, inference/storage/exposure, performance evaluation and accuracy monitoring. Sub-clause(s) may be added to capture details.

**SP-241325-Nokia**

### 5.2.A	Rel-18 SA1 WID - AI/ML model transfer in 5GS (AIML_MT)

### 5.2.A.1	Description

The objective of this work item is to specify performance requirements (for end-to-end latency, experienced data rate, communication service availability) and service requirements (for AI/ML QoS management, AI/ML model /data distribution/transfer, network performance and resource utilization monitoring/prediction) for 5GS to support the following AI/ML operations for various applications (e.g. image/speech recognition, media editing/enhancements, robot control, automotive):

-	AI/ML operation splitting between AI/ML endpoints

-	AI/ML model/data distribution and sharing over 5G system

-	Distributed/Federated Learning over 5G system

### 5.2.B	Rel-19 SA1 SID - AI/ML Model Transfer Phase 2 (FS_AIML_MT_Ph2)

### 5.2.B.1	Description

The objective of this study is to explore new use cases and potential service and performance requirements to support efficient AI/ML operations using direct device connections. This includes:

-	Distributed AI training and inference based on direct device connections, such as traffic KPIs, various QoS, and functional requirements for sidelink transmission.

-	Considerations for charging and security aspects.

### 5.2.C	Rel-19 SA1 WID - AI/ML Model Transfer Phase 2 (AIML_MT_Ph2)

### 5.2.C.1	Description

The objective of this work item is to specify KPI and functional requirements for 5GS to support the AIML data transfer by leveraging direct device connection under 5G network control. These objectives were derived based on outcome of Rel-19 study in SA1 that relates to how the 5GS supports the transmissions of AI/ML-based services over the application layer. The study addressed use cases and potential performance requirements for 5G system support of application layer Artificial Intelligence (AI)/Machine Learning (ML) model distribution and transfer (download, upload, updates, etc.), and identified traffic characteristics of AI/ML model distribution, transfer and training for various applications, e.g. video/speech recognition, robot control, automotive, other verticals.

### 5.2.D	Rel-18 SA2 WID - Enablers for Network Automation for 5G – phase 3 (eNA_Ph3)

### 5.2.D.1	Description

The objective of this work item is to further enhance NWDAF, based on what has been specified in the previous releases to allow 5GS to support network automation. This work item focuses on architecture enhancement, new scenarios and the necessary inputs and outputs to the NWDAF based on the conclusions of the study in Rel-18. The work focuses on 10 key aspects as follows:

-	Improve correctness of NWDAF analytics

-	NWDAF-assisted application detection

-	Data and analytics exchange in roaming case

-	Enhancements on Data collection and Storage

-	Enhancements on trained ML Model sharing

-	NWDAF-assisted URSP

-	Enhancements on QoS Sustainability analytics

-	Supporting Federated Learning in 5GC

-	Enhancement of NWDAF with finer granularity of location information

-	Interactions with MDAS/MDAF

**SP-241336-vivo**

### 5.2.X	AI/ML related activities on WID# R18 Enablers for Network Automation for the 5G System (5GS) Phase 3

### 5.2.X.1	Description

Editor's Note:	This clause will investigate and identify AI/ML related activities of all TSG SA&CA working groups of Rel-18 features and Rel-19.

In Rel-15, Rel-16, and also Rel-17 the framework and solutions of eNA had been investigated for supporting network automation leveraging 5GC information exposure and network data analytics. The eNA_Ph3 aims at further enhancements for NWDAF, new cases and even Rel-17 leftover.

The issues deprioritized or unsolved from R17 includes how to improve correctness of NWDAF analytics, how NWDAF could assist application detection and how to support data and analytics exchange in roaming case.

To further enhance the features in R17, the eNA_Ph3 investigate the enhancement on Data collection and Storage, the enhancement on training ML Model sharing among different vendors, how NWDAF could assist URSP and the enhancement on QoS sustainability analytics.

The eNA_Ph3 also focus on new scenarios, such as supporting Federated Learning in 5GC, the enhancement of NWDAF with finer granularity of location information and how NWDAF interact with MDAS/MDAF.

### 5.2.X.2	Terminology

Editor's Note:	This clause describes AI/ML related terminology (i.e. set of definitions, acronyms)

This clause describes AI/ML related terminology (i.e. set of definitions, acronyms)

Abbreviations:

AI	Artificial Intelligence

ADRF	Analytics Data Repository Function

AnLF	Analytics Logical Function

DCCF	Data Collection Coordination Function

DCAF	Data Collection Application Function

FL	Federated Learning

RE-NWDAF	Roaming Exchange Network Data Analytics Function

ML	Machine Learning

MFAF	Messaging Framework Adaptor Function

MTLF	Model Training Logical Function

NWDAF	Network Data Analytics Function

Definitions:

Analytics logical function (AnLF): A logical function in NWDAF, which performs inference, derives analytics information (i.e. derives statistics and/or predictions based on Analytics Consumer request) and exposes analytics service.

Model Training logical function (MTLF): A logical function in NWDAF, which trains Machine Learning (ML) models and exposes new training services.

Analytics Accuracy Information: Represent a performance measure of an analytics ID provided by an NWDAF containing AnLF, which is composed of the number of correct predictions of the analytics ID out of all predictions and the corresponding number of samples.

ML Model Accuracy Information: Represent a performance measure of a ML Model provided by an NWDAF containing MTLF, which is composed of the number of correct predictions by the ML Model out of all predictions and the corresponding number of samples.

Analytics Feedback Information: Indicates that the consumer NF has taken action(s) influenced by the previously provided analytics, which may or may not affect the ground truth data.

Federated learning: a machine learning technique in core network that trains an ML Model across multiple decentralized entities holding local data set, without exchanging/sharing local data set.

Analytics/ML Model Accuracy Monitoring: achieved by comparing the predictions using the current trained ML Model and its corresponding ground truth data i.e. the corresponding true observed events.

Analytics Feedback Information: indicates that the consumer NF has taken an action(s) influenced by the previously provided analytics, which may or may not affect the ground truth data corresponding to analytic ID requested at the time which the prediction refers to, and consequently affect the ML Model Accuracy Monitoring.

### 5.2.X.3	Activities Summary

Editor's note:	This clause describes high-level AI/ML activities e.g. LCM for AI/ML, data collection/storage/exposure, model training/delivery/ (de)-activation/inference emulation, inference/storage/exposure, performance evaluation and accuracy monitoring. Sub-clause(s) may be added to capture details.

Data collection

Figure 5.2.X.3: Data Delivery via DCCF or MFAF

Each Event Notification received from a Data Source NF is sent to the DCCF which propagates it to all Data Consumers / Notification Endpoints specified by the Data Consumers or determined by the DCCF. Each Data Consumer may specify in its request to the DCCF multiple notification endpoints, which may include the requesting Data Consumer, an ADRF or other NFs. The DCCF may also select an ADRF or other notification endpoint based on configuration. The DCCF supports formatting and processing for each Consumer / notification endpoint so notifications comply with the data requests received from each Consumer NF.

The Messaging Framework formats and processes data received from the Data Source NF and sends notifications to all Data Consumers and Notification Endpoints specified by Data Consumers or determined by the DCCF. Each Data Consumer may specify in its request to the DCCF multiple notification endpoints, which may include the requesting Data Consumer, an ADRF or other NFs. The DCCF may also select an ADRF or other notification endpoint based on configuration. While the Messaging Framework is not standardized by 3GPP, a Messaging Framework Adaptor NF (MFAF) offers 3GPP defined services that allow the 5GS to interact with the Messaging Framework.

ML Model Storage

The ADRF offers services that enable a consumer to store, retrieve and delete data and analytics in Rel-17. The ADRF offers services that enable a consumer to store, delete and retrieve ML Models in Rel-18.

ML Model(s) may be stored in the ADRF by a consumer sending the ADRF a storage request containing the ML Model or ML Model address to be stored. The ADRF response provides a result indication.

ML Model(s) may be retrieved from the ADRF by a consumer sending the ADRF a retrieval request containing one or more tuples of unique ML Model identifier stored in ADRF or Storage Transaction Identifier. The ADRF response provides one or more tuples of unique ML Model identifiers and ML Model file address of ML Model file stored in ADRF.

Analytics/ML Model Accuracy Monitoring

Input data is collected from Data Producer NF(s) when there is a request for inference/prediction per analytics ID in NWDAF for a specific time period in future. Ground truth data are collected from those Data Producer NF corresponding to the requested analytic ID at the time to which the prediction refers.

The NWDAF (containing AnLF/MTLF) with accuracy checking capability decides to initiate Analytics Accuracy Monitoring based on:

-	A request from an analytics accuracy consumer. The analytics accuracy consumer may be an NWDAF containing AnLF, NWDAF containing MTLF or an analytics consumer NF.

-	Analytics Feedback Information which may be provided by an Analytics Consumer NF.

**end-SP-241336-vivo

**SP-241339-China Telecom**

- 5.2.X	AI/ML related activities on eNA_Ph3

- 5.2.X.1	Description

The eNA_ph3 work item in Rel-18 focused on enhancing network automation within the 5G system, building upon the work done in Rel-15 and Rel-16, and further exploring architecture enhancements, new cases, and leftover issues from Rel-17.

Deprioritized issues and unresolved questions from Rel-17 were investigated, including studying possible mechanisms for improving the correctness of NWDAF analytics, how NWDAF could assist in application detection without degrading UPF performance and how to support data and analytics exchange in roaming scenarios.

Several Rel-17 features were further enhanced in this work item which included data collection and storage enhancements, such as improvements to DCCF and ADRF, investigation on how to enhance the sharing of trained ML models among different vendors, with necessary coordination with SA5 for AI/ML management, data reporting from the UPF to the NWDAF to support analytics , interactions between NWDAF leveraging MDAS/MDAF functionality and enhancement on analytics for QoS sustainability with finer granularity and additional input data.

This WID also focused on enhancing architecture and new use cases, studying the possibility of enhancing the architecture to support federated learning in the 5GC and considering NWDAF enhancements for location information with finer granularity than TA and cell level.

- 5.2.X.2	Terminology

Acronyms

AI	Artificial Intelligence

ADRF	Analytics Data Repository Function

AnLF	Analytics Logical Function

DCCF	Data Collection Coordination Function

DCS	Data Synthesis and Compression

FL	Federated Learning

KPI	Key Performance Indicator

ML	Machine Learning

MFAF	Messaging Framework Adaptor Function

MTLF	Model Training Logical Function

NWDAF	Network Data Analytics Function

Terms

Analytics logical function (AnLF): A logical function in NWDAF, which performs inference, derives analytics information (i.e. derives statistics and/or predictions based on Analytics Consumer request) and exposes analytics service i.e. Nnwdaf_AnalyticsSubscription or Nnwdaf_AnalyticsInfo.

Analytics Accuracy Information: Represent a performance measure of an analytics ID provided by an NWDAF containing AnLF, which is composed of the number of correct predictions of the analytics ID out of all predictions and the corresponding number of samples.

Analytics Feedback Information: Indicates that the consumer NF has taken action(s) influenced by the previously provided analytics, which may or may not affect the ground truth data.

ML Model Accuracy Information: Represent a performance measure of a ML Model provided by an NWDAF containing MTLF, which is composed of the number of correct predictions by the ML Model out of all predictions and the corresponding number of samples.

Model Training logical function (MTLF): A logical function in NWDAF, which trains Machine Learning (ML) models and exposes new training services (e.g. providing trained ML Model) .

- 5.2.X.3	Activities Summary

- 5.2.X.3.1 Data collection, storage and exposure

In this work item, data collection and storage were enhanced based on eNA-ph2 in Rel-17, including DCCF and ADRF enhancements, e.g., DCCF relocation, ADRF selection, ML model storage. Data and analytics exchange in roaming case which had no conclusion from the previous phase was also included.

ML model storage：

NWDAF containing AnLF can retrieve the models directly from an NWDAF containing MTLF, this service is already available in Rel-17. In this work item, ADRF was suggested as an intermediate storage of models.

MTLF can store ML model in ADRF based on MTLF policy. MTLF requests to store a (set of) ML Model(s) to the ADRF, optionally including allowed NF instance list for the ML Model identifier as described in TS 33.501. The ADRF locally maintains the association between the ML Model identifier and the NF instance ID of NWDAF containing MTLF and allowed NF instance list.

Analytics Exposure in Roaming Case:

Based on roaming agreements (e.g. SLA), analytics may be exchanged between PLMNs (i.e. HPLMN and VPLMN of a UE served by the NWDAF analytics consumer). In this case, an NWDAF with roaming exchange capability (RE-NWDAF) is used as entry point in a PLMN to exchange analytics in roaming scenario with other PLMNs. It authorizes the analytics request according to roaming agreements, and filters the information exposed to other PLMNs.

Data collection coordination and DCCF relocation:

DCCF introduced in Rel-17 coordinates the collection and distribution of data requested by NF consumers, preventing data sources from having to handle multiple subscriptions for the same data and send multiple notifications containing the same information due to uncoordinated requests from data consumers. The Data consumer may use an NRF to perform NF discovery and selection to find a DCCF that can coordinate data collection.

Due to e.g. UE mobility, the source DCCF or MFAF may no longer be able to serve the UE. In such cases, the data consumer may select a new DCCF, or source DCCF may execute the DCCF relocation procedure. The data consumer may indicate in the subscription request whether the DCCF should execute the relocation procedure or send a notification to the data consumer as specified in the reselection procedure. If the data consumer does not send an indicator in the subscription request allowing DCCF relocation initiated by the DCCF the consumer may execute DCCF reselection based on internal logic.

ADRF/NWDAF data management enhancement:

When a consumer requests data or analytics to be stored in an ADRF, it may specify Storage Handling information requesting:

-	a lifetime indicating how long the data or analytics should be stored; and/or

-	that a notification alerting the consumer be sent prior to data deletion from the ADRF.

The ADRF, DCCF or NWDAF may be configured with default operator storage policies. The policies specify how long data or analytics are to be stored and conditions when the default policy supersedes Storage Handling Information provided in a request from the Data or Analytics Consumer. Based on the default operator policy and the Storage Handling information, the ADRF, DCCF or NWDAF determines the Storage Approach that will be applied for the data or analytics.

The consumer may include in the storage request the DataSetTag attribute which the data records are to be associated to when stored by ADRF. Data records can be associated to multiple DataSetTag attributes. The DataSetTag may be included in the service requests to AnLF and MTLF to indicate the data set to be used by the AnLF, respectively by MTLF.

Data can be stored at and retrieved from ADRF using techniques of data compression and/or data synthesis. The consumer may include in the storage request the Data Synthesis and Compression (DSC) information, indicating the usage and information of data synthesis and/or compression technique. The detail of DSC information is up to implementation.

The NWDAF may decide to reduce the amount of data collected to reduce signalling load, by either prioritizing requests received from analytics consumers, or reducing the extent (e.g. duration, scope) of data collection, or modifying the sampling ratios.

- 5.2.X.3.2 Model sharing

Based on the work done in Rel-17, ML Model sharing is enhanced in this release to support model sharing between different vendors.

Parameters enabling trained ML model sharing :

Interoperability indicator is added to enable ML model sharing between different vendors:

-	Interoperability indicator which comprises a list of NWDAF providers (vendors) that are allowed to retrieve ML Models from this NWDAF containing MTLF. It also indicates that the NWDAF containing MTLF supports the interoperable ML Models requested by the NWDAFs from the vendors in the list.

ML model sharing in FL:

Model sharing between NWDAF instances is a prerequisite for FL operation. During FL procedure,  FL servers provides initial ML model information to FL clients and exchanges interim ML model information with clients.

- 5.2.X.3.3 Performance evaluation and accuracy monitoring

A NWDAF may have the accuracy checking capability for Analytics and/or ML Models. The NWDAF may provide the accuracy information to consumers when requested or use it for its internal processes. Analytics/ML Model Accuracy Monitoring is to be achieved by comparing the predictions using the current trained ML Model and its corresponding ground truth data i.e. the corresponding true observed events.

Input for accuracy check:

The AnLF with analytics accuracy checking capability is able to determine Analytics Accuracy Information based on e.g.:

-	Comparing predictions and its corresponding ground truth data, which are collected corresponding to the requested analytic ID at the time which the prediction refers to.

-	Comparing changes in internal configuration for the analytics ID generation (e.g. change of data collection parameters, change in data distribution from a Data Source).

-	Previous existent records of Analytics Accuracy Information.

-	Accuracy Feedback Information provided by an NF consumer.

-	Determining analytics accuracy by comparing analytics accuracy using multiple ML Models that serve the same Analytics ID.

The MTLF with ML Model accuracy checking capability is able to determine ML Model degradation based on e.g.:

-	comparing/evaluating the data: including input data, analytics output and the ground truth data either collected from various data source NFs, DCCF, AnLF, ADRF or configured by OAM;

-	or AnLF providing notifications of the Analytics Accuracy Information; or

-	AnLF providing Analytics Feedback Information of the analytics generated by the ML Model.

Triggers of performance check:

The NWDAF (containing AnLF/MTLF) with accuracy checking capability decides to initiate Analytics Accuracy Monitoring based on:

-	A request from an analytics accuracy consumer. The analytics accuracy consumer may be an NWDAF containing AnLF, NWDAF containing MTLF or an analytics consumer NF.

-	Analytics Feedback Information which may be provided by an Analytics Consumer NF.

Actions after accuracy check:

The AnLF with analytics accuracy checking capability is able to provide or notify the accuracy information of Analytics IDs to the analytics consumers of such service and when the analytics accuracy does not meet the analytics consumer's requirements, the analytics consumer may stop using analytics for a period of time or obtain new analytics. In addition, updated analytics for the provided Analytics IDs may be provided to analytics consumers as requested, if the updated analytics is able to be generated within the correction time period.

The NWDAF containing MTLF may reselect a new ML Model or retrain the existing ML Model and consequently notify the ML Model accuracy degradation to the ML Model consumer(s). In addition, the NWDAF containing MTLF may consider the rating of untrusted AF(s) when used as data sources.

Accuracy monitoring in FL:

The ML Model Accuracy threshold can be used to indicate the target ML Model Accuracy of the training process and the FL server NWDAF may stop the training process when the ML Model Accuracy threshold is achieved during the training process.

**end-SP-241339-China Telecom**

**SP-241325-Nokia**

### 5.2.E	Rel-18 SA2 WID - System Support for AI/ML-based Services (AIMLsys)

### 5.2.E.1	Description

This work item implements the conclusions of the Rel-18 study on the 5GS architectural and functional extensions to enable 5GS to assist the Application AI/ML operations. The normative text will be defined based on the agreed conclusions on 6 key issues, including unfinished discussions on proposals on how to define the aspects that are left to be finalized during normative work, and ensuring consistency with other 5GS features. The agreed conclusions focus on the following aspects:

-	Monitoring of network resource utilization to support the Application AI/ML operations

-	Expsoure of 5GC information to authorized 3rd party for Application AI/ML operations

-	Enhancement of external parameter provisioning in 5GC to assist the Application AI/ML operations

-	Enhancement in 5GC to enable Application AI/ML traffic transport

-	Enhancement of QoS and Policy control to support Application AI/ML data transport over 5GS

-	5GS assistance to federated learning operation

### 5.2.F	Rel-19 SA2 SID - Core Network Enhanced Support for Artificial Intelligence (AI)/Machine Learning (ML) (FS_AIML_CN)

### 5.2.F.1	Description

The aim of this study is to investigate and identify potential architectural and system-level enhancements to support AI/ML enhancements. Specifically, the objectives include:

- AI/ML Cross-Domain Coordination Aspects: Investigate enhancements to support AI-enabled RAN based on the conclusions of the RAN study in [a]. This task will discuss whether and how to support cross-domain (i.e., UE, RAN, 5GC, OAM, and AF) collaborative AI/ML mechanisms for the aspects described below.

  - Enhancements to LCS for AI/ML-Based Positioning: Examine whether and how to consider enhancements to LCS to support AI/ML-based positioning.

- Collaborative AI/ML Operations for Vertical Federated Learning (VFL): Determine potential enhancements needed to enable the 5G system to assist in collaborative AI/ML operations involving 5GC/NWDAF and/or AF for “Vertical Federated Learning (VFL).” This work will be based solely on and limited to the scope of justified use cases.

- Enhancements to Support NWDAF-Assisted Policy Control and Address Network Abnormal Behavior:

  - Investigate additional support needed to enhance 5GC NF operations (i.e., policy control and QoS) assisted by NWDAF. This task will first identify specific use cases to define the appropriate scope. It will analyze the impacts on NWDAF (e.g., the need to understand specific NF functionality) and the compatibility of new solutions with existing analytics to determine the necessity and benefits of new solutions.

  - Study the prediction, detection, prevention, and mitigation of network abnormal behaviors, such as signaling storms, with the assistance of NWDAF.

### 5.2.G	Rel-19 SA2 WID - Core Network Enhanced Support for Artificial Intelligence (AI)/Machine Learning (ML) (AIML_CN)

### 5.2.G.1	Description

The objective of this work item is to specify the following enhancements to 5GS as per the conclusions reached within the Rel-19 study for the following aspects:

- Enhancements to LCS to Support Direct AI/ML-Based Positioning

- LMF Enhancements: The LMF will be enhanced to perform location calculations based on an ML model. The triggers for data collection and model training within the LMF will be implementation-specific.

- MTLF and LMF Enhancements: Both the MTLF and the LMF will be enhanced to facilitate ML model training for AI/ML-based positioning.

- Procedure Development: Related procedures for data collection will be developed in coordination with RAN WGs.

- 5GC Support for Vertical Federated Learning

  - 5GC Enhancements: The 5GC will be enhanced to support vertical federated learning (VFL), a technique that does not involve exchanging or sharing local datasets or ML models, in the following scenarios:

-	VFL among NWDAFs within a single PLMN.

-	VFL between NWDAF(s) within a single PLMN and AF(s).NWDAF-Assisted Policy Control and QoS Enhancement

- NWDAF-Assisted Policy Control and QoS Enhancement

  - Assistance Information: Based on PCF requests, the NWDAF may provide assistance information to the PCF to aid in the determination and modification of QoS parameters.

- NWDAF Enhancements to Support Network Abnormal Behaviours Mitigation and Prevention

  - Signalling Storm Mitigation: The NWDAF will support signalling storm mitigation and prevention by providing analytics related to the detection and prediction of signalling storms caused by massive signalling from UEs and/or NFs.

### 5.2.H	Rel-18 SA3 WID - Security aspects of enablers for Network Automation for 5G - phase 3 (eNA_SEC_PH3)

### 5.2.H.1	Description

The main objective of this work is to produce normative specification based on the conclusions from Rel-18 study.  More specifically, the following objectives are expected to be specified:

-	Protection of data and analytics exchange in roaming case

-	Security for AI/ML model storage and sharing

-	Authorization of selection of participant NWDAF instances in the Federated Learning group

### 5.2.I	Rel-19 SA3 SID - Security aspects of Core Network Enhanced Support for AIML (FS_AIML_CN_SEC)

### 5.2.I.1	Description

The objectives of this study are the following:

- Security Aspects on Enhancements to LCS: Study security aspects on enhancements to LCS to support AI/ML-based positioning, considering the conclusions in [a] and [b].

- Security Aspects of Cross-Domain Vertical Federated Learning (VFL):

  - Authorization of VFL Group Members: Examine the authorization of members of the VFL group.

  - Security Aspects of Enhancements on SA2 Architecture: Investigate security aspects of enhancements on SA2 architecture to support VFL.

### 5.2.J	Rel-19 SA4 SID - Artificial Intelligence (AI) and Machine Learning (ML) for Media (FS_AI4Media)

### 5.2.J.1	Description

The primary objective of this study item is to identify relevant interoperability requirements and implementation constraints of AI/ML in 5G media services. The specific objectives include:

- Use Cases for Media-Based AI/ML Scenarios: List and describe the use cases for media-based AI/ML scenarios, based on those defined in [c].

- Media Service Architecture and Service Flows: Describe the media service architecture and relevant service flows for the scenarios. Identify the impacts on the architecture for each use case, including any potential gaps with existing 5G media service architectures. Also, describe the model operation configurations for each use case, including split AI/ML operations, and identify where certain AI/ML operations occur.

- Data Formats and Protocols: Identify and document the available data formats and suitable protocols for the exchange of different data components of various AI/ML models, such as model data, metadata, media data, and intermediate data necessary for such model operation configurations. Investigate the data traffic characteristics of these data components for delivery over the 5G system, including any needs and potentials for data rate reduction.

- Key Performance Indicators (KPIs): Identify and study key performance indicators for such scenarios, based on the initial considerations in [d]. Emphasize the use cases, model operation configurations, and data components identified in earlier objectives, focusing on objective performance metrics considering the identified KPIs.

- Normative Work and Collaboration: Identify potential areas for normative work as the next phase. Communicate and align with SA2 and other potential 3GPP working groups on relevant aspects related to the study.

### 5.2.K	Rel-18 SA5 WID - AI/ML management (AIML_MGT)

### 5.2.K.1	Description

The objective of this work is to specify the AI/ML management capabilities, including use cases, requirements and solutions for each phase of the AI/ML operational workflow for managing the AI/ML capabilities in 5GS (i.e., management and orchestration, 5GC and NG-RAN), including:

- Management capabilities for ML training phase, which includes control of producer-initiated ML training, data management for ML training, performance evaluation for ML training, ML entity validation, ML context management, ML entity capability discovery and ML entity testing

- Management capabilities for ML deployment phase, including management of ML entity loading

- Management capabilities for AI/ML inference phase.

To describe the deployment scenarios of the AI/ML management capabilities, with consideration of alignment with other relevant 3GPP WGs (e.g., RAN3, SA2) and ETSI ISG ZSM.

### 5.2.L	Rel-19 SA5 SID - AI/ML management - phase 2 (FS_AIML_MGT_Ph2)

### 5.2.L.1	Description

The objectives of this study item include:

- Continuation of AI/ML Studies: Continue the study on AI/ML emulation, AI/ML inference coordination, and ML knowledge transfer that are leftover from Rel-18.

- Management Aspects of AI/ML Functionalities Defined by Other 3GPP WGs:

  - AI/ML Model Transfer in 5GS: Study the management aspects (LCM, CM, and PM) of AI/ML model transfer in 5GS, as defined in SA1.

  - 5GS Support for AI/ML-Based Services: Investigate the management aspects of 5GS support for AI/ML-based services, as defined in SA2.

  - Support for AI/ML Services at Application Enablement Layer: Examine the management aspects of support for AI/ML services at the application enablement layer, as defined in SA6.

- Management Aspects of AI/ML Functionalities Defined by 3GPP SA5 WG:

  - Management Data Analytics (MDA) Phase 3: Study the management aspects (LCM, CM, and PM) of AI/ML functionalities defined by 3GPP SA5 WG, including MDA phase 3.

- AI/ML Management and Operation Capabilities: Investigate the AI/ML management and operation capabilities to support different types of AI/ML technologies needed for AI/ML in 5GS, such as Federated Learning, Reinforcement Learning, Online and Offline Training, Distributed Learning, and Generative AI.

- Sustainability Aspects of AI/ML:

  - Energy Consumption/Efficiency Impacts: Evaluate the energy consumption and efficiency impacts associated with AI/ML solutions for all operational phases (training, emulation, deployment, inference).

- Trustworthiness Aspects Related to AI/ML Functionalities in 5GS:

  - Concept of Trustworthiness: Further study the concept of trustworthiness for AI/ML in the context of OAM.

  - Data for Trustworthiness Indicators: Identify and analyze data (e.g., measurements, events) to support the calculation of trustworthiness indicators.

### 5.2.M	Rel-19 SA6 SID - Application layer support for AI/ML services (FS_AIMLAPP)

### 5.2.M.1	Description

The objective of this study is to enable support for AI/ML services at the application enablement layer. This includes the following:

- Analysis of Rel-18 and Rel-19 Requirements: Analyze the requirements in [d] related to AI/ML model distribution, transfer, and training. Identify key issues and develop corresponding architectural requirements at the application enablement layer, along with potential enhancements to the application layer architecture.

- Architectural and Functional Implications: Study the architectural and functional implications on existing SA6 application enablers (e.g., ADAES, other SEAL services, EDGEAPP) for supporting AI/ML lifecycle operations. This includes operations such as data collection, data preparation, training, inference, and federated learning for ML models used in ADAE layer analytics.

- Potential Solutions and APIs: Identify potential solutions, including information flows and developer-friendly application enablement APIs, to satisfy the architectural requirements and enhancements identified in the previous points.

- Impact on Deployments and Business Models: Investigate the possible impacts of application layer support for AI/ML services on different deployments and business models.

### 5.2.M	Rel-19 SA6 WID - Application enablement for AI/ML services (AIML_App)

### 5.2.M.1	Description

The objectives of this work include the following:

Develop Stage 2 normative technical specification for AIML enablement service as a new SEAL service, based on the key issues, architecture, solutions, and conclusions captured in [e]. The Stage 2 normative technical specification will include the following aspects:

-	Architecture requirements, deployment models and application architecture for AIML service enablement over 3GPP networks.

-	Procedures, information flows and APIs supporting concluded solutions related to AIML enablement capabilities for AI/ML, FL (e.g., Vertical FL among VAL UEs, Horizontal FL), Transfer Learning. Such capabilities include:

- Support AIML client management (e.g., registration, discovery) and selection.

- Support AIML service lifecycle management aspects (e.g., training, inference, data management).

- Support AIML operation split and ML model distribution operations.

- Support AIML operations in edge / distributed deployments.

-	Procedures information flows and APIs supporting concluded solutions for application layer support capabilities related to new ADAE analytics services. Such new analytics services include:

- DN Energy Analytics.

- Analytics for supporting FL.

Identify potential enhancements to other enablement frameworks (e.g. SEAL, EGDEAPP and CAPIF) based on the specified solutions for the above objectives.

** end SP-241325-Nokia**

**SP-241338-China Telecom**

- 5.2.X	AI/ML related activities on FS_PAIDC-UPF

- 5.2.X.1	Description

In Rel-18, the UPF offers services to the NEF, AF, SMF, NWDAF, DCCF, MFAF via the Nupf service based interface for data collecting in AI/ML related activities. In Rel-19, CT4 is studying “Protocol for AI Data Collection from UPF”, which aims at studying UPF data Collection for AI/ML and whether alternative protocols, or enhancements to the existing SBI protocol, are needed to optimize the AI/ML data collection while ensuring secure, scalable and reliable data transfers across the core network identifies.

- 5.2.X.2	Terminology

AF	Application Function

DCCF	Data Collection Coordination Function

MFAF	Messaging Framework Adaptor Function

NEF	Network Exposure Function

NWDAF	Network Data Analytics Function

UPF	User Plane Function

SMF	Session Management Function

- 5.2.X.3	Activities Summary

Three modes have been defined for collecting AI/ML related data from the UPF:

-	Mode 1: NF consumer subscribes the UPF event (e.g. QoS Monitoring) via SMF using the PFCP protocol via N4 interface. The AI/ML related consumers could be NEF, AF, NWDAF, DCCF, MFAF.

-	Mode 2: NF consumer subscribes the UPF event (e.g. User Data Usage Measurement, User Data Usage Trends) via SMF using the HTTP protocol via the Nupf SBI interface. The AI/ML related consumers could be NWDAF, DCCF, MFAF, and the target could be a specific UE or a group of UEs; or any UE with AoI, BSSID/SSID or DNAI information.

-	Mode 3: NF consumer subscribes the UPF event (e.g. User Data Usage Measurement, User Data Usage Trends) directly using the HTTP protocol via the Nupf SBI interface. The AI/ML related consumers could be NWDAF, DCCF, MFAF, and the target could be Target to any UE without any AoI, BSSID/SSID or DNAI information.

In these three modes, after the UPF receives the subscription, the UPF uses SBI based protocol to send notification to the NF consumer.

Considering AI/ML adoption grows for 5G use cases like network automation, analytics and others, which results in massive network signalling messages, the potential optimisations on collecting information from UPF, i.e. identifying and lowering the network performance impacts of intensive data collection from UPF, is being studied Rel-19 as described in TR 29.889.

Furthermore, for different application traffic and different use cases, different protocols may be used to request the UPF to deliver those event reports to the relevant NF service consumer in more efficient ways. Only SBI based protocol is used to report the notification data in Rel-18 as described in clause 5.2.2.3 of TS 29.564 while other protocols and corresponding protocol selection are being studied in Rel-19 as described in TR 29.889.

**end-SP-241338-China Telecom**

**SP-241234-SA5**

- 5.2.X	AI/ML related activities in TSG SA5

- 5.2.X.1	Description

Editor's Note:	This clause will investigate and identify AI/ML related activities of all TSG SA working groups of Rel-18 features and Rel-19.

- 5.2.X.2	Terminology

Editor's Note:	This clause describes AI/ML related terminology (i.e. set of definitions, acronyms)

The following terms and definitions have been developed and documented in Rel-18 TS 28.105 [x2].

Start of quoted text (From TS 28.105)

ML model: a manageable representation of an ML model algorithm.

NOTE 1:	an ML model algorithm is a mathematical algorithm through which running a set of input data can generate a set of inference output.

NOTE 2:	ML model algorithm is proprietary and not in scope for standardization and therefore not treated in this specification.

NOTE 3:	ML model may include metadata. Metadata may include e.g. information related to the trained model, and applicable runtime context.

ML model training: a process performed by an ML training function to take training data, run it through an ML model algorithm, derive the associated loss and adjust the parameterization of that ML model iteratively based on the computed loss and generate the trained ML model.

ML model initial training: a process of an initial version of an ML model.

ML model re-training: a process of training a previous version of an ML model and generate a new version.

NOTE 4:	a new version of a trained ML model supports the same type of inference as the previous version of the ML model, i.e., the data type of inference input and data type of inference output remain unchanged between the two versions of the ML model, but parameter values might be different for the re-trained model.

ML model joint training: a process of training a group of ML models.

ML training function: a logical function with ML model training capabilities.

ML model testing: a process of testing an ML model using testing data.

ML testing function: a logical function with ML model testing capabilities.

AI/ML inference: a process of running a set of input data through a trained ML model to produce set of output data, such as predictions.

NOTE 5:	the inference represents the process to realize the AI capabilities by utilizing a trained ML model and other AI enablers if needed, hence the AI/ML prefix is used when referring to inference as compared to training and testing.

AI/ML inference function: a logical function that employs trained ML model(s) to conduct inference.

AI/ML inference emulation: running the inference process to evaluate the performance of an ML model in an emulation environment before deploying it into the target environment.

ML model deployment: a process of making a trained ML model available for use in the target environment.

End of quoted text (From TS 28.105)

##### 5.2.X.2.1	Observations and analyses: terminology

- Developing a unified set of terms for AI/ML specifications forms the baseline for defining unified 3GPP AI/ML framework and help towards preventing subsequent duplication and inconsistency of the work among the 3GPP WGs.

- The terms documented in TS 28.105[x2] could be considered as guideline terms, with all or at least a subset of them can serving as a baseline by the 3GPP WGs.

- The 3GPP WGs may of course still define additional AI/ML related terms if deemed necessary.

- Based on the above, it is recommended that the 3GPP adopts a harmonised set of AI/ML-related terminologies as much as possible, taking into consideration the definitions introduced by SA5 in TS 28.105 [x2]. Effort should be made to avoid developing different definitions for the same terms, unless necessary to address the specific needs of AI/ML domain-specific specifications being developed by different WGs.

- 5.2.X.3	ML model life cycle management (LCM)

Rel-18 specification [x2] addressed the AI/ML LCM management capabilities (including wide range of use cases, corresponding requirements (stage 1) and solutions (stage 2 NRMs & stage 3 OpenAPIs) for the ML model, including ML model training (which also includes validation), testing, AI/ML inference emulation, deployment and AI/ML inference steps of the lifecycle as shown below for managing the entire lifecycle of the ML model:

Start of quoted text (from TS 28.105)

- 4a.0	ML model lifecycle

AI/ML techniques are widely used in 5GS (including 5GC, NG-RAN, and management system), the generic AI/ML operational workflow in the lifecycle of an ML model, is depicted in Figure 4a.0-1.

Figure 4a.0-1: ML model lifecycle

The ML model lifecycle includes training, testing, emulation, deployment, and inference. These steps are briefly described below:

-	ML model training: training, including initial training and re-training, of an ML model or a group of ML models. It also includes validation of the ML model to evaluate the performance when the ML model performs on the training data and validation data. If the validation result does not meet the expectation (e.g., the variance is not acceptable), the ML model needs to be re-trained.

-	ML model testing: testing of a validated ML model to evaluate the performance of the trained ML model when it performs on testing data. If the testing result meets the expectations, the ML model may proceed to the next step If the testing result does not meet the expectations, the ML model needs to be re-trained.

-	AI/ML inference emulation: running an ML model for inference in an emulation environment. The purpose is to evaluate the inference performance of the ML model in the emulation environment prior to applying it to the target network or system.

NOTE:	The AI/ML inference emulation is considered optional and can be skipped in the AI/ML operational workflow.

-	ML model deployment: ML model deployment includes the ML model loading process (a.k.a. a sequence of atomic actions) to make a trained ML model available for use at the target AI/ML inference function.

ML model deployment may not be needed in some cases, for example when the training function and inference function are co-located.

-	AI/ML inference: performing inference using a trained ML model by the AI/ML inference function. The AI/ML inference may also trigger model re-training or update based on e.g., performance monitoring and evaluation.

End of quoted text

##### 5.2.x.3.1	Observations and analyses: AI/ML LCM

- The AI/ML workflow defined by SA5 [x2] represents a general framework encapsulating the various life cycle management (LCM) operations for ML model (i.e., model training, testing, emulation, deployment, and inference).

- The AI/ML LCM capabilities defined by SA5 for each of the operational steps are generic for managing of 3GPP system including the Management and orchestration, CN and RAN domains.

- It is important to recognise that “domain-specific” ML model life cycle related tasks can be developed for the specific domains by the relevant 3GPP WGs, e.g., the RAN WGs can specify data collection within the RAN domain needed to train the UE-side, network-side, or the two-sided UE/network ML models, and specific LCM operations for UE-side model over air-interface.

- While ML model and AI/ML inference function life cycle can be specified by the relevant 3GPP WG for the specific domain (i.e., RAN, CN or Management & Orchestration), the “management aspects” of life cycle (i.e., life cycle management) remains to be primarily a “management task” that falls within the responsibility of SA5.

- The ML models and the associated “Life Cycle” can be a use case and/or domain specific, the management of the Life Cycle (i.e., LCM) is a higher layer task which is typically a role of the OAM that encompasses the process of e.g., the governance, automation, and operational practices applied to the entire AI/ML lifecycle. It is therefore imperative to distinguish the difference between Life Cycle and Life Cycle Management.

- Where feasible, the ML model LCM workflow and associated management capabilities specified by SA5 in TS 28.105 [x2] could be considered by 3GPP for the currently ongoing and future relevant specification development. The 3GPP WG(s) should potentially provide AI/ML LCM-related requirements, if any, to SA5 to avoid duplication and contention of effort.

NOTE: SA5 Rel-18 specification in TS 28.105 on ML model LCM and the associated management capabilities [x2] does not address the UE-side and UE/Network-side Model LCM.

#### 5.2.x.4	ML model lifecycle management capabilities

Each step in the ML model lifecycle. i.e., the ML model training, ML model testing, AI/ML emulation, ML model deployment and AI/ML inference correspond to number of dedicated management capabilities. The specified capabilities are developed based on corresponding use cases and requirements. The management capabilities specified by SA5 [x2] are highlighted below:

Start of quoted text (From TS 28.105)

- 6.1	ML model lifecycle management capabilities

Each operational step in the ML model lifecycle (see clause 4a.0.1) is supported by one or more AI/ML management capabilities as listed below.

Management capabilities for ML model training

-	ML model training management: allowing the MnS consumer to request the ML model training, consume and control the producer-initiated training, and manage the ML model training/re-training process. The training management capability may include training performance management and setting a policy for the producer-initiated ML model training.

-	ML model training capability also includes validation to evaluate the performance of the ML model when performing on the validation data, and to identify the variance of the performance on the training and validation data. If the variance is not acceptable, the ML model would need to be re-trained before being made available for the next step in the operational workflow (e.g., ML model testing).

Management capabilities for ML testing

-	ML model testing management:  allowing the MnS consumer to request the ML model testing, and to receive the testing results for a trained ML model. It may also include capabilities for selecting the specific performance metrics to be used or reported by the ML testing function. MnS consumer may also be allowed to trigger ML model re-training based on the ML model testing performance results.

Management capabilities for AI/ML inference emulation:

- AI/ML inference emulation: a capability allowing an MnS consumer to request an ML inference emulation for a specific ML model or models (after the training, validation, and testing) to evaluate the inference performance in an emulation environment prior to applying it to the target network or system.

Management capabilities for ML model entity deployment:

-	ML entity loading management: allowing the MnS consumer to trigger, control and/or monitor the ML model loading process.

Management capabilities for AI/ML inference:

-	AI/ML inference management: allowing an MnS consumer to control the inference, i.e., activate/deactivate the inference function and/or ML model/models, configure the allowed ranges of the inference output parameters. The capabilities also allow the MnS consumer to monitor and evaluate the inference performance and when needed trigger an update of an ML model or an AI/ML inference function.

The use cases and corresponding requirements for AI/ML management capabilities are specified in the following clauses.

End of quoted text

##### 5.2.x.4.1	Observations and analyses: ML model lifecycle management capabilities

- ML model lifecycle management (LCM) capabilities are crucial for the effective deployment, operation, and optimization of AI/ML-enabled features and capabilities in both the NG-RAN and 5GC. These capabilities ensure that AI/ML models are not only developed and trained correctly but also tested, deployed, evaluated, and operated efficiently in the network environment.

- The management capabilities outlined in TS 28.105 offer a structured approach to managing the various steps of the ML model lifecycle. This structured approach is applicable to AI/ML-enabled features and capabilities in NG-RAN, 5GC, and management system, ensuring consistency and reliability in the deployment and operation of AI/ML technologies for different domains.

- The AI/ML LCM management capabilities are foundational for integrating advanced AI/ML features into 5G networks. By ensuring that ML models are effectively managed from the training step through to inference, these capabilities provide robust and reliable AI/ML-driven network enhancements.

- The AI/ML LCM workflow and associated management capabilities specified by SA5 in [x2] should be considered as the baseline for the AI/ML E2E framework for the 3GPP. These capabilities provide a comprehensive foundation for ensuring that AI/ML models and related processes are consistently managed across all steps of their lifecycle, promoting seamless integration and operation for all domain within the 5G system.

#### 5.2.x.5	AI/ML functionalities management scenarios

The Rel-18 specification [x2] also documented AI/ML functionalities management scenarios in relation with managed AI/ML features which describe the possible locations of ML training function and AI/ML inference function involving the various 3GPP system domains.

Start of quoted text (From TS 28.105)

- 4a.2	AI/ML functionalities management scenarios (relation with managed AI/ML features)

The ML training function and/or AI/ML inference function can be located in the RAN domain MnS consumer (e.g. cross-domain management system) or the domain-specific management system (i.e. a management function for RAN or CN), or Network Function.

For MDA, the ML training function can be located inside or outside the MDAF. The AI/ML inference function is in the MDAF.

For NWDAF, the ML training function can be located in the MTLF of the NWDAF or the management system, the AI/ML inference function is in the AnLF.

For RAN, the ML training function and AI/ML inference function can both be located in the gNB, or the ML training function can be located in the management system and AI/ML inference function is located in the gNB.

Therefore, there might exist several location scenarios for ML training function and AI/ML inference function.

Scenario 1:

The ML training function and AI/ML inference function are both located in the 3GPP management system (e.g. RAN domain management function). For instance, for RAN domain-specific MDA, the ML training function and AI/ML inference functions for MDA can be located in the RAN domain-specific MDAF. As depicted in figure 4a.2-1.

Figure 4a.2-1: Management for RAN domain specific MDAF

Similarly, for CN domain-specific MDA the ML training function and AI/ML inference function can be located in CN domain-specific MDAF.

Scenario 2:

For RAN AI/ML capabilities the ML training function is located in the 3GPP RAN domain-specific management function while the AI/ML inference function is located in gNB. See figure 4a.2-2.

Figure 4a.2-2: Management where the ML model training is located in RAN domain management function and AI/ML inference is located in gNB

Scenario 3:

The ML training function and AI/ML inference function are both located in the gNB. See figure 4a.2-3.

Figure 4a.2-3: Management where the ML model training and AI/ML inference are both located in gNB

Scenario 4:

For NWDAF, the ML training function and AI/ML inference function are both located in the NWDAF. See figure 4a.2-4.

Figure 4a.2-4: Management where the ML model training and AI/ML inference are both located in CN

End of quoted text

##### 5.2.x.5.1	 Observations and analyses: AI/ML functionalities management 	scenarios

- The functional arrangement scenarios defined by SA5 specifications demonstrate that different part of the ML model life cycle can be managed depending on the use case.

- The functional arrangements represent management deployment scenarios where for example ML model training related tasks can either be a domain specific or as a cooperative multi-domain task involving for example RAN and management & orchestration or CN and management & orchestration (OAM) domains.

- The LCM workflow defined by SA5 serves as a management framework to accommodate and enable all the possible functional arrangement scenarios within or cross-domains in the 3GPP system.

- The functional arrangement scenarios, coupled with the ML model LCM as defined by SA5 in TS 28.105 [x2],  can be considered in the ongoing and any future 3GPP relevant specification development.

- 5.2.X.6	Activities Summary

Editor's note:	This clause describes high-level AI/ML activities e.g. LCM for AI/ML, data collection/storage/exposure, model training/delivery/ (de)-activation/inference emulation, inference/storage/exposure, performance evaluation and accuracy monitoring. Sub-clause(s) may be added to capture details.

**end-SP241234-SA5**

*** End of Change ***