TSG SA Meeting #SP-105	SP-24xxxx

10 – 13 September 2024, Melbourne, Australia

- Title:				chapter 5.3 AI/ML related activities in TSG RAN Working Groups

- Source:

- Document for:				Approval

Agenda Item:				7

Work Item / Release:	FS_AIML_CAL/ Rel-19

- 1. Introduction

Chapter 5.3 AI/ML related activities in TSG RAN Working Groups was addressed by following documents:

| SP-241125 | 22.850: Overview of AI/ML related activities in all Working Groups | MediaTek Inc. | List of features Rel18 and Rel19 of RAN1, Ran2, Ran3 under Chapter 5.1 General |
| --- | --- | --- | --- |
| SP-241317 | 22.850: AI/ML related activities in AI/ML for NG-RAN | NEC Corporation | NR_AIML_NGRAN NR_AIML_NGRAN-Core |
| SP-241325 | 22.850: Rel-19 pCR TR 22.850 AIML related activities in 3GPP | Nokia | Rel-18 RAN1 SID (FS_NR_AIML_Air) Rel-19 RAN1/RAN4 WID (NR_AIML_air) Rel-19 RAN2 SID (FS_NR_AIML_Mob) Rel-18 RAN3 WID (NR_AIML_NGRAN-Core) Rel-19 RAN3 SID (FS_NR_AIML_NGRAN_enh) |
| SP-241335 | 22.850: AI/ML Framekwork/Performance monitoring in R18 AIML for air interface | vivo | R18 Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface |
| SP-241351 | 22.850: AI/ML-Related Activities in RAN Working Groups | OPPO | FS_NR_AIML_Air (RAN1) NR_AIML_Air (RAN1) FS_NR_AIML_Mob (RAN2) SID NR_AIML_NGRAN and NR_AIML_NGRAN-Core (RAN3) |

- 2. Proposal

Incorporate the following change in TR 22.850.

*** Begin of Change ***

## 5.3	AI/ML related activities in TSG RAN Working Groups

### 5.3.X	AI/ML related activities on WID/SID#X

### 5.3.X.1	Description

Editor's Note:	This clause will investigate and identify AI/ML related activities of all TSG RAN working groups of Rel-18 features and Rel-19.

### 5.3.X.2	Terminology

Editor's Note:	This clause describes AI/ML related terminology (i.e. set of definitions, acronyms)

### 5.3.X.3	Activities Summary

Editor's note:	This clause describes high-level AI/ML activities e.g. LCM for AI/ML, data collection/storage/exposure, model training/delivery/ (de)-activation/inference emulation, inference/storage/exposure, performance evaluation and accuracy monitoring. Sub-clause(s) may be added to capture details.

**SP-241335-vivo**

### 5.3.X	AI/ML related activities on SID# R18 Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

### 5.3.X.1	Description

Editor's Note:	This clause will investigate and identify AI/ML related activities of all TSG RAN working groups of Rel-18 features and Rel-19.

In Release 18, study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact.

Use cases to focus on:

- CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1]

- Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1]

- Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1]

### 5.3.X.2	Terminology

Editor's Note:	This clause describes AI/ML related terminology (i.e. set of definitions, acronyms)

The Terminology of AI/ML is introduced in this subsection.

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

AI/ML-enabled Feature: refers to a Feature where AI/ML may be used.

AI/ML Model: A data driven algorithm that applies AI/ML techniques to generate a set of outputs based on a set of inputs.

AI/ML model delivery: A generic term referring to delivery of an AI/ML model from one entity to another entity in any manner. Note: An entity could mean a network node/function (e.g., gNB, LMF, etc.), UE, proprietary server, etc.

AI/ML model Inference:  A process of using a trained AI/ML model to produce a set of outputs based on a set of inputs.

AI/ML model testing: A subprocess of training, to evaluate the performance of a final AI/ML model using a dataset different from one used for model training and validation. Differently from AI/ML model validation, testing does not assume subsequent tuning of the model.

AI/ML model training: A process to train an AI/ML Model [by learning the input/output relationship] in a data driven manner and obtain the trained AI/ML Model for inference.

AI/ML model transfer: Delivery of an AI/ML model over the air interface in a manner that is not transparent to 3GPP signalling, either parameters of a model structure known at the receiving end or a new model with parameters. Delivery may contain a full model or a partial model.

AI/ML model validation: A subprocess of training, to evaluate the quality of an AI/ML model using a dataset different from one used for model training, that helps selecting model parameters that generalize beyond the dataset used for model training.

Data collection: A process of collecting data by the network nodes, management entity, or UE for the purpose of AI/ML model training, data analytics and inference.

Federated learning / federated training: A machine learning technique that trains an AI/ML model across multiple decentralized edge nodes (e.g., UEs, gNBs) each performing local model training using local data samples. The technique requires multiple interactions of the model, but no exchange of local data samples.

Functionality identification: A process/method of identifying an AI/ML functionality for the common understanding between the NW and the UE. Note: Information regarding the AI/ML functionality may be shared during functionality identification. Where AI/ML functionality resides depends on the specific use cases and sub use cases.

Management instruction: Information needed to ensure proper inference operation. This information may include selection/(de)activation/switching of AI/ML models or AI/ML functionalities, fallback to non-AI/ML operation, etc.

Model activation: enable an AI/ML model for a specific AI/ML-enabled feature.

Model deactivation: disable an AI/ML model for a specific AI/ML-enabled feature.

Model download: Model transfer from the network to UE.

Model identification: A process/method of identifying an AI/ML model for the common understanding between the NW and the UE. Note: The process/method of model identification may or may not be applicable. Note: Information regarding the AI/ML model may be shared during model identification.

Model monitoring: A procedure that monitors the inference performance of the AI/ML model.

Model parameter update: Process of updating the model parameters of a model.

Model selection: The process of selecting an AI/ML model for activation among multiple models for the same AI/ML enabled feature. Note: Model selection may or may not be carried out simultaneously with model activation.

Model switching: Deactivating a currently active AI/ML model and activating a different AI/ML model for a specific AI/ML-enabled feature.

Model update: Process of updating the model parameters and/or model structure of a model.

Model upload: Model transfer from UE to the network.

Network-side (AI/ML) model: An AI/ML Model whose inference is performed entirely at the network.

Offline field data: The data collected from field and used for offline training of the AI/ML model.

Offline training: An AI/ML training process where the model is trained based on collected dataset, and where the trained model is later used or delivered for inference. Note: This definition only serves as a guidance. There may be cases that may not exactly conform to this definition but could still be categorized as offline training by commonly accepted conventions.

Online field data: The data collected from field and used for online training of the AI/ML model.

Online training: An AI/ML training process where the model being used for inference) is (typically continuously) trained in (near) real-time with the arrival of new training samples. Note: the notion of (near) real-time vs. non real-time is context-dependent and is relative to the inference time-scale. Note: This definition only serves as a guidance. There may be cases that may not exactly conform to this definition but could still be categorized as online training by commonly accepted conventions. Note: Fine-tuning/re-training may be done via online or offline training.

Reinforcement Learning (RL): A process of training an AI/ML model from input (a.k.a. state) and a feedback signal (a.k.a.  reward) resulting from the model’s output (a.k.a. action) in an environment the model is interacting with.

Semi-supervised learning: A process of training a model with a mix of labelled data and unlabelled data.

Supervised learning: A process of training a model from input and its corresponding labels.

Test encoder/decoder for TE: AI/ML model for UE encoder/gNB decoder implemented by TE.

Two-sided (AI/ML) model: A paired AI/ML Model(s) over which joint inference is performed, where joint inference comprises AI/ML Inference whose inference is performed jointly across the UE and the network, i.e, the first part of inference is firstly performed by UE and then the remaining part is performed by gNB, or vice versa.

UE-side (AI/ML) model: An AI/ML Model whose inference is performed entirely at the UE.

Unsupervised learning: A process of training a model without labelled data.

Proprietary-format models: ML models of vendor-/device-specific proprietary format, from 3GPP perspective. They are not mutually recognizable across vendors and hide model design information from other vendors when shared. Note: An example is a device-specific binary executable format.

Open-format models: ML models of specified format that are mutually recognizable across vendors and allow interoperability, from 3GPP perspective. They are mutually recognizable between vendors and do not hide model design information from other vendors when shared.

### 5.3.X.3	Activities Summary

Editor's note:	This clause describes high-level AI/ML activities e.g. LCM for AI/ML, data collection/storage/exposure, model training/delivery/ (de)-activation/inference emulation, inference/storage/exposure, performance evaluation and accuracy monitoring. Sub-clause(s) may be added to capture details.

As defined in TR 21.905, the following the definition of components are studied in LCM:

-	Data collection

-	Model training

-	Functionality/model identification

-	Model delivery/transfer

-	Model inference operation

-	Functionality/model selection, activation, deactivation, switching, and fallback operation.

-	Functionality/model monitoring

-	Model update

-	UE capability

#### 5.3.X.3.1 Functional framework

This section introduces the functional framework for AI/ML for NR air interface illustrated in Figure 5.3.X.3.1-1. As As defined in TR 21.905, the aim of this framework is to cover a general functional architecture addressing both model-ID-based LCM and functionality-based LCM. Therefore, some of the functions or data/information/instruction flows (i.e., the arrows) shown in the figure might not always be relevant for a given LCM approach.

As an illustrative example, consider a scenario where the network performs functionality-based LCM and where models are not identified in the network, while the UE concurrently performs model-level management (e.g., model selection/switching/(de)activation, etc…). In this hypothetical case, the "Model Training" or "Model Storage" functions with their respective procedures, may be regarded as irrelevant from the network’s perspective.

Note: The functional framework and high-level procedures defined in this TR should not prevent from "thinking beyond" them during a normative phase if any use case requires so.

Figure 5.3.X.3.1-1: Functional framework for AI/ML for NR Air Interface

As seen in Figure 5.3.X.3.1-1, the general framework consists of the following:

-	Data Collection is a function that provides input data to the Model Training, Management, and Inference functions.

-	Training Data: Data needed as input for the AI/ML Model Training function.

-	Monitoring Data: Data needed as input for the Management of AI/ML models or AI/ML functionalities.

-	Inference Data: Data needed as input for the AI/ML Inference function.

-	Model Training is a function that performs AI/ML model training, validation, and testing which may generate model performance metrics which can be used as part of the model testing procedure. The Model Training function is also responsible for data preparation (e.g., data pre-processing and cleaning, formatting, and transformation) based on Training Data delivered by a Data Collection function, if required.

-	Trained/Updated Model: In case of having a Model Storage function, this is used to deliver trained, validated, and tested AI/ML models to the Model Storage function, or to deliver an updated version of a model to the Model Storage function.

-	Management is a function that oversees the operation (e.g., selection/(de)activation/switching/fallback) and monitoring (e.g., performance) of AI/ML models or AI/ML functionalities. This function is also responsible for making decisions to ensure the proper inference operation based on data received from the Data Collection function and the Inference function.

-	Management Instruction: Information needed as input to manage the Inference function. Concerning information may include selection/(de)activation/switching of AI/ML models or AI/ML-based functionalities, fallback to non-AI/ML operation (i.e., not relying on inference process), etc…

-	Model Transfer/Delivery Request: Used to request model(s) to the Model Storage function.

-	Performance Feedback / Retraining Request: Information needed as input for the Model Training function, e.g., for model (re)training or updating purposes.

-	Inference is a function that provides outputs from the process of applying AI/ML models or AI/ML functionalities, using the data that is provided by the Data Collection function (i.e., Inference Data) as an input. The Inference function is also responsible for data preparation (e.g., data pre-processing and cleaning, formatting, and transformation) based on Inference Data delivered by a Data Collection function, if required.

-	Inference Output: Data used by the Management function to monitor the performance of AI/ML models or AI/ML functionalities.

-	Model Storage is a function responsible for storing trained/updated models that can be used to perform the Inference function.

-	Note: The Model Storage function in Figure 4.4-1 is only intended as a reference point (if any) when applicable for protocol terminations, model transfer/delivery, and related processes. It should be stressed that its purpose does not encompass restricting the actual storage locations of models. Therefore, the specification impact of all data/information/instruction flows (i.e., the arrows in Figure 4.4-1) to/from this function should be studied case by case.

-	Model Transfer/Delivery: Used to deliver an AI/ML model to the Inference function.

#### 5.3.X.3.2 Performance monitoring

As defined in TR 21.905, the following metrics/methods for AI/ML model monitoring in lifecycle management per use case are considered:

-	Monitoring based on inference accuracy, including metrics related to intermediate KPIs

-	Monitoring based on system performance, including metrics related to system performance KPIs

-	Other monitoring solutions, at least the following 2 options.

-	Monitoring based on data distribution

-	Input-based: e.g., Monitoring the validity of the AI/ML input, e.g., out-of-distribution detection, drift detection of input data, or SNR, delay spread, etc.

-	Output-based: e.g., drift detection of output data

-	Monitoring based on applicable condition

Note:	Monitoring metric calculation may be done at NW or UE

**end-SP241335-vivo**

**SP-241351-OPPO**

### 5.3.X	AI/ML related activities on WID/SID FS_NR_AIML_Air (RAN1)

#### 5.3.X.1	Description

Editor's Note:	This clause will investigate and identify AI/ML related activities of all TSG RAN working groups of Rel-18 features and Rel-19.

This section lists AI/ML-related activities on Rel-18 SID FS_NR_AIML_AIR (WI #940084). The study was led by RAN1 and worked together with other RAN WGs.

| Unique ID | Title | Acronym | WGs | WID/SID | Output |
| --- | --- | --- | --- | --- | --- |
| 940084 | Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface | FS_NR_AIML_air | R1 | RP-221348 | 38.843 |

#### 5.3.X.2	Terminology

Editor's Note:	This clause describes AI/ML related terminology (i.e. set of definitions, acronyms)

Terminologies introduced in this SID are captured in TR 38.843 and referenced here.

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

AI/ML-enabled Feature: refers to a Feature where AI/ML may be used.

AI/ML Model: A data driven algorithm that applies AI/ML techniques to generate a set of outputs based on a set of inputs.

AI/ML model delivery: A generic term referring to delivery of an AI/ML model from one entity to another entity in any manner. Note: An entity could mean a network node/function (e.g., gNB, LMF, etc.), UE, proprietary server, etc.

AI/ML model Inference:  A process of using a trained AI/ML model to produce a set of outputs based on a set of inputs.

AI/ML model testing: A subprocess of training, to evaluate the performance of a final AI/ML model using a dataset different from one used for model training and validation. Differently from AI/ML model validation, testing does not assume subsequent tuning of the model.

AI/ML model training: A process to train an AI/ML Model [by learning the input/output relationship] in a data driven manner and obtain the trained AI/ML Model for inference.

AI/ML model transfer: Delivery of an AI/ML model over the air interface in a manner that is not transparent to 3GPP signalling, either parameters of a model structure known at the receiving end or a new model with parameters. Delivery may contain a full model or a partial model.

AI/ML model validation: A subprocess of training, to evaluate the quality of an AI/ML model using a dataset different from one used for model training, that helps selecting model parameters that generalize beyond the dataset used for model training.

Data collection: A process of collecting data by the network nodes, management entity, or UE for the purpose of AI/ML model training, data analytics and inference.

Federated learning / federated training: A machine learning technique that trains an AI/ML model across multiple decentralized edge nodes (e.g., UEs, gNBs) each performing local model training using local data samples. The technique requires multiple interactions of the model, but no exchange of local data samples.

Functionality identification: A process/method of identifying an AI/ML functionality for the common understanding between the NW and the UE. Note: Information regarding the AI/ML functionality may be shared during functionality identification. Where AI/ML functionality resides depends on the specific use cases and sub use cases.

Management instruction: Information needed to ensure proper inference operation. This information may include selection/(de)activation/switching of AI/ML models or AI/ML functionalities, fallback to non-AI/ML operation, etc.

Model activation: enable an AI/ML model for a specific AI/ML-enabled feature.

Model deactivation: disable an AI/ML model for a specific AI/ML-enabled feature.

Model download: Model transfer from the network to UE.

Model identification: A process/method of identifying an AI/ML model for the common understanding between the NW and the UE. Note: The process/method of model identification may or may not be applicable. Note: Information regarding the AI/ML model may be shared during model identification.

Model monitoring: A procedure that monitors the inference performance of the AI/ML model.

Model parameter update: Process of updating the model parameters of a model.

Model selection: The process of selecting an AI/ML model for activation among multiple models for the same AI/ML enabled feature. Note: Model selection may or may not be carried out simultaneously with model activation.

Model switching: Deactivating a currently active AI/ML model and activating a different AI/ML model for a specific AI/ML-enabled feature.

Model update: Process of updating the model parameters and/or model structure of a model.

Model upload: Model transfer from UE to the network.

Network-side (AI/ML) model: An AI/ML Model whose inference is performed entirely at the network.

Offline field data: The data collected from field and used for offline training of the AI/ML model.

Offline training: An AI/ML training process where the model is trained based on collected dataset, and where the trained model is later used or delivered for inference. Note: This definition only serves as a guidance. There may be cases that may not exactly conform to this definition but could still be categorized as offline training by commonly accepted conventions.

Online field data: The data collected from field and used for online training of the AI/ML model.

Online training: An AI/ML training process where the model being used for inference) is (typically continuously) trained in (near) real-time with the arrival of new training samples. Note: the notion of (near) real-time vs. non real-time is context-dependent and is relative to the inference time-scale. Note: This definition only serves as a guidance. There may be cases that may not exactly conform to this definition but could still be categorized as online training by commonly accepted conventions. Note: Fine-tuning/re-training may be done via online or offline training.

Reinforcement Learning (RL): A process of training an AI/ML model from input (a.k.a. state) and a feedback signal (a.k.a.  reward) resulting from the model’s output (a.k.a. action) in an environment the model is interacting with.

Semi-supervised learning: A process of training a model with a mix of labelled data and unlabelled data.

Supervised learning: A process of training a model from input and its corresponding labels.

Test encoder/decoder for TE: AI/ML model for UE encoder/gNB decoder implemented by TE.

Two-sided (AI/ML) model: A paired AI/ML Model(s) over which joint inference is performed, where joint inference comprises AI/ML Inference whose inference is performed jointly across the UE and the network, i.e, the first part of inference is firstly performed by UE and then the remaining part is performed by gNB, or vice versa.

UE-side (AI/ML) model: An AI/ML Model whose inference is performed entirely at the UE.

Unsupervised learning: A process of training a model without labelled data.

Proprietary-format models: ML models of vendor-/device-specific proprietary format, from 3GPP perspective. They are not mutually recognizable across vendors and hide model design information from other vendors when shared. Note: An example is a device-specific binary executable format.

Open-format models: ML models of specified format that are mutually recognizable across vendors and allow interoperability, from 3GPP perspective. They are mutually recognizable between vendors and do not hide model design information from other vendors when shared.

5.3.X.3	Activities Summary

Editor's note:	This clause describes high-level AI/ML activities e.g. LCM for AI/ML, data collection/storage/exposure, model training/delivery/ (de)-activation/inference emulation, inference/storage/exposure, performance evaluation and accuracy monitoring. Sub-clause(s) may be added to capture details.

Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface in Rel-18 was leading by RAN1 and worked together with RAN2, focusing on the following areas:

- Objective:

  - Explore the benefits of augmenting the air-interface with AI/ML features.

  - Study the 3GPP framework for AI/ML concerning performance, complexity, and specification impact for each use case.

- Use Cases:

  - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1]

  - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1]

  - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1]

- Framework Identification:

  - Develop a common AI/ML framework and define functional requirements.

  - Characterize AI/ML models, including generation, inference, and lifecycle management.

  - Identify UE-gNB collaboration levels.

    - No collaboration

    - Various levels of UE/gNB collaboration

- Performance Evaluation:

  - Evaluate performance benefits of AI/ML algorithms for selected use cases against legacy methods.

  - Determine common and use-case specific KPIs.

- Specification Impact:

  - Assess potential specification impacts on PHY and protocol layers.

  - Identify common notation and terminology for AI/ML functions.

  - Ensure AI/ML processing capabilities meet existing minimum requirements without specifying specific AI/ML models.

##### 5.3.X.3.1 General AI/ML framework

This work studied general AI/ML framework aspect including descriptiong of AI/ML stages, life cycle manamgnet, collaboration levels, and functional framework details.

- General AI/ML Framework:

  - Identify common notation and terminology for AI/ML-related functions, procedures, and interfaces.

  - Stages include model generation (training, validation, testing) and inference operations. The treatment of datasets for these stages is also documented.

- Life Cycle Management (LCM):

  - LCM covers the entire lifecycle of AI/ML models, from data collection, model training, and deployment to monitoring and updating, including:

    - Data collection

    - Model training

    - Functionality/model identification

    - Model delivery/transfer

    - Model inference operation

    - Functionality/model selection, activation, deactivation, switching, and fallback operation.

    - Functionality/model monitoring

    - Model update

    - UE capability

- Model Identification:

  - Models are categorized into type A and type B based on how they are identified, either through over-the-air signaling or offline processes. The identification process can be initiated by the network or the UE

  - Models can be functionality-based or model-ID-based, with models identified by a model ID that may be logical or physical.

- Scenario-Specific Models:

  - Scenario/configuration specific (including site-specific configuration/channel conditions) models may provide performance benefits in some studied use cases (i.e., when a single model cannot generalize well to multiple scenarios/configurations/sites.

  - Various approaches for achieving good performance across different scenarios/configurations were studied, including model generalizatio, model switching, and model update.

- Data Collection:

  - Data collection is crucial for various stages like model training, inference, monitoring, and updating.

  - The study emphasizes the importance of latency requirements for real-time data collection, especially in the RRC_CONNECTED state for all types of offline model training (i.e., UE- /NW-/ two-sided model training.

- Collaboration Levels:

  - Various levels of collaboration between the network and UE are defined

    - Level x: No collaboration.

    - Level y: Signalling-based collaboration without model transfer. Note: this level includes cases without model delivery.

    - Level z: Signalling-based collaboration with model transfer.

Figure 5.3.x.3.1-x: Collaborabtions between the network and the UE

  - Different cases of model delivery/transfer are outlined, each with implications for storage location, training location, and transfer format.

- Functional Framework:

  - The functional framework for AI/ML details functions like data collection, model training, inference, management, and model storage.

  - The framework is designed to be flexible, covering both model-ID-based and functionality-based LCM.

Figure 5.3.x.3.1-x: Functional framework for AI/ML for NR Air Interface

##### 5.3.X.3.2 Performance evaluation

Performance benefits of AI/ML based algorithms for the agreed use cases in the final representative set are evaluated.

- Evaluation Methodology:

  - The 3GPP channel models (TR 38.901) serve as the standard for evaluations. However, results based on other datasets are also acceptable.

- Common Key Performance Indicators (KPIs):

  - 3GPP channel models (TR 38.901) are used as the baseline for evaluations

  - The following KPIs are studied:

    - Performance Metrics

    - Over-the_air overhead Metrics

    - Inference complexity, including complexity for pre- and post-processing

    - Training complexity

    - LCM related complexity and storage overhead

- Model Monitoring KPIs:

  - Accuracy and relevance (i.e., how well does the given monitoring metric/methods reflect the model and system performance)

  - Overhead (e.g., signalling overhead associated with model monitoring)

  - Complexity (e.g., computation and memory cost for model monitoring)

  - Latency (i.e., timeliness of monitoring result, from model failure to action, given the purpose of model monitoring)

##### 5.3.X.3.3 Potential specification impact assessment

The work studies the potential specification impacts from physical layer and protocol layer perspective.

- Physicaly layer aspects:

  - Consider AI model lifecycle management, dataset construction, and performance monitoring across different use cases and collaboration levels.

  - Include considerations for new signaling mechanisms, assistance information, and feedback systems necessary to support the training, validation, and monitoring of AI/ML models.

- Protocol layer aspects:

  - Consider the use cases and as per RAN1 input, aspects related to life cycle management signalling procedures, model identification, data collection, model transfer/delivery, UE capability reporting, and applicability-related reporting are studied.

- Interoperability and testability aspects:

  - Study requirements and testing frameworks to validate AI/ML based performance enhancements and ensure that UE and gNB with AI/ML meet or exceed the existing minimum requirements, if applicable.

### 5.3.Y	AI/ML related activities on WID/SID NR_AIML_Air (RAN1)

#### 5.3.Y.1	Description

This section lists AI/ML-related activities on Rel-19 WID NR_AIML_AIR (WI #1020093). The work item has been led by RAN1 and worked together with other RAN WGs.

| Unique ID | Title | Acronym | WGs | WID/SID | Output |
| --- | --- | --- | --- | --- | --- |
| 1020093 | Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface | NR_AIML_air | R1 | RP-240774 | 38.843 38.300 38.214 38.215 38.331 38.305 37.355 38.455 38.133 38.133 |

#### 5.3.Y.2	Terminology

Editor's Note:	To be added aligning with progress of this work item in RAN WGs.

#### 5.3.Y.3	Activities Summary

Editor's Note:	To be added aligning with progress of this work item in RAN WGs.

### 5.3.Z	AI/ML related activities on WID/SID FS_NR_AIML_Mob (RAN2)

#### 5.3.Z.1	Description

This section lists AI/ML-related activities on Rel-19 SID FS_NR_AIML_Mob (WI #1020084). The work item has been led by RAN2 and worked together with other RAN WGs.

| Unique ID | Title | Acronym | WGs | WID/SID | Output |
| --- | --- | --- | --- | --- | --- |
| 1020084 | Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR | FS_NR_AIML_Mob | R2 | RP-240082 | 38.744 |

#### 5.3.Z.2	Terminology

Editor's Note:	To be added aligning with progress of this work item in RAN WGs.

#### 5.3.Z.3	Activities Summary

Editor's Note:	To be added aligning with progress of this work item in RAN WGs.

### 5.3.A	AI/ML related activities on WID/SID NR_AIML_NGRAN and NR_AIML_NGRAN-Core (RAN3)

#### 5.3.A.1	Description

This section lists AI/ML-related activities on Rel-18 WID/SID NR_AIML_NGRAN and NR_AIML_NGRAN-Core (WI #941010). The study was led by RAN3 and worked together with other WGs.

| Unique ID | Title | Acronym | WGs | WID/SID | Output |
| --- | --- | --- | --- | --- | --- |
| 941010 | Artificial Intelligence (AI) / Machine Learning (ML) for NG-RAN | NR_AIML_NGRAN | R3 | RP-233441 | 38.300 38.401 38.420 38.423 |
| 941110 | Artificial Intelligence (AI) / Machine Learning (ML) for NG-RAN | NR_AIML_NGRAN-Core | R3 | RP-233441 | 38.300 38.401 38.420 38.423 |

#### 5.3.A.2	Terminology

This work builds on the previous work from Rel-17 study item 880076, “Study on Enhancements for Data Collection for NR and EN-DC” The terminologies used in this section are derived from TR 37.817, which is based on study item 880076.

Data collection: Data collected from the network nodes, management entity or UE, as a basis for AI/ML model training, data analytics and inference.

AI/ML Model: A data driven algorithm by applying machine learning techniques that generates a set of outputs consisting of predicted information and/or decision parameters, based on a set of inputs

AI/ML Training: An online or offline process to train an AI/ML model by learning features and patterns that best present data and get the trained AI/ML model for inference.

AI/ML Inference: A process of using a trained AI/ML model to make a prediction or guide the decision based on collected data and AI/ML model.

#### 5.3.A.3	Activities Summary

##### 5.3.A.3.1 Summary from Rel-18 work

A summary of this work is already captured in clause 11.2 in TR 21.918 (directly referenced from TR 21.918)

- Objective and scope

The objective is to improve network performance and user experience, through analysing the data collected and autonomously processed by the NG-RAN, which can yield further insights, e.g., for Network Energy Saving, Load Balancing, Mobility Optimization.

Signalling procedures used for the exchange of information to support AI/ML in NG-RAN are use case and data type agnostic

Support of AI/ML in NG-RAN does not apply to ng-eNB

- Focused use cases:

  - Network Energy Saving

    - AI/ML-based Network Energy Saving is to optimize the overall energy consumption in an area of the Operator’s network by leveraging on the data collected in the RAN network. AI/ML based Energy Saving at a NG-RAN node essentially is for a trained AI model to make real-time decisions to either deactivate or activate a cell or a NG-RAN node to save network energy consumption.

  - Load Balancing

    - Load balancing is to distribute load evenly among cells and among areas of cells, or to transfer part of the traffic from congested cells or from congested areas of cells, or to offload users from one cell, cell area, carrier or RAT to improve network performance. The AI/ML-based Load Balancing is introduced to improve the load balancing performance by using load predictions from AI/ML models and feedback information mirroring the performance of a UE after an offloading action.

  - Mobility Optimization

    - Mobility management allows to guarantee service-continuity during mobility by minimizing call drops, radio link failures, unnecessary handovers, and ping-pong. AI/ML-based mobility optimization is introduced to improve successful handover rates and determine which is the best mobility target for maximisation of efficiency and performance.

- Deployment scenarios:

  - AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the NG-RAN node.

  - AI/ML Model Training and AI/ML Model Inference are both located in the NG-RAN node:

##### 5.3.A.3.2 Summary from Rel-17 TR 37.817, Study on enhancement for Data Collection for NR and EN-DC

Since this work item is based on the conclusions captured in TR 37.817, it is useful to include activities summary from TR 37.817.

- High-level Principles

  - The detailed AI/ML algorithms and models for use cases are implementation specific and out of RAN3 scope.

  - The study focuses on AI/ML functionality and corresponding types of inputs/outputs.

  - The input/output and the location of the Model Training and Model Inference function should be studied case by case.

  - The study focuses on the analysis of data needed at the Model Training function from Data Collection, while the aspects of how the Model Training function uses inputs to train a model are out of RAN3 scope.

  - The study focuses on the analysis of data needed at the Model Inference function from Data Collection, while the aspects of how the Model Inference function uses inputs to derive outputs are out of RAN3 scope.

  - Where AI/ML functionality resides within the current RAN architecture, depends on deployment and on the specific use cases.

  - The Model Training and Model Inference functions should be able to request, if needed, specific information to be used to train or execute the AI/ML algorithm and to avoid reception of unnecessary information. The nature of such information depends on the use case and on the AI/ML algorithm.

  - The Model Inference function should signal the outputs of the model only to nodes that have explicitly requested them (e.g., via subscription), or nodes that take actions based on the output from Model Inference.

  - An AI/ML model used in a Model Inference function has to be initially trained, validated and tested by the Model Training function before deployment.

  - NG-RAN SA is prioritized; EN-DC and MR-DC are down-prioritized, but not precluded from Rel.18.

  - Functional framework and high-level procedures defined in this TR should not prevent from “thinking beyond” them during normative phase if a use case requires so.

  - User data privacy and anonymisation should be respected during AI/ML operation

- Functional Framework

- Figure 5.3.A.3.2-x: Functional Framework for RAN Intelligence

  - Data Collection is a function that provides input data to Model training and Model inference functions. AI/ML algorithm specific data preparation (e.g., data pre-processing and cleaning, formatting, and transformation) is not carried out in the Data Collection function.  
Examples of input data may include measurements from Ues or different network entities, feedback from Actor, output from an AI/ML model.

    - Training Data: Data needed as input for the AI/ML Model Training function.

    - Inference Data: Data needed as input for the AI/ML Model Inference function.

  - Model Training is a function that performs the AI/ML model training, validation, and testing which may generate model performance metrics as part of the model testing procedure. The Model Training function is also responsible for data preparation (e.g., data pre-processing and cleaning, formatting, and transformation) based on Training Data delivered by a Data Collection function, if required.

    - Model Deployment/Update: Used to initially deploy a trained, validated, and tested AI/ML model to the Model Inference function or to deliver an updated model to the Model Inference function.

    - Note: Details of the Model Deployment/Update process as well as the use case specific AI/ML models transferred via this process are out of RAN3 Rel-17 study scope. The feasibility to single-vendor or multi-vendor environment has not been studied in RAN3 Rel-17 study.

  - Model Inference is a function that provides AI/ML model inference output (e.g., predictions or decisions). Model Inference function may provide Model Performance Feedback to Model Training function when applicable. The Model Inference function is also responsible for data preparation (e.g., data pre-processing and cleaning, formatting, and transformation) based on Inference Data delivered by a Data Collection function, if required.

    - Output: The inference output of the AI/ML model produced by a Model Inference function.

    - Model Performance Feedback:  It may be used for monitoring the performance of the AI/ML model, when available.

  - Actor is a function that receives the output from the Model Inference function and triggers or performs corresponding actions. The Actor may trigger actions directed to other entities or to itself.

    - Feedback: Information that may be needed to derive training data, inference data or to monitor the performance of the AI/ML Model and its impact to the network through updating of KPIs and performance counters.

**end-SP-241351-OPPO**

**SP-241325-Nokia**

### 5.3.A	Rel-18 RAN1 SID - Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface (FS_NR_AIML_Air)

### 5.3.A.1	Description

The objective of this study is to investigate the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact.

- Target Use Cases and Aspects to Consider:

  - Initial Use Cases:

    - CSI Feedback Enhancement:

      - Objectives: Overhead reduction, improved accuracy, prediction.

      - Relevant Group: [RAN1]

    - Beam Management:

      - Objectives: Beam prediction in time and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement.

      - Relevant Group: [RAN1]

    - Positioning Accuracy Enhancements:

      - Objectives: Improved accuracy in scenarios with heavy NLOS conditions.

      - Relevant Group: [RAN1]

  - Finalization of Representative Sub Use Cases:

    - Characterize and evaluate baseline performance.

    - Ensure diversity in AI/ML approaches to support various gNB-UE collaboration levels.

- Framework Investigation:

  - AI/ML Model Characterization:

    - Model Generation:

      - Includes model training (input/output, pre-/post-process, online/offline), validation, and testing.

    - Inference Operation:

      - Includes input/output and pre-/post-process as applicable.

  - Collaboration Levels:

    - No Collaboration:

      - Implementation-based AI/ML algorithms without information exchange (for comparison).

    - Various Levels of Collaboration:

      - Separate or joint ML operations between UE and gNB.

  - Lifecycle Management:

    - Model training, deployment, inference, monitoring, and updating.

  - Datasets:

    - For training, validation, testing, and inference.

  - Common Notation and Terminology:

    - For AI/ML functions, procedures, and interfaces.

    - Consider work done for FS_NR_ENDC_data_collect where appropriate.

- Performance Evaluation:

  - Methodology:

    - Based on statistical models.

    - Extensions of 3GPP evaluation methodology for AI/ML techniques.

    - Discussion on the need for field data to assess real-world performance and robustness.

    - Common assumptions for dataset construction for training, validation, and testing.

    - Adequate model training strategy, collaboration levels, and implications.

    - Agreed-upon base AI models for calibration.

    - Reporting AI model description and training methodology for cross-checking.

  - KPIs:

    - Common KPIs and requirements for AI/ML operations.

    - Use-case specific KPIs and benchmarks.

    - Comparison of AI/ML algorithms’ performance, inference latency, and computational complexity with state-of-the-art baselines.

    - Consideration of overhead, power consumption, memory storage, hardware requirements, and generalization capability.

- The assessment of potential specification impacts for AI/ML use cases involves detailed considerations across PHY layer, protocol, and interoperability aspects. Each layer requires specific guidelines and protocols to manage AI/ML model lifecycle, collaboration levels, and performance validation, ensuring seamless integration and enhanced performance in the 3GPP framework.

### 5.3.B	Rel-19 RAN1/RAN4 WID - Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface (NR_AIML_air)

### 5.3.B.1	Description

The objective of this work is to provide specification support for the following aspects:

- AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]:

  - Signalling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback

    - Identification related signalling is part of the above objective

  - Necessary signalling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring, data collection (except for the purpose of CN/OAM/OTT collection of UE-sided model training data) for both UE-sided and NW-sided models

  - Signalling mechanism of applicable functionalities/models

- Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]:

  - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”)

  - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”)

  - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any

  - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]:

  - Direct AI/ML positioning:

    - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning

    - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

    - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

  - AI/ML assisted positioning

    - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning

    - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

  - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any

  - Investigate and specify the necessary signalling of necessary measurement enhancements (if any)

  - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases

- Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]:

  - Specify necessary RAN4 core requirements for the above two use cases.

  - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring.

Study objectives with corresponding checkpoints in RAN#105 (Sept ’24):

- CSI feedback enhancement [RAN1]:

  - For CSI compression (two-sided model), further study ways to:

    - Improve trade-off between performance and complexity/overhead

      - e.g., considering extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction (compared to Rel-18 non-AI/ML based approach), etc.

    - Alleviate/resolve issues related to inter-vendor training collaboration.

  - For CSI prediction (UE-sided model), further study performance gain over Rel-18 non-AI/ML based approach and associated complexity, while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the [a] (e.g., cell/site specific model could be considered to improve performance gain).

- Necessity and details of model Identification concept and procedure in the context of LCM [RAN2/RAN1]

- CN/OAM/OTT collection of UE-sided model training data [RAN2/RAN1]:

  - For the FS_NR_AIML_Air study use cases, identify the corresponding contents of UE data collection

  - Analyse the UE data collection mechanisms identified during the FS_NR_AIML_Air (clause 7.2.1.3.2 of [a]) study along with the implications and limitations of each of the methods

- Model transfer/delivery [RAN2/RAN1]:

  - Determine whether there is a need to consider standardised solutions for transferring/delivering AI/ML model(s) considering at least the solutions identified during the FS_NR_AIML_Air study

- Testability and interoperability [RAN4]:

  - Finalize the testing framework and procedure for one-sided models and further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least:

    - Relation to legacy requirements

    - Performance monitoring and LCM aspects considering use-case specifics

    - Generalization aspects

    - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.)

    - UE processing capability and limitations

    - Post-deployment validation due to model change/drift

  - RAN5 aspects related to testability and interoperability to be addressed on a request basis

- For Beam Management and Positioning Accuracy enhancement use cases, specify performance requirements and test cases for AI/ML LCM procedures (including performance monitoring) and UE features enabled by UE-sided models

  - Specify necessary performance requirements and tests (including metrics) for the above-mentioned use cases

  - Specify necessary test cases and performance requirements for LCM procedure, including performance monitoring.

### 5.3.C	Rel-19 RAN2 SID - AIML for mobility in NR (FS_NR_AIML_Mob)

### 5.3.C.1	Description

The study will focus on mobility enhancement in RRC_CONNECTED mode over air interface by following existing mobility framework, i.e., handover decision is always made in network side. Mobility use cases focus on standalone NR PCell change. UE-side and network-side AI/ML model can be both considered, respectively. The investigation is to evaluate potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover, considering the following aspects:

- AI/ML based RRM measurement and event prediction,

  - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2]

    - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2]

  - HO failure/RLF prediction (UE sided model) [RAN2]

  - Measurement events prediction (UE sided model) [RAN2]

- Study the need/benefits of any other UE assistance information for the network side model [RAN2]

- The evaluation of the AI/ML aided mobility benefits should consider HO performance KPIs (e.g., Ping-pong HO, HOF/RLF, Time of stay, Handover interruption, prediction accuracy, and measurement reduction) etc.) and complexity tradeoffs [RAN2]

- Potential AI mobility specific enhancement should be based on the Rel19 AI/ML-air interface WID general framework (e.g. LCM, performance monitoring etc) [RAN2]

- Potential specification impacts of AI/ML aided mobility [RAN2]

- Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4]

### 5.3.D	Rel-18 RAN3 WID - Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN (NR_AIML_NGRAN-Core)

### 5.3.D.1	Description

The objective of this work is to specify data collection enhancements and signaling support within existing NG-RAN interfaces and architecture (including non-split architecture and split architecture) for AI/ML-based Network Energy Saving, Load Balancing and Mobility Optimization.

Support of AI/ML for NG-RAN, as a RAN internal function, is used to facilitate Artificial Intelligence (AI) and Machine Learning (ML) techniques in NG-RAN. The objective of AI/ML for NG-RAN is to improve network performance and user experience, through analysing the data collected and autonomously processed by the NG-RAN, which can yield further insights, e.g., for Network Energy Saving, Load Balancing, Mobility Optimization.

Support of AI/ML in NG-RAN requires inputs from neighbour NG-RAN nodes (e.g., predicted information, feedback information, measurements) and/or UEs (e.g., measurement results).

Signalling procedures used for the exchange of information to support AI/ML in NG-RAN are use case and data type agnostic, which means that the intended usage of the data exchanged via these procedures (e.g., input, output, feedback) is not indicated. The collection and reporting of information are configured through the Data Collection Reporting Initiation procedure, while the actual reporting is performed through the Data Collection Reporting procedure.

Support of AI/ML in NG-RAN does not apply to ng-eNB.

For the deployment of AI/ML in NG-RAN, the following scenarios may be supported:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the NG-RAN node.

-	AI/ML Model Training and AI/ML Model Inference are both located in the NG-RAN node.

### 5.3.E	Rel-19 RAN3 SID - Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN (FS_NR_AIML_NGRAN_enh)

### 5.3.E.1	Description

The objective of this study is to further investigate new AI/ML based use cases and identify enhancements to support AI/ML functionality, and further discussions on the Rel-18 leftovers. The detailed objectives of the study are listed as follows:

- Study two new AI/ML based use cases, i.e., Network Slicing and CCO, with existing NG-RAN interfaces and architecture (including non-split architecture and split architecture).

- Rel-18 leftovers as candidates for normative work, based on the Rel-18 principles, as follows:

  - Mobility optimization for NR-DC

  - Split architecture support for Rel-18 use cases based on the conclusions from Rel-18 WI

  - Energy Saving enhancements, e.g., Energy Cost Prediction

  - Continuous MDT collection targeting the same UE across RRC states

  - Multi-hop UE trajectory across gNBs.

**end-SP-241325-Nokia**

**SP-241317-NEC**

### 5.3.X	AI/ML related activities on WID/SID#941010

### 5.3.X.1	Description

Editor’s Note:	This clause will investigate and identify AI/ML related activities of all TSG RAN working groups of Rel-18 features and Rel-19.

This clause highlights the AI/ML-related activities for NG-RAN for Rel-18, which is a progression of the Rel-17 study which is documented in TR 37.817 [x7].  It outlines AI/ML related terminologies and other relevant specifications work  along with relevant references provided.

The clause 11.2 in TR 21.918 [x1] describes AI/ML for NG-RAN.  The impacted TSs and TRs shown in the table below are extracted from the 3GPP workplan.

Table 5.3.x.1-1: list of relevant TSs/TRs for NG-RAN AI/ML related work

| Unique_ID | Name | Acronym | Working Groups | SID/WID | Impacted TSs and TRs |
| --- | --- | --- | --- | --- | --- |
| 941010 | Artificial Intelligence (AI) / Machine Learning (ML) for NG-RAN | NR_AIML_NGRAN |  | RP-233441 | 38.300, 38.401, 38.420, 38.423 |
| 941110 | Core part: Artificial Intelligence (AI) / Machine Learning (ML) for NG-RAN | NR_AIML_NGRAN-Core | R3 | RP-233441 | 38.300, 38.401, 38.420, 38.423 |

### 5.3.X.2	Terminology

Editor's Note:	This clause describes AI/ML related terminology (i.e. set of definitions, acronyms)

### 5.2.X.2.1			Abbreviations

The clause 3.1 in TS38.300 [x2] lists the following AI/ML related abbreviations.

AI	Artificial Intelligence

ML	Machine Learning

The clause 3.1 in TS38.401[x3] lists the following AI/ML related abbreviations.

AI	Artificial Intelligence

The clause 3.2 in TS38.420 [x4] lists the following AI/ML related abbreviations.

AI	as defined in TS 38.300

The clause 3.2 in TS38.423[x5] lists the following AI/ML related abbreviations.

AI	Artificial Intelligence

ML	Machine Learning

### 5.2.X.2.2			Definitions

The clause 3.2 in TS38.300 [x2] lists the following AI/ML related definitions.

None

The clause 3.1 in TS38.401[x3] lists the following AI/ML related definitions.

AI/ML Model Inference: follows the definition of “AI/ML inference” as specified in clause 3.1 of TS 28.105 [x6].

AI/ML Model Training: follows the definition of “ML model training” as specified in clause 3.1 of TS 28.105 [x6].

The clause 3.1 in TS38.420 [x4] lists the following AI/ML related definitions.

None

The clause 3.1 in TS38.423 [x5] lists the following AI/ML related definitions.

None

### 5.3.X.3	Activities Summary

Editor's note:	This clause describes high-level AI/ML activities e.g. LCM for AI/ML, data collection/storage/exposure, model training/delivery/ (de)-activation/inference emulation, inference/storage/exposure, performance evaluation and accuracy monitoring. Sub-clause(s) may be added to capture details.

As described in the clause 11.2 in TR 21.918 [x1], the Rel-18 Artificial Intelligence (AI) / Machine Learning (ML) for NG-RAN specifies the data collection enhancements and signalling support within existing NG-RAN interfaces and architecture (only limited to non-split architecture) for three use cases, including AI/ML-based Network Energy Saving, Load Balancing and Mobility Optimization.

This work item is based on the conclusions captured in TR37.817, and the objectives of Artificial Intelligence (AI) / Machine Learning (ML) for NG-RAN are included in [x1].

The Release 18 AI/ML specifications for NG-RAN focused on data collection improvements and signaling support within existing NG-RAN interfaces, specifically for non-split architectures. The work builds on the Release 17 study documented in TR 37.817 [x7] and targets three main use cases: Network Energy Saving, Load Balancing, and Mobility Optimization.

Key Features and Use Cases:

- Network Energy Saving: It is introduced to optimize the overall energy consumption in an area of the Operator’s network by leveraging on the data collected in the RAN network. AI/ML based Energy Saving at a NG-RAN node essentially is for a trained AI model to make real-time decisions to either deactivate or activate a cell or a NG-RAN node to save network energy consumption.

- Load Balancing: It is introduced to improve the load balancing performance by using load predictions from AI/ML models and feedback information mirroring the performance of a UE after an offloading action.

- Mobility Optimization: 	It is introduced to improve successful handover rates and determine which is the best mobility target for maximisation of efficiency and performance.

The implementation supports different deployment scenarios where AI/ML model training and inference can occur in either NG-RAN nodes or the OAM systems. Signaling for AI/ML support in NG-RAN is designed to be use-case and data type agnostic, facilitating seamless integration across various network functions.

These AI/ML activities do not apply to ng-eNB and are aimed at improving overall network performance and user experience by leveraging data-driven insights within the NG-RAN.

**end-SP-241317-NEC**

*** End of Change ***