TSG SA Meeting #SP-105	SP-24xxxx

10 – 13 September 2024, Melbourne, Australia

- Title:		chapter 5.2 AI/ML related activities in TSG SA & CT Working Groups

- Source:

- Document for:				Approval

Agenda Item:				7

Work Item / Release:	FS_AIML_CAL/ Rel-19

- 1. Introduction

Chapter 5.2 AI/ML related activities in TSG SA & CT Working Groups was addressed by following documents:

| SP-241325 | 22.850: Rel-19 pCR TR 22.850 AIML related activities in 3GPP | Nokia | Rel-18 SA1 WID (AIML_MT) Rel-19 SA1 SID (FS_AIML_MT_Ph2) Rel-19 SA1 WID (AIML_MT_Ph2) Rel-18 SA2 WID (eNA_Ph3) Rel-18 SA2 WID (AIMLsys) Rel-19 SA2 SID (FS_AIML_CN) Rel-19 SA2 WID (AIML_CN) Rel-18 SA3 WID (eNA_SEC_PH3) Rel-19 SA3 SID (FS_AIML_CN_SEC) Rel-19 SA4 SID (FS_AI4Media) Rel-18 SA5 WID (AIML_MGT) Rel-19 SA5 SID (FS_AIML_MGT_Ph2) Rel-19 SA6 SID (FS_AIMLAPP) Rel-19 SA6 WID (AIML_App) |
| --- | --- | --- | --- |
| SP-241326 | 22.850: Rel-19 pCR TR 22.850 AIML terminology analysis across 3GPP | Nokia | Extract of terminology in chapter 5.2 to a separate chapter |
| SP-241336 | 22.850: Data collection, ML Model Storage Analytics and ML Model Accuracy Monitoring in Rel-18 eNA_Ph3 | vivo | R18 Enablers for Network Automation for the 5G System (5GS) Phase 3 (eNA_Ph3) |
| SP-241338 | 22.850: AI/ML related activities on FS_PAIDC_UPF | China Telecom | FS_PAIDC_UPF |
| SP-241339 | 22.850: AI/ML related activities on eNA_Ph3 | China Telecom | eNA_Ph3 |
| SP-241347 | 22.850: AI/ML related activities in SA & CT Working Groups | Rakuten Mobile, Inc | Table introduced, but more fits under 5.1 |

- 2. Proposal

Incorporate the following change in TR 22.850.

*** Begin of Change ***

## 5.2	AI/ML related activities in TSG SA & CT Working Groups

### 5.2.X	AI/ML related activities on WID/SID#X

### 5.2.X.1	Description

Editor's Note:	This clause will investigate and identify AI/ML related activities of all TSG SA&CT working groups of Rel-18 features and Rel-19.

### 5.2.X.2	Terminology

Editor's Note:	This clause describes AI/ML related terminology (i.e. set of definitions, acronyms)

### 5.2.X.3	Activities Summary

Editor's note:	This clause describes high-level AI/ML activities e.g. LCM for AI/ML, data collection/storage/exposure, model training/delivery/ (de)-activation/inference emulation, inference/storage/exposure, performance evaluation and accuracy monitoring. Sub-clause(s) may be added to capture details.

**SP-241325-Nokia**

### 5.2.A	Rel-18 SA1 WID - AI/ML model transfer in 5GS (AIML_MT)

### 5.2.A.1	Description

The objective of this work item is to specify performance requirements (for end-to-end latency, experienced data rate, communication service availability) and service requirements (for AI/ML QoS management, AI/ML model /data distribution/transfer, network performance and resource utilization monitoring/prediction) for 5GS to support the following AI/ML operations for various applications (e.g. image/speech recognition, media editing/enhancements, robot control, automotive):

-	AI/ML operation splitting between AI/ML endpoints

-	AI/ML model/data distribution and sharing over 5G system

-	Distributed/Federated Learning over 5G system

### 5.2.B	Rel-19 SA1 SID - AI/ML Model Transfer Phase 2 (FS_AIML_MT_Ph2)

### 5.2.B.1	Description

The objective of this study is to explore new use cases and potential service and performance requirements to support efficient AI/ML operations using direct device connections. This includes:

-	Distributed AI training and inference based on direct device connections, such as traffic KPIs, various QoS, and functional requirements for sidelink transmission.

-	Considerations for charging and security aspects.

### 5.2.C	Rel-19 SA1 WID - AI/ML Model Transfer Phase 2 (AIML_MT_Ph2)

### 5.2.C.1	Description

The objective of this work item is to specify KPI and functional requirements for 5GS to support the AIML data transfer by leveraging direct device connection under 5G network control. These objectives were derived based on outcome of Rel-19 study in SA1 that relates to how the 5GS supports the transmissions of AI/ML-based services over the application layer. The study addressed use cases and potential performance requirements for 5G system support of application layer Artificial Intelligence (AI)/Machine Learning (ML) model distribution and transfer (download, upload, updates, etc.), and identified traffic characteristics of AI/ML model distribution, transfer and training for various applications, e.g. video/speech recognition, robot control, automotive, other verticals.

### 5.2.D	Rel-18 SA2 WID - Enablers for Network Automation for 5G – phase 3 (eNA_Ph3)

### 5.2.D.1	Description

The objective of this work item is to further enhance NWDAF, based on what has been specified in the previous releases to allow 5GS to support network automation. This work item focuses on architecture enhancement, new scenarios and the necessary inputs and outputs to the NWDAF based on the conclusions of the study in Rel-18. The work focuses on 10 key aspects as follows:

-	Improve correctness of NWDAF analytics

-	NWDAF-assisted application detection

-	Data and analytics exchange in roaming case

-	Enhancements on Data collection and Storage

-	Enhancements on trained ML Model sharing

-	NWDAF-assisted URSP

-	Enhancements on QoS Sustainability analytics

-	Supporting Federated Learning in 5GC

-	Enhancement of NWDAF with finer granularity of location information

-	Interactions with MDAS/MDAF

**SP-241336-vivo**

### **end-SP-241336-vivo

**SP-241339-China Telecom**

**end-SP-241339-China Telecom**

**SP-241325-Nokia**

### 5.2.E	Rel-18 SA2 WID - System Support for AI/ML-based Services (AIMLsys)

### 5.2.E.1	Description

This work item implements the conclusions of the Rel-18 study on the 5GS architectural and functional extensions to enable 5GS to assist the Application AI/ML operations. The normative text will be defined based on the agreed conclusions on 6 key issues, including unfinished discussions on proposals on how to define the aspects that are left to be finalized during normative work, and ensuring consistency with other 5GS features. The agreed conclusions focus on the following aspects:

-	Monitoring of network resource utilization to support the Application AI/ML operations

-	Expsoure of 5GC information to authorized 3rd party for Application AI/ML operations

-	Enhancement of external parameter provisioning in 5GC to assist the Application AI/ML operations

-	Enhancement in 5GC to enable Application AI/ML traffic transport

-	Enhancement of QoS and Policy control to support Application AI/ML data transport over 5GS

-	5GS assistance to federated learning operation

### 5.2.F	Rel-19 SA2 SID - Core Network Enhanced Support for Artificial Intelligence (AI)/Machine Learning (ML) (FS_AIML_CN)

### 5.2.F.1	Description

The aim of this study is to investigate and identify potential architectural and system-level enhancements to support AI/ML enhancements. Specifically, the objectives include:

- AI/ML Cross-Domain Coordination Aspects: Investigate enhancements to support AI-enabled RAN based on the conclusions of the RAN study in [a]. This task will discuss whether and how to support cross-domain (i.e., UE, RAN, 5GC, OAM, and AF) collaborative AI/ML mechanisms for the aspects described below.

  - Enhancements to LCS for AI/ML-Based Positioning: Examine whether and how to consider enhancements to LCS to support AI/ML-based positioning.

- Collaborative AI/ML Operations for Vertical Federated Learning (VFL): Determine potential enhancements needed to enable the 5G system to assist in collaborative AI/ML operations involving 5GC/NWDAF and/or AF for “Vertical Federated Learning (VFL).” This work will be based solely on and limited to the scope of justified use cases.

- Enhancements to Support NWDAF-Assisted Policy Control and Address Network Abnormal Behavior:

  - Investigate additional support needed to enhance 5GC NF operations (i.e., policy control and QoS) assisted by NWDAF. This task will first identify specific use cases to define the appropriate scope. It will analyze the impacts on NWDAF (e.g., the need to understand specific NF functionality) and the compatibility of new solutions with existing analytics to determine the necessity and benefits of new solutions.

  - Study the prediction, detection, prevention, and mitigation of network abnormal behaviors, such as signaling storms, with the assistance of NWDAF.

### 5.2.G	Rel-19 SA2 WID - Core Network Enhanced Support for Artificial Intelligence (AI)/Machine Learning (ML) (AIML_CN)

### 5.2.G.1	Description

The objective of this work item is to specify the following enhancements to 5GS as per the conclusions reached within the Rel-19 study for the following aspects:

- Enhancements to LCS to Support Direct AI/ML-Based Positioning

- LMF Enhancements: The LMF will be enhanced to perform location calculations based on an ML model. The triggers for data collection and model training within the LMF will be implementation-specific.

- MTLF and LMF Enhancements: Both the MTLF and the LMF will be enhanced to facilitate ML model training for AI/ML-based positioning.

- Procedure Development: Related procedures for data collection will be developed in coordination with RAN WGs.

- 5GC Support for Vertical Federated Learning

  - 5GC Enhancements: The 5GC will be enhanced to support vertical federated learning (VFL), a technique that does not involve exchanging or sharing local datasets or ML models, in the following scenarios:

-	VFL among NWDAFs within a single PLMN.

-	VFL between NWDAF(s) within a single PLMN and AF(s).NWDAF-Assisted Policy Control and QoS Enhancement

- NWDAF-Assisted Policy Control and QoS Enhancement

  - Assistance Information: Based on PCF requests, the NWDAF may provide assistance information to the PCF to aid in the determination and modification of QoS parameters.

- NWDAF Enhancements to Support Network Abnormal Behaviours Mitigation and Prevention

  - Signalling Storm Mitigation: The NWDAF will support signalling storm mitigation and prevention by providing analytics related to the detection and prediction of signalling storms caused by massive signalling from UEs and/or NFs.

### 5.2.H	Rel-18 SA3 WID - Security aspects of enablers for Network Automation for 5G - phase 3 (eNA_SEC_PH3)

### 5.2.H.1	Description

The main objective of this work is to produce normative specification based on the conclusions from Rel-18 study.  More specifically, the following objectives are expected to be specified:

-	Protection of data and analytics exchange in roaming case

-	Security for AI/ML model storage and sharing

-	Authorization of selection of participant NWDAF instances in the Federated Learning group

### 5.2.I	Rel-19 SA3 SID - Security aspects of Core Network Enhanced Support for AIML (FS_AIML_CN_SEC)

### 5.2.I.1	Description

The objectives of this study are the following:

- Security Aspects on Enhancements to LCS: Study security aspects on enhancements to LCS to support AI/ML-based positioning, considering the conclusions in [a] and [b].

- Security Aspects of Cross-Domain Vertical Federated Learning (VFL):

  - Authorization of VFL Group Members: Examine the authorization of members of the VFL group.

  - Security Aspects of Enhancements on SA2 Architecture: Investigate security aspects of enhancements on SA2 architecture to support VFL.

### 5.2.J	Rel-19 SA4 SID - Artificial Intelligence (AI) and Machine Learning (ML) for Media (FS_AI4Media)

### 5.2.J.1	Description

The primary objective of this study item is to identify relevant interoperability requirements and implementation constraints of AI/ML in 5G media services. The specific objectives include:

- Use Cases for Media-Based AI/ML Scenarios: List and describe the use cases for media-based AI/ML scenarios, based on those defined in [c].

- Media Service Architecture and Service Flows: Describe the media service architecture and relevant service flows for the scenarios. Identify the impacts on the architecture for each use case, including any potential gaps with existing 5G media service architectures. Also, describe the model operation configurations for each use case, including split AI/ML operations, and identify where certain AI/ML operations occur.

- Data Formats and Protocols: Identify and document the available data formats and suitable protocols for the exchange of different data components of various AI/ML models, such as model data, metadata, media data, and intermediate data necessary for such model operation configurations. Investigate the data traffic characteristics of these data components for delivery over the 5G system, including any needs and potentials for data rate reduction.

- Key Performance Indicators (KPIs): Identify and study key performance indicators for such scenarios, based on the initial considerations in [d]. Emphasize the use cases, model operation configurations, and data components identified in earlier objectives, focusing on objective performance metrics considering the identified KPIs.

- Normative Work and Collaboration: Identify potential areas for normative work as the next phase. Communicate and align with SA2 and other potential 3GPP working groups on relevant aspects related to the study.

### 5.2.K	Rel-18 SA5 WID - AI/ML management (AIML_MGT)

### 5.2.K.1	Description

The objective of this work is to specify the AI/ML management capabilities, including use cases, requirements and solutions for each phase of the AI/ML operational workflow for managing the AI/ML capabilities in 5GS (i.e., management and orchestration, 5GC and NG-RAN), including:

- Management capabilities for ML training phase, which includes control of producer-initiated ML training, data management for ML training, performance evaluation for ML training, ML entity validation, ML context management, ML entity capability discovery and ML entity testing

- Management capabilities for ML deployment phase, including management of ML entity loading

- Management capabilities for AI/ML inference phase.

To describe the deployment scenarios of the AI/ML management capabilities, with consideration of alignment with other relevant 3GPP WGs (e.g., RAN3, SA2) and ETSI ISG ZSM.

### 5.2.L	Rel-19 SA5 SID - AI/ML management - phase 2 (FS_AIML_MGT_Ph2)

### 5.2.L.1	Description

The objectives of this study item include:

- Continuation of AI/ML Studies: Continue the study on AI/ML emulation, AI/ML inference coordination, and ML knowledge transfer that are leftover from Rel-18.

- Management Aspects of AI/ML Functionalities Defined by Other 3GPP WGs:

  - AI/ML Model Transfer in 5GS: Study the management aspects (LCM, CM, and PM) of AI/ML model transfer in 5GS, as defined in SA1.

  - 5GS Support for AI/ML-Based Services: Investigate the management aspects of 5GS support for AI/ML-based services, as defined in SA2.

  - Support for AI/ML Services at Application Enablement Layer: Examine the management aspects of support for AI/ML services at the application enablement layer, as defined in SA6.

- Management Aspects of AI/ML Functionalities Defined by 3GPP SA5 WG:

  - Management Data Analytics (MDA) Phase 3: Study the management aspects (LCM, CM, and PM) of AI/ML functionalities defined by 3GPP SA5 WG, including MDA phase 3.

- AI/ML Management and Operation Capabilities: Investigate the AI/ML management and operation capabilities to support different types of AI/ML technologies needed for AI/ML in 5GS, such as Federated Learning, Reinforcement Learning, Online and Offline Training, Distributed Learning, and Generative AI.

- Sustainability Aspects of AI/ML:

  - Energy Consumption/Efficiency Impacts: Evaluate the energy consumption and efficiency impacts associated with AI/ML solutions for all operational phases (training, emulation, deployment, inference).

- Trustworthiness Aspects Related to AI/ML Functionalities in 5GS:

  - Concept of Trustworthiness: Further study the concept of trustworthiness for AI/ML in the context of OAM.

  - Data for Trustworthiness Indicators: Identify and analyze data (e.g., measurements, events) to support the calculation of trustworthiness indicators.

### 5.2.M	Rel-19 SA6 SID - Application layer support for AI/ML services (FS_AIMLAPP)

### 5.2.M.1	Description

The objective of this study is to enable support for AI/ML services at the application enablement layer. This includes the following:

- Analysis of Rel-18 and Rel-19 Requirements: Analyze the requirements in [d] related to AI/ML model distribution, transfer, and training. Identify key issues and develop corresponding architectural requirements at the application enablement layer, along with potential enhancements to the application layer architecture.

- Architectural and Functional Implications: Study the architectural and functional implications on existing SA6 application enablers (e.g., ADAES, other SEAL services, EDGEAPP) for supporting AI/ML lifecycle operations. This includes operations such as data collection, data preparation, training, inference, and federated learning for ML models used in ADAE layer analytics.

- Potential Solutions and APIs: Identify potential solutions, including information flows and developer-friendly application enablement APIs, to satisfy the architectural requirements and enhancements identified in the previous points.

- Impact on Deployments and Business Models: Investigate the possible impacts of application layer support for AI/ML services on different deployments and business models.

### 5.2.M	Rel-19 SA6 WID - Application enablement for AI/ML services (AIML_App)

### 5.2.M.1	Description

The objectives of this work include the following:

Develop Stage 2 normative technical specification for AIML enablement service as a new SEAL service, based on the key issues, architecture, solutions, and conclusions captured in [e]. The Stage 2 normative technical specification will include the following aspects:

-	Architecture requirements, deployment models and application architecture for AIML service enablement over 3GPP networks.

-	Procedures, information flows and APIs supporting concluded solutions related to AIML enablement capabilities for AI/ML, FL (e.g., Vertical FL among VAL UEs, Horizontal FL), Transfer Learning. Such capabilities include:

- Support AIML client management (e.g., registration, discovery) and selection.

- Support AIML service lifecycle management aspects (e.g., training, inference, data management).

- Support AIML operation split and ML model distribution operations.

- Support AIML operations in edge / distributed deployments.

-	Procedures information flows and APIs supporting concluded solutions for application layer support capabilities related to new ADAE analytics services. Such new analytics services include:

- DN Energy Analytics.

- Analytics for supporting FL.

Identify potential enhancements to other enablement frameworks (e.g. SEAL, EGDEAPP and CAPIF) based on the specified solutions for the above objectives.

** end SP-241325-Nokia**

**SP-241338-China Telecom**

**end-SP-241338-China Telecom**

**SP-241234-SA5**

- **end-SP241234-SA5**

**SP-241326-Nokia**

### 5.2.Y		AI/ML related terminology

#### 5.2.Y.1		TSG SA WG2

The following definitions are provided in clause 3 of 3GPP TS 23.288 [a]:

- Analytics Accuracy Information: Represent a performance measure of an analytics ID provided by an NWDAF containing AnLF, which is composed of the number of correct predictions of the analytics ID out of all predictions and the corresponding number of samples.

- ML Model Accuracy Information: Represent a performance measure of a ML Model provided by an NWDAF containing MTLF, which is composed of the number of correct predictions by the ML Model out of all predictions and the corresponding number of samples.

- Analytics Feedback Information: Indicates that the consumer NF has taken action(s) influenced by the previously provided analytics, which may or may not affect the ground truth data.

The following definitions are provided in clause 5 of 3GPP TS 23.288 [a]:

- Analytics logical function (AnLF): A logical function in NWDAF, which performs inference, derives analytics information (i.e. derives statistics and/or predictions based on Analytics Consumer request) and exposes analytics service i.e. Nnwdaf_AnalyticsSubscription or Nnwdaf_AnalyticsInfo.

- Model Training logical function (MTLF): A logical function in NWDAF, which trains Machine Learning (ML) models and exposes new training services (e.g. providing trained ML Model) as defined in clause 7.5 and clause 7.6.

The following definitions are provided in clause 3 of 3GPP TR 23.700-84 [b]:

- Horizontal Federated Learning (HFL): a federated learning technique without exchanging/sharing local data set, wherein the local data set in different FL clients for local model training have the same feature space for different samples (e.g. UE IDs).

- Vertical Federated Learning (VFL): a federated learning technique without exchanging/sharing local data set, wherein the local data set in different VFL Participant for local model training have different feature spaces for the same samples (e.g. UE IDs).

- Label: A label is the training objective in supervised machine learning.

- VFL Server: An NWDAF or AF that integrates local training results, computes gradient information or loss information and send them to VFL client(s) for the local ML model update in VFL training process. It also coordinates the VFL training process by discovering and selecting VFL clients. In VFL inference process, The VFL server aggregates local inference results from VFL clients to generate the final VFL inference result and sends the final VFL inference result to the consumer. Only one VFL server may exist for each VFL process.

- VFL Client: An NWDAF or AF that holds the local dataset and performs local training and inference as asked by VFL Server. There can be multiple VFL Clients in VFL training and inference.

The following definitions are provided in clause 6.15 of 3GPP TR 23.700-84 [b]:

- VFL active participant: A VFL function that owns part of an ML model for an analytic ID and knows the labels for the ML model. The active participant is the main function for training an ML model for an analytic ID.

- VFL passive participant: A VFL function that owns part of an ML model for an analytic ID but does not know the labels of the ML model but is able to collect local data for one or more features.

#### 5.2.Y.2		TSG SA WG5

The following definitions are provided in clause 3 of 3GPP TS 28.105 [c]:

- ML model: a manageable representation of an ML model algorithm.

  - NOTE 1: an ML model algorithm is a mathematical algorithm through which running a set of input data can generate a set of inference output.

  - NOTE 2: ML model algorithm is proprietary and not in scope for standardization and therefore not treated in this specification.

  - NOTE 3: ML model may include metadata. Metadata may include e.g. information related to the trained model, and applicable runtime context.

- ML model training: a process performed by an ML training function to take training data, run it through an ML model algorithm, derive the associated loss and adjust the parameterization of that ML model iteratively based on the computed loss and generate the trained ML model.

- ML model initial training: a process of training an initial version of an ML model.

- ML model re-training: a process of training a previous version of an ML model and generate a new version.

  - NOTE 4: a new version of a trained ML model supports the same type of inference as the previous version of the ML model, i.e., the data type of inference input and data type of inference output remain unchanged between the two versions of the ML model, but parameter values might be different for the re-trained model.

- ML model joint training: a process of training a group of ML models.

- ML training function: a logical function with ML model training capabilities.

- AI/ML inference function: a logical function that employs an ML model to conduct inference.

- ML model testing: a process of testing an ML model using testing data.

- ML testing function: a logical function with ML model testing capabilities.

- AI/ML inference: a process of running a set of input data through a trained ML model to produce set of output data, such as predictions.

  - NOTE 5: the inference represents the process to realize the AI capabilities by utilizing a trained ML model and other AI enablers if needed, hence the AI/ML prefix is used when referring to inference as compared to training and testing.

- AI/ML inference function: a logical function that employs trained ML model(s) to conduct inference.

- AI/ML inference emulation: running the inference process to evaluate the performance of an ML model in an emulation environment before deploying it into the target environment.

- ML model deployment: a process of making a trained ML model available for use in the target environment.

#### 5.2.Y.3		TSG SA WG6

The following definitions are provided in clause 3 of 3GPP TR 23.700-82 [d]:

- ML model: According to 3GPP TS 28.105 [c], mathematical algorithm that can be "trained" by data and human expert input as examples to replicate a decision an expert would make when provided that same information.

- ML model lifecycle: The lifecycle of an ML model includes data collection, data processing, model training, model verification, model, instantiation and deployment, model monitoring and termination of ML model components.

- ML model training: According to 3GPP TS 28.105 [c], ML model training includes capabilities of an ML training function or service to take data, run it through an ML model, derive the associated loss and adjust the parameterization of that ML model based on the computed loss.

- ML model inference: According to 3GPP TS 28.105 [c], ML model training includes capabilities of an ML model inference function that employs an ML model and/or AI decision entity to conduct inference.

- AI/ML enablement: an application enablement framework consisting of one or more AI/ML enabler capabilities based on implementation. Such function can be deployed as (or within) an enablement layer server (e.g. SEAL or ADAES) or client.

- AI/ML client: an application layer entity (also referred as ML client) which is an AI/ML endpoint and performs client-side operations (e.g. related to the ML model lifecycle). Such AI/ML client can be a VAL client or AIML enabler client and may be configured e.g. to provide ML model training and inference locally e.g. at the VAL UE side.

- AI/ML server: an application layer entity which is an AI/ML endpoint and performs server-side operations (e.g. related to the ML model lifecycle). Such AI/ML server can be a VAL server or AIML enabler server.

- FL member: An FL member or participant is an entity which has a role in the FL process. An FL member can be an FL client performing ML model training, or an FL server performing aggregation/collaboration for the FL process. The FL member in this study is assumed to be either a functionality at the VAL UE or at the network/server side (AIML enablement server, VAL server).

- FL client: An FL member which locally trains the ML model as requested by the FL server.

- FL server: An FL member which generates global ML model by aggregating local model information from FL clients.

**end-SP-241326-Nokia**

*** End of Change ***