TSG SA Meeting #SP-105	SP-241409

10 – 13 September 2024, Melbourne, Australia

# Title:				pCR on TR 22.850 clause 5.3

# Source:				Deutsche Telekom, Mediatek, NEC, Nokia, vivo, OPPO, Samsung

# Document for:				Approval

Agenda Item:				7

Work Item / Release:	FS_AIML_CAL/ Rel-19

# 1. Introduction

Clause 5.3 AI/ML related activities in TSG RAN Working Groups was addressed by following documents:

| SP-241125 | 22.850: Overview of AI/ML related activities in all Working Groups | MediaTek Inc. | List of features Rel18 and Rel19 of RAN1, Ran2, Ran3 under Chapter 5.1 General |
| --- | --- | --- | --- |
| SP-241317 | 22.850: AI/ML related activities in AI/ML for NG-RAN | NEC Corporation | NR_AIML_NGRAN NR_AIML_NGRAN-Core |
| SP-241325 | 22.850: Rel-19 pCR TR 22.850 AIML related activities in 3GPP | Nokia | Rel-18 RAN1 SID (FS_NR_AIML_Air) Rel-19 RAN1/RAN4 WID (NR_AIML_air) Rel-19 RAN2 SID (FS_NR_AIML_Mob) Rel-18 RAN3 WID (NR_AIML_NGRAN-Core) Rel-19 RAN3 SID (FS_NR_AIML_NGRAN_enh) |
| SP-241326 | 22.850: Rel-19 pCR TR 22.850 AIML terminology analysis across 3GPP | Nokia | Extract terminology section as a separate section |
| SP-241335 | 22.850: AI/ML Framekwork/Performance monitoring in R18 AIML for air interface | vivo | R18 Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface |
| SP-241351 | 22.850: AI/ML-Related Activities in RAN Working Groups | OPPO | FS_NR_AIML_Air (RAN1) NR_AIML_Air (RAN1) FS_NR_AIML_Mob (RAN2) SID NR_AIML_NGRAN and NR_AIML_NGRAN-Core (RAN3) |

# 2. Proposal

Incorporate the following change in TR 22.850.

*** Begin of Change ***

## 5.3	AI/ML related activities in TSG RAN Working Groups

Editor's Note:	Description clause will be further clarified whether to be based on WI summaries or WID objectives.

### 5.3.X	AI/ML related activities on WID/SID#X

### 5.3.X.1	Description

Editor's Note:	This clause will investigate and identify AI/ML related activities of all TSG RAN working groups of Rel-18 features and Rel-19.

### 5.3.X.2	Terminology

Editor's Note:	This clause describes AI/ML related terminology (i.e. set of definitions, acronyms)

### 5.3.X.3	Activities Summary

Editor's note:	This clause describes high-level AI/ML activities e.g. LCM for AI/ML, data collection/storage/exposure, model training/delivery/ (de)-activation/inference emulation, inference/storage/exposure, performance evaluation and accuracy monitoring. Sub-clause(s) may be added to capture details.

### 5.3.B	Rel-19 RAN1/RAN4 WID - Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface (NR_AIML_air)

### 5.3.B.1	Description

The objective of this work is to provide specification support for the following aspects:

- AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]:

  - Signalling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback

    - Identification related signalling is part of the above objective

  - Necessary signalling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring, data collection (except for the purpose of CN/OAM/OTT collection of UE-sided model training data) for both UE-sided and NW-sided models

  - Signalling mechanism of applicable functionalities/models

- Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]:

  - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”)

  - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”)

  - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any

  - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]:

  - Direct AI/ML positioning:

    - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning

    - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

    - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

  - AI/ML assisted positioning

    - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning

    - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

  - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any

  - Investigate and specify the necessary signalling of necessary measurement enhancements (if any)

  - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases

- Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]:

  - Specify necessary RAN4 core requirements for the above two use cases.

  - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring.

Study objectives with corresponding checkpoints in RAN#105 (Sept ’24):

- CSI feedback enhancement [RAN1]:

  - For CSI compression (two-sided model), further study ways to:

    - Improve trade-off between performance and complexity/overhead

      - e.g., considering extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction (compared to Rel-18 non-AI/ML based approach), etc.

    - Alleviate/resolve issues related to inter-vendor training collaboration.

  - For CSI prediction (UE-sided model), further study performance gain over Rel-18 non-AI/ML based approach and associated complexity, while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the [a] (e.g., cell/site specific model could be considered to improve performance gain).

- Necessity and details of model Identification concept and procedure in the context of LCM [RAN2/RAN1]

- CN/OAM/OTT collection of UE-sided model training data [RAN2/RAN1]:

  - For the FS_NR_AIML_Air study use cases, identify the corresponding contents of UE data collection

  - Analyse the UE data collection mechanisms identified during the FS_NR_AIML_Air (clause 7.2.1.3.2 of [a]) study along with the implications and limitations of each of the methods

- Model transfer/delivery [RAN2/RAN1]:

  - Determine whether there is a need to consider standardised solutions for transferring/delivering AI/ML model(s) considering at least the solutions identified during the FS_NR_AIML_Air study

- Testability and interoperability [RAN4]:

  - Finalize the testing framework and procedure for one-sided models and further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least:

    - Relation to legacy requirements

    - Performance monitoring and LCM aspects considering use-case specifics

    - Generalization aspects

    - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.)

    - UE processing capability and limitations

    - Post-deployment validation due to model change/drift

  - RAN5 aspects related to testability and interoperability to be addressed on a request basis

- For Beam Management and Positioning Accuracy enhancement use cases, specify performance requirements and test cases for AI/ML LCM procedures (including performance monitoring) and UE features enabled by UE-sided models

  - Specify necessary performance requirements and tests (including metrics) for the above-mentioned use cases

  - Specify necessary test cases and performance requirements for LCM procedure, including performance monitoring.

### 5.3.C	Rel-19 RAN2 SID - AIML for mobility in NR (FS_NR_AIML_Mob)

### 5.3.C.1	Description

The study will focus on mobility enhancement in RRC_CONNECTED mode over air interface by following existing mobility framework, i.e., handover decision is always made in network side. Mobility use cases focus on standalone NR PCell change. UE-side and network-side AI/ML model can be both considered, respectively. The investigation is to evaluate potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover, considering the following aspects:

- AI/ML based RRM measurement and event prediction,

  - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2]

    - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2]

  - HO failure/RLF prediction (UE sided model) [RAN2]

  - Measurement events prediction (UE sided model) [RAN2]

- Study the need/benefits of any other UE assistance information for the network side model [RAN2]

- The evaluation of the AI/ML aided mobility benefits should consider HO performance KPIs (e.g., Ping-pong HO, HOF/RLF, Time of stay, Handover interruption, prediction accuracy, and measurement reduction) etc.) and complexity tradeoffs [RAN2]

- Potential AI mobility specific enhancement should be based on the Rel19 AI/ML-air interface WID general framework (e.g. LCM, performance monitoring etc) [RAN2]

- Potential specification impacts of AI/ML aided mobility [RAN2]

- Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4]

### 5.3.D	Rel-18 RAN3 WID - Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN (NR_AIML_NGRAN-Core)

### 5.3.D.1	Description

The objective of this work is to specify data collection enhancements and signaling support within existing NG-RAN interfaces and architecture (including non-split architecture and split architecture) for AI/ML-based Network Energy Saving, Load Balancing and Mobility Optimization.

Support of AI/ML for NG-RAN, as a RAN internal function, is used to facilitate Artificial Intelligence (AI) and Machine Learning (ML) techniques in NG-RAN. The objective of AI/ML for NG-RAN is to improve network performance and user experience, through analysing the data collected and autonomously processed by the NG-RAN, which can yield further insights, e.g., for Network Energy Saving, Load Balancing, Mobility Optimization.

Support of AI/ML in NG-RAN requires inputs from neighbour NG-RAN nodes (e.g., predicted information, feedback information, measurements) and/or UEs (e.g., measurement results).

Signalling procedures used for the exchange of information to support AI/ML in NG-RAN are use case and data type agnostic, which means that the intended usage of the data exchanged via these procedures (e.g., input, output, feedback) is not indicated. The collection and reporting of information are configured through the Data Collection Reporting Initiation procedure, while the actual reporting is performed through the Data Collection Reporting procedure.

Support of AI/ML in NG-RAN does not apply to ng-eNB.

For the deployment of AI/ML in NG-RAN, the following scenarios may be supported:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the NG-RAN node.

-	AI/ML Model Training and AI/ML Model Inference are both located in the NG-RAN node.

### 5.3.E	Rel-19 RAN3 SID - Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN (FS_NR_AIML_NGRAN_enh)

### 5.3.E.1	Description

The objective of this study is to further investigate new AI/ML based use cases and identify enhancements to support AI/ML functionality, and further discussions on the Rel-18 leftovers. The detailed objectives of the study are listed as follows:

- Study two new AI/ML based use cases, i.e., Network Slicing and CCO, with existing NG-RAN interfaces and architecture (including non-split architecture and split architecture).

- Rel-18 leftovers as candidates for normative work, based on the Rel-18 principles, as follows:

  - Mobility optimization for NR-DC

  - Split architecture support for Rel-18 use cases based on the conclusions from Rel-18 WI

  - Energy Saving enhancements, e.g., Energy Cost Prediction

  - Continuous MDT collection targeting the same UE across RRC states

  - Multi-hop UE trajectory across gNBs.

### 5.3.Y		AI/ML related terminology

#### 5.3.Y.1		TSG RAN WG1

The following definitions are provided in clause 3 of 3GPP TR 38.843 [e]:

- AI/ML-enabled Feature: refers to a Feature where AI/ML may be used.

- AI/ML Model: A data driven algorithm that applies AI/ML techniques to generate a set of outputs based on a set of inputs.

- AI/ML model delivery: A generic term referring to delivery of an AI/ML model from one entity to another entity in any manner. Note: An entity could mean a network node/function (e.g., gNB, LMF, etc.), UE, proprietary server, etc.

- AI/ML model ID:  A logical AI/ML model is identified by a Model ID. The Model ID, if needed, can be used in a Functionality (defined in functionality-based LCM) for LCM operations.

- AI/ML model Inference:  A process of using a trained AI/ML model to produce a set of outputs based on a set of inputs.

- AI/ML model testing: A subprocess of training, to evaluate the performance of a final AI/ML model using a dataset different from one used for model training and validation. Differently from AI/ML model validation, testing does not assume subsequent tuning of the model.

- AI/ML model training: A process to train an AI/ML Model [by learning the input/output relationship] in a data driven manner and obtain the trained AI/ML Model for inference.

- AI/ML model transfer: Delivery of an AI/ML model over the air interface in a manner that is not transparent to 3GPP signalling, either parameters of a model structure known at the receiving end or a new model with parameters. Delivery may contain a full model or a partial model.

- AI/ML model validation: A subprocess of training, to evaluate the quality of an AI/ML model using a dataset different from one used for model training, that helps selecting model parameters that generalize beyond the dataset used for model training.

- Data collection: A process of collecting data by the network nodes, management entity, or UE for the purpose of AI/ML model training, data analytics and inference.

- Federated learning / federated training: A machine learning technique that trains an AI/ML model across multiple decentralized edge nodes (e.g., UEs, gNBs) each performing local model training using local data samples. The technique requires multiple interactions of the model, but no exchange of local data samples.

- Functionality identification: A process/method of identifying an AI/ML functionality for the common understanding between the NW and the UE. Note: Information regarding the AI/ML functionality may be shared during functionality identification. Where AI/ML functionality resides depends on the specific use cases and sub use cases.

- Management instruction: Information needed to ensure proper inference operation. This information may include selection/(de)activation/switching of AI/ML models or AI/ML functionalities, fallback to non-AI/ML operation, etc.

- Model activation: enable an AI/ML model for a specific AI/ML-enabled feature.

- Model deactivation: disable an AI/ML model for a specific AI/ML-enabled feature.

- Model download: Model transfer from the network to UE.

- Model identification: A process/method of identifying an AI/ML model for the common understanding between the NW and the UE. Note: The process/method of model identification may or may not be applicable. Note: Information regarding the AI/ML model may be shared during model identification.

- Model monitoring: A procedure that monitors the inference performance of the AI/ML model.

- Model parameter update: Process of updating the model parameters of a model.

- Model selection: The process of selecting an AI/ML model for activation among multiple models for the same AI/ML enabled feature. Note: Model selection may or may not be carried out simultaneously with model activation.

- Model switching: Deactivating a currently active AI/ML model and activating a different AI/ML model for a specific AI/ML-enabled feature.

- Model update: Process of updating the model parameters and/or model structure of a model.

- Model upload: Model transfer from UE to the network.

- Network-side (AI/ML) model: An AI/ML Model whose inference is performed entirely at the network.

- Offline field data: The data collected from field and used for offline training of the AI/ML model.

- Offline training: An AI/ML training process where the model is trained based on collected dataset, and where the trained model is later used or delivered for inference. Note: This definition only serves as a guidance. There may be cases that may not exactly conform to this definition but could still be categorized as offline training by commonly accepted conventions.

- Online field data: The data collected from field and used for online training of the AI/ML model.

- Online training: An AI/ML training process where the model being used for inference) is (typically continuously) trained in (near) real-time with the arrival of new training samples.

- Reinforcement Learning (RL): A process of training an AI/ML model from input (a.k.a. state) and a feedback signal (a.k.a.  reward) resulting from the model’s output (a.k.a. action) in an environment the model is interacting with.

- Semi-supervised learning: A process of training a model with a mix of labelled data and unlabelled data.

- Supervised learning: A process of training a model from input and its corresponding labels.

- Test encoder/decoder for TE: AI/ML model for UE encoder/gNB decoder implemented by TE.

- Two-sided (AI/ML) model: A paired AI/ML Model(s) over which joint inference is performed, where joint inference comprises AI/ML Inference whose inference is performed jointly across the UE and the network, i.e, the first part of inference is firstly performed by UE and then the remaining part is performed by gNB, or vice versa.

- UE-side (AI/ML) model: An AI/ML Model whose inference is performed entirely at the UE.

- Unsupervised learning: A process of training a model without labelled data.

- Proprietary-format models: ML models of vendor-/device-specific proprietary format, from 3GPP perspective. They are not mutually recognizable across vendors and hide model design information from other vendors when shared. Note: An example is a device-specific binary executable format.

- Open-format models: ML models of specified format that are mutually recognizable across vendors and allow interoperability, from 3GPP perspective. They are mutually recognizable between vendors and do not hide model design information from other vendors when shared.

#### 5.3.Y.2		TSG RAN WG3

The following definitions are provided in clause 16.20 of 3GPP TS 38.300 [f]:

- AI/ML Model Training follows the definition of the "ML model training" as specified in clause 3.1 of TS 28.105 [c].

- AI/ML Model Inference follows the definition of the "AI/ML inference" as defined in clause 3.1 of TS 28.105 [c].

*** End of Change ***