3GPP TSG SA WG 1 Meeting #107 	S1-242029

Maastricht, The Netherlands, 19 – 23 Aug 2024	(revision of S1-24xxxx)

Source:	Nokia

pCR Title:	22.883 pCR on New Use case on UE tolerance to QoS degradation due to network energy saving

Draft Spec:	3GPP TR 22.883

Agenda item:	7.2 (FS_EnergyServ_Ph2)

Document for:	Approval

Contact:	Laurent-Walter Goix <laurent-walter.goix@nokia.com>

Abstract: This pCR proposes requirements to better control service adjustments targeting network energy saving.

1. Introduction

This pCR proposes a new use case related to UEs’ tolerance to service adjustments caused by network energy saving actions, which could vary based on subscription policies, the target service performance characteristics, or potential incentives.

2. Reason for Change

The main problem with known energy saving techniques is that it is unclear what tradeoff among power saving and QoS degradation is acceptable. Depending on load and network deployment, some changes to configuration due to energy saving cannot be adhered to without service/QoS degradation.

UEs may have different requirements with respect to QoS degradation when it comes to save network energy, especially when contractual SLAs are in place.

Yet users may be more tolerant if stimulated by some incentive, being it charging-related or service-related, thus allowing the network to perform actions targeting higher energy savings.

The present use case introduces different situations – and the associated requirements - for which the network willing to save energy checks for UE-acceptable “ES actions” before applying them.

3. Conclusions

None.

4. Proposal

It is proposed to agree the following use case and add it to TR 22.883.

FIRST CHANGE

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

ES action: a network action targeting energy saving (ES) that would result in service performance adjustments (e.g. degradation) of a data flow, if performed. An ES action consists of: service performance characteristics; charging and/or incentives characteristics; and applicability conditions (e.g. time, location, slice).

SECOND CHANGE (NEW TEXT)

## 5.x	Use case on UE tolerance to QoS degradation due to network energy saving

### 5.x.1	Description

By default, mobile networks seek least energy consumption. Network energy saving techniques try to optimize energy consumption without QoS degradation. Sometimes, they may identify further energy saving opportunities, but at the cost of QoS degradation. Actions from the network in order to save energy may target one or more UEs: in some cases, if a UE is generating very high energy consumption on a base station due to its location/radio conditions and heavy traffic, it may alone be identified by the network as potential candidate to propose an incentive in turn of service performance adjustments that could lower such energy consumption from a network perspective.

The main problem is that it is unclear what tradeoff is acceptable (e.g. tolerated by the application / end user) between energy saving and service adjustments, such as QoS degradation. Different kinds of behaviors are envisioned:

- not tolerant to any QoS degradation

- tolerant to some QoS degradation upon explicit/informed consent

- tolerant to some QoS degradation, although upon explicit/informed notification

- tolerant to any QoS degradation

Hence, tolerance to QoS degradation can vary case by case depending on the current UE/user activity, in particular based on the specific application/service. QoS degradation could take the form of a change in 5QI to accommodate more relaxed KPIs for example for conversational or streaming video services in particular when using GBR. However, there is a limit to a tolerable QoS degradation, which still needs to satisfy policy rules or operator policies in place for a service, even in degraded conditions. Note that this use case does not apply to best effort traffic, as the network would not be able to evaluate an alternative configuration of the QoS attributes.

Furthermore, users may be more tolerant if stimulated by some incentive, being it charging-related or service-related, thus favoring the network to perform network saving actions. In general, the network should trace energy saving evidence at the cost of QoS degradation, to avoid reducing the QoS performance to the minimum without any compensation from UE/user side.

The present use case introduces different situations for which the network has identified one or more alternative “candidate ES actions” to save energy but needs to check against UE/subscriber tolerance to the associated QoS degradation.

An ES action typically contains:

- (newly proposed) Service performance characteristics (e.g. reduced/augmented bitrate, latency, 5QI);

- Charging/incentives characteristics, e.g. lower energy credit consumption rate, rewarded credits, or related to future dynamic adjustments of the provided communication service (e.g. temporary improved QoS or service performance);

- Applicability conditions, which can be (semi-) permanent, or limited in terms of e.g., applicable network slice, time or area, application/service. Such conditions can also refer to explicit actions from UEs and/or users (e.g. move to another location, stay at a given location for a minimal duration).

Candidate ES actions can be checked against subscription policies, without UE involvement, before being applied by the network. Subscription policies can for example indicate whether the subscriber prefers to be informed when an ES action is applied, or to verify the candidate ES action before it is applied, or to reject by default, or to accept by default without being informed – also depending on the applicability conditions and the charging/incentive characteristics. When requiring verification, answers to candidate ES actions can be automatically handled by the UE or request explicit user involvement, including based on UE policies. ES action information can include human-readable information.

### 5.x.2	Pre-conditions

Tom is a site manager at a construction company. His company has subscribed a QoS-based 5G service for their mobile rugged devices, which are used for various professional tasks.

Their fleet of professional devices is optimized to prioritize QoS performance operation mode. However, they can signal their preference for energy saving operation mode e.g. when the battery goes below a certain threshold, i.e. if it can save sufficient battery in the device, then they can accept certain QoS degradation - but want to be notified.

Tom’s mobile network operator cares about energy saving and runs optimization algorithms to this end. In some cases the algorithms allow for saving without any QoS degradation, but in other cases, their recommendations would degrade the QoS of some UEs. This results in one or more candidate “ES actions” that could save more or less energy on the network side, whilst impacting differently the service performance of some session.

### 5.x.3	Service Flows

- Tom goes for a site inspection with his professional rugged tablet. As the inspection goes on, he notices some cracks in the concrete of foundations and decides to use its RemoteExpertise app to interact with one of the few remote experts of his company on video to understand the potential risks. Based on Tom’s subscription, this application leverages a dedicated slice.

- At some point during this video/AR session, the 5G network wants to save energy locally (e.g. based on energy price or supply mix change during some time period and some algorithm suggesting energy saving opportunities), which would degrade the QoS of UEs it is serving in that area. Based on internal logic, the network derives the following candidate ES actions based on the potential impacted UEs:

  - ES action 1 would allow the network to switch off the capacity cell, i.e. move Tom under the coverage layer,

  - ES action 2 would remove the video flow and only keep the audio flow

  - ES action 3 would degrade the QoS (e.g. by adjusting PDB/PER) of the video flow, probably resulting in lower resolution/quality.

When checking Tom’s subscription data and UE context, the network identifies that none of these candidate ES actions comply with its SLA and thus keeps serving him under the same service performance conditions.

- Tom is now having his weekly video call on his rugged tablet with his boss Alice. Part of it includes a tour of the construction site to let her see the progress. As the site building rises up, coverage quality varies across the site but still its QoS contract allows him to have good quality video call. However from a network operator perspective, such QoS is costly to maintain around the site.

- After 10mn of high-quality video call, Tom’s tablet starts to go low on battery and informs the network it can tolerate some QoS degradation. Based on this information, as Tom’s UE traffic is quite resource-consuming, the network tries again to identify opportunities to save energy. Similar candidate ES actions 1 to 3 are derived and checked against Tom subscription data and UE context. Given the recent indication from Tom’s UE, ES action 2 is now acceptable and thus the network applies it – as it saves more energy than ES action 3. As per Tom’s subscription, the network notifies him that some energy saving has been applied in respect of the contract.

### 5.x.4	Post-conditions

Tom continues to have a high-quality service despite some seldom notifications when his mobile network operator degrades QoS to save energy. In turn, his network operator manages to keep its energy consumption quite low over its serving area and attractive subscription plans despite providing good QoS.

### 5.x.5	Existing features partly or fully covering the use case functionality

The QoS Parameter Notification control method allows SMF to indicate whether notifications are requested from the NG-RAN when the "GFBR can no longer (or can again) be guaranteed" for a QoS flow during the lifetime of the QoS flow. This method also allows the NG-RAN to apply an Alternative QoS Profile for the QoS flow.

UE can already provide updated parameter list within the QoS flow descriptor, or even new 5QI for energy saving with relaxed QoS parameters values that most influence the energy consumption at the UE side. However, such mechanisms imply the UE to know and request specific QoS profiles, rather than enabling the network to maximize the trade-off between QoS and energy saving and adjust its service accordingly, whilst still meeting the UE requirements.

Editor’s Note: FFS to align with Rel-19 Stage-2 study conclusion on Energy Behaviour Assistance Information.

### 5.x.6	Potential new requirements needed to support the use case

[PR.5.x.6-1] Subject to operator’s policy, regulatory requirements and subscription policies, the 5G system shall enable to indicate and update UE tolerance to ES actions at least based on service performance adjustments, their applicability criteria (e.g. time, geographical area, slice) and charging/incentive characteristics.

NOTE 1: Tolerance can be indicated e.g. as simple flag or include specific parameters such e.g. 5QI, QoS characteristics.

[PR.5.x.6-2] The 5G network shall be able to identify UEs causing high energy consumption in the network.

NOTE 2: This requirement does not assume that “per-specific-UE” energy consumption is measured by the network. Rather this can be based on existing measurements (e.g. radio conditions, data volume) and statistical models.

[PR.5.x.6-3]  Subject to operator’s policy, regulatory requirements, subscription policies and user consent, the 5G network shall be able to identify candidate ES actions and perform service performance adjustments for UEs causing high energy consumption, including based on the energy-related characteristics of the network (i.e. energy consumption, energy supply mix, carbon footprint, energy capacity and availability conditions), on the applicability criteria (e.g. time, location, slice) and on tolerance indication of the target UEs.

NOTE 3: Candidate ES actions can be evaluated against subscription data and/or UE context and do not mandate any real-time involvement of the UE.

[PR.5.x.6-4] Subject to operator’s policy, regulatory requirements, subscription policies and user consent, the 5G network performing ES actions that degrade UEs service performance (e.g. different 5QI or relaxed QoS parameters) shall be able to inform timely the impacted UEs.

[PR.5.x.6-5] Subject to operator’s policy and regulatory requirements, the 5G network shall be able to trigger charging events related to each impacted UE once ES actions degrading service performance are executed. Such charging events shall include the service performance characteristics of the related data flow before and after the ES action.

NOTE 4: Such charging events can result in incentives to the subscriber e.g. by the 5G network, resulting in additional energy credits being provided to the subscriber based on the ES action, or by the operator providing discounts etc

END OF CHANGES