3GPP TSG-SA WG1 Meeting #107 	 S1-242277

Maastricht, The Netherlands, 19-23 August 2024

Title:	Use case on using a multi orbit satellite system to improve service reliability and availability

Agenda Item:	7.3

Source:	Viasat Inc., Inmarsat, Novamint, SES

Contact:		Nandan Das Nandan.das@viasat.com
Jim Petranovich jim.petranovich@viasat.com

Abstract:  In this note we present multiple use cases for multi-orbit satellite systems.  We consider satellites in geostationary, i.e. geosynchronous equatorial plane, orbits (GEO) orbits as well as non-geostationary (NGSO) orbits.  NGSO orbits could include medium (MEO), low (LEO), and highly elliptical (HEO) Earth orbits.  These use cases are envisaged for a system that has multiple satellites across different orbits, potentially with different characteristics and potentially using different frequency bands, coexisting in one network.

# Introduction:

Different satellite orbits bring different characteristics that prove advantageous in different applications.  E.g. GEO orbits, where the satellite has a field of view (FOV) of about 1/3 of the earth’s surface area is great for coverage and for flexibly moving bandwidth and capacity from areas of low demand to areas of high demand in real time. LEO orbits with an altitude of a few hundred km are useful for latency sensitive applications and in some cases can provide line-of-sight (LoS) advantages, link budget advantages (this may or may not be the case depending on the satellite size and orbit altitude) and in some cases polar or high latitude coverage depending on the constellation configuration.  HEOs are very effective at covering latitudes close to the polar regions, and typically cover polar regions of the Earth very effectively.   A system that combines some or all these orbits in a multi-orbit satellite network will be able to take advantage of these characteristics to cover a wide range of use cases that would be unlikely to be satisfied using only one orbit.

In this paper, we describe one of multiple such use cases.

---------- Use Case template ----------

## 2.2	Use case on using a multi orbit satellite system to improve service reliability and availability.

### 2.2.1	Description

NGSO satellite communications necessarily require the UE to track one or more satellites as they move through the sky.  This often results in blockage (and thus interrupted communications) due to trees, buildings, mountains etc., especially when the satellites are at a low elevation angle in the sky.  Moreover, when the NGSO constellation is being deployed, initially, there may not be a satellite in view of a UE at all times.  In fact, NGSO constellations can be designed in a way where areas of low usage may not see a satellite at all times even when the NGSO constellation is fully deployed.  The GEO satellite, however, is at fixed spot (relative to the user) at all times.  Thus, a GEO satellite can be used to improve availability of the satellite network as the UE can use the GEO satellite when the NGSO is blocked or unavailable.  NGSO satellites often have limited active duty cycles and may be turned off in some locations to conserve satellite power

Conversely, the GEO satellite itself may be blocked if the UE doesn’t have a good visual of the GSO arc.  At high latitudes, closer to the poles, where the look angles to the GEO satellite can be very low, availability can be enhanced by using NGSO satellites that have good coverage over the polar regions.

Other than due to line-of-sight blockages, one satellite path may be unavailable due to other reasons (e.g. congestion, partial network outage etc).  In these cases as well, the availability of a multi orbit alternate communication path enhances the reliability of the network.

### 2.2.2	Pre-conditions

- The network supports both GEO and NGSO satellites

- The network supports handover between GEO satellite(s) and NGSO satellite(s)

- UEs can close a link to both GEO and NGSO satellites

- UEs are able to handle the Doppler of NGSO satellites

- UEs are able to tolerate a differential delay inherent in the GEO and NGSO networks

- GEO satellites provide broad coverage via (potentially) spot beams or regional beams

- NGSO satellites with focused coverage via on-demand spot beams

- A network plan that deconflicts frequency allocations between the GEO and NGSO satellites.

- The network is able to determine the location of a user to a sufficient granularity in order to be able to activate or redirect the user to the correct NGSO coverage beam or cell

- The different satellite orbits employed by the network may be supported by the same or different RAT types

- The network must be able to switch traffic between GEO and NGSO satellites quickly, or receive and transmit traffic from multiple GEO and NGSO satellites simultaneously on the same coverage area.

- The network must be able withstand blockages of either the GEO or the NGSO link.

- The network must be able to detect blockages/outages of either the GEO or NGSO link.

### 2.2.3	Service Flows

- During communication, the network shall detect traffic interruption.

- The network shall determine the optimal path to route traffic and send traffic through that path.

### 2.2.4	Post-conditions

The end-user should have a more reliable network with higher availability.

### 2.2.5	Existing features partly or fully covering the use case functionality

3GPP Rel 19 NTN features which support the use of satellites shall cover the use case partially.

### 2.2.6	Potential New Requirements needed to support the use case

[PR 5.x.6.001] A 5G system with satellite access supporting multi orbit operation, shall be able to support a mechanism to split the same service flow down multiple paths with large differential delay profiles.