3GPP TSG-SA WG1 Meeting #107 	 S1-242361

Maastricht, The Netherlands, 19-23 August 2024	(revision of S1-242351)

Title:	Use case on using GEO for initial network entry and service initialization

Agenda Item:	7.3

Source:	Viasat Inc., Inmarsat, Novamint, SES

Contact:		Nandan Das Nandan.das@viasat.com
Jim Petranovich jim.petranovich@viasat.com

Abstract:  In this note we present multiple use cases for multi-orbit satellite systems.  We consider satellites in geostationary, i.e. geosynchronous equatorial plane, orbits (GEO) orbits as well as non-geostationary (NGSO) orbits.  NGSO orbits could include medium (MEO), low (LEO), and highly elliptical (HEO) Earth orbits.  These use cases are envisaged for a system that has multiple satellites across different orbits, potentially with different characteristics and potentially using different frequency bands, coexisting in one network.

# Introduction:

Different satellite orbits bring different characteristics that prove advantageous in different applications.  E.g. GEO orbits, where the satellite has a field of view (FOV) of about 1/3 of the earth’s surface area is great for coverage and for flexibly moving bandwidth and capacity from areas of low demand to areas of high demand in real time. LEO orbits with an altitude of a few hundred km are useful for latency sensitive applications and in some cases can provide line-of-sight (LoS) advantages, link budget advantages (this may or may not be the case depending on the satellite size and orbit altitude) and in some cases polar or high latitude coverage depending on the constellation configuration.  HEOs are very effective at covering latitudes close to the polar regions, and typically cover polar regions of the Earth very effectively.   A system that combines some or all these orbits in a multi-orbit satellite network will be able to take advantage of these characteristics to cover a wide range of use cases that would be unlikely to be satisfied using only one orbit.

In this paper, we describe one of multiple such use cases.

* * * First Change * * *

## 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

satellite access type: in the context of this study, it refers to a satellite access with a specific set of characteristics (including orbit type, orbital characteristics, satellite capabilities, IoT or NR, …)

network entry initialization: in the context of this study, it refers to initial steps required in entering the network, sufficient to validate that the UE is a valid authorized device and exchanges enough information to allow the UE to connect to a network of the preferred satellite access type.

network entry completion: in the context of this study, it refers to any remaining steps after network entry initiation has been completed to allow full registration of the UE.

* * Next Change * * *

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

GEO	Geostationary satellite Earth Orbit

GSO	Geosynchronous Orbit

LEO	Low-Earth Orbit

MEO	Medium-Earth Orbit

NGSO	Non-Geostationary Satellite Orbit

* * Next Change * * *

---------- Use Case template ----------

## 2.1	Use case on using GEO for initial network entry and service initialization

### 2.1.1	Description

GEO satellites have a much wider field of view than NGSO satellites and typically have longer active duty cycles than NGSO systems.  They can be used to transmit cell broadcast signals and system information on a very large coverage area very efficiently, where a GEO satellite cell coverage area may be of a size that is equivalent to one or several NGSO satellite footprints areas (and thus to several NGSO cells). Cell search and network entry can be a simpler process using GEO satellites, while cell search and network entry can be an extended process for a UE served by NGSO satellites exclusively.

Furthermore, satellites often present significant payload power budget constraints, which can be more severe for NGSO satellites. In such scenarios, an NGSO satellite’s active coverage footprint might be selectively reduced in order to conserve power.  This has however the effect of creating coverage holes under the footprint of the satellite, in which cells may not be activated.

The presence of a GEO satellite coverage overlay on top of NGSO allows to bridge coverage gaps by providing broadcast signal coverage and initial access for the network.  In this mode, we assume that upon power-on, an UE will perform initial cell search, initial access and network registration (also known as “network entry”)  into a GEO satellite.  During the initial access and registration procedure the network may choose to handover the UE to an available NGSO satellite.  The NGSO satellite may be instructed by the network to activate a dormant beam or cell with a specific set of resources in the targeted coverage area, or the UE may be instructed to join an existing NGSO beam or cell.

This concept can be generalized from GEO and NGSO to multiple satellite access types but the concept is most easily understood and justified when considering GEO and NGSO.

### 2.1.2	Pre-conditions

- The network supports multiple satellite access types

- The network supports handover between multiple satellite access types

- A network plan that deconflicts frequency allocations between the component satellite access types.

- The networks supports network entry initiation on one satellite access network and network entry completion on a different satellite access network.

### 2.1.3	Service Flows

- UE is turned on and wants to establish a connection

- UE does initial network entry on a GEO satellite (or more generally a satellite of the appropriate satellite access type)

- The system provides the UE enough information to enable network entry completion on a satellite of the NGSO (or more generally a satellite of the appropriate satellite network type)

- UE does network entry complete on an NGSO (or more generally a satellite of the completes

- UE  does network entry completion on an NGSO spot beam

### 2.1.4	Post-conditions

The UE is registered on the preferred satellite access network and carries out communication on that satellite network.

### 2.1.5	Existing features partly or fully covering the use case functionality

3GPP Rel 19 NTN features which support the use of satellites shall cover the use case partially.

### 2.1.6	Potential New Requirements needed to support the use case

[PR 5.x.6.001] A 5G system with multi orbit satellite access shall be able to support network entry initiation via one satellite access type and network entry completion via a different satellite access type.

[PR x.1.6-002] A 5G System supporting multi orbit satellite access shall enable the UE to select the relevant GSO for network entry initiation.