3GPP TSG SA WG 1 Meeting #107 	S1-242366

Maastricht, The Netherlands, 19-23 August 2024	(revision of S1-242360)

Source:	TNO, ESA, Thales, Novamint, SES, Viasat, Inmarsat

pCR Title:	Pseudo-CR on Use Case on multi-orbit satellite backhauling

Draft Spec:	3GPP TR 22.887 v0.1.0

Agenda item:	7.3

Document for:	Approval

Contact:	Relja Djapic (relja.djapic@tno.nl)

Abstract: Satellite backhauling exploiting and/or combining connectivity over multiple orbits needs to be supported by NTN systems. A requirement is proposed to address multi-orbit satellite backhauling.

1. Introduction

Multi-orbit satellite access makes part of the the objectives of the Study Item on satellite access - phase 4. The goal is to identify use-cases not yet covered in previous Releases and corresponding potential new service requirements. Rel-17 addressed the basic satellite backhauling over a single satellite. Rel-18 addressed satellite backhauling that allows for multiple hops over several satellites on a single satellite orbit.

The aim of this contribution is to introduce a use-case that would allow for backhauling over multiple satellite orbits.

2. Reason for Change

TR 22.887 v0.1.0 does not yet contain any use case that covers multi-orbit satellite backhauling.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.887 v0.1.0.

* * * First Change * * * *

## 5.x	Use case on Multi-orbit satellite backhauling

### 5.x.1	Description

Backhaul links are used as connectivity between a Core Network and Base Stations (gNBs). Various techniques could be used to provision backhauling, e.g., terrestrial fiber optical-, mmWave- or satellite-links. Simultaneously combining various backhaul link types could be of interest in cases when a single backhaul link cannot meet the prescribed requirements (e.g., capacity, latency). Specific characteristic of each backhaul link needs to be taken into account when combining it with another backhaul link type in order to achieve required QoS of each user within a group of users (aggregate QoS).

- MLK is an airline company known for its high end on-board services. MLK takes good care of its passenger satisfaction and intends to upgrade its on-board entertainment services with on-line messaging, internet access, video streaming and gaming. MLK is looking for connectivity solutions that will enable the upgrade of its on-board entertainment. Cost efficiency of the connectivity solution is of great importance for MLK in order to remain competitive on the tight airline market. In addition to passenger entertainment services, the airplane crew should exploit the connectivity solution for their mission critical communication related to e.g. traffic management.

- MOOI is a mobile network operator providing services on the mainland as well as the overseas territories, i.e. the island that is located in a remote geographical location. MOOI provides mobile connectivity services to the residents of the island through several base stations that are connected with the Core Network using satellite backhauling. Due to its natural beaty the island is occasionally visited by large yachting regattas. In such cases QoS of the end users might deteriorate due to the limited backhaul capacity. MOOI seeks for a solution that would assure requested QoS to the end users also in cases when large number of tourists visit the island. MOOI is searching for cost effective solutions.

### 5.x.3	Service Flows

A satellite operator SAOP having GEO and NGSO satellites in its satellite fleet offers backhauling connectivity solutions for the airplanes in MLK fleet anywhere around the globe. SAOP offers the most cost efficient solution for MLK as the different service types e.g., messaging, internet, video/gaming, will transfer their traffic over satellite backhauling links optimal for the selected service e.g., GEO, MEO, LEO, respectively.

While on the ground the airplane mounted base station is connected through a terrestrial backhaul. The passengers on board the airplane acquire wide variety of services (latency sensitive, latency tolerant, messaging, video or gaming) through the terrestrial backhaul or terrestrial mobile network. The airplane cockpit crew communicates (voice, data) with the air traffic control through a dedicate air traffic management communication links at the given airport as part of the take-off and flight preparation.

After the take-off, airplane mounted base station loses the terrestrial backhaul connectivity and switches over to satellite backhauling through multiple satellite orbits i.e., LEO, MEO and/or GEO satellites. Satellite backhauling through distinct orbits have different properties with respect to latency, data rate, spectrum, coverage and availability of the satellites in different orbits. 5G system is capable of differentiating traffic among various service types of the airplane passengers and the airplane crew. For example, 5G system will route latency tolerant services (e.g. messaging) through GEO satellite backhaul, latency sensitive services (e.g. mission critical communication, on-line gaming gaming)  through LEO satellite backhaul and video on demand through MEO satellite backhaul.

### 5.x.4	Post-conditions

MLK improved offer of their services in a cost efficient manner. MLK is able to provide wide range of services (latency sensitive, latency tolerant, high data rate and high reliability) by simultaneously exploiting multiple satellite backhauling paths. The cabin (cockpit) crew is able to establish mission critical communication through the satellite backhaul having the lowest latency among various multi-orbit backhauling paths.

### 5.x.5	Existing features partly or fully covering the use case functionality

3GPP TS 22.261 [2] specifies the requirements for satellite backhauling i.e. that the 5G system shall support a mechanism to determine suitable QoS parameters for traffic over a satellite backhaul, based e.g. on the latency and bandwidth of the specific backhaul.

3GPP TS 22.261 [2] specifies the requirement that the 5G system shall be able to support mechanisms to differentiate charging information for traffic carried over satellite backhaul.

The above listed requirement are focused on usage of a single satellite orbit. The cases of possible simultaneous usage of multi-orbit satellite connectivity is not explicitly considered.

### 5.x.6	Potential New Requirements needed to support the use case

[PR 5.x.6-001] The 5G system using multi orbit satellite network for backhauling shall be able to take into account the respective capabilities (e.g., latency, data rate) and availability of the different satellite access (e.g. over GSO, MEO, LEO) to map the backhaul traffic with a given aggregated QoS.

[PR 5.x.6-002] The 5G system using multi orbit satellite network shall be able to map the backhaul traffic related to mission critical services (MCS) over the appropriate satellite access and meet MCS requirements.

[PR 5.x.6-003] The 5G system shall be able to collect charging information related to backhaul traffic over different satellite access.

* * * End of Changes * * * *