3GPP TSG-SA WG1 Meeting #107 	 S1-242510

Maastricht, The Netherlands, 19-23 August 2024	(revision of S1-242387)

Title:	Use case on using GEO to land data in a desirable geographical location

Agenda Item:	7.3

Source:	Viasat Inc., Inmarsat, Novamint, SES

Contact:		Nandan Das Nandan.das@viasat.com
Jim Petranovich jim.petranovich@viasat.com

Abstract:  In this note we present multiple use cases for multi-orbit satellite systems.  We consider satellites in geostationary, i.e. geosynchronous equatorial plane, orbits (GEO) orbits as well as non-geostationary (NGSO) orbits.  NGSO orbits could include medium (MEO), low (LEO), and highly elliptical (HEO) Earth orbits.  These use cases are envisaged for a system that has multiple satellites across different orbits, potentially with different characteristics and potentially using different frequency bands, coexisting in one network.

# Introduction:

Different satellite orbits bring different characteristics that prove advantageous in different applications.  E.g. GEO orbits, where the satellite has a field of view (FOV) of about 1/3 of the earth’s surface area is great for coverage and for flexibly moving bandwidth and capacity from areas of low demand to areas of high demand in real time. LEO orbits with an altitude of a few hundred km are useful for latency sensitive applications and in some cases can provide line-of-sight (LoS) advantages, link budget advantages (this may or may not be the case depending on the satellite size and orbit altitude) and in some cases polar or high latitude coverage depending on the constellation configuration.  HEOs are very effective at covering latitudes close to the polar regions, and typically cover polar regions of the Earth very effectively.   A system that combines some or all these orbits in a multi-orbit satellite network will be able to take advantage of these characteristics to cover a wide range of use cases that would be unlikely to be satisfied using only one orbit.

In this paper, we describe one of the multiple such use cases.

* * * First Change * * *

## 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Satellite ground station location: in the context of this study, it refers to the geographical area where a 5G system with multi orbit satellite access is providing the network access to a UE

* * Next Change * * *

---------- Use Case template ----------

## 2.3	Use case on using GEO to land data in a desirable geographical location

### 2.3.1	Description

NGSO satellites typically have a much smaller field of view (FOV) than a GEO satellite.  Thus NGSO satellites have to either download traffic from a UE to a gateway within its FOV, or transfer via inter satellite links (ISL) the data to another NGSO satellite in order to land the data outside the FOV of the satellite connecting to the UE.  These ISLs sometimes often do not have as much bandwidth as the primary links.  ISLs may also be unavailable under various conditions due to satellite duty cycle constraints or routing conflicts.

It is also desirable for many customers to download data to network based in a specific geographic areas.  This may be for intellectual property, security, or national security requirements.  Many customers may want to avoid areas that belong to adversarial nations or have geopolitical instability. We will refer to this as data with a preferred satellite ground station location.

Using a multi-orbit satellite system allows differentiated data landing based on customer needs.  E.g. the UE can connect to a NGSO and then have its data sent to a GEO satellite from which the data can be landed anywhere in the field of view of the GEO.  It may also be the case that the UE with data with preferred satellite ground station location, directly communicates with a GEO satellite bypassing the NGSO entirely.

### 2.3.2	Pre-conditions

- The network supports both GEO and NGSO satellites

- The network supports handover between GEO satellite(s) and NGSO satellite(s)

- A network plan that deconflicts frequency allocations between the GEO and NGSO satellites.

- The network is able to determine the location of a user to a sufficient granularity in order to be able to activate or redirect the user to the correct NGSO coverage beam or cell

- The network must be able to determine which data has a preferred satellite ground station location.

- The network must be able to determine whether each satellite network has access to the preferred satellite ground station location.

### 2.3.3	Service Flows

- During communication, the network or UE determines that data will be exchanged that has a preferred satellite ground station location

- The network shall determine the optimal path to route traffic and send traffic through that path.

### 2.3.4	Post-conditions

Data with preferred satellite ground station location is received securely at preferred location(s).

### 2.3.5	Existing features partly or fully covering the use case functionality

3GPP Rel 19 NTN features which support the use of satellites shall cover the use case partially.

### 2.3.6	Potential New Requirements needed to support the use case

[PR 5.x.6.001] A 5G system with multi orbit satellite access shall be able to support a mechanism for the network to deliver specific data traffic of a UE using preferred satellite ground station location.

NOTE: 	The link between satellite and satellite ground station location is out of 3GPP scope