SA WG2 Meeting #165	S2-2410768

- Hyderabad, India, 14 – 18 October 2024                                      revision of S2-2410227

Source:	OPPO

Title:	Interim conclusion on temporary ID

Spec:	3GPP TR 23.700-13

Agenda item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Document for:	Approval

Abstract of the contribution: This contributions is to conclude the temporary ID

1. Introduction

The AIoT Device ID can be allocated by the operator and by the 3rd party. The device ID will have some information to point to the domain which holds the subscription.

In the reply LS from RAN1 (S2-2409592)

RAN1 is studying several different A-IoT device architectures, which all are assumed to include a memory block described as follows:

- Memory can include two types of memory: 1) Non-Volatile Memory (NVM) such as EEPROM for permanently storing device ID, etc, and 2) registers for temporarily keeping any information required for its operation only while energy is available in energy storage.

Therefore, it can be assumed that an A-IoT device can incorporate an NVM in the device design, i.e., include an NVM in the BoM.

RAN1 would like to provide the following answer:

- An Ambient IoT device will be able to update its non-volatile memory at some point after receiving a trigger from the Reader. Writing may not always be possible at all times.

- The power consumption during writing to the NVM is higher than during reading.

- RAN1 thinks that frequent or recurring writing to NVM should be avoided.

- RAN1 has not discussed energy status and energy storage capabilities in relation to an Ambient IoT device ability to update its non-volatile memory.

From the above statement, it is seen that update the temporary ID the NVM is possible for AIoT devices. However it is also pointed that updating is not always possible.

Temporary ID is only required if the privacy protection is required and whether the privacy protection is in the scope of SA3. If the temporary ID allocation is required for the privacy protection, then the NW should be aware whether the temporary ID can be stored in the NVM.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-13.

* * * * First Change * * * *

## 8.2	Interim Conclusion on Key Issue #2

### 8.2.1	Identifier and Identification Management

The following aspects and principles are considered and agreed for the interim conclusion on Identifier  and Identification Management:

An ambient IoT Device Identifier can be assigned by an operator or by a third party.

Editor's note:	How it is used is FFS.

Editor's note:	Whether the temporary ID in the NAS layer is required for the privacy protection is FFS and is pending SA3 decision.

* * * * End of Changes * * * *