3GPP TSG-SA WG4 Meeting #128	S4-240883

Korea, Jeju, 20 – 24 May 2024

Source:	Fraunhofer Heinrich Hertz Institute (HHI), Nokia Corporation

Title:	[FS_AI4Media] NNC results for compression of model data for automatic speech recognition without data-driven tools

Agenda item:	9.6

Document for:	Agreement

- Introduction

# An objective of the study item on “Artificial Intelligence (AI) and Machine Learning (ML) for Media” (SP-230538) is to establish a framework for the evaluation of different AI/ML scenarios. One of the agreed scenarios is the “Transmission of compressed AI/ML model data for automatic speech recognition”. In this document, we present coding results obtained with an encoder producing bitstreams conforming to the NNC standard [1].

# More specifically, clause 2 of this document comprises the coding results, which have already been presented (in document S4aV240013) and discussed at the last online-meeting. They are now provided in this document with minor revisions to incorporate them to the evaluation permanent document.

# Clause 3 of this document describes of the software framework used to generate the results, which is currently available at https://vcgit.hhi.fraunhofer.de/tech/ai4media and attached to this document. The software framework is based on the existing scripts for model compression (which are already available in the 5G-MAG repository) and adds the following new features:

- Compression with NNC

- Running the compression pipeline with multiple configurations

- Plotting the RD-curves from the results

# Accordingly, clause 3 of this document extends the description of the existing framework in clause 2.1.2 of the evaluation permanent document by a description of the new features.

# We propose to:

- Add the coding results in clause 2 of this document to clause 10.1 of the evaluation permanent document.

- Add the modified framework description in clause 3 of this document to clause 2.1.2 of the evaluation permanent document.

- Add the modified scripts to the 5G-MAG AI-ML repository.

- Incorporate the updated clauses 2.1.2 and 10.1 of the evaluation permanent document also to the TR 26.847 (Evaluation of AIML in 5G).

- Modifications for clause 10.1 of the evaluation permanent document

Add the following sub-clause:

# 10.1.15 Coding results for NNC without data-dependent tools

To quantify possible data rate reductions with NNC [1], the weight tensors of the WAV2VEC2_ASR_BASE_960H model and the HUBERT_ASR_LARGE model (as described in clause 10.1.4) have been encoded with the software framework described in clause 2.1.2. This means, NNCodec [2], which is an open implementation of the NNC standard, has been used.

The models have been encoded without enabling enhanced or data-dependent encoding tools and also without employing encoder-only pre-processing techniques to the model. Consequently, results are representative for a straightforward use-case, which does not require test or training data for additional encoder-side re-training or additional inference steps for encoder-decisions. Table 10.1.15-1 provides details on the enabled NNCodec tools.

| NNCodec parameter | Value | Description |
| --- | --- | --- |
| use_dq | True | Dependent scalar quantization |
| codebook_mode | False | Integer codebook for transmission |
| param_opt | True | Parameter optimization for DeepCabac |
| cabac_unary_length_minus1 | 10 | Length of unary binarization part |
| opt_qp | True | QP optimization based on tensor statistics |
| ioq | False | Inference-optimized quantization |
| bnf | False | Batch-norm folding |
| lsa | False | Training-based local scaling adaptation |
| fine_tune | False | Training-based tuning of non-weight tensors |

Table 10.1.15-1: Enabled NNC tools as described in [2], other parameters are set to NNCodec’s default values.

To achieve different trade-offs between compressed model size and model performance, the models have been encoded with different quantization step sizes. More specifically, NNCodec’s quantization parameter (QPs) has been varied in the range from −15 and −45.

Figure 10.1.15-1 shows the results: The performance of the compressed model is reported as word error rate (wer). The size of the compressed model (cSize) is reported in percent of the original uncompressed model size (sizeAnc). In summary, the results show that NNC reduces the model size to about 10% to 15% with negligible model performance losses in a setup without any data-driven tools, or optimization techniques that modify the models before encoding. Higher reductions can might be possible when enabling more sophisticated encoding tools, as e.g. also data-dependent tools or additional encoder-only model optimization techniques.

Figure 10.1.15-1: Compressed model size and model performance achieved for different QPs. 
For reference, the anchor performance werAnc is shown as red line.

- Modifications for clause 2.1.2 of the evaluation permanent document

Add the revisions given by the changemarks.

### 2.1.2 	Scripts for the evaluation of compressed AI/ML model transmission

This clause describes the software framework for the evaluation of compressed AI/ML model transmission. The framework comprises python scripts implementing three main functionalities:

- A codec pipeline for encoding, decoding, and evaluation of AI/ML models with different parameters to obtain their performances and sizes before and after encoding (pipeline/run.py). The pipeline can be extended with new scenarios and compression methods in a modular way.

- An NNC evaluation script to evaluate the compression of the ASR scenario (as defined in clause 10.1) with NNC. The script invokes the codec pipeline multiple times with different QPs and stores results in csv-files (evaluations/evalNncAsr.py).

- A graph plotter creating pdfs from the csv-files obtained by the NNC evaluation script (evaluations/plotGraphs.py).

Furthermore, the framework includes bash-scripts to a) create a docker image comprising the python scripts and required dependencies and to b) run docker containers from the docker image.

#### 2.1.2.1	Software repository and installation

The software is available in the following git-repository:

- https://github.com/5G-MAG/rt-ai-ml-evaluation-framework/tree/main/scripts/asr/compression

The scripts can be used with or without a docker. Without docker, the framework can be installed as follows:

- Install a python 3.10 environment.

- Install packages listed in requirements.txt

pip install -r requirements.txt

- Install NNCodec as follows:

git clone --branch v0.3.1 https://github.com/fraunhoferhhi/nncodec.git

cd nncodec

pip install .

To run the scripts with docker, the framework comprises a Dockerfile to build an image with Ubuntu 22.04, python 3.10, packages in the requirements.txt, Nvidia GPU support, and NNCodec.

The image can be built as follows:

cd docker

./buildContainer.sh

Containers have been tested with CUDA version 12.0 and docker engine version 24.0.5.

#### 2.1.2.2	Codec pipeline

Figure 2.1.2.2-1 shows the process executed by the codec pipeline as simplified pseudo-code. First, the process instantiates a scenario object and a coder object. Then, the process obtains the anchor model from the scenario object. It derives the size of the anchor model and uses the scenario object to derive the anchor model’s performance. Subsequently, the coder object encodes the anchor model to a bitstream and decodes the bitstream to obtain the reconstructed model. Finally, the process derives the size of the bitstream, uses the scenario object to derive the reconstructed model’s performance and writes the results to a file as comma separated values (csv).

scenario  = scenario_factory.get( cfg )
  coder     = coder_factory.get( cfg, scenario )

  anc_model = scenario.get_model()

  results["anc_size"] = get_size( anc_model )
  results["anc_perf"] = scenario.get_performance( anc_model )

  bit_stream  = coder.encode( anc_model )
  rec_model   = coder.decode( bit_stream )

  results["rec_size"] = get_size( bit_stream )
  results["rec_perf"] = scenario.get_performance( rec_model )

  write_to_csv( results )

Figure 2.1.2.2-1: The main evaluation process (simplified pseudo-code)

#### 2.1.2.2.1	Configuration

The codec pipeline can be configured as shown in Table 2.1.2.2-1. Marks C, S, and R in the last column indicate that the parameters are directly forwarded to the coder object, the scenario object, and the result csv-file, respectively.

| Parameter name | Description | Forward |
| --- | --- | --- |
| coder_name | Name of the compression method | C,R |
| scenario_name | Name of the scenario | S,R |
| data_set_name | Name of the dataset | S,R |
| model_name | Name of the model (valid values depend on the scenario) | S,R |
| enc_cfg_file_name | Name of a config-file for the compression method | C |
| unique_tag | Unique tag added to output file-names | C,R |
| out_dir | Directory to store the csv-file, the bitstream, and other data | C |
| data_dir | Directory to model data and datasets | S |
| batch_size | Evaluation batch size (currently ignored) | S |
| workers | Number of workers for the data loader | S |
| disable_progress_bar | Disable progress bar | C, S |
| eval_compression | Compress and evaluate reconstructed model | R |
| eval_anchor | Evaluate anchor model | R |
| download_only | Only download models and datasets |  |

Table 2.1.2.2.1-1: Configuration parameters

The available scenarios, model and compression methods that are currently available are shown in Table 2.1.2.2.1-2.

| Type | Name | Description |
| --- | --- | --- |
| Scenario | asr | Automatic speech recognition. Available models: wav2vec_asr_base_960h and hubert_asr_large |
| Coder | dummy | Dummy methods. Writes parameters as unmodified 32-bit floating point values. Copies the anchor model to the reconstructed model. |
| Coder | nnc | NNCodec |

Table 2.1.2.2.1-2: Implemented scenarios and compression methods

The pipeline can, for example be started with:

python ./run --scenario_name="asr" --model_name="hubert_asr_large" 
       --coder_name="dummy"`

#### 2.1.2.2.2	Encoder configuration files

Encoder configuration files for NNC are provided in evaluations/cfg and can be specified using the parameter enc_cfg_file_name. They can for example be used as follows:

python ./run --scenario_name="asr" --model_name="wav2vec_asr_base_960h"
       --coder_name="nnc" --enc_cfg_file_name="../evaluations/cfg/QP_-22.json"

A description of the parameters can be found at https://github.com/fraunhoferhhi/nncodec/wiki/Usage. The parameters bitstream_path, model_name, and dataset_path are programmatically set by the codec pipeline. Since the framework currently only supports non-data driven tools, the parameters ioq, lsa, and fine_tune must always be equal to 0.

#### 2.1.2.2.3	Result csv file

Table 2.1.2.2.3-1 shows the results that are written to the result csv-file. Additionally, the configuration parameters marked with R in Table 2.1.2.2.1-1 are added.

| Name | Description | Unit |
| --- | --- | --- |
| anc_size | Size of the anchor model | byte |
| rec_size | Size of the bitstream | byte |
| compress_ratio | rec_size / anc_size | - |
| metric_name | Name of the metric | - |
| anc_perf | Performance of anchor model | Unit of metric_name |
| rec_perf | Performance of reconstructed model | Unit of metric_name |
| anc_eval_time | Evaluation time for anchor model | seconds |
| rec_eval_time | Evaluation time for reconstructed model | seconds |
| enc_time | Encoding time | seconds |
| dec_time | Decoding time | seconds |

Table 2.1.2.2.3-1: Results written to the csv-file

#### 2.1.2.2.4	Running the codec pipeline with docker

The codec pipeline can be run within a docker container by calling docker/runPipeline.sh from within the docker directory, as follows

cd docker

runPipeline.sh host_directory [other_parameters...]

with the following parameters:

- host_directory is a directory on the host system to which models/datasets are downloaded and to which results will be written.

- other_parameters are the other codec pipeline parameters, as specified above.

The script operates as follows:

- It mounts host_directory to the container.

- It sets the parameters out_dir and data_dir to host_directory\out and host_directory\data, respectively.

- It forwards other parameters to the invocation of pipeline/run.py within the container.

Example:

runPipeline.sh ~/myAiMlData --scenario_name="asr" --model_name="hubert_asr_large"
  --coder_name="dummy"

Configuration files for NNC, which are stored in evaluations/cfg, are also available within the docker image in directory framework/evaluations/cfg. The container can be run with a specific cfg-file, e.g. as follows:

runPipeline.sh ~/myAiMlData --scenario_name="asr" --model_name="hubert_asr_large"
   --coder_name="nnc" --eval_anchor=0 --unique_tag="test_hubert -22"
   --enc_cfg_file_name="/framework/evaluations/cfg/QP_-22.json"

#### 2.1.2.2.5	Extending the codec pipeline

The software framework allows to add new scenarios in a modular way. New scenarios must be provided as package containing a python class having the interface shown in Figure 2.1.2. 2.5-1. The parameters marked with S in Table 2.1.2.2.1-1 are forwarded to the init function of the Scenario class within the opts variable.

class Scenario():
  def __init__(self, opts):
    self.metric_name = "MetricOfScenario"
    # Input:
    # - opts: an object with members defining the scenario configuration
    # Should:
    # - define self.metric_name as string denoting the performance metric of

#   the scenario, which will be forwarded to the result csv-file
    # - init object from opts

  def get_model(self, pre_trained):
    # Input:

# - pre_trained a boolean indicating whether to provide the pre-trained model
    # Should download model data and datasets, when not already done
    # Output:
    # - If pre_trained is true, model should be a pre-trained model,
    #   Otherwise, model should be an un-initialized model
    return model

  def download_data_and_models(self):
    # Should download model data and datasets, when not already done

  def get_perf(self, model, partition, enforce_higher_is_better=False):
    # Inputs:
    # - model: the model to get the performance for
    # - partition: the partition of the dataset used for evaluation:
    #   - "test"  The test partition for final performance measurement should be used
    #   - "valid" The validation partition for data-driven methods should be used
    # - enforce_higher_is_better: if true perf should be increasing with increasing 
    #   model performance
    # Outputs:
    # - perf: the performance
    # - infer_time: the inference plus measurement time
    return perf, infer_time

Figure 2.1.2.2.5-1: Interface required to be implement for new scenarios

The software framework allows to add new compression methods in a modular way. New compression methods must be provided as package containing a python class having the interface shown in Figure 2.1.2.2.5-1. The parameters marked with C in Table 2.1.2.2.2-1 are forwarded to the init function of the Coder class within the opts variable.

class Coder():
  def __init__(self, opts ):
    self.__opts = opts
    # Inputs:
    # - opts: an object with members defining the coder configuration:
    #   - opts.file_names["bit"]: the bitstream filename
    #   - opts.file_names["dec"]: the decoded model filename
    #   - opts.scenario: the scenario object
    # Should init the coder object from the opts object

  def encode(self, model ):
    # Inputs:
    # - model: the model to encode
    # Should:
    # - Encode the state_dict() of model to the file given in
    #   self.__opts.file_names["bit"]

  def decode(self, rec_model):
    # Inputs:
    # - rec_model: the model to write the reconstructed parameters to
    # Should:
    # - decode the bitstream file given in self.__opts.file_names["bit"]
    # - store the decoded parameters in the state_dict of rec_model

Figure 2.1.2.2.5-2: Interface required to be implemented for new compression methods

Encoder-only optimization methods might use:

- the encode function to write optimized model parameters in a raw-byte format to the bitstream

- the decode function to read them back to rec_model.

#### 2.1.2.3 NNC evaluation script

The NNC evaluation script (evaluations/evalNncAsr.py) calls the codec pipeline multiple times while varying NNC’s quantization parameter (QP).

##### 2.1.2.3.1 Command line options

The command line parameters of the NNC evaluation script are shown in Table 2.1.2.3.1-1.

| Parameter name | Description |
| --- | --- |
| data_directory | Directory to store downloaded model data and results |
| model_name | Name of the model to evaluate |
| qp_start | Start of the QP range |
| qp_end | End of the QP range |
| qp_step | Step size of QP increments |

Table 2.1.2.3.1-1: Configuration parameters for the NNC evaluation script

The parameter model_name can be hubert_asr_large or wav2vec_asr_base_960h. The data_directory must have enough space, as also reconstructed models are stored

#### 2.1.2.3.2 Running the NNC evaluation script with docker

Docker containers with the NNC evaluation script can be run by calling docker/runEvalNncAsr.sh from within the docker directory, as follows:

cd docker

runEvalNncAsr.sh host_directory

The parameter host_directory specifies a directory on the host system to which models and datasets are downloaded and to which results are written.

The parameters passed to the NNC evaluation script in the container are currently hardcoded at the end of docker/runEvalNncAsr.sh. The codec pipeline is started with models wav2vec_asr_base_960h and hubert_asr_large and QPs in the range of −15 to −45, inclusive, for both models.

More specifically, for each of the two models, the docker/runEvalNnrAsr.sh script operates as follows:

- It mounts host_directory to the container.

- It calls evaluations/evalNncAsr.py within the container with data_directory=host_directory, model_name equal to the respective model, and qp_start=-45, qp_end=-15, and qp_step=1

When calling the codec pipeline, the pipelines’ parameters out_dir and data_dir are set to host_directory\out and host_directory\data, respectively.

Encoding the two models with all QPs on a single machine requires a significant amount of time. For testing purposes, it is recommended to reduce the QP range.

#### 2.1.2.4	Graph plotter

The graph plotter (evaluations/plotGraphs.py) creates pdfs from the csv-files generated by the NNC evaluation script.

#### 2.1.2.4.1 Command line options

The graph plotter supports the parameters shown in Table 2.1.2.4.1-1.

| Parameter name | Description |
| --- | --- |
| data_directory | Directory with csv-files; the PDF containing the plots will be written here |
| prefix | Prefix of the csv-files to be used for plotting |

Table 2.1.2.4.1-1: Configuration parameters for the Graph Plotter

The graph plotter considers the csv-files starting with prefix from data_directory. The created pdfs are also stored in data_directory.

#### 2.1.2.4.2 Running the graph plotter with docker

Docker containers with the graph plotter can be run by calling docker/runPlotGraphs.sh from within the docker directory, as follows:

cd docker

runPlotGraphs.sh host_directory

The parameters that are passed to the graph plotter in the container are currently hardcoded at the bottom of docker/runPlotGraphs.sh, so that results from calling docker/runEvalNncAsr.sh are plotted.

In particular, this means that the following is expected:

- csv-files are located in host_directory/out

- csv-file for the hubert_asr_large model start with asr_nnc_hubert_asr_large__

- csv-file for the wav2vec_asr_base_960h model start with asr_nnc_wav2vec_asr_base_960h__

The docker/runPlotGraphs.sh script operates as follows:

- It mounts the host_directory from the host to /container_directory in the container.

- It runs evaluations/plotGraphs.py in the container with data_directory=host_directory/out.

This creates two pdfs in host_directory/out. Examples for the outputs are given in evaluations/examples/. Because of floating point arithmetic, the results might not be exactly reproducible on different machines/environments.

runContainer.sh host_directory [other parameters...]

runContainer.sh ~/myAiMlData --scenario_name="asr" --model_name="hubert_asr_large" \

--coder_name="dummy"

- References

- ISO/IEC JTC 1/SC 29, “Compression of neural networks for multimedia content description and analysis (ISO/IEC 15938-17:2022)”, August 2022.

- Fraunhofer Neural Network Encoder/Decoder (NNCodec) [Computer software]. https://github.com/fraunhoferhhi/nncodec v0.3.1.