3GPP TSG SA WG4 Meeting #128	S4-240

# Jeju, Korea, 20th–24th May 2024

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.956 | CR | pseduo | rev |  | Current version: |  |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_Beyond2D] Scenario: Beyond2D Live Streaming |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile Com. Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_Beyond2D |  |  |  |  |  | Date: |  |  | 2024-05-20 |
| Category: |  |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Introduces a scenario on “ Beyond 2D Live Streaming”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Provide the following information on Beyond 2D Live Streaming: - Motivation - Description of the Anticipated Application - B2D Video Formats - Encoding and Decoding Constraints |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6 (add a new clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	Allied Market Research, “3D Technology Market Size, Share, Competitive Landscape and Trend Analysis Report by Product, Application : Global Opportunity Analysis and Industry Forecast, 2021-2030.”,  www.alliedmarketresearch.com/3d-technology-market.

[3]	Mordor Intelligence, “Mobile 3D Market Size & Share Analysis - Growth Trends & Forecasts (2024 - 2029).”,  https://www.mordorintelligence.com/industry-reports/mobile-3d-market.

[4]	Grand View Research, “Immersive Technology Market Size, Share & Trends Analysis Report By Component (Hardware, Software, Services), By Technology, By Application, By Industry, By Region, And Segment Forecasts, 2023 - 2030.”,  https://www.grandviewresearch.com/industry-analysis/immersive-technology-market-report.

[5]	3GPP TS 26.119: "Media Capabilities for Augmented Reality".

[6]	3GPP TS 26.118: "Virtual Reality (VR) profiles for streaming applications".

[7]	Wang, Y., Lu, Z., Cao, P. et al. How Live Streaming Changes Shopping Decisions in E-commerce: A Study of Live Streaming Commerce. Comput Supported Coop Work 31, 701–729 (2022). https://doi.org/10.1007/s10606-022-09439-2

[8]	Coresight Research, “Inside the $50 billion shopping spree happening on livestreams.”,  https://www.cnbc.com/video/2023/06/09/inside-the-50-billion-shopping-spree-happening-on-livestreams.html

[9]	3GPP TR 26.955: “5G Video Codec Characteristics”

[11]	3GPP TS 26.511: “5G Media Streaming (5GMS); Profiles, codecs and formats”

[12]	3GPP TS 26.116: “Television (TV) over 3GPP services; Video profiles”

Next change

6.X	Scenario X: Beyond 2D Live Streaming

6.X.1		Motivation

Live Streaming services can be deployed across various platforms, including social media platforms like YouTube Live, Facebook Live, and TikTok, as well as though e-commerce platforms such as eBay and Taobao [7]. It significantly impact marketing by providing a dynamic and interactive channel to directly connect  markets and their target audiences in real time. In 2023, US livestream sales are projected to reach $50 billion, according to Coresight Research [8].

In 3GPP TR 26.955 [9] clause 6.2, Full HD Streaming has been introduced, complemented by 5G Media Streaming [11] and the TV Video Profiles [12] specifications. Additionally, TS 26.118 [6] defines VR profiles for streaming applications, focusing on the coded representation of 360 VR distribution signals. However, advancements in capturing devices (e.g., ToF cameras, phones equipped with depth sensors, spatial cameras) and displays technology (e.g., HMDs, AR Glasses, MR HMDs, glasses-free autostereoscopic displays, and multiscopic displays), are reshaping video services beyond traditional two-dimensional formats. The integration of beyond 2D video into Live Streaming services can create a more life-like and immersive experience.

The scenario is shown in the below figure (NOTE that for TR, the figure likely cannot be re-used). For UE capable of directly capturing beyond 2D video on the device (e.g., UE equipped with ToF, LiDAR or Spatial camera), it pre-processes the captured video frames into a well-defined B2D format and sends them to the encoder as input. The encoded B2D video streams are then streamed to the streaming server within the network, where the server may transcode them into different bitrates and distribute them to various audiences. The receiving end decodes B2D video streams and perform post-processing to adapt to the rendering system.

For UE limited to capturing only 2D video (e.g., UE with a monocular camera), the UE initially encodes the regular 2D video and streams it to a cloud server capable of real-time 2D-to-beyond 2D conversion (the process is described in the pipeline below). The cloud server then encodes the converted B2D video and streams it to the streaming server.

- Pipeline for 2D-to-beyond 2D conversion:

On the other hand, AI-generated representations, particularly highlighted by techniques like NeRF/3DGS, have gradually emerged in the domain of beyond 2D video formats. These technologies show great promise in achieving high-resolution photorealistic results, real-time rendering, and bandwidth reduction.

For NeRF, it uses a single neural network to represent an entire 3D scene. As for 3DGS, proposed in the middle of the year 2023, it reconstructs 3D scenes by training Gaussian points in 3D space. Both methods can allow the obtaining of images directly from specified viewpoints. The training process for AI-generated representations only require images and camera parameters as input. The corresponding output images can be obtained by simply inputting the camera parameters.

[The informative knowledge:

Observations:

- Performance: the latest 3DGS model can perform fast-on-the-fly per-frame reconstruction within 12 seconds and real-time rendering at 200 FPS, and AI-generated representations are continually being optimized to meet the requirement to be used in streaming services.

- UE Support: 3DGS/NeRF can be used in mobile devices, e.g.,:

  - KIRI Engine - 3D Gaussian Splatting on Android and iOS mobile devices: https://rebusfarm.net/news/kiri-engine-3d-gaussian-splatting-on-android-and-ios

  - Generating 3D scenes from 2D images more efficiently – Mobile NeRF rendering using Vulkan on Adreno GPU

      - https://www.qualcomm.com/developer/blog/2023/12/generating-3d-scenes-2d-images-more-efficiently-mobile-nerf-rendering-using-vulkan-adreno-gpu

- Market services: These technologies are incorporated into services available in the market.

  - Nividia Instant NeRF: https://www.nvidia.com/en-us/research/nerf-vr-contest/

]

Based on the knowledge of AI-generated representations provided above, the workflow for AI-generated representations is described as follows:

UE first streams the encoded 2D video to a cloud server, which employs on-the-fly AI training to generate B2D video sequences (e.g., multi-view, multi-view+depth...) in real-time. These B2D video frames are then encoded in the server and streamed to a streaming server for distribution to the receiving ends. The receiving end decodes B2D video streams and perform post-processing to adapt to the rendering system.

6.X.2	Description of the Anticipated Application

In the context of 3GPP services, 5G Media Streaming [11] as well as the VR Profiles for streaming applications [6] are specifications address this streaming scenario. Both, 5G Media Streaming [11] and VR Profiles for streaming applications [6] builds on DASH distribution. From 3GPP TS 26.118, the following operation points may be considered in the scope of the Beyond 2D Streaming Scenario:

-	Basic H.264/AVC Operation Point as defined in TS 26.118 [6], clause 5.1.4

-	Main H.265/HEVC Operation Point as defined in TS 26.118 [6], clause 5.1.5

-	Flexible H.265/HEVC Operation Point as defined in TS 26.118 [6], clause 5.1.6

-	8K H.265/HEVC Operation Point as defined in TS 26.118 [6], clause 5.1.7

The considered scenario is the distribution of B2D video content through DASH/CMAF/HLS based streaming. Important aspects that are expected to be considered when evaluating a codec in the context of this:

-	Quality and Coding Efficiency:

-	High and uninterrupted visual quality, talking into account the service constraints

-	Any savings can provide significant benefits due to the expected large volume of the traffic either in quality or network utilization

- 	Considered settings for encoding:

-		Low latency Settings

-		Regular Random Access, typically every 1-2 seconds

-	Encoding in this scenario is typically done as:

-		Live and On-Demand distribution and encoding

- 	Server and Cloud-based Encoding

6.X.3	B2D Video Formats

Table 6.X.3-1 provides an overview of typical beyond 2D source signal properties for UE-to-UE Live Streaming. This information is used to select proper test sequences.

Table 6.x.3-1 Beyond 2D Source Format Properties for UE-to-UE Live Streaming

| Source format properties | B2D Live Streaming |
| --- | --- |
| Number of views | 2 |
| Spatial resolution for each view | - Full Width: - 3840 x 1080 - Full Height: - 1920 x 2160 - 2560 x 3200 - Half Width/Height: - 1920 x 1080 2560 x 1600 |
| Chroma format | Y’CbCr, RGB |
| Chroma subsampling | 4:2:0 |
| Picture aspect ratio | - 32:9 - 16:9 - 16:10 |
| Frame rates | 25, 30, 60, 90, 120 Hz |
| Bit depth | 8, 10 |
| Colour space formats | BT.709,BT.2020 |
| Transfer characteristics | BT.709,BT.2020 |

- 6.x.4	Encoding and Decoding Constraints

Table 6.x.4-1 provides an overview of encoding and decoding constraints for H.264/AVC and H.265/HEVC for UE-to-UE Live Streaming scenario. This information supports the definition of detailed anchor conditions.

Table 6.x.4-1 Encoding and Decoding Configurations

| Encoding and Decoding Constraints | H.264/AVC | H.265/HEVC |
| --- | --- | --- |
| Relevant Codec and Codec Profile/Levels | H.264/AVC Progressive High Profile 4.2 | H.265/HEVC Main 10 Profile Level 4.1, 5.1 |
| Random access frequency | 1 second | 1 seconds |
| Bit rates and quality configuration | Fixed QP CBR - Half Width/Height: 4-8Mbps - Full Width/Height: 8-16Mbps Capped-VBR | Fixed QP CBR - Half Width/Height: 5-8Mbps - Full Width/Height: 8-16Mbps Capped-VBR |
| Bit rate parameters (CBR, VBR, CAE, HRD parameters) | Covering a range of relevant bitrates and qualities | Covering a range of relevant bitrates and qualities |
| Latency requirements and specific encoding settings | Low latency requirements | Low latency requirements |
| Encoding complexity context | - Real-time encoding, Cloud-based encoding | Real-time encoding, Cloud-based encoding |
| Required decoding capabilities | H.264/AVC Progressive High Profile 4.2 | H.265/HEVC Main 10 Profile Level 4.1, 5.1 |

6.x.5	Performance Metrics

The following metrics are expected to be reported

<tbd>

End of changes