3GPP TSG-SA WG4 Meeting #128	S4-241008

Jeju Island, Korea, 20-25 May 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.253 | CR | - | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [ISAR] Metadata Size Field |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Fraunhofer IIS |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | ISAR, IVAS_codec |  |  |  |  |  | Date: |  |  | 2024-05-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | When packing the ISAR bitstream to RTP, it may be necessary to separate the metadata section and the audio section of each frame, for example when ISAR is transmitted via a system that uses an established transport protocol for the binaural audio for the center pose. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Size of the pose correction metadata section is written to each ISAR frame. Split rendering configurations with pose correction. Split rendering without pose correction (0-DOF) is unaffected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | High burden to use ISAR pose correction metadata with an existing transmission system that allows transmission of binaural audio. Without the metadata section size being known, that section would have to be decoded in full to arrive at the audio section. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.6.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 26.250 (pseudo-CR) TS 26.254 CR 0001 TS 26.255 CR 0001 TS 26.258 CR 0001 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Code is available at https://forge.3gpp.org/rep/ivas-codec-pc/ivas-codec/-/merge_requests/1547/ |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional information

## Motivation

The source verified the impact of the change by running a PEAQ analysis on listening test material according to the ISAR selection relative to the switch being disabled and relative to the selected ISAR version that has undergone the ISAR selection testing. It has been processed using the available ISAR selection scripts according to the setups for BS1534-1, BS1534-2, BS1534-3, and BS1534-4 and analyzed using the PEAQ objective audio analysis tool to provide insights into the effect of the change. All ISAR Selection operation modes were evaluated for the configuration at that time, i.e. 768kbps with LCLD or LC3plus being the default codecs for SBA/MASA or ISM/MC, respectively.

## Test conditions

Three test conditions were in the test, termed UT1, UT2, and UT3 as descreibed in the following

### UT1

ISAR executables based on the code available at https://forge.3gpp.org/rep/ivas-codec-pc/ivas-codec/-/merge_requests/1547/  (which is based on the most recent main available by the IVAS Public Collaboration, sha: a0f5a8423 on branch isar_metadata_size_field ) with the switch SPLIT_REND_METADATA_SECTION_SIZE enabled.

### UT2

Same as UT1 with the switch SPLIT_REND_METADATA_SECTION_SIZE disabled.

### UT3

ISAR executable based on the code that has been selected (tag 2024_03_14-isar-selection).

## Results

PEAQ analysis of the change suggest that there is no significant difference due to the reduction of the total bit rate by 13 bits per frame, which is 13/(1920*8) bits, resulting in 0.08% of additional bit rate. In addition, there is no significant difference between UT1, UT2 and UT3 while UT1 provides the additional benefit of flexibility in selecting transmission methods that use a consolidated ISAR bit stream or other embedding methods that require a different ordering, enabling applications such as pose correction for legacy systems that are e.g. based on LC3plus.

* * * First Change * * * *

### 7.6.7	Bit allocation for Split rendering

Detailed bit allocation principles at different bitrates of the Split rendering operation are provided in Table 7.6-21, Table 7.6-22, Table 7.6-23,Table 7.6-24 and Table 7.6-25.

Table 7.6-21: Bit allocation for split rendering with 1, 2 and 3 DOF pose correction with LCLD or LC3plus codecs

| Description | 384 kbps | 512 kbps | 768 kbps |
| --- | --- | --- | --- |
| Total number of bits per 20ms frame | 7680 | 10240 | 15360 |
| DOF (degree of freedom) | 2 | 2 | 2 |
| HQ mode | 1 | 1 | 1 |
| Rotation axes | 3 | 3 | 3 |
| Reference Pose P’ (yaw, pitch and roll angle) | 27 | 27 | 27 |
| Coding strategy | 1 | 1 | 1 |
| Quantization strategy | 2 | 2 | 2 |
| Metadata size | 13 | 13 | 13 |
| Metadata bits | variable | variable | variable |
| LCLD or LC3plus bits | variable | variable | variable |

Table 7.6-22: Bit allocation for split rendering with 0 DOF (no pose correction) with 5ms split rendering frame size

| Description | 256 kbps | 384 kbps | 512 kbps |
| --- | --- | --- | --- |
| Total number of bits per 5ms frame | 1280 | 1920 | 2560 |
| LCLD or LC3plus bits | 1280 | 1920 | 2560 |

Table 7.6-23: Bit allocation for split rendering with 0 DOF (no pose correction) with 10ms split rendering frame size

| Description | 256 kbps | 384 kbps | 512 kbps |
| --- | --- | --- | --- |
| Total number of bits per 10ms frame | 2560 | 3840 | 5120 |
| LCLD or LC3plus bits | 2560 | 3840 | 5120 |

Table 7.6-24: Bit allocation for split rendering with 0 DOF (no pose correction) with 20ms split rendering frame size

| Description | 256 kbps | 384 kbps | 512 kbps |
| --- | --- | --- | --- |
| Total number of bits per 20ms frame | 5120 | 7680 | 10240 |
| LCLD or LC3plus bits | 5120 | 7680 | 10240 |

Table 7.6-25: Bit allocation for split rendering with 1, 2 and 3 DOF pose correction with PCM binaural output

| Description | Bitrate >= 50 kbps |
| --- | --- |
| Total number of bits per 20ms frame | >= 1000 |
| DOF (degree of freedom) | 2 |
| HQ mode | 1 |
| Rotation axes | 3 |
| Reference Pose P’ (yaw, pitch and roll angle) | 27 |
| Coding strategy | 1 |
| Quantization strategy | 2 |
| Metadata size | 13 |
| Metadata bits | Remaining bits |

* * * End of Changes * * * *