3GPP TSG-SA4 Meeting # 128 	S4-241078

# Jeju, KR, 20th - 24th May 2024

Source:	Apple Inc.

Title:	Updates on Colour Format Enhancement Derivation in HEIF

Agenda item:	X.X

Document for:	Information

# 1 Overview

This document provides an update to the SA4 on the progress done at MPEG for definition of an image derivation item in HEIF that can be used for chroma enhancement applications. This approach can be used to enhance a base layer image coded in a lower capability colour format representation, such as 4:2:0 YCbCr, to produce a higher capability colour format representation, such as 4:4:4 YCbCr or RGB, using the newly defined metadata. One benefit of this approach is that it permits decoders that are not capable of native 4:4:4 HEVC coding to still be able to decode 4:4:4 content through simple software support.

An item with an item_type value of 'cfen' is currently defined in the HEIF amendment as a colour format enhancement derived image item whose reconstructed image is formed from one or more input images that carry components in the luma plane that are used to reconstruct a picture with an enhanced colour format [1], [2].

The input images are ordered using the reference type 'dimg' for this colour format enhancement derived image item within the ItemReferenceBox, where the value of from_item_ID identifies the colour format enhancement derived image item, and the values of to_item_ID identify the input images. The reference_count gives the number of input image items and shall be greater or equal to one. The currently defined syntax in the working draft is as follows:

aligned(8) class ColourFormatEnhancement {
	unsigned int(8) version = 0;
	for (i=0; i<reference_count; i++) {
		bit(7) reserved = 0;
		unsigned int(1) is_packed_flag;
		if(is_packed_flag == 1) {
			// packed replacement of components
			unsigned int(3) num_cols_minus1;
			unsigned int(3) num_rows_minus1;
			unsigned int(5) hor_guard_band_log2;
			unsigned int(5) ver_guard_band_log2;
			for(i=0; i<num_cols_minus1+num_rows_minus1+2; i++) {
				unsigned int(8) channel_id;
			}
		}
		else {
			unsigned int(8) channel_id;
		}
	}
}

This allows applications such as those shown in the following figures to be realized with a conventional 4:2:0 YCbCr video codec.

Figure 1: YCbCr 4:4:4 with YCbCr 4:2:0 as a fallback and a hidden YCbCr 4:2:0 packed image.

Figure 2: YCbCr 4:4:4 with YCbCr 4:2:0 as a fallback and two YCbCr 4:2:0 as independent hidden image items.

Figure 3: RGB with YCbCr 4:2:0 as a fallback and three YCbCr 4:2:0 as independent hidden image items carrying R, G, and B channels in the luma plane.

Figure 4: RGB with YCbCr 4:2:0 as a fallback and a single YCbCr 4:2:0 packed image items carrying R, G, and B channels in the luma plane.

# 2 Proposal

This is a purely informative contribution to inform SA4 about the current state of development in HEIF with regard to Colour Format Enhancement Derivation.

# References

- WD of ISO/IEC 23008-12 3rd edition AMD 2 Support for tone map derivation and others, MDS23524_WG03_N01157, Output document MPEG #145

- WD of ISO/IEC 23008-12 3rd edition AMD 2 Support for tone map derivation and others, MDS23837_WG03_N01227, Output document MPEG #146