3GPP SA4 #128	S4-241112

Jeju, Korea, 20 May – 24 May 2024

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.822 | CR | pseudo | rev | - | Current version: | 0.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_5G_RTP_Ph2] Benefits of Application Layer FEC Awareness for PDU Set Handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5G_RTP_Ph2 |  |  |  |  |  | Date: |  |  | 05/20/2024 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This addresses Key issue #4: Application-layer FEC awareness for PDU Set handling This paper adds evidence for the benefits of Application-layer FEC awareness for PDU Set handling as background information of the Key issue. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding evidence of application-layer FEC awareness for PDU Set handling |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 3GPP may not be vested to pursue solutions to enabling application-layer FEC awareness for PDU Set handling. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

.

# Proposed changes

# * * * * 1st change * * * *

## 5.4.x	Benefits of Application-layer FEC awareness for PDU Set handling

When the RTP source adds redundant PDUs for an ADU, the redundancy is over budgeted to account for error in the estimation of the packet loss rate in the network. That is, there are more packets than needed for reconstructing the ADU. At the last hop of the PDU Set delivery, if the base station is aware of AL-FEC, it can drop PDUs that are no longer needed for rescontructing the ADU. This has two benefits:

- Reducing the usage of resources and hence improving the spectral efficiency (the amount of resources per PDU Set)

- Lowering the power consumption of the UE because the network can let the UE go to the sleep mode earlier.

This AL-FEC aware PDU Set handling is illustrated in the figure below. Packets 0 and 1 are served in the first time slot (which is a ‘D’ slot), and packets 2 and 3 in the second slot, and so on. In Case-1, without AL-FEC awareness, the redundant packets are still transmitted, which wastes network resources and keeps the UE awake longer before the network lets the UE go to the sleep mode. In contrast, in Case-2, with AL-FEC awareness, the use of network resources becomes more efficient, and the UE goes to the sleep mode earlier.

Figure 5.4.x-1 Benefits of AL-FEC awareness at RAN

Simulation study is carried out for the following scenario to quantify the benefits:

TDD with slot pattern DDDSU, 30kHz SCS, HARQ Retransmission (up to 3 times), 100MHz bandwidth, IBLER = 10%, 30fps, MDS AL-FEC code, redundancy ratio 30% (FEC code rate = 22%). The power consumption values are presented by relative power as defined in TS 38.840.

Table 5.4-1 Benefits of AL-FEC aware PDU Set handling at RAN

|  | 99-Percentile Latency (ms) | Power Consumption | Network loading |
| --- | --- | --- | --- |
| Case 1 – without AL-FEC awareness at RAN | 27 | 312 | 91.08% |
| Case 2 – with AL-FEC awareness at RAN | 27 (0%) | 270 (-13%) | 74.03% (-19%) |

We see from the table that AL-FEC aware handling reduces power consumption of the UE by 13%, which is significant for the UE. It also reduces the network loading by 19%, and this allows the network to accommodate more users.

Observation 1: AL-FEC aware PDU Set handling can greatly reduce the UE power consumption and reduce the network loading.

# * * * * End of 1st change * * * *