3GPP TSG SA WG4 Meeting #128	S4-241163

# Jeju, Korea, 20th–24th May 2024                                                   Revision of S4-240937

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.506 | CR | 0005 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Terminology alignment in RTC architecture |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, BBC, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | GA4RTAR |  |  |  |  |  | Date: |  |  | 2024-05-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In developing generic architecutre and APIs between 5GMS and RTC, it was raised that some of names and terminologies should be rephased. In particular, the term RTC endpoint is indiscreetly used to indicate the function both in UE and RTC AS. The protocol requirements should be different between RTC endpoint in UE and another in RTC AS, so this should be distinguished. In addition, the existing RTC architecture does not sufficiently address the peer-to-peer media transport case, which is essntial. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Provides more detailed definition for architecture functions: RTC endpoint, RTC Client, RTC Access Function, WebRTC Framework - Adds missing reference points: RTC-10 (AS to AS), RTC-12 (peer-to-peer) - W3C JS API to be included to RTC Application at reference point RTC-7 - Updates relevant text according to re-specified definitions/reference points |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent terminologies between 5GMS (as specified in TS 26.501 and 26.510) and RTC architecture |  |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.1, 4.1.1, 4.1.2, 4.2.4, 4.2.5, 4.2.6, 4.2.7, 4.2.8, 4.2.9, 4.2.10, 4.2.11, 4.2.12 (new), 4.3.3, 4.3.4, 4.3.5, 4.3.6, 4.3.7, 4.3.7A (new), 4.3.8, 4.3.9 (new), 4.4.2.1, 5.4, 5.5, 6.1, 6.2, Annex A.1, A.2, A.3, A.4, B (new), B.1 (new), B.2 (new), B.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1 – new clause (4.2.12) for RTC Access Function, WebRTC API in figure changed to dotted line, more clarificatino on WebRTC Framework definition |  |  |  |  |  |  |  |  |

FIRST CHANGE

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 26.998: "Support of 5G glass-type Augmented Reality / Mixed Reality (AR/MR) devices".

[3]	3GPP TS 26.119: "Media Capabilities for Augmented Reality".

[4]	3GPP TS 26.113: "Enabler for Immersive Real-time Communication".

[5]	3GPP TR 26.930: "Study on the enhancement for Immersive Real-Time communication for WebRTC".

[6]	3GPP TS 26.501: "5G Media Streaming (5GMS); General description and architecture".

[7]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[8]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

[9]	3GPP TS 36.321: "LTE; Medium Access Control (MAC) protocol specification".

[10]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia telephony; Media handling and interaction".

[11]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[12]	3GPP TS 23.548: "5G System Enhancements for Edge Computing; Stage 2".

[13]	IETF RFC 8825: “Overview: Real-Time Protocols for Browser-Based Applications”.

Next change

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

RTC Application: A Native WebRTC Application or a Web App that is compliant with the profile of a WebRTC-based application defined in the present document.

RTC endpoint: A WebRTC endpoint incorporating an instance of the WebRTC Framework that is capable of participating in an RTC session and which is deployed either in the RTC Access Function of a UE or in the Media Function of an RTC AS.

RTC Client: UE function comprising an RTC Access Function and an RTC Media Session Handler which interacts with functions in the network and UE applications.

RTC Access Function: A set of functions including an instance of the WebRTC Framework. The RTC Access Function exchanges real-time media with one or more RTC endpoints via reference point RTC-4m or RTC-12, and the RTC Access Function exchanges signalling messages with WebRTC Signalling Function via reference point RTC-4s. Also, the RTC Access Function exposes client APIs defined in the present document to the RTC Application at reference point RTC-7 and to the RTC Media Session Handler at reference point RTC-11.

WebRTC Framework: A well-defined subset of the WebRTC protocol stack for data transport and data framing that supports real-time media communication between an RTC endpoint and its peer(s) as specified in RFC 8825 [13].

Next change

4.1	Overall architecture for Real-Time media Communication (RTC)

4.1.1	Definition of RTC architecture

Real-Time media Communication (RTC) over 5G system in the context of this specification is defined as the delivery of delay-sensitive media from one peer to another with support of 5G network. AR conversational service described in TR 26.998 [2] is a typical use cases for RTC, which enables end-users to directly communicate real-time media including AR/MR media content as specified in TS 26.119 [3]. As identified in clause 8.4 of TR 26.998, there may be different options to enable such AR conversational service, for example re-use of parts of MTSI as defined in TS 26.114 [10] such as the IMS data channel or 5G Media Streaming for managed services.

The overall RTC architecture is shown in Figure 4.1.1-1 as below.

Figure 4.1.1-1: Real-time media communication (RTC) in 5G System

NOTE:	The functions indicated by the yellow filled boxes are in scope of the present document for RTC. The functions indicated by the grey boxes are defined in 5G System specifications. The functions indicated by the blue boxes are neither in scope of 5G RTC nor 5G System specifications.

The media data is exchanged between two or more RTC endpoints over a 5G System as defined in TS 23.501 [11]. An RTC endpoint is an instance of the WebRTC Framework configured by the RTC System defined in the present document. An RTC endpoint is typically realised by the RTC Client of a UE, but an RTC AS, possibly deployed as an edge computing server as defined in clause 4.4.2, may also play the role of RTC endpoint. The Application Provider provides a RTC Application on the UE to make use of RTC endpoint and network functions using interfaces and APIs. RTC architecture provides the core functions and entities to support WebRTC-based service over 5G System, two main functions are defined in the trusted DN.

-	RTC AF: An Application Function as defined in TS 26.501 [6], but dedicated to real-time media communication.

-	RTC AS: An Application Server dedicated to real-time media communication.

NOTE:	If both the RTC AF and RTC AS are deployed in an external DN, this is out of scope of the present document.

The detailed RTC architecture mapping to the overall high-level architecture in figure 4.1.1-1 is shown in figure 4.1.1-2 below.

NOTE 1:	Some subfunctions may not be required depending on the collaboration scenario. Description of collaboration scenario and its architecture variant are specified in annex A.

NOTE 2:	Void.

NOTE 3:	Red ovals indicate API provider functions.

NOTE 4:	The RTC Access Function may be realised by a web browser in deployments of the RTC Client that support Web App through the W3C defined JavaScript APIs including WebRTC API.

Figure 4.1.1-2: RTC General Architecture

The WebRTC Signalling Function may be co-located with the RTC AF. In such deployments, the WebRTC Signalling Function acts as an RTC AF with access to the 5G Core, and some of the RTC AF interactions with the WebRTC Signalling Function may be replaced to avoid concurrent/redundant requests from the RTC endpoint in the UE. Specifically, media session handling interactions between the RTC AF and the UE at reference point RTC-5 may be replaced by the equivalent WebRTC signalling interactions defined at reference point RTC-4.

The subfunctions inside the RTC AF, RTC AS and the RTC Client are defined in clause 4.2 and the reference points shown in figure 4.1.1-2 are defined in clause 4.3.

Two types of RTC Application are defined in the present document:

-	Native WebRTC App: An RTC Application running on the UE that makes use of client APIs at reference points RTC-6 and RTC-7.

-	Web App: A web application running in a web browser on the UE that makes use of the W3C-defined WebRTC APIs.

NOTE:	Detailed deployment architecture for the Native WebRTC Application and the Web App are described in annex B.

4.1.2	Generalized Media Delivery architecture

4.1.2.1	Generalized Media Delivery in the 5G System

This clause and subsequent subclauses of clause 4.1.2 define a generalized Media Delivery architecture of which the architecture for Real-Time Communication (RTC) defined elsewhere in the present document is one possible realisation. In case of any misalignment between the two, the RTC architecture has precedence over this generalised architecture.

Due to the similarity of the 5GMS architecture (as defined in TS 26.501 [6]) to the architecture for Real-Time media Communication (RTC) defined in the present document, the RTC functions and 5GMS functions may share or may make use of many common functionalities for both media session handling and media delivery. A generalized Media Delivery architecture that integrates 5GMS and RTC functionality in the 5G System is defined in figure 4.1.2.1-1.

NOTE:	Full integration of 5GMS and RTC is not addressed in the present document.

Figure 4.1.2.1-1: Generalized Media Delivery architecture within the 5G System

In this representation:

-	The Media Application Provider plays the role of the RTC Application Provider.

-	The Media-aware Application plays the role of the Native WebRTC App.

-	The RTC AF is one possible realisation of the general Media AF.

-	The RTC AS is one possible realisation of the general Media AS.

-	The RTC Client is part of the general Media Client.

4.1.2.2	Reference architecture for Media Delivery

A functional description with additional details as well as reference points is provided below, as illustrated in figure 4.1.2.2-1.

NOTE 1:	Exposed APIs are named in italics.

NOTE 2:	If the Media Client is deployed as a monolithic functional block, it may choose not to expose interfaces externally at reference point M11.

Figure 4.1.2.2-1: Generalized Media Delivery architecture

4.1.2.3	Network Functions and UE entities

Functional definitions may be generalized as follows:

-	Media AF: An Application Function as defined in clause 6.2.10 of TS 23.501 [11] dedicated to Media Delivery.

-	Media AS: An Application Server dedicated to Media Delivery.

-	Media Client: A UE internal function dedicated to Media Delivery comprising:

-	Media Session Handler: An entity on the UE that communicates with the Media AF in order to establish, control and support the delivery of a media session.

-	Media Access Function: An entity on the UE that communicates with the Media AS in order to access and deliver media content. The media access function for example may be further sub-divided into content delivery protocols, codecs, media types and metadata representation.

-	Media-aware Application: An application entity on the UE that makes use of 3GPP-defined APIs to invoke the Media Session Handler and/or the Media Access Function in order to support Media Delivery.

NOTE:	An application (e.g., a web browser application) that does not invoke either the Media Session Handler or the Media Access Function using 3GPP-defined APIs is not considered a Media-aware Application and is not mapped into the generalized Media Delivery reference architecture.

Table 4.1.2.3-1: Mapping of RTC functions to generalized Media Delivery architecture

| Generalized media architecture function |  | RTC function |
| --- | --- | --- |
| Media AF |  | RTC AF |
| Media AS |  | RTC AS |
| Media Client |  | RTC Client |
|  | Media Session Handler | RTC Media Session Handler |
|  | Media Access Function | RTC Access Function |
| Media Application Provider |  | RTC Application Provider |
| Media-aware Application |  | Native WebRTC App |

4.1.2.4	Reference points

The following reference points are defined for Media Delivery:

M1:	Reference point between the Media Application Provider and the Media AF for the provisioning of Media Delivery.

M2:	Reference point between the Media Application Provider and the Media AS for the purposes of ingesting media into the Media AS or egesting media from the Media AS.

NOTE 1:	Reference point M2 is not defined by the RTC architecture in this release.

M3:	Reference point between the Media AF and the Media AS for the purposes of Media AS configuration and/or for media session handling in relation to Media Delivery.

NOTE 2:	Reference point M3 is defined by the RTC architecture in this release but specification is for future study.

M4:	Reference point between the Media AS and the Media Access Function in the UE for the purpose of downlink transport of media from the Media AS to the Media Access Function ("content distribution") or uplink transport of media from the Media Access Function to the Media AS ("content contribution").

NOTE 3:	Session setup signalling at reference point RTC-4 lies outside the scope of reference point M4.

M5:	Reference point between the Media AF and the Media Session Handler in the Media Client for the purpose of media session handling in relation to Media Delivery.

M6:	Reference point between the Media-aware Application and the Media Session Handler for the purpose of configuring the Media Session Handler.

M7:	Reference point between the Media-aware Application and the Media Access Function for the purpose of media access control.

M8:	Reference point between the Media-aware Application and the Media Application Provider.

NOTE 4:	Reference point M8 is private and therefore beyond the scope of standardisation.

M9:	Reference point between one instance of the Media AF and another for the purpose of Media AF instance chaining.

NOTE 5:	Reference point M9 is not defined by the RTC architecture in this release.

M10:	Reference point between one instance of the Media AS and another for the purpose of distributed service chaining over multiple Media AS instances.

NOTE 6:	Reference point M10 is defined by the RTC architecture but is not further specified in this release.

M11:	Reference point between the Media Session Handler and the Media Access Function (both in the Media Client) for the purpose of configuring the Media Session Handler and/or media access control.

M12:	Reference point between one RTC Access Function in a UE and another for the purpose of peer-to-peer media transport between different Media Clients when this is permitted by the 5G System.

Table 4.1.2.4-1: Mapping of RTC reference points to generalized Media Delivery architecture

| Generalized Media Delivery architecture reference point | RTC reference point |
| --- | --- |
| M1 | RTC-1 |
| M2 | Not defined |
| M3 | RTC-3 |
| M4 | RTC-4 |
| M5 | RTC-5 |
| M6 | RTC-6 |
| M7 | RTC-7 |
| M8 | RTC-8 |
| M9 | Not defined |
| M10 | RTC-10 |
| M11 | RTC-11 |
| M12 | RTC-12 |

Next change

4.2.4	RTC Media Session Handler (MSH)

The RTC Media Session Handler is an entity running on the UE, which assists with the 5G integration of the RTC Application. It exchanges, on behalf of the application, information about the RTC sessions with the network.

The RTC MSH receives information about a new WebRTC session from the application. It relays the information to the Network Support Function. It also receives events and other network information about the RTC session from the Network Support Function, which it may relay to the application.

In addition, one of subfunction in RTC MSH is the metric collection and reporting. It executes the collection of QoS and QoE metrics measurements from the RTC Access Function and the RTC Application and sends metrics reports to the RTC AF for the purpose of metrics analysis or to enable potential transport optimizations by the network.

4.2.5	Network Support Function

The support functionality includes the following:

-	Network Support Function receives information from the UE and/or other ASs about a RTC session and its state.

-	Network Support Function requests the network that QoS should be allocated (or satisfied) for a starting or modified session.

-	Network Support Function receives notification from the network about changes to the QoS allocation for the ongoing RTC session.

-	Network Support Function exchanges information about the RTC session with the trusted STUN/TURN/Signalling function, e.g. to identify a RTC session and associate it with a QoS template.

NOTE:	The integration/co-location of this RTC AF and WebRTC signalling function is possible. A co-located WebRTC Signalling Function is able to act as an RTC AF which is accessible to 5GC, and replace some of this RTC AF’s interfaces and APIs with the WebRTC Signalling Function. For example, reference point RTC-5 between the RTC AF and the RTC Media Session Handler is replaced to avoid concurrent/redundant requests from UEs.

4.2.6	ICE Function

The MNO may offer trusted ICE functions to the RTC Application to be used during the WebRTC ICE gathering phase. These functions may be STUN and TURN services that facilitate NAT and firewall traversal. In the context of the present document, a TURN service relays WebRTC content at reference point RTC-4m between multiple RTC Access Functions that cannot communicate directly with each other at reference point RTC-12 because the 5G System policy does not permit peer-to-peer communication.

When the ICE functions are provided by the MNO, they may assist with the 5G integration of RTC Applications. This may be achieved by triggering Network Assistance for starting or ongoing RTC sessions.

4.2.7	WebRTC Signalling Function

The WebRTC Signalling Function is used to set up and manage RTC Applications. It offers a standardized signalling protocol for session setup to RTC session participants. The WebRTC Signalling Function handles the offer/answer exchange and has access to the SDP in both directions.

The WebRTC Signalling Function may use that knowledge to offer network assistance and other 5G features to the endpoints of the RTC session.

The WebRTC Signalling Function manages media flow sessions in both uplink and downlink directions.

4.2.8	Interworking Function

This function provides interworking functionality to enable MNO-facilitated RTC sessions that involve endpoints across different MNOs. They may for example provide cross-network signalling functionality to allow WebRTC signalling server that are hosted in different networks to communicate, in order to establish and manage the RTC sessions.

4.2.9	Transport Gateway Function

A Transport Gateway Function may be offered by the MNO to support cross-operator RTC sessions. It may offer the border control function for user plane (e.g., topology hiding, IPv4-IPv6 translation) as a gateway, which is located at the network boundary where different operators or third-party network connects. It works under the control of the Interworking Function.

4.2.10	Media Function

A Media Function may be offered by the MNO to support RTC sessions. It may offer a wide range of functionality such as:

-	A content server that serves content to the RTC Access Function, e.g. through a data channel

-	Media processing used to perform tasks such as media transcoding, recording, 3D reconstruction, etc.

-	Scene composition in which the Media Function composes a 3D scene and distribute it to several point-to-point RTC sessions.

-	Multi-point Control Unit (MCU): the Media Function offers multi-party conferencing functionality to merge a number of point-to-point RTC sessions.

-	Selective Forwarding Unit (SFU): the Media Function offers the selection, copy, and forwarding functionality of RTC sessions produced by multiple RTC Clients  or WebRTC session participants external to the RTC System.

-	Maintain uplink and downlink flow context (QoS, remote control and etc.) by interacting with the WebRTC Signalling Function in the RTC AS (see clause 4.2.7).

The Media Function is an RTC endpoint that incorporates an implementation of a WebRTC Framework as the basis of the above functionality.

4.2.11	Application-supporting Web Function

A web server may be offered by the MNO to support RTC Applications by providing web service entry point, authorization/authentication functionality, sharing of files, or scheduling conferencing sessions.

4.2.12	RTC Access Function

A RTC Access Function is a set of functions in RTC Client in terminal that offers:

-	Access to real-time media exchanged by WebRTC Framework with one or more RTC endpoints.

-	Provision of client APIs to the Native WebRTC App, as well as W3C-defined JavaScript API including WebRTC API to Web App via RTC-7, and client APIs to the RTC Media Session Handler via RTC-11.

-	Real-time media stream and associated signalling delivery to RTC endpoint(s) over 5G System.

Next change

4.3.3	RTC-4: Media-centric transport interface via RTC AS

Reference point RTC-4 is used to exchange the WebRTC media traffic between the RTC Access Function and the Media Function of the RTC AS (see clause 4.2.10) as well as to exchange signalling information relating to the WebRTC session with the RTC AS such as WebRTC Signalling Function (see clause 4.2.7).

The traffic includes:

-	Media streams sent over RTP

-	Application data sent over data channel

-	WebRTC Signalling data along with STUN and TURN servers

-	Other application data

RTC-4 may further be grouped into two subsidiary reference points as follows.

RTC-4s:

The RTC-4s is a Subsidiary reference point between the RTC Access Function and the RTC AS such as WebRTC Signalling Function. This interface is used for the exchange of signalling information relating to the RTC session between two or more RTC endpoints via the RTC AS.

RTC-4m:

This subsidiary reference point is used for transmission of media and other related data between two or more RTC endpoints when at least one of RTC endpoints participating in an RTC session is instantiated in the RTC AS.

The traffic at subsidiary reference point RTC-4m includes:

-	Media data transmitted over SRTP

-	Application data transmitted using Data channel.

-	Media related meta-data transmitted using Data channel.

-	Other application data.

NOTE 1:	The Media Function of the RTC AS maintains the status for both uplink and downlink traffic.

NOTE 2:	An RTC Client supports WebRTC streaming functions for uplink and downlink traffic.

4.3.4	RTC-5: Control transport interface

Reference point RTC-5 is used to convey configuration information from the RTC AF to the RTC Media Session Handler and is used by the RTC Media Session Handler to request media session handling support from the RTC AF for RTC sessions. The configuration information may consist of static information such as the following:

-	Recommendations for media configurations.

-	Configurations of STUN and TURN server locations.

-	Configuration about consumption and QoE reporting.

-	Discovery information for WebRTC signalling and data channel servers and their capabilities

The support functionality includes the following:

-	RTC MSH receives the configuration information

-	RTC MSH informs the RTC AF about a RTC session and its state

-	RTC MSH requests QoS allocation for a starting or modified session

-	RTC MSH receives notification about changes to the QoS allocation for the ongoing RTC session

-	RTC MSH receives the updated information about the RTC session with the RTC STUN/TURN/Signalling function, e.g. to identify a RTC session and associate it with a QoS template

The RTC functionality that offers application functions to the RTC Application may equally be provided by Application Servers (RTC AS) instead of by the RTC AF. These then use a dedicated interface RTC-3 to request configurations and network support for the ongoing RTC sessions from the RTC AF.

4.3.5	RTC-6: Client API

The RTC Media Session Handler is a function in the UE that provides access to RTC support functions to the Native WebRTC \Applications. These functions may be offered on request, i.e., through reference point RTC-6, or transparently without direct involvement of the application. The RTC MSH may assist indirectly in the ICE negotiation by providing a list of STUN and TURN server candidates that offer RTC functionality. The RTC MSH also collects QoE metrics reports and submits consumption reports. It may also offer media configuration recommendations to the application through RTC-6.

4.3.6	RTC-7: Client interface

Reference point RTC-7 is used to provide the capability of RTC Access Function as API between the RTC Access Function and the RTC Application. The API includes Client API provided for the Native WebRTC App and W3C defined JavaScript APIs including WebRTC API provided for the Web App (i.e., JavaScript application) running on a browser.

4.3.7	RTC-8: Application interface

RTC-8 is a proprietary reference point between the RTC Application and the RTC Application Provider, which may be used to exchange configuration information related to the RTC session or the application.

4.3.7A	RTC-10: RTC AS to another RTC AS interface

RTC-10 is a reference point between one instance of the Media AS and another for the purpose of distributed service chaining over multiple RTC AS instances. This may be present, for example when different RTC Clients are using the services of different Media Functions on separate RTC AS instances.

4.3.8	RTC-11: RTC Client configuration APIs

Reference point RTC-11 is used by the RTC Access Function to configure media session handling in the RTC Media Session Handler and/or used by the RTC Media Session Handler to configure media access in the RTC Access Function.

Because it is internal to the RTC Client, RTC-11 may not be exposed as an API to application developers but may instead be in the form of an internal API.

The RTC Access Function hides details of QoS allocation and network support from the Native WebRTC Application and Web App. It autonomously and transparently invokes the functions offered by the RTC MSH to provide support for the RTC session.

4.3.9	RTC-12: Peer-to-peer media-centric transport interface

Reference point RTC-12 is used to exchange WebRTC traffic between RTC Access Functions in different UEs when the 5G System permits peer-to-peer media transport. The protocols supported at this reference point shall be a subset of those at reference point RTC-4m.

NOTE:	In case of peer-to-peer communication at reference point RTC-12, UPF provides the functionalities as specified in TS 23.501 [11], such as traffic usage reporting for billing and the Lawful Intercept (LI) collector interface.

zNext change

4.4.2	Extended RTC architecture for Edge Computing

4.4.2.1	General

The RTC architecture can be extended to add support for media processing in the edge. The extended architecture is an integration of the RTC architecture defined in TS 26.506 with the architecture for enabling Edge Applications defined in TS 23.558 [7] and TS 26.501 [6].

The extended RTC architecture supports both client-driven as well as Application Function-driven management of the edge processing session.

The RTC Application Provider may request the deployment of edge resources as part of the Provisioning Session.

-	The RTC Application Provider provisions edge resources through reference point RTC-1, in a similar fashion as defined in TS 26.501 [6], enabling client-driven and/or AF-driven edge configuration.

-	In the client-driven approach, the Native WebRTC Application becomes aware of the support of edge processing in the network and takes steps, such as using the APIs at reference point EDGE-5, to discover and locate a suitable RTC AS instance in the Edge DN, similar to the process defined in clause 8.1 of [6].

-	In the AF-driven approach, the RTC Application Provider provisions the RTC AF to deploy edge processing for the media sessions of the corresponding Provisioning Session, similar to the process defined in clause 8.2 of [6]. In this case, instantiating edge resources is the responsibility of the RTC AF, based on monitoring the load of the deployed EAS instances. A WebRTC Application may become aware of the deployed EAS through the RTC Application Provider from the RTC MSH through reference point RTC-5 (and possibly RTC-6). This appropriate EAS instance for a particular EAS Client to use is discovered by means of DNS resolution with support from the DNS server (e.g., EASDF/DNS resolver) as defined in TS 23.548 [12].

When the RTC Application is a web application, the implementation of the interface at reference point EDGE-5 to discover the RTC AS/EAS location by accessing the EEC is difficult if the Web browser does not implement interfaces necessary for supporting edge-enabled RTC applications/services.

NOTE:	Other methods that can be used for sharing EAS information (e.g., sharing EAS hostname to the RTC Application by RTC-8 or by other means and then using DNS resolution) are FFS.

NOTE:	This figure corresponds to collaboration scenario 2. Other subfunctions of the RTC AS required to support collaboration scenarios 3 and 4 are not depicted.

Figure 4.4.2-1: Edge-enabled RTC architecture

Next change

5.4	Call flow for Network-supported RTC sessions (CS#2)

The MNO offers access to trusted ICE functionality to UEs that wish to participate in RTC sessions. The session establishment takes into account the configured trusted ICE functions.

The call flow is as follows.

Figure 5.4-1: Call flow for network-supported RTC session (collaboration scenario 2)

The working assumptions are:

-	The Application on UE1 and the remote RTC endpoint use an external WebRTC Signalling Function to establish the RTC session.

Initially:

0.	A Provisioning Session is created by the RTC Application Provider at the RTC AF. This may include a list of trusted STUN/TURN servers to be used by the RTC Client for establishing RTC sessions.

The steps are as follows:

1.	The RTC Media Session Handler retrieves Service Access Information from the RTC AF via reference point RTC-5. This includes configuration for the RTC Client, including a list of trusted STUN/TURN servers that it may use for establishing RTC sessions.

2.	The RTC Application on UE1 queries the RTC MSH for the list of trusted ICE servers.

3.	The RTC Application on UE1 discovers and tests the ICE candidates to validate that they are suitable for the connection to the remote RTC endpoint.

4.	The RTC Application on UE1 and the remote RTC endpoint use an external RTC Signalling Function to exchange information about ICE candidates and to exchange the SDP offer/answer.

When an RTC session is established using the most suitable ICE candidate:

5.	The STUN or TURN server in ICE function, upon reception of the allocation request by the application (or RTC Access Function) may extract the 5-Tuple information for each of the media sessions and convey the information to the Network Support AF in RTC AF for requesting QoS assistance.

6.	The RTC AF interacts with the PCF at reference point N5 to request QoS allocation. It may request differential charging based on pre-existing provisioning for these sessions.

7.	Confirmation of QoS allocation is notified to the RTC AF and the RTC AS.

8.	The RTC AF also subscribes to events relating to the QoS flows of the RTC session with the PCF and SMF.

9.	The RTC AF receives notifications about any changes to the QoS flows of the RTC session from the PCF or the SMF. Then, the RTC AF sends notifications to the ICE function (STUN/TURN server).

10.	The STUN/TURN server may forward the bit rate recommendation to the RTC MSH, if the allocation session is still active.

11. Alternatively, the RTC Media Session Handler may interact with the UE Modem to trigger to query the recommended bit rate on the uplink or downlink direction.

12.	The UE Modem then sends the ANBRQ (Access Network Bit Rate Query) signalling to the RAN as defined in TS 38.321 [8] for NR access and TS 36.321 [9] for LTE access.

The RAN, based on the network status, returns the recommended bit rate to the UE modem as requested. The recommended bit rate is in kbps at the physical layer at the time when the decision is made.

NOTE 1:	The UE may determine the corresponding IP layer bit rate based on the long-term average of the IP packet sizes, L2 header sizes, and ROHC header sizes, but the translation methodologies and the estimation error levels required to implement accurate media bit rate adaptation have not been specified. The UE may determine the corresponding IP layer bit rate based on the long-term average of the IP packet sizes, L2 header sizes, and ROHC header sizes, but the translation methodologies and the estimation error levels required to implement accurate media bit rate adaptation have not been specified.

NOTE 2:	The eNodeB may determine the corresponding IP layer bit rate based on the long-term average of the IP packet sizes, L2 header sizes, and ROHC header sizes, but the translation methodologies and the estimation error levels required to implement accurate media bit rate adaptation have not been specified.

NOTE 3:	The recommended/queried bit rate as signalled over the LTE and NR access is defined to be in kbps at the physical layer. The uplink/downlink bit rate at the physical layer is rUL/DL=kLkT, where Lkis the bit-length of the k-th successfully transmitted/received TB by the UE within the window T. In TS 36.321[9] and 38.321[8], a window length of 2000 ms is applied.

13.	The UE modem notifies the recommended bit rate to the RTC Media Session Handler.

14.	The RTC Media Session Handler notifies the recommended bit rate to the RTC Application.

15.	The RTC Application may act on the bit rate recommendation, e.g. by reducing the uplink media bit rate.

16.	Media traffic is exchanged with the remote RTC endpoint. If a TURN server is present in the RTC Client configuration obtained in step 1, media is relayed to the other RTC endpoint by the ICE Function of the RTC AS via reference point RTC-4, possibly with additional media processing by the Media Function of the RTC AS, as required by the RTC session.

5.5	Call flow for MNO-facilitated RTC sessions (CS#3)

In collaboration scenario 3, MNO hosts the RTC sessions by providing a trusted WebRTC signalling function in the RTC AS. In addition, a trusted media server is also present in RTC AS to support SFU and MCU functionality.

NOTE:	When the RTC Application is a web-based application (i.e., a Web App), the RTC MSH function is not supported.

The call flow for this scenario when RTC MSH is involved are as shown in figure 5.5-1.

Figure 5.5-1: Call flow for MNO-facilitated RTC session (collaboration scenario 3)

The RTC Application Provider may create a Provisioning Session with the RTC AF and starts provisioning the usage of the RTC Streaming session between two endpoints. During the establishment phase, the used features such as content consumption measurement, logging, collection and reporting; QoE metrics measurement, logging, collection and reporting; dynamic policy; network assistance; are negotiated and detailed configurations are exchanged.

The RTC Application Provider Provisioning Session phase is optionally performed prior to the establishment of any related RTC sessions by the RTC Application Provider. Detailed procedure is addressed in clause 5.2.1.

The ICE candidate discovery phase is performed with the following steps in an MNO-facilitated RTC system:

1.	Retrieve Service Access Information: The RTC MSH retrieves Service Access Information from the RTC AF that includes RTC Client configuration: a list of trusted STUN/TURN servers, trusted WebRTC Signalling Function and data channel service endpoint addresses and their capabilities. The RTC Client may use this configuration information for establishing RTC sessions.

2.	ICE Servers request: The RTC Application queries the RTC MSH for the list of trusted ICE servers.

3.	ICE candidate validation: The UE discovers and tests the ICE candidates to validate that they are suitable for the connection.

The RTC session establishment phase is performed with the following steps in an MNO-facilitated RTC system:

4.	Query configuration information: In response to a request from the RTC Application, the RTC Access Function queries the RTC MSH for the WebRTC signalling function information. In some cases where the signalling is not handled by RTC Access Function, the Native WebRTC application may directly query the RTC MSH for the WebRTC Signalling server information.

5.	Configuration information: RTC MSH sends the WebRTC signalling function and data channel servers and their capabilities information to the RTC Access Functionor in some cases to the Native WebRTC application.

In the RTC session establishment phase, two or more RTC endpoints exchange signalling information relating to the RTC session, such as ICE candidates, SDP offer/answer using the WebRTC Signalling Function in the RTC AS.

NOTE:	Figure 5.5-1 illustrates that SDP offer is generated by the RTC Application. However, in SFU/MCU mode, SDP offer is generated by the Media Function in the RTC AS.

6.	SDP offer: The RTC Application creates a request with an SDP offer which includes the ICE candidates and sends it to the WebRTC Signalling Function.

7.	Determine remote endpoint location: The WebRTC Signalling Function uses the registration information to locate the remote endpoint

8.	SDP offer: The WebRTC Signalling Function forwards the request to remote endpoint.

9.	SDP answer: Upon accepting the offer, remote endpoint responds to signalling function with SDP answer.

10.	SDP answer: WebRTC signalling function sends the SDP answer to the UE1.

With this, an RTC session is established between RTC endpoints using the most suitable ICE candidate and the WebRTC Signalling Function.

The Dynamic policy phase is then performed to allocate QoS for the media streams of the RTC session with the following steps:

11.	QoS request: The WebRTC Signalling Function in the RTC AS sends a request to the RTC AF for the allocation of QoS for the RTC session. The RTC AF sends a request to the PCF to allocate QoS for the media streams of the RTC session

12.	Confirmation: The PCF or SMF confirms the successful allocation of network support or QoS allocation.

If the Network Support Function is supported in the RTC AF, the Network Support Function AF (NS-AF) offers the bit rate recommendation request API based on existing policy templates, through the usage of either the Npcf_PolicyAuthorization API over N5 interface, or the Nnef_AFSessionWithQoS over N33 interface to the PCF. If no corresponding AF application session context already exists, the NS-AF uses the Npcf_PolicyAuthorization_Create method over N5 interface with the appropriate service information to create and provision an application session context. The Network Assistance phase is performed with the following steps in an MNO-facilitated RTC system.

13.	Subscribe to QoS events: The NS-AF also subscribes to events related to the QoS flows of the RTC session with the PCF and SMF.

14.	QoS events: The NS-AF receives notifications about any changes to the QoS flows of the RTC session from the PCF or the SMF.

15.	QoS notifications or bit rate recommendations: The NS-AF may send notifications to the RTC MSH about the changes in QoS flow. When the allocated session is active, the RTC MSH forwards the bit rate recommendation to the application.

16.	Adjust media bit rate: An RTC Application may act on adjusting the bit rate recommendation, e.g., by reducing the uplink media bit rate.

After successful establishment of an RTC session and the bit rate negotiations, the actual RTC session over 5G may start:

17.	Media transfer:

a)	In some cases, peer-to-peer communication between two RTC Clients is possible, and the media is exchanged directly with the remote RTC endpoint via reference point RTC-12.

b)	Otherwise, the RTC Application connects to the selected TURN server and/or Media Function in the RTC AS through reference point RTC-4, and the media is relayed to the remote endpoint by the ICE Function of the RTC AS, possibly with additional media processing by the Media Function of the RTC AS as required by the RTC session.

18.	Method calls and notifications: Supporting information about the RTC session is passed from the RTC Access Function to the RTC MSH.

19.	Reporting, network assistance, and dynamic policy: The RTC MSH exchanges supporting information about the RTC session with the RTC AF.

20.	End session: The RTC Application informs the RTC Access Funtion (via RTC-6) and WebRTC Signalling Function (via RTC-4s) that the RTC session has ended.

21.	Session ending event: The RTC Access Function informs the RTC MSH about the end of the RTC session.

22.	Final reporting: The RTC MSH performs any final reporting to the RTC AF.

Next change

6.1	Client-driven management of RTC edge processing

The detailed call flow for client-driven management of edge processing session is shown in figure 6.1-1.

Figure 6.1-1. Client-driven management of RTC edge processing

The Edge Computing Provisioning phase is a provisioning phase, that may be repeated several times (e.g., to extend edge processing coverage to new geographical areas or to increase the capacity of an already provisioned area). All steps in this phase are optional and performed on need basis. The steps are:

1.	Spawn ECS: A new ECS instance is instantiated to manage new or increased demand for edge processing.

2.	Spawn RTC AF: A new RTC AF that is edge-enabled is instantiated to handle new or increased demand for RTC sessions with edge processing.

3.	EES Configuration: The EES is configured for a specific Edge Data Network (EDN).

4.	EES Registration with ECS: The EES registers with the ECS that is in authority over the target EDN.

The RTC Application Provider Provisioning phase is performed prior to the establishment of any related RTC sessions by the RTC Application Provider. Subsequent updates to the provisioning session are possible.

5.	Create Provisioning Session: The RTC Application Provider creates a new provisioning session in the RTC AF.

6.	Provision RTC features: The RTC Application Provider may create different configurations such as QoS support, charging, collection of consumption, offering STUN/TURN servers, WebRTC Signalling Function, Edge Processing, etc.

The Native WebRTC Application initiates a new RTC session:

7.	Application initialization: The user launches the Native WebRTC Application. The Native WebRTC Application performs any required initialization steps.

8.	Start RTC session: The Native WebRTC Application invokes the RTC Access Function with appropriate streaming access parameters.

9.	Session starting event: The Native WebRTC Application informs the RTC MSH about the start of a new RTC session.

10.	Retrieve Service Access Information: The RTC MSH retrieves Service Access Information from the RTC AF, including RTC Client configuration information provisioned in step 6.

11.	Determine eligibility for requesting edge resources: Using information from the Service Access Information, the RTC MSH determines whether the RTC session is eligible for requesting edge resources.

If the eligibility criteria are met in the previous step, the UE discovers an EAS instance offering RTC AS functionality in the Client-based Edge Computing Discovery phase:

12.	Locate EAS instances: The RTC MSH asks the EEC to discover the location of one or more suitable EAS instances offering the "RTC AS" capability that can serve the application.

13.	Locate local EES: The EEC queries the ECS for a suitable EES (EDGE-4 API).

14.	Register with EES: The EEC registers with the selected EES (EDGE-1 API).

15.	Request list of "RTC AS" EAS instances: The EEC queries the EES for one or more EAS instances offering the “RTC AS” capability that can serve the session, using EAS discovery filters (see Table 8.5.3.2-2 in [2]) provided by the Application Client, e.g. “RTC AS” for EAS type, appropriate values for service feature(s), and other EAS characteristics.

The optional sub-flow RTC AS Provisioning is for provisioning an additional RTC AS instance if a suitable EAS instance offering the "RTC AS" capability cannot be located. The steps are:

16.	Check resource template: The RTC AF checks the provisioned edge processing resource template for the related application to determine the requirements of the application.

17.	Instantiate new "RTC AS" EAS instance: The RTC AF requests the MnS to instantiate a new RTC AS EAS instance with the specified requirements and considering parameters provided in the query by the EEC.

18.	Spawn "RTC AS" EAS instance: The MnS creates a new instance of the EAS offering RTC AS capability with the requested placement and resources.

19.	EAS instance configuration: The newly instantiated "RTC AS" EAS instance is configured.

20.	Register EAS instance with EES: The newly instantiated EAS instance registers itself with the triggering EES.

21.	Configure provisioned features: This may include configuring and launching the server-side application in the RTC AS.

Completion of UE Edge Computing Discovery phase:

22.	List of suitable "RTC AS" EAS instances: The EES/RTC AF responds to the EEC with a list of "RTC AS" EAS instances and their characteristics in an EAS discovery response (see table 8.5.3.3-1 in [16]).

23.	Select preferred "RTC AS" EAS instance: The AC and/or EEC select(s) a “RTC AS” EAS instance from the provided list, based on the AC’s desired criteria.

After successful discovery of a “RTC AS” EAS instance, the actual RTC session over 5G may start:

24.	Media transfer: The Native WebRTC Application connects to the selected "RTC AS" EAS instance and the exchange of WebRTC media begins.

25.	Method calls and notifications: Supporting information about the RTC session is passed from the RTC Access Function to the RTC MSH.

26.	Reporting, network assistance, dynamic policy: The RTC MSH exchanges supporting information about the RTC session with the RTC AF.

27.	End session: The Native WebRTC Application informs the RTC Access Function that the RTC session has ended.

28.	Session ending event: The RTC Access Function informs the RTC MSH about the end of the RTC session.

29.	Final reporting: The RTC MSH performs any final reporting to the RTC AF.

6.2	AF-driven management of RTC edge processing

The detailed call flow for AF-driven management of edge processing session by using the RTC Media Session Handler is shown in figure 6.2-1.

Figure 6.2-1. AF-driven management of RTC edge processing

The steps are:

1.	Steps 1-4 as described in clause 6.1.

2.	Create Provisioning Session: In this step, the RTC Application Provider creates a new provisioning session.

3.	Provision RTC features: The RTC Application Provider may create different configurations such as QoS support, charging, collection of consumption, offering STUN/TURN servers, WebRTC signalling function, edge processing, etc.

4.	RTC AS provisioning (if needed), as described in figure 6.1-1, steps 16-21.

The Native WebRTC Application initiates a new RTC session:

5.	Start session: The WebRTC Application invokes the RTC Access Function with appropriate real-time streaming access parameters.

6.	Session starting event: The application informs the RTC MSH about the start of a new RTC session.

7.	Retrieve Service Access Information: The RTC MSH retrieves Service Access Information from the RTC AF, including RTC Client configuration information.

8.	Instantiate edge resources: The RTC AF determines whether edge resources need to be instantiated to support the RTC session, for example by requesting that the MnS instantiates a new "RTC AS" EAS.

9.	Start the media streaming as defined in figure 6.1-1, steps 24-26.

10.	Continue the final steps as defined in figure 6.1-1, steps 27-29.

Next change

A.1	General

This clause addresses the derivative architecture for each of the collaboration scenarios. The four collaboration scenarios are summarized below and further details is specified in Table A.1-1.

The four collaboration scenarios are specified based on the location of required functional entities in trusted domain as defined as follows.

-	Over the top WebRTC: the RTC session runs completely over the top. However, the MNO may offer support in form of QoS allocation, bitrate recommendations, and QoE report collection based on request by the UE.

-	MNO-provided WebRTC functions: On top of over the top scenario, the MNO additionally offers ICE Function in RTC AS.

-	MNO-facilitated WebRTC services: the MNO hosts and facilitates RTC sessions by providing a WebRTC Signalling Function, which may also offer 5G network assistance.

-	Interoperable WebRTC services: collaboration scenario 3 is extended with functions to support MNO to MNO interoperability.

NOTE:	Collaboration scenario 4 is in the scope of this specification. Some of its details, which are not specified in the current version of the document, is FFS.

The list of key functional entities in trusted domain differs from collaboration scenarios as described in Table A.1-1.

Table A.1-1: Mapping of key functions to each collaboration scenarios

| Functions | Collaboration scenario 1 | Collaboration scenario 2 | Collaboration scenario 3 | Collaboration scenario 4 |
| --- | --- | --- | --- | --- |
| Provisioning Function | Optional | Optional | Optional | Optional |
| Configuration Function | Optional | Required | Optional (maybe fulfilled by WebRTC Signalling Function) | Optional (maybe fulfilled by WebRTC Fignalling Function) |
| RTC Media Session Handler | Required | Optional | Optional | Optional |
| Network Support Function | Required | Required | Optional (maybe fulfilled by WebRTC Signalling Function) | Optional (maybe fulfilled by WebRTC Signalling Function) |
| ICE Function | N/A | Required | Optional | Optional |
| WebRTC Signalling Function | N/A | N/A | Required | Required |
| Media Function | N/A | Optional | Optional | Optional |
| NOTE: The collaboration scenario 3 may further split depending on the role of MNO, as addressed in TR 26.930. |  |  |  |  |

A.2	Collaboration scenario 1:

Figure A.2-1 shows the architecture variant for the collaboration scenario 1 when the RTC session is completely running over the top. For this case, many of WebRTC-related entities are not the scope of this specification. However, Network Support Function is present in the RTC AF to support QoS allocation, bitrate recommendations, and QoE report collection.

Figure A.2-1: Derivative RTC architecture for collaboration scenario 1

A.3	Collaboration scenario 2:

Figure A.3-1 shows the architecture variant for the collaboration scenario 2 when MNO provides ICE Function in the RTC AS. It also instantiates the Configuration Function in the RTC AF to support the network-assisted RTC sessions over 5G System.

Figure A.3-1: Derivative RTC architecture for collaboration scenario 2

NOTE:	RTC-4 interface is present only when the ICE function contains the TURN server in this scenario.

A.4	Collaboration scenario 3:

Figure A.4-1 shows the architecture variant for the collaboration scenario 3 when MNO hosts the RTC sessions by providing the WebRTC Signalling Function in the RTC AS. In addition, the Media Function is present in the RTC AS to provide the role of RTC endpoint.

Figure A.4-1: Derivative RTC architecture for collaboration scenario 3

Next change

Annex B (normative):
Architecture variants for RTC Application

B.1	General

This annex describes variants of the RTC reference architecture (see figure 4.1.1-2) for different kinds of RTC Application.

B.2	RTC Application is a Native WebRTC App

The Native WebRTC App implements reference point RTC-6 for invoking media session handling functionality and reference point RTC-7 for establishing and controlling RTC session by using API provided by the RTC Access Function. The Native WebRTC App also uses reference point RTC-7 and RTC-4s to exchange RTC session signalling with the WebRTC Signalling Function in the RTC AS.

Figure B.2-1. RTC architecture variants for Native WebRTC App

B.3	RTC architecture for Web App

The Web App does not instantiate reference points RTC-6. Instead, it uses the W3C defined JavaScript APIs (including WebRTC API) which is exposed by RTC Access Function via RTC-7 to exchange RTC session signalling with the WebRTC Signalling Function in the RTC AS.

Figure B.3-1. RTC architecture variants for Web App

End of changes