3GPP TSG SA WG4 128	S4-241236

Jeju, South Korea, 20 - 24 May, 2024

Title:	LS Reply on FS_XRM Ph2

Response to:	LS (S2-2405625/S4-240874) from SA2

Release:	Release 19

Work Item:	FS_XRM Ph2, FS_5G_RTP_Ph2

Source:	SA WG4

To:	SA WG2,

Cc:	RAN WG2, RAN WG3

Contact Person:

Name:	Qi Pan

E-mail Address:	panqi8 AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

- 1. Overall Description:

SA4 thanks SA2 for the LS on FS_XRM Ph2 (S2-2405625/S4-240874) and would like to provide the feedback as following:

- Question1 [for SA4, RAN2 and RAN3]: PDU Set correlation information (Sol#23) provides the dependency relationship among PDU Sets. Does SA4, RAN2 and RAN3 see any improvement with adding inter-PDU set correlation information to assist RAN making PDU set discarding decision as comparing to the existing (R18) PDU Set information that is already provided by the AS?

Reply 1: Release 18 TS 26.522 clause 4.2.6.2 contains guidelines that assign PDU Set Importance implicitly taking the dependencies between PDU Sets into account. There is no consensus to add another solution besides the PDU Set importance in SA4. However, SA4 may further study this.

- Question2 [for SA4]: In Sol#29, PDU Set QoS or ordinary per packet based QoS (e.g. PER, PDB) can be applied for different media streams multiplexed in an IP flow, SA2 would like to ask SA4 whether a media stream (e.g. a video RTP stream) can include packet which is not related to PDU Set?

Reply 2: Yes, it may be possible. However, for a single media stream, i.e., a video stream, the common understanding from SA4 is that each PDU should be marked if PDU Set marking is enabled for this video stream. SA4 is also further studying whether and how to handle packets not related to a PDU Set in FS_5G_RTP_Ph2 from the media application perspective.

- Question4 [for SA4 and RAN2]: In Sol#30, the PSA UPF may identify the size of incoming burst based on N6 protocol, and send it to NG-RAN to assist RAN scheduling.

  - To SA4: is it possible that the application server provides the burst size in the first packet of the burst via N6?

Reply 4: Yes, but not always. It is possible since there are cases where the application server can become aware of the data burst size before sending it out. However, it depends on the application implementations and there are cases where it may introduce additional latency. Therefore, it needs to be confirmed by the application layer that such feature is available, or even feasible given the application requirements. SA4 is studying the enhancement of data burst related traffic characteristics in FS_5G_RTP_Ph2.

- Question5 [for SA4]: Some of the solutions support only QUIC-based media delivery. Can SA4 provide feedback on choosing only solutions for PDU Set identification for encrypted traffic that only support QUIC as transport protocol?

Reply 5:

SA4 has identified protocols with encrypted payloads other than QUIC for which PDU Set identification may be required, such as secured TURN and RTP over DTLS.

(For your information, SA4 is studying QUIC as transport protocol for segmented media delivery in its FS_AMD feasibility study, but has not yet studied the use of QUIC for Real-Time media Communication.)

- 2. Actions:

- To SA2:

ACTION: SA4 kindly asks SA2 to take the above into account and provide feedback if any.

- 3. Date of Next SA4 Meetings:

SA4#129-e	19th August – 23rd August 2024			Online

SA4#130	18th Nov – 22nd Nov 2024				Orlando, FL, USA