3GPP TSG-S4 Meeting # 128	S4-241198

Jeju, Korea,  May 20th - 24th, 2024                                                  (revision of S4-240926)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.143 | CR | 0002 | rev |  | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on Corrections and Transcoding Guidelines |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc., Fraunhofer IIS, Dolby |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | PROMISE |  |  |  |  |  | Date: |  |  | 14th May 2024 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There are small mistakes and typos in clause 5.8 on 3D scenes and assets. There are also no examples on messages that carry a 3D scene or asset. Furthermore, it is not clear how to handle 3D scenes and assets when the receiving UE does not supoprt this feature. Some inconsistencies in the signaling of the AOT for Audio codec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correction to the capabilities, 3D scenes and assets section, addition of an example, and addition of a new Annex on transcoding. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification will be erroneous and support of legacy devices is ambiguous. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.5.1, 5.5.2, 6.2.1, 5.8, B.1; C (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	Khronos glTF 2.0, glTF™ 2.0 Specification (khronos.org)

[3] 	ISO/IEC 23090-14 AMD 2, Information technology — Coded representation of immersive media — Part 14: Scene description — Amendment 2: Support for haptics, augmented reality, avatars, Interactivity, MPEG-I audio, and lighting

[4]	3GPP TS 26.511: "5G Media Streaming (5GMS); Profiles, Codecs and Formats".

[5]	3GPP TS 26.117: "5G Media Streaming (5GMS); Speech and audio profiles".

[6]	IETF Draft draft-ietf-mimi-content-01: "More Instant Messaging Interoperability (MIMI) message content", 	Rohan Mahy

[7]	3GPP TS 22.140: "Multimedia Messaging Service (MMS); Stage 1".

[8]	Open Mobile alliance, "MMS Architecture" OMA-AD-MMS-V1_3-20110913-A.

[9]	Open Mobile alliance, "Multimedia Messaging Service Encapsulation Protocol" OMA-TS-MMS_ENC-V1_3-20110913-A.

[10]	GSMA "RCS Universal Profile Service Definition Document", Version 2.6, 19 December 2022

[11]	GSMA PRD RCC.07 version 13.0 - "Rich Communication Suite - Advanced Communications Services and Client Specification" 19 December 2022

[12]	IETF RFC 2046, "Multipurpose Internet Mail Extensions (MIME) Part Two: Media Types"

[13]	ISO/IEC 14496-12: "Information technology - Coding of audio-visual objects -Part 12: ISO base media file format".

[14]	ISO/IEC 23000-24:2023 Preliminary Draft of: Information technology — Multimedia application format (MPEG-A) — Part 24: Messaging Media Application Format (MeMAF) ".

NOTE: 	A preliminary draft of this standard is available as MDS23345_W03_N1082 here: https://www.mpeg.org/wp-content/uploads/mpeg_meetings/144_Hannover/w23345.zip

[15]	3GPP 23.140: "Multimedia Messaging Service (MMS); Functional Description; Stage 2".

[16]	ITU-T Recommendation T.81: "Information technology; Digital compression and coding of continuous-tone still images: Requirements and guidelines".

[17]	"JPEG File Interchange Format", Version 1.02, September 1, 1992.

[18]	"Exchangeable image file format for digital still cameras: EXIF 2.2", Specification by the Japan Electronics and Information Technology Industries Association (JEITA), April 2002, URL: http://www.exif.org/

[19]	CompuServe Incorporated: "GIF Graphics Interchange Format: A Standard defining a mechanism for the storage and transmission of raster-based graphics information", Columbus, OH, USA, 1987.

[20]	Compuserve Incorporated, Columbus, Ohio (1990): "Graphics Interchange Format (Version 89a)".

[21]	IETF RFC 2083: "PNG (Portable Networks Graphics) Specification version 1.0 ", T. Boutell, et. al., March 1997.

[22]	ISO/IEC 23000-22:2019 Information technology — Multimedia application format (MPEG-A) — Part 22: Multi-image application format (MIAF)

[23]	IETF RFC 2045, "Multipurpose Internet Mail Extensions (MIME) Part One: Format of Internet Message Bodies", November 1996

[24]	ISO/IEC 23008-12:2019 Information technology — High efficiency coding and media delivery in heterogeneous environments — Part 12: Image File Format

[25]	ITU-T Recommendation H.265 (02/2018): "High efficiency video coding".

[26]	3GPP TS 26.244: "Transparent end-to-end packet switched streaming service (PSS); 3GPP file format (3GP)"

[27]	3GPP TS 26.245: "Transparent end-to-end packet switched streaming service (PSS); Timed text format"

[28]	ISO/IEC 14496-30: "Information technology - Coding of audio-visual objects - Part 30: Timed text and other visual overlays in ISO base media file format".

[29]	IETF RFC 2387, "The MIME Multipart/Related Content-type"

[30]	IETF RFC 6381, "The 'Codecs' and 'Profiles' Parameters for "Bucket" Media Types"

[31]	3GPP TS 26.307, "Presentation Layer for 3GPP Services"

[32]	3GPP TS 26.140, "Multimedia Messaging Service (MMS); Media formats and codecs"

[33]	IETF RFC 2077, "The Model Primary Content Type for Multipurpose Internet Mail Extensions"

[34]	3GPP TS 26.119, "Media Capabilities for Augmented Reality"

2nd Change

### 5.5.1	Player and Decoding capabilities

The capability 26143_AUDIO_EVS is defined as the capability of playing back (decoding and rendering) a file that

-	is decodable by a decoder capable of the EVS decoding capabilities as defined in clause 5.2 of TS 26.117 [5] and the receiver requirements in clause 6.2.4.2 of TS 26.117 [5],

-	is encapsulated in an ISO BMFF Track [14] conforming with the requirements of the sample entry 'sevs' as defined in TS 26.244 [26],

-	is contained in a 3GP file that conforms to the 26143_CONTAINER_MP4_3GP9 capability as defined in clause 5.2.

In the context of this specification, the media type for files with this capability 26143_AUDIO_EVS shall be signalled with audio/mp4, profile="3gp9" codecs="sevs" or an equivalently compatible media type.

The capability 26143_AUDIO_AMR-WB is defined as the capability of playing back (decoding and rendering) a file that

-	is decodable by a decoder capable of the AMR-WB decoding capabilities as defined in clause 5.2 of TS 26.117 [5] and the receiver requirements in clause 6.2.3.2 of TS 26.117 [5],

-	is encapsulated in an ISO BMFF Track [14] conforming with the requirements of the sample entry 'sawb' as defined in TS 26.244 [26],

-	is contained in a 3GP file that conforms to the 26143_CONTAINER_MP4_3GP9 capability as defined in clause 5.2.

In the context of this specification, the media type for files with this capability 26143_AUDIO_EVS shall be signalled with audio/mp4, profile="3gp9" codecs="sawb" or an equivalently compatible media type.

The capability 26143_AUDIO_AMR is defined as the capability of playing back (decoding and rendering) a file that

-	is decodable by a decoder capable of the AMR decoding capabilities as defined in clause 5.2 of TS 26.117 [5] and the receiver requirements in clause 6.2.2.2 of TS 26.117 [5],

-	is encapsulated in an ISO BMFF Track [14] conforming with the requirements of the sample entry 'samr' as defined in TS 26.244 [26],

-	is contained in a 3GP file that conforms to the 26143_CONTAINER_MP4_3GP9 capability as defined in clause 5.2.

In the context of this specification, the media type for files with this capability 26143_AUDIO_EVS shall be signalled with audio/mp4, profile="3gp9" codecs="samr" or an equivalently compatible media type.

The capability 26143_AUDIO_XHE-AAC is defined as the capability of playing back (decoding and rendering) a file that

-	is decodable by a decoder capable of the xHE-AAC stereo decoding capabilities as defined in clause 5.2 of TS 26.117 [5] and the receiver requirements in clause 6.4.2.2 of TS 26.117 [5],

-	is encapsulated in an ISO BMFF Track [14] conforming with the requirements of the sample entry 'mp4a.40.42',

-	is contained in a 3GP file that conforms to the 26143_CONTAINER_MP4_3GP9 capability as defined in clause 5.2.

In the context of this specification, the media type for files with this capability 26143_AUDIO_XHE-AAC shall be signalled with audio/mp4, profile="3gp9" codecs="mp4a.40.42" or an equivalently compatible media type.

The capability 26143_AUDIO_EAAC+ is defined as the capability of playing back (decoding and rendering) a file that

-	is decodable by a decoder capable of the eAAC+ decoding capabilities as defined in clause 5.2 of TS 26.117 [5] and the receiver requirements in clause 6.3.2.2 of TS 26.117 [5],

-	is encapsulated in an ISO BMFF Track [14] conforming with the requirements of the sample entry 'mp4a.40.5',

-	is contained in a 3GP file that conforms to the 26143_CONTAINER_MP4_3GP9 capability as defined in clause 5.2.

In the context of this specification, the media type for files with this capability 26143_AUDIO_EAAC+ shall be signalled with audio/mp4, profile="3gp9" codecs="mp4a.40.5" or an equivalently compatible media type.

3rd Change

### 5.5.2	MMBP Content Generator capabilities

The capability 26143_AUDIO_ENC_EVS for a content generator is defined as the combination of the following capabilities:

-	the capability to generate a file from an audio signal in real-time, such that the file can be played back by a player with the capability 26143_AUDIO_EVS,

-	the EVS encoding capabilities as defined in clause 5.3 of TS 26.117 [5] and the sender requirements in clause 6.2.4.3 of TS 26.117 [5],

-	the capability to generate an ISO BMFF track that conforms with the requirements of the sample entry 'sevs' as defined in TS 26.244 [26].

-	the generation of a 3GP file from the ISO BMFF track that conforms to the 26143_CONTAINER_MP4_3GP9 capability as defined in clause 5.2.

-	the provisioning of media type signalling with the generated file using audio/mp4, profile="3gp9" codecs="sevs" or an equivalently compatible media type.

The capability 26143_AUDIO_ENC_AMR-WB for a content generator is defined as the combination of the following capabilities:

-	the capability to generate a file from an audio signal in real-time, such that the file can be played back by a player with the capability 26143_AUDIO_AMR-WB,

-	the AMR-WB encoding capabilities as defined in clause 5.3 of TS 26.117 [5] and the sender requirements in clause 6.2.3.3 of TS 26.117 [5],

-	the capability to generate an ISO BMFF track that conforms with the requirements of the sample entry 'sawb' as defined in TS 26.244 [26].

-	the generation of a 3GP file from the ISO BMFF track that conforms to the 26143_CONTAINER_MP4_3GP9 capability as defined in clause 5.2.

-	the provisioning of media type signalling with the generated file using audio/mp4, profile="3gp9" codecs="sawb" or an equivalently compatible media type.

The capability 26143_AUDIO_ENC_AMR for a content generator is defined as the combination of the following capabilities:

-	the capability to generate a file from an audio signal in real-time, such that the file can be played back by a player with the capability 26143_AUDIO_AMR,

-	the EVS encoding capabilities as defined in clause 5.3 of TS 26.117 [5] and the sender requirements in clause 6.2.2.3 of TS 26.117 [5],

-	the capability to generate an ISO BMFF track that conforms with the requirements of the sample entry 'samr' as defined in TS 26.244 [26].

-	the generation of a 3GP file from the ISO BMFF track that conforms to the 26143_CONTAINER_MP4_3GP9 capability as defined in clause 5.2.

-	the provisioning of media type signalling with the generated file using audio/mp4, profile="3gp9" codecs="samr" or an equivalently compatible media type.

The capability 26143_AUDIO_ENC_XHE-AAC for a content generator is defined as the combination of the following capabilities:

-	the capability to generate a file from an audio signal in real-time, such that the file can be played back by a player with the capability 26143_AUDIO_XHE-AAC,

-	the xHE-AAC stereo encoding capabilities as defined in clause 5.3 of TS 26.117 [5] and the sender requirements in clause 6.4.2.3 of TS 26.117 [5],

-	the capability to generate an ISO BMFF track that conforms with the requirements of the sample entry 'mp4a.40.42' as defined in TS 26.244 [26].

-	the generation of a 3GP file from the ISO BMFF track that conforms to the 26143_CONTAINER_MP4_3GP9 capability as defined in clause 5.2.

-	the provisioning of media type signalling with the generated file using audio/mp4, profile="3gp9" codecs="mp4a.40.42" or an equivalently compatible media type.

The capability 26143_AUDIO_ENC_EAAC+ for a content generator is defined as the combination of the following capabilities:

-	the capability to generate a file from an audio signal in real-time, such that the file can be played back by a player with the capability 26143_AUDIO_EAAC+,

-	the eAAC+ encoding capabilities as defined in clause 5.3 of TS 26.117 [5] and the sender requirements in clause 6.3.2.3 of TS 26.117 [5],

-	the capability to generate an ISO BMFF track that conforms with the requirements of the sample entry 'mp4a.40.5' as defined in TS 26.244 [26].

-	the generation of a 3GP file from the ISO BMFF track that conforms to the 26143_CONTAINER_MP4_3GP9 capability as defined in clause 5.2.

-	the provisioning of media type signalling with the generated file using audio/mp4, profile="3gp9" codecs="mp4a.40.5" or an equivalently compatible media type.

4th Change

### 6.2.1	Overview

The baseline MMBP Player profile is aligned with TS 26.140 [32].

The container format is based on IETF RFC 2045 [23] as the format for the MMBPs. Offering of alternative content is the container is permitted. In addition, the profile permits to encapsulate encapsulation of real-time video into the 3GP file format using the baseline profile. The container does not support external bodies, i.e. the MMBP is expected to be delivered as a single message. For details on the container format requirements, refer to clause 6.2.2.

The media types address basic text, audio/speech, images, video, text/subtitle, 3D scenes including AR as well as simple HTML-5 presentations.

Content conforming to the baseline MMBP player may include media types that are not explicitly supported by the media capabilities as defined in clause 6.2.3. Receivers shall ignore non-recognized media types. However, based on the container requirements, ignoring media types may results in specific processing requirements, for example pick an alternative, or ignore the entire MMBP.

Content generated to be compatible for playback on players for this profile as well as players expose their capabilities should use the URN identifier "urn:3GPP:26143:18:baseline-mmbp-player".

5th Change

## 5.8	3D scenes and assets

The capability 26143_SCENE_GLTF20 is defined as the capability of rendering glTF 2.0 scenes as specified in [2] for which all components of the 3D scene are included as multiple parts in a related MMBP as defined in clause 4.4, for which the root MMBP is a glTF2.0 JSON document.

The capability 26143_SCENE_GLTF20_GLB is defined as the capability of rendering glTF 2.0 scenes as specified in [2] for which all components of the 3D scene are either

-	encapsulated in a GLB file, or

-	included as multiple parts in a related MMBP, for which the root MMBP is a GLB file.

The capability 26143_SCENE_GLTF20_AR is defined as the capability of 26143_SCENE_GLTF20 with the addition, that the scene may include MPEG_anchor extension, EXT_lights_image_based and MPEG_lights_texture_based extensions as defined in [3].

The capability 26143_SCENE_GLTF20_GLB_AR is defined as the capability of 26143_SCENE_GLTF20_GLB with the addition, that the scene may include MPEG_anchor extension, EXT_lights_image_based and MPEG_lights_texture_based extensions as defined in [3].

It is recommended that clients supporting such capabilities determine if the media can be safely processed prior to any processing and rendering.

In the context of this specification, the media type for scenes with this capability 26143_SCENE_GLTF20 and 26143_SCENE_GLTF20_ AR shall be signalled with model/gltf+json as defined in [2].

In the context of this specification, the media type for scenes with this capability 26143_SCENE_GLTF20_GLB and 26143_SCENE_GLTF20_GLB_AR shall be signalled with model/gltf-binary as defined in [2].

Note: Annex C provides guidelines on the transcoding of 3D scenes and assets in messages for legacy devices.

6th Change

Annex B (informative):
Examples

## B.1 MMBP message with a 3D asset

In this example we show an excerpt of a message that contains a 3D asset. The root entry is a glTF 2.0 file. The message has two additional parts: a binary file that contains the geometry of the 3D asset and an image that provides the texture of the 3D asset.

MIME-Version: 1.0 Content-Type: multipart/related; boundary="===============" =============== Content-Type: model/gltf+json Content-Disposition: attachment; filename="sofa.gltf" { "asset": { "version": "2.0" }, "buffers": [ { "uri": "buffer.bin", "byteLength": 1024 } ], "images": [ { "uri": "texture.jpg" } ], "scenes": [ { "nodes": [0] } ], "nodes": [ { "mesh": 0 } ], "meshes": [ { "primitives": [ { "attributes": { "POSITION": 0 }, "indices": 1 } ] } ], "accessors": [ { "bufferView": 0, "componentType": 5126, "count": 24, "type": "VEC3" } ], "bufferViews": [ { "buffer": 0, "byteOffset": 0, "byteLength": 288 } ] } =============== Content-Type: application/octet-stream Content-Disposition: attachment; filename="buffer.bin" … binary data … =============== Content-Type: image/jpeg Content-Disposition: attachment; filename="texture.jpg" … binary data … ===============

7th Change

Annex C (informative)
Transcoding Guidelines

## C.1 Transcoding 3D scenes and assets

Transcoding may need to be performed to support UEs that lack the capabilities to render 3D scenes and assets as defined in clause 5.8. The transcoding operation for 3D scenes and assets is effectively a rendering operation, where the 3D scene or asset is rendered into an image, image sequence, or video. The Messaging Server (e.g. MMS Server/Relay) may select an appropriate pose or set of poses (e.g. along a pre-defined path) that it uses to render the 3D scene or asset.