3GPP TSG-SA5 Meeting #157 	S5-245979

Hyderabad, India, 14 - 18 October 2024

Title:	Reply LS on AIML Data Collection

Response to:	S5-245373

Release:	Rel-19

Work Item:	NR_AIML_Air

Source:	SA5

To:	RAN2

Cc:	RAN1, RAN2, RAN3, TSG SA, WG SA2, WG SA3

Contact person:	Sivaramakrishnan Swaminathan

sivaramakrishnan.swaminathan@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

SA5 received LS from RAN on AIML data collection in S5-245373/RP-242389. The LS provides an analysis of different data collection options for UE-side model training. SA5 thanks RAN for sharing the requirements for data collection.

SA5 expects the RAN provide the clarification for the following:

- Clarify which standardized data are collected?

- The endpoint “Server for data collection for UE -side model training” needs further clarification whether it is a “Network entity fully control by MNO” or a “Network entity not control by MNO”.

SA5 has defined the following measurement collection mechanism regarding UE data collection.

- Trace & MDT are documented in TS 32.422, which can collect M1 ~ M7 measurements.

- NG-RAN UE level measurements collection and reporting are supported by MDT mechanisms for both signalling and management-based activation, which are documented in TS 28.558.

SA5 would like to provide views on the analysis of different data collection options for UE-side model training as follows:

- For option3 CP tunnel: The CP tunnel indicated in option 3 can be realized using MDT feature and per-UE measurements specified in SA5. The OAM configures the gNB to configure the UEs to collect specific MDT measurements as specified in RAN2. The UE collects the MDT measurements and transfer to gNB. The gNB subsequently forwards the MDT measurements to either an IP address of the Trace Collection Entity in case of file transfer mechanism or a streaming URI in case of streaming mechanism. The MDT data is transferred from UE to gNB via RRC signaling. This approach is feasible provided the data volume remains within the RRC signaling capacity. If the data volume is high, there might be issues in using RRC signaling, this limitation comes from the RRC protocol layer. There is no limitation from OAM point of view. The user privacy, anonymity and user consent is respected as MDT is triggered only when user consent is available. The MNO has full control over the data and the transfer process with OAM solution providing the Trace job to configure the MDT and collect the measurements and the MNO has full visibility of the standardized data. Therefore, the CP tunnel indicated in option 3 whether can be realized using MDT feature specified in SA5 depending on RAN standardized data.

- For option3 UP tunnel, The destination information of the MDT measurements at OAM in the current solution is not known to the UE for security reasons. The UP tunnel between UE and OAM is feasible but the existing solution cannot be re-used due to security concerns. This solution would require additional specification work to address the security concerns depending on the data that needs to be collected. It would be better to have clarification on the data needs to be collected in the UP option.

- For option 1b and option 2, in case of “Server for data collection for UE -side model training” is a Network entity that fully control by MNO, the OAM configurations on the Server for data collection for UE -side model training/CN/gNB to support the data collection may needed.

Conclusion:

- The CP tunnel approach using MDT measurements discussed as part of option 3 is supported in SA5 already and the solution satisfies all the requirements proposed by RAN in their LS provided if the data is standardized. The UP tunnel indicated in option 3 at OAM is feasible but this needs additional specification work to address the security concerns.

- For option 1b and option 2, there may be management and control requirements for SA5. If there are management support for these two options, the discussion of option1b and option2 should have SA5 involved.

# 2	Actions

To RAN

ACTION: 	SA5 would like to ask RAN to take above conclusion into account.

# 3	Dates of next TSG SA WG 5 meetings

SA5#158		18 November - 22 November 2024		Orlando, US

SA5#159		17 February - 21 February 2025		Sophia-Antipolis, France