3GPP TSG-SA5 Meeting #157	S5-246001

Hyderabad, India, 14 - 18 October 2024

- Source:	Huawei, Ericsson, ZTE, Intel

- Title:	Rel-19 pCR TR 28.858 add use case of managing NG-RAN AI/ML-based Coverage and Capacity Optimization

- Document for:	Approval

Agenda Item:	6.19.1

- 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

- 2	References

[1]		SP-231780 " New SID: Study on AI/ML management - phase 2".

[2]		TR 38.743 Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN

[3]		TS 28.105 Artificial Intelligence / Machine Learning (AI/ML) management

- 3	Rationale

In TS 28.105 [3], the Managing NG-RAN AI/ML-based use case defined in clause 6.5.4.2 including Managing NG-RAN AI/ML-based distributed Network Energy Saving, Managing NG-RAN AI/ML-based distributed Mobility Optimization and Managing NG-RAN AI/ML-based distributed Load Balancing. These three use cases have independent requirements.

TR 38.743 [2] defines the use case of AI/ML based Coverage and Capacity Optimization, and states that:

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU

Therefore, we propose to add new use case of Managing NG-RAN AI/ML-based Coverage and Capacity Optimization to support AI/ML based Coverage and Capacity Optimization.

This contribution proposes to add the use case and solutions of Managing NG-RAN AI/ML-based Coverage and Capacity Optimization.

- 4	Detailed proposal

Start of First change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.105: Artificial Intelligence / Machine Learning (AI/ML) management".

[3]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics".

[4]	3GPP TS 28.541: " Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[x]	3GPP TR 38.743 Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN.

Next change

## 5.5	AI/ML inference

### 5.5.x 	AI/ML inference capability configuration management for NG-RAN

#### 5.5.x.1 	Description

In TS 28.105 [2], the existing use cases (AI/ML inference capability configuration management in clause 6.5.4) are described to support managing NG-RAN AI/ML-based related use case.

5.5.x.2	Use cases

TR 38.743 [x] define the use case of AI/ML based Coverage and Capacity Optimization. The MnS producer for AI/ML inference management needs to provide a capability for configuration of the AI/ML based Coverage and Capacity Optimization inference function.

##### 5.5.x.2.1	Managing NG-RAN AI/ML-based Coverage and Capacity Optimization

An NG-RAN AI/ML-based Coverage and Capacity Optimization capability may use one or more ML models or AI/ML Inference Functions to derive CCO recommendations.

The MnS consumer monitors the network performance and determines whether to, and when to activate or deactivate an AI/ML Inference Functions related to an AI/ML-based Coverage and Capacity Optimization function. The activation and deactivation actions for AI/ML Inference Functions related to an AI/ML-based Coverage and Capacity Optimization conducted by the MnS producer may also be triggered by some defined policies provided by the consumer.

##### 5.5.x.3	Potential Requirements

REQ-AIML_INF-01: The MnS producer of NG-RAN AI/ML-based Coverage and Capacity Optimization should have a capability to allow an authorized MnS consumer to manage the ML model and/or AI/ML Inference Function related to the Coverage and Capacity Optimization functions.

#### 5.5.x.2	Potential solutions

Enhance the CCOFunction IOC (TS 28.541 [4]) with the attribute mLModelRefList and aIMLInferenceFunctionRefList. Attribute mLModelRefList indicates that the ML models that are used to support this function. Attribute aIMLInferenceFunctionRefList indicates the AI/ML Inference Function that is used to support this function.

The proxyClass of AIMLSupportedFunction defined in “NRM fragment for AI/ML inference function” needs to be extended with CCOFunction IOC.

The existing AIMLInferenceFunction IOC defined in TS 28.105 [2] with activationStatus and managedActivationScope can be reused to support managing NG-RAN AI/ML-based Network Slicing function.

Editor notes: This use case and related solution is based on RAN use case AI/ML-based Coverage and Capacity Optimization described in TR 38.743. Any modification in RAN study may require changes to SA5 solution.

End of First change