# 3GPP TSG-RAN2 Meeting #127	R2-2407796

# Maastricht, Netherlands, August 19-23, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | 5042 | rev | 1 | Current version: | 14.17.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

8

| Title: | Correction to MIB-MBMS systemFrameNumber field description |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Samsung, ABS, SJTU, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | MBMS_LTE_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Currently the field description of systemFrameNumber in MIB-MBMS is as follows: MasterInformationBlock-MBMS field descriptions systemFrameNumber Defines the 6 most significant bits of the SFN of the MBMS-dedicated cell. As indicated in TS 36.211 [21, 6.6.1], the 4 least significant bits of the SFN are acquired implicitly in the P-BCH decoding, i.e. timing of 160ms P-BCH TTI indicates 4 least significant bits (within 40ms P-BCH TTI, the first radio frame: 00, the fourth radio frame: 01, the eighth radio frame: 10, the last radio frame: 11). The problem is with the second part (yellow highlight above). It looks like this was due to copy-paste from non-MBMS MIB, shown below: MasterInformationBlock field descriptions systemFrameNumber Defines the 8 most significant bits of the SFN. As indicated in TS 36.211 [21], 6.6.1, the 2 least significant bits of the SFN are acquired implicitly in the P-BCH decoding, i.e. timing of 40ms P-BCH TTI indicates 2 least significant bits (within 40ms P-BCH TTI, the first radio frame: 00, the second radio frame: 01, the third radio frame: 10, the last radio frame: 11). For non-MBMS MIB, Section 5.2.1.2 captures the following: The MIB uses a fixed schedule with a periodicity of 40 ms and repetitions made within 40 ms. The first transmission of the MIB is scheduled in subframe #0 of radio frames for which the SFN mod 4 = 0, and repetitions are scheduled in subframe #0 of all other radio frames. So, it is clear that the 2 LSBs of SFN can be inferred from the MIB transmission as described in the field description. In contrast, one should note that the periodicity (and repetition) of MIB and MIB-MBMS are not the same. The MIB-MBMS uses a fixed schedule with a periodicity of 160 ms and repetitions made within 160 ms. The first transmission of the MIB-MBMS is scheduled in subframe #0 of radio frames for which the SFN mod 16 = 0, and repetitions are scheduled in subframe #0 of all other radio frames for which the SFN mod 4 = 0. This is captured in TS 36.211 section 6.6.4 as follows: The block of complex-valued symbols for each antenna port shall - for an MBMS-dedicated cell, be transmitted during 4 consecutive radio frames fulfilling , starting in each radio frame fulfilling , and - otherwise, be transmitted during 4 consecutive radio frames, starting in each radio frame fulfilling . In addition, TS 36.211 clause 6.6.1 states the following about PBCH: The scrambling sequence shall be initialised with in each radio frame fulfilling . For an MBMS-dedicated cell, the scrambling sequence shall be initialised with in each radio frame fulfilling . This means, unlike MIB, although 4 LSBs are indicated by P-BCH decoding, in reality the 2 LSBs in the systemFrameNumber in MBMS-MIB are always ‘00’ and the remaining 2 out of the 4 LSBs are indicated implicitly based on the P-BCH timing as specified in TS 36.211. So, the correct intent of the above field description is as follows: For an MBMS-dedicated cell, within the 160ms P-BCH periodicity, the 2 LSBs are always ‘00’ and the 3rd/4th LSBs indicated implicitly by the P-BCH decoding. I.e., the P-BCH transmitted in the 1st RF indicates 0000 (i.e. first radio frame within 160ms, not within 40ms), the P-BCH transmitted in the second 40ms duration indicates 0100 (i.e. the fifth radio frame within 160ms, not the fourth radio frame nor within the 40ms), the P-BCH transmitted in the 3rd 40ms duration indicates 1000 (i.e. the ninth radio frame within 160ms, not the 8th, nor within 40ms), the P-BCH transmitted in the 4th 40ms duration indicates 1100 (i.e. the 13th radio frame within 160ms, not the last, nor within 40ms). Additionally, regarding “within 40ms P-BCH TTI”: - For MBMS, the P-BCH TTI is 160ms, not 40ms. - there cannot be ‘eighth’ radio frame within 40ms P-BCH TTI. So, the existing text is clearly wrong at multiple places. In summary, the field description is not only unclear, but also incorrect as well, needing an essential correction. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct the field description of systemFrameNumber in MasterInformationBlock-MBMS. Impact Analysis: Impacted feature: LTE MBMS Interoperability Analysis: - Implementations done according to TS 36.211 are not impacted. Since the incorrect text in the concerned field description is impossible to implement (e.g. there is no 8th radio frame in a 40ms period), no interoperability issue is foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The RRC specification for MBMS remains ambiguous, incorrect, and misaligned with TS 36.211. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

### 6.2.2	Message definitions

<<unchanged text skipped>>

#### –	MasterInformationBlock-MBMS

The MasterInformationBlock-MBMS includes the system information transmitted on BCH.

Signalling radio bearer: N/A

RLC-SAP: TM

Logical channel: BCCH

Direction: E-UTRAN to UE

MasterInformationBlock-MBMS

-- ASN1START

MasterInformationBlock-MBMS-r14 ::=			SEQUENCE {

dl-Bandwidth-MBMS-r14						ENUMERATED {

n6, n15, n25, n50, n75, n100},

systemFrameNumber-r14					BIT STRING (SIZE (6)),

additionalNonMBSFNSubframes-r14		INTEGER (0..3),

spare									BIT STRING (SIZE (13))

}

-- ASN1STOP

MasterInformationBlock-MBMS field descriptions

additionalNonMBSFNSubframes Configures additional non-MBSFN subframes where SystemInformationBlockType1-MBMS and SystemInformation-MBMS may be transmitted. Value 0, 1, 2, 3 mean zero, one, two, three additional non-MBSFN subframes are configured after each subframe which has PBCH.

dl-Bandwidth-MBMS Parameter: transmission bandwidth configuration, NRB in downlink, see TS 36.101 [42, table 5.6-1]. n6 corresponds to 6 resource blocks, n15 to 15 resource blocks and so on.

systemFrameNumber Defines the 6 most significant bits of the SFN of the MBMS-dedicated cell. As indicated in TS 36.211 [21, 6.6.1], the 4 least significant bits of the SFN are acquired implicitly in the P-BCH decoding, i.e. timing of 160ms P-BCH TTI indicates 4 least significant bits (within 160ms P-BCH TTI, the first radio frame: 0000, the fifth radio frame: 0100, the ninth radio frame: 1000, the thirteenth radio frame: 1100).

End of Changes