3GPP TSG-RAN WG2 Meeting #127	R2-2406957

Maastricht, Netherlands, 19 – 23 August 2024

Agenda item:	8.11.3

Source:	Nokia

Title:	Other Aspects of SBFD

WID/SID:	NR_duplex_evo-Core - Release 19

Document for:	Discussion and Decision

# 1 	Introduction

In RAN#102 meeting, a Work Item (WI) on Evolution of NR duplex operation: Subband full duplex (SBFD) was approved [1]. The work item description has been updated in RAN#104 meeting and contain the following objectives [RP-241614]:

For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier: - Specify semi-static indication of time location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2] - Indication of time location of SBFD subbands in SIB is not precluded - Specify semi-static indication of frequency domain location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2] - Indication of frequency domain location of SBFD subbands in SIB is not precluded - Specify SBFD operation to support random access in SBFD symbols by UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode [RAN1, RAN2] - Specify UE transmission, reception and measurement behavior and procedures in SBFD symbols and/or non-SBFD symbols for SBFD aware UE [RAN1, RAN2] - Transmission and reception behaviours on SBFD subbands configured in DL and/or flexible symbol indicated by TDD-UL-DL-ConfigCommon - UL transmissions within UL subband only - DL receptions within DL subband(s) only, except for CLI measurement by the UE outside of the DL subbands Note: When flexible symbols are used, it is not expected that any legacy Uplink symbol is converted to Downlink/SBFD symbols - Enhancement on resource allocation in frequency domain in SBFD symbols, including - resource allocation in frequency domain for PDSCH/CSI-RS across two DL subbands in SBFD symbols - handling of unaligned boundaries between SBFD subband(s) and RBG, CSI reporting subband, CSI-RS resource, PRG - Enhancements on physical channels/signals and procedure across SBFD symbols and non-SBFD symbols in different slots, where each transmission/reception within a slot has either all SBFD or all non-SBFD symbols, including - resource allocation in frequency domain for transmission or reception in SBFD symbols and non-SBFD symbols with different available frequency resource in different slots - CSI report of which associated CSI-RS instances occur in both SBFD symbols and non-SBFD symbols in different slots - Configurations for SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols, e.g., resources, frequency hopping parameters, UL power control parameters and/or beam/spatial relation - Collision handling between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol - Followings are assumed based on TR 38.858 - SBFD at the gNB side - Half duplex operation at the UE side - FR1 and FR2-1 - SBFD operation Option 4, i.e., both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs - Coexistence between non-SBFD aware UEs (including legacy UEs) and SBFD aware UEs in the cell operating SBFD at gNB side - SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies - One UL subband for SBFD operation in an SBFD symbol (excluding legacy UL symbol/slot) within a TDD carrier - Mechanisms for SBFD operation shall also consider the adjacent channel coexistence between two operators - Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - Information exchange among gNBs, including [RAN3] - Semi-static cell-specific SBFD time and frequency location configuration - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS - Strongest DL beam information - CLI-mitigation request - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain - PUSCH resource mapping, i.e., rate-matching around the muted REs - UCI resource determination in symbols with muted REs - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource - At least aperiodic reporting - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - UCI bits generation - Priority rules for multiple CSI reporting - CLI measurement accuracy requirement - Note: Without dedicated optimization for dynamic/flexible TDD. - Specify BS RF requirements for SBFD operation at gNB [RAN4] - Specify applicable RRM core requirements for CLI handling mechanisms [RAN4] - Specify other RRM core requirements for SBFD operation, if identified [RAN4] - Note: RAN3 will not specify enhancements to network signalling to support inter-operator coordination for CLI handling

In this contribution we provide our view on relevant RAN2-aspects according to the above objectives regarding need for reselection control for SBFD supporting UEs We start with a general overview of SBFD in Section 2, taking into account the agreements reached during the RAN1 Rel-18 study item phase and Rel-19 RAN1 normative work so far, followed by more detailed discussion

# 2	Status of Subband Full duplex (SBFD) standardization in RAN1

## 2.1	SBFD High-level overview

SBFD was deeply studied as part of the Evolution of Duplex Operation Study item that took place in Release 18 in RAN1 and RAN4 working groups (together with enhancements for dynamic and flexible TDD operation, although these are deprioritized in the current Rel-19 work item phase). RAN1 focused on performance evaluation aspects as well as physical layer enablers for SBFD, while RAN4 primarily focused on the feasibility of SBFD operation from BS RF perspective, as well as coexistence of SBFD with legacy TDD in adjacent channel; The conclusions of the Rel-18 study can be found in TR 38.858. The work item phase started in RAN1 in February 2024, where some of the key agreements reached are further described in coming sections.

SBFD refers to a new mode of operation in unpaired spectrum where separate set of RBs within a TDD carrier are used for UL and DL transmission simultaneously. The intention is to allow more UL transmission opportunities compared to the typical DL-heavy TDD configurations used in today’s TDD deployments. This bring benefits in terms of UL coverage and UL latency especially in wide area macro networks where many UEs are in power-limited conditions in the UL direction.

More specifically, ‘SBFD subbands’ are defined as 1 RB or more consecutive RBs which are used for the same transmission direction (either UL or DL). The following high-level principles on SBFD operation were agreed during the Rel-18 study item phase (as also noted in the work item description):

- Simultaneous transmission and reception (on non-overlapping set of RBs) is assumed at the gNB side, while half-duplex operation is assumed the UE side. RAN1 is currently specifying UE behavior for handling collision and priorities between UL and DL channels.

- Only one UL subband, while one or at most two DL subbands within a TDD carrier. These are commonly referred as DU (or UD) or DUD operation, respectively – see Figure 1.

- UL transmissions take place within UL subband only, while DL receptions take place within DL subband(s) only.

- SBFD-aware UEs is used to refer to UEs supporting (at least some) SBFD-related features to be standardized in Rel-19.

- As baseline, it is assumed that both time and frequency locations of subbands for SBFD operation are known to SBFD-aware UEs, while this information is not available for legacy (pre Rel-19) UEs.

- Legacy UEs are not aware of SBFD operation and thus behave in similar manner as in today’s TDD system. The gNB (e.g. by implementation) may ensure that UL transmission and DL receptions are confined within the corresponding subbands.

- The Rel-18 study concluded that non-SBFD aware UEs, including legacy UEs, and SBFD aware UEs can coexist in cells with SBFD operation at gNB side from RAN1 specification point of view.

Observation 1: For SBFD operation within a TDD carrier, RAN1 has agreed on the following principles:

- Simultaneous transmission and reception (on non-overlapping set of RBs) is assumed at the gNB side, while half-duplex operation is assumed the UE side.

- Only one UL subband, while one or at most two DL subbands within a TDD carrier, also referred as DU/UD or DUD operation, respectively.

- UL transmissions take place within UL subband only, while DL receptions take place within DL subband(s) only.

- As baseline, it is assumed that both time and frequency locations of subbands for SBFD operation are known to SBFD-aware UEs, while this information is not available for legacy (pre Rel-19) UEs.

- From RAN1 specification point of view, non-SBFD aware UEs, including legacy UEs, and SBFD aware UEs can coexist in cells with SBFD operation at gNB side.

Figure 1 illustrates an example arrangement of UL and DL resources for both DU/UD (left) and DUD (right) operation. SBFD symbols refer to those symbols (or slots) where both a UL and DL subband(s) are simultaneously available, whole non-SBFD symbols refer to (legacy) uni-directional symbols. Different time-domain sequences were discussed by RAN1 in the TR 38.858, e.g. DXXXU, XXXXX, or XXXXU where D, U, X refers to downlink, uplink and SBFD slots, respectively. One or more RBs may be used as guard band between the UL and DL subbands to facilitate rejection of cross-link interference; Similarly, a guard time or period of one or more OFDM symbols may take place for the transition between different symbol/slot types.

Figure 1: SBFD resources in time and frequency domain

## 2.2	Signaling of SBFD time and frequency configuration

RAN1 has already reached a few agreements regarding how to indicate the location of SBFD subbands to SBFD-aware UEs. Detailed agreements can be found in [x]. Time-domain aspects and frequency-domain aspects of the signaling were separately discussed, where the former provides the location of the SBFD symbols (i.e. symbols where SBFD operation shall take place) and the latter provides the split of RBs between DL and UL direction in such SBFD symbols.

An example of the agreed signaling framework in time-domain is shown in Figure 1:

- SBFD symbols can be configured in DL and/or flexible symbols configured in TDD-UL-DL-ConfigCommon.

- The SBFD symbols are configured in consecutive manner and independently for each TDD-UL-DL pattern period. Note that the specs allow one or two TDD-UL-DL pattern periods in TDD-UL-DL-ConfigCommon (when two periods are configured, they repeat periodically after each other: pattern1, pattern2, pattern1, pattern2, …).

- The configured SBFD symbols can start from any symbol within a slot and can end in any symbol within a slot.

- referenceSubcarrierSpacing in TDD-UL-DL-ConfigCommon is used as reference SCS.

Figure : Example of time-domain configuration of SBFD subbands

In frequency domain,

- The maximum number of UL subbands for SBFD operation in an SBFD symbol within a TDD carrier is one. The UL subband can be located at one side of the carrier or can be located at the middle part of the carrier.

- Frequency locations of UL subband and DL subband(s) are explicitly configured. Guardband(s) if any are implicitly derived as the RBs which are not within UL subband or DL subband(s).

- The subband frequency-domain resources are same across different SBFD symbols within a TDD carrier. Frequency location of cell specific UL subband, and DL subband(s) if explicitly indicated, are indicated with reference to CRB grid.

- RB-level granularity is supported for semi-static indication of SBFD subband frequency location.

- Cell-specific frequency locations of SBFD subbands are separately configured for each SCS configuration in SCS-SpecificCarrierList. For each SCS configuration, the reference starting PRB is the PRB determined by the SCS configuration and offsetToCarrier corresponding to this subcarrier spacing.

Last but not least, RAN1 has also agreed on the following general principles for the SBFD signaling:

- For RRC connected mode UEs, at least cell-specific configuration on time and frequency (working assumption) location of SBFD subbands is supported within a TDD carrier. It is FFS whether to additionally support UE-specific configuration on time and/or frequency locations of SBFD subbands

- A symbol configured as SBFD symbol via cell-specific configuration cannot be reverted to a non-SBFD symbol via any UE-specific configuration or group-common signaling. Similarly, a symbol not configured as SBFD symbol via cell-specific configuration cannot be reverted to an SBFD symbol via any UE-specific configuration or group-common signaling.

Observation 2: RAN1 has agreed on the following regarding SBFD resource configuration in time-domain:

- SBFD symbols can be configured in DL and/or flexible symbols configured in TDD-UL-DL-ConfigCommon.

- The SBFD symbols are configured in consecutive manner and independently for each TDD-UL-DL pattern period.

- The configured SBFD symbols can start from any symbol within a slot and can end in any symbol within a slot.

Observation 3: RAN1 has agreed on the following regarding SBFD resource configuration in frequency-domain:

- The maximum number of UL subbands for SBFD operation in an SBFD symbol within a TDD carrier is one. The UL subband can be located at one side of the carrier or can be located at the middle part of the carrier.

- Frequency locations of UL subband and DL subband(s) are explicitly configured. Guardband(s) if any are implicitly derived as the RBs which are not within UL subband or DL subband(s).

- The subband frequency-domain resources are same across different SBFD symbols within a TDD carrier.

- RB-level granularity is supported for semi-static indication of SBFD subband frequency location.

# 3	RAN2 aspects of SBFD

## 3.1	Cell selection or reselection

As noted earlier, RAN1 has concluded that non-SBFD aware UEs, e.g. legacy UEs, and SBFD aware UEs can coexist in cells with SBFD operation at gNB side at least from RAN1 specification point of view. As one example, assuming a cell time-domain configuration of DXXXU, where D, X, U  refer to a DL-only, SBFD and UL-only slot, respectively, the gNB may prioritize scheduling of legacy UEs in the DL-only and UL-only slot, while SBFD-aware UEs may be prioritized in SBFD slots. We also foresee possible to schedule legacy UEs in DL direction in SBFD slots (by proper scheduling such that DL allocations are confined to the DL subband), while UL transmissions in the UL subband may not be possible for legacy-UEs if SBFD operation is configured on top of a traditional DDDSU configuration in TDD-UL-DL-ConfigCommon. In other words, only SBFD-aware UEs may get the coverage and latency benefit of SBFD operation, while legacy UEs may not experience any performance improvement or may even have a performance loss instead.

In scenarios with a mix of SBFD- and non-SBFD cells, while there is no need to fully prevent legacy UEs to connect to SBFD cells, it is desirable that SBFD-aware UEs prioritize SBFD cells during cell (re-)selection over cells not supporting SBFD. Similarly, legacy UEs may get better performance if they are served by non-SBFD cells. For this reason, we think the knowledge on whether the cell supports or not SBFD operation should be available to the SBFD-aware UEs for cell (re-)selection purposes.

Observation 4: RAN1 has concluded that non-SBFD aware UEs, including legacy UEs, and SBFD aware UEs can coexist in cells with SBFD operation at gNB side. Nevertheless, only SBFD-aware UEs may get the coverage and latency benefit of SBFD, while legacy UEs may not experience any performance improvement or may even be subject to a performance loss instead.

Proposal 1: Knowledge on whether the cell supports or not SBFD operation should be available to the UE for cell selection purposes.

Proposal 2: Support SBFD-aware UEs to prioritize SBFD cells over non-SBFD cells.

# Conclusion

Observation 4: RAN1 has concluded that non-SBFD aware UEs, including legacy UEs, and SBFD aware UEs can coexist in cells with SBFD operation at gNB side. Nevertheless, only SBFD-aware UEs may get the coverage and latency benefit of SBFD, while legacy UEs may not experience any performance improvement or may even be subject to a performance loss instead.

Proposal 1: Knowledge on whether the cell supports or not SBFD operation should be available to the UE for cell selection purposes.

Proposal 2: Support SBFD-aware UEs to prioritize SBFD cells over non-SBFD cells.

# Appendix – RAN1 agreements

- RAN1#116 agreements on SBFD random access

R1-2401676	Summary#1 on SBFD random access operation	Moderator (CMCC)

Working assumption:

For SBFD aware UEs in RRC CONNECTED state, support CBRA and CFRA in SBFD symbols.

Conclusion

No new PRACH format is introduced in Rel-19 duplex WI.

Agreement

For random access operation for SBFD-aware UEs in RRC CONNECTED state, at least consider the following options:

- Option 1: Use one single RACH configuration with possible enhancement

- The ROs within UL subband in SBFD symbols can be valid for SBFD-aware UE

- FFS: Further details

- Option 2: Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration

- The ROs within UL subband in SBFD symbols configured by the additional RACH configuration can be valid for SBFD-aware UE

- FFS: Further details

Agreement

For SBFD aware UEs in RRC CONNECTED state, support Type-1 random access procedure (4-step RACH) in SBFD symbols.

- FFS Type-2 random access procedure (2-step RACH)

R1-2401677	Summary#2 on SBFD random access operation	Moderator (CMCC)

Conclusion

If PRACH is allowed in SBFD symbols for SBFD-aware UEs in RRC_IDLE/INACTIVE mode, RAN1 observed the following:

- The benefits include at least one or more of the following:

- reduced random access latency

- reduced PRACH collision probability or allowing more contiguous frequency resources for PUSCH in UL slots

- improved coverage of PRACH with sparse UL resources

- increased cell range of PRACH with sparse UL resources

- PRACH transmissions in UL subband in SBFD symbols may cause UE-to-UE CLI (similar to the case of RRC connected mode UEs) for some deployment scenarios. Initial studies based on two companies’ evaluation results, the DL performance degradation due to UE-to-UE CLI caused by PRACH transmission in SBFD symbols is not significant for indoor office scenario and Urban Macro scenario.

For future meetings

Companies to consider whether the existing random access configuration tables for unpaired spectrum (i.e., Table 6.3.3.2-3 for FR1 and Table 6.3.3.2-4 for FR2) need to be enhanced.

Agreement

For SBFD aware UEs in RRC CONNECTED state, at least PRACH without repetition is supported in SBFD symbols.

- FFS PRACH repetition in SBFD symbols.

- FFS PRACH repetition across SBFD symbols and non-SBFDs symbols.

R1-2401678	Summary#3 on SBFD random access operation	Moderator (CMCC)

Agreement

For SBFD-aware UEs in RRC CONNECTED state, further study the following two options:

- Option 1: a valid RO can only be on SBFD symbols or on non-SBFD symbols

- a configured RO across SBFD and non-SBFD symbols in the same slot or across slots is invalid

- Option 2: a valid RO can be across SBFD and non-SBFD symbols in the same slot or across slots

RAN1 to leverage the study in Rel-18 as baseline.

Agreement

For SBFD-aware UEs in RRC CONNECTED state, at least further study whether/how to enable Msg2, Msg3 and Msg4 related transmission/reception in SBFD symbols taking into account the following aspects:

- Msg2[/Msg4 PDSCH] reception in DL subband(s)

- Msg3 PUSCH[/Msg4 HARQ-ACK PUCCH] frequency resource allocation and frequency hopping

- Msg3 repetition

- Msg3 PUSCH[/Msg4 HARQ-ACK PUCCH] power control

- FFS whether/how gNB to identify whether a UE is SBFD aware UE or non-SBFD aware UE

Note: Strive to make progress in accordance to the discussion in AI 9.3.1.

For future meetings:

In RAN1#116bis meeting, at least the following issues will be discussed:

- Whether to support random access in SBFD symbols for UEs in RRC_IDLE/INACTIVE mode

- Details of the two options for configuring ROs for SBFD aware UEs in RRC CONNECTED mode, including RO validation rules, SSB-RO mapping rules, whether/how to allow SBFD aware UE and non-SBFD aware UE to use different PRACH preamble formats.

- Whether/how to support separate PRACH power control parameters configuration in SBFD symbols and non-SBFD symbols.

- Whether/how to enhance the existing random access configuration tables for unpaired spectrum

- Whether/how to support PRACH repetition

- RAN1#116bis agreements on random access:

R1-2403468	Summary#1 on SBFD random access operation	Moderator (CMCC)

Agreement

Confirm the working assumption:

Working assumption:

For SBFD aware UEs in RRC CONNECTED state, support CBRA and CFRA in SBFD symbols.

Agreement

For Option 1 (i.e., use one single RACH configuration with possible enhancement) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, consider the following alternatives to derive the time and frequency resources of the configured ROs in SBFD symbols.

- Alt 1-1: only based on the existing parameters of the single RACH configuration (e.g., prach-ConfigurationIndex, msg1-FDM and msg1-FrequencyStart in rach-ConfigCommon).

- FFS the details

- FFS: Alt 1-2: based on the existing parameters of the single RACH configuration (e.g., prach-ConfigurationIndex, msg1-FDM and msg1-FrequencyStart in rach-ConfigCommon) and newly introduced parameter(s).

Agreement

For Option 1 (i.e., use one single RACH configuration with possible enhancement) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, no separate prach-ConfigurationIndex to be configured in this option.

Agreement

For Option 1 (i.e., use one single RACH configuration with possible enhancement) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, use existing random access configurations tables for unpaired spectrum (i.e., Table 6.3.3.2-3 for FR1 and Table 6.3.3.2-4 for FR2 in TS38.211).

R1-2403469	Summary#2 on SBFD random access operation	Moderator (CMCC)

Working Assumption

For SBFD-aware UEs in RRC CONNECTED state, both RACH configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration) and RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) are supported. Enabling both options at the same time for a UE is not supported.

- For Option 1 with Alt 1-1, FFS whether/how to reinterpret msg1-FrequencyStart in rach-ConfigCommon, RO validation rules and SSB-RO mapping rules, etc.

- For Option 2, FFS the RO validation rules, SSB-RO mapping rules, whether all the parameters currently in rach-ConfigCommon are necessary to be included in the additional RACH configuration, etc.

UE is not required to support both options.

Agreement

For RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, down-select (in RAN1#117) from the following alternatives:

- Alt 2-3:

- The additional-ROs in non-SBFD symbols configured by additional RACH configuration are invalid for SBFD-aware UEs.

- FFS: The case where the additional-ROs partially overlap with non-SBFD symbols

- Alt 2-4:

- The additional-ROs in non-SBFD symbols configured by additional RACH configuration can be valid for SBFD-aware UEs.

For the legacy-ROs configured by legacy RACH configuration, the legacy RO validation rules and the legacy SSB-RO mapping rules are followed for SBFD aware UEs.

Proposal

Support random access in SBFD symbols for UEs in RRC_IDLE/INACTIVE mode.

- Supported by 24 companies: New H3C, CMCC, ZTE, IDC, Xiaomi, HW/HiSi, Samsung, Nokia, NEC, Google, TCL, Sharp, Wilus, LGE, Fujitsu, ETRI, Sony, QC, Lenovo, SKT, MTK, CATT, Panasonic

- Objected by Ericsson

R1-2403437	SBFD Random access operation	Ericsson

R1-2403470	Summary#3 on SBFD random access operation	Moderator (CMCC)

Agreement

For SBFD-aware UEs in RRC CONNECTED state, and for RACH configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration),

- For the legacy-ROs, including the ROs in non-SBFD symbols and the ROs in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon (if any), the legacy SSB-RO mapping is followed.

- For the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, separate SSB-RO mapping will be used

Agreement

For RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, and for interpretation of the parameter prach-ConfigurationIndex provided by the additional RACH configuration,

- For FR2, consider from the following alternatives:

- Alt 1: use existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-4 in TS38.211)

- FFS whether to introduce new parameter(s) to determine the slot number for ROs in SBFD symbols.

- Alt 3: Introduce new entries on top of existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-4 in TS38.211)

- For FR1, consider from the following alternatives:

- Alt 1: Use existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-3 in TS38.211)

- FFS whether to introduce new parameter(s) to determine the subframe number for ROs in SBFD symbols.

- Alt 2: Use existing random access configurations table for paired spectrum/supplementary uplink (i.e., Table 6.3.3.2-2 in TS38.211)

- Alt 3: Introduce new entries on top of existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-3 in TS38.211)

Agreement

For Option 1 (i.e., use one single RACH configuration with possible enhancement) to support random access operation for SBFD-aware UEs in RRC CONNECTED state,

- no enhancements for the RO validation rule for the ROs in non-SBFD symbols and the ROs in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon (if any).

- FFS: the ROs in non-SBFD symbols that are valid for non-SBFD aware UEs are also valid for SBFD aware UEs.

- FFS: It’s up to network configuration to ensure the ROs in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon, which are valid for non-SBFD aware UEs based on legacy RO validation rule, are also valid for SBFD aware UEs (i.e., the configured ROs in SBFD symbols, if configured as flexible by tdd-UL-DL-ConfigurationCommon, are within the UL usable PRBs)

- the RO in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon is valid if at least:

- Time and frequency resource of the RO are fully within UL usable PRBs, and not overlapped with SSB

- FFS: Other condition.

Note: For the case that all the SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, there is no restriction that all the configured ROs in SBFD symbols should be within the UL usable PRBs.

- RAN1#117 agreements on random access:

Agreement

A RO across SBFD symbols and non-SBFD symbols in the same slot or across slots is invalid by default.

A configured RO starting from SBFD symbol and ending in non-SBFD symbol either in the same slot or across different slots can be valid based on network configuration. This is only supported for RACH configuration option 2 and only supported for the ROs configured by the additional RACH configuration. If network configures such RO as a valid RO, UE should treat the RO as an additional-RO in SBFD symbols, and the followings are assumed by network and UE:

- The same frequency resources are used for both the SBFD segment and non-SBFD segment of the PRACH.

- The same UL transmit power is used for both the SBFD segment and non-SBFD segment of the PRACH.

- The same UL spatial domain filter is used for both the SBFD segment and non-SBFD segment of the PRACH.

- UE doesn’t stop PRACH transmission in the transition period/gap (if any) between SBFD and non-SBFD symbols

- There are no phase coherency requirements on the UE between the SBFD segment and non-SBFD segment of the PRACH.

- Other assumptions are not precluded.

NOTE: For FR2, network may need to ensure that the additional-RO and the legacy RO, which overlap with each other in time domain, are mapped to the same SSB.

Agreement

Support random access in SBFD symbols for UEs in RRC_IDLE/INACTIVE mode.

Agreement

Confirm the following working assumption.

Working Assumption

For SBFD-aware UEs in RRC CONNECTED state, both RACH configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration) and RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) are supported. Enabling both options at the same time for a UE is not supported.

- For Option 1 with Alt 1-1, FFS whether/how to reinterpret msg1-FrequencyStart in rach-ConfigCommon, RO validation rules and SSB-RO mapping rules, etc.

- For Option 2, FFS the RO validation rules, SSB-RO mapping rules, whether all the parameters currently in rach-ConfigCommon are necessary to be included in the additional RACH configuration, etc.

UE is not required to support both options.

R1-2405656	Summary#4 on SBFD random access operation	Moderator (CMCC)

Conclusion

For SBFD-aware UEs in RRC CONNECTED state, and for RACH configuration Option 1 with Alt 1-1:

- The legacy ROs that are valid for non-SBFD aware UEs are also valid for SBFD aware UEs.

- It is up to the network to configure ROs in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon.

- If legacy ROs are configured in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon, network ensures the ROs are within the UL usable PRBs

Agreement

For SBFD-aware UEs in RRC CONNECTED state, and for RACH configuration Option 1 with Alt 1-1, for the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, consider whether additional condition(s) on top of what was agreed in RAN1#116bis for RO validation are necessary. For example,

- Condition#1: A valid RO starts at least X>=0 (FFS the value) symbols after a last downlink non-SBFD symbol.

- Condition#2: A valid RO starts at least X>0 (FFS the value) symbols after the SSB.

- Condition#3: A valid RO does not precede a SSB in the PRACH slot.

Agreement

Update the following agreement made in RAN1#116-bis meeting:

For RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, and for interpretation of the parameter prach-ConfigurationIndex provided by the additional RACH configuration,

- For FR2, adopt the following:

- Alt 1: use existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-4 in TS38.211)

- FFS whether to introduce new parameter(s) to determine the slot number for ROs in SBFD symbols.

- For FR1, consider from the following alternatives:

- Alt 1: Use existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-3 in TS38.211)

- FFS whether to introduce new parameter(s) to determine the subframe number for ROs in SBFD symbols.

- Alt 2: Use existing random access configurations table for paired spectrum/supplementary uplink (i.e., Table 6.3.3.2-2 in TS38.211)