3GPP TSG-RAN WG2 Meeting #127	R2-2407046

# Maastricht, Netherlands, 19th Aug – 23rd Aug 2024

Agenda Item:	7.5.3

Source:	Ericsson

Title:	Discussion on user plane corrections

Document for:	Discussion, Decision

# 1	Introduction

In this paper we discuss Rel-18 user plane corrections for XR.

# 2	Discussion on MAC correction

## 2.1 DSR reserved bit field

Rel-18 XR introduced a new MAC CE format for Delay Status Reporting (DSR). This format contains a reserved bit that is not used in Rel18, see figure 1.

Figure 1: DSR MAC CE with reserved bit in the R-field.

However in the MAC specification there is a missing a description of this R field and it would thus be good to clarify the format, as is done in similar cases for reserved bits in other MAC CEs. It can be noted that this reserved field may get usage in the ongoing advancements of the DSR feature in Rel19.

Add description of the reserved bit field for the DSR MAC CE in 38.321.

An example TP is provided in Annex.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Add description of the reserved bit field for the DSR MAC CE in 38.321.

# References

TS 38.321, V18.2.0

# Annex: TP for TS 38.321

===== OMITTED UNCHANGED LINES =======

#### 6.1.3.72	Delay Status Report MAC CE

The Delay Status Report (DSR) MAC CE is identified by MAC subheader with an eLCID as specified in Table 6.2.1-2b.

The fields in the DSR MAC CE are defined as follows:

-	LCGi: This field indicates the presence of delay information (i.e. the Remaining Time and Buffer Size fields) for the LCG i. The LCGi field set to 1 indicates that the delay information for the LCG i is reported. The LCGi field set to 0 indicates that the delay information for the LCG i is not reported;

-	Remaining Time: This field indicates the shortest remaining value of running PDCP discardTimer (described in clause 7.3 in TS 38.323 [4]) among all PDCP SDUs that are buffered for an LCG but have not been transmitted in any MAC PDU, at the time of the first symbol of the first PUSCH transmission that includes this DSR MAC CE. The length of this field is 6 bits. This field is present only if the buffer size indicated by the corresponding Buffer Size field is not zero; otherwise, this field is reserved and set to 0. If present, the value r in this field indicates a remaining time within the range of (r, r + 1] msec;

-	BT: This field is present only if the corresponding LCG is configured with additionalBS-TableAllowed and the buffer size indicated by the corresponding Buffer Size field is not zero; otherwise, this field is reserved and set to 0. If present, the BT field set to 1 indicates that the buffer sizes specified in Table 6.1.3.1-3 are used to set the value of the Buffer Size field, while the BT field set to 0 indicates that the buffer sizes specified in Table 6.1.3.1-2 are used instead;

-	Buffer Size: The Buffer Size field indicates the total amount of delay-critical UL data for an LCG according to the data volume calculation procedure specified in clause 5.5 in TS 38.322 [3] and clause 5.15 in TS 38.323 [4] for the associated RLC and PDCP entities, respectively, after the MAC PDU has been built. If the corresponding LCG is configured with additionalBS-TableAllowed and the amount of delay-critical UL data for an LCG is within the buffer sizes specified in Table 6.1.3.1-3, the MAC entity shall use the buffer sizes specified in Table 6.1.3.1-3 to set the value of this field; otherwise, the MAC entity shall use Table 6.1.3.1-2 instead. This field is indicated in number of bytes. The length of this field is 8 bits.

-	R: Reserved bit, set to 0;

The DSR MAC CE shall include delay information of all LCGs which have pending DSRs when the MAC PDU containing this DSR MAC CE is to be built. The Remaining Time, the BT, and the Buffer Size fields for an LCG shall be reported in two consecutive octets. These three fields for different LCGs shall be included in a DSR MAC CE in ascending order based on the LCGi.

Figure 6.1.3.72-1: DSR MAC CE

===== OMITTED UNCHANGED LINES =======