3GPP TSG-RAN WG2 Meeting #127	R2-2407291

# Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda item:		8.2.5 (FS_Ambient_IoT_solutions)

Source:	LG Electronics Inc.

Title: 	Discussion on topology 2 considerations of ambient IoT

Document for:	Discussion and Decision

1.	Introduction

This document discusses topology 2 considerations of ambient IoT (A-IoT). In this contribution, we discuss mainly topology 2 related aspects between gNB and reader.

2.	Discussion

The following two connectivity topologies are mainly studied for ambient IoT (A-IoT) communication.

- Topology 1: BS <--> Ambient IoT Device

- Topology 2: BS <--> intermediate node <--> Ambient IoT Device

Regarding architecture for two topologies, RAN3 considers the architecture shown in Figure 1 for Topology 1.

Figure 1 Architecture for Topology 1

In Figure 1, two function blocks are defined as follows.

- Common reader function: a function that communicates with the AIoT device by means of AIoT radio.

- AIoT RAN node function: a function residing in AIoT RAS. It contains e.g. the control of the AIoT radio resources used towards the AIoT device.

For Topology 2, the RAN architecture for Topology 2 is still under discussion. However, considering that common reader function block is for communicating with A-IoT device using A-IoT radio, it seems obvious that the intermediated node (i.e., the UE reader) has common reader function block for communicating with A-IoT device. Also, RAN2 already agreed in a previous meeting (R2-125bis) that from RAN2 perspective, the aim is that the design on the interface between reader and A-IoT device is common for Topology 1 and Topology 2. So, the common reader function block (or something corresponds to the common reader functions block) seems same for Topology 1 and Topology 2 except its location (i.e., BS in Topology 1 and UE reader in Topology 2).

In Topology 2, the UE reader needs to have a connection to the network by means of Uu interface over NR radio. Through the connection between the UE reader and gNB, A-IoT related messages (e.g., A-IoT control messages, A-IoT data) are transferred between common reader function in the UE reader and the correspond network functions in the network. According to TR23.700-13, proposed solutions consider delivering A-IoT messages on top of Uu AS layer. For an example, A-IoT messages can be delivered using a RRC container like a NAS message delivery. For other example, A-IoT messages can be delivered using a PDU session.

When A-IoT messages are delivered on top of Uu AS layer between the UE reader and gNB, the expected impact is limited to upper layers only. Therefore, for Topology 2, no RAN2 impact on Uu interface between UE reader and gNB is expected.

Proposal 1. For Topology 2, RAN2 assumes no spec. impact on Uu interface between UE reader and gNB from RAN2 perspective.

3.	Conclusion

This document discusses topology 2 related aspects between gNB and reader.

Proposal 1. For Topology 2, RAN2 assumes no spec. impact on Uu interface between UE reader and gNB from RAN2 perspective.