3GPP TSG-RAN2 Meeting #127	R2-2407317

Maastricht, Netherlands, Aug 19-23, 2024

Agenda item:	8.2.4

Source:	Qualcomm Incorporated

Title:	Views on Random Access Aspects of Ambient IoT

WID/SID:	FS_Ambient_IoT_solutions – Release 19

Document for:	Discussion and Decision

# Introduction

Rel-19 Ambient IoT SID [1] describes the RAN2 objectives and Ambient IoT random access should be studied.

In RAN2 #125bis meeting, the following agreements were made regarding the Ambient IoT random access.

- RAN2 confirms slotted-ALOHA is the baseline for Ambient IoT random access

- We will study the support for access triggering for a single device, group of devices, or all devices.    RAN2 to discuss the contention-based and contention-free access procedures and detailed solutions.

- Random Access is triggered by the reader

- Reader provides the information that the device needs to respond to the random access trigger.  FFS what those parameters are

- Study the solution and benefits of both 2-step like random access procedure and 4-step like random access procedure.  FFS the details on each procedure and how we call it.

- Handling of contention resolution failure and access failure at the device will be studied in RAN2, including failure detection and re-access.  FFS details

- For the very first access message from the device to reader in random access an ID is included.  RAN2 to discuss whether a temporary identifier is included, or the permanent device ID is included (considering other WGs input as well).

Further, RAN2 #126 meeting made the following agreements related to Ambient IoT random access.

Agreements on “4 step” RA

1	A-IoT Msg1: the device sends an ID to the reader.  ID is a random ID generated by device (FFS how it is generated, e.g. randomly generated or generated based on Device ID).  FFS on ID size.  This doesn’t preclude any other RAN1 agreed information

2	A-IoT Msg2: the reader echos the ID received in Msg1.   Further information may be included in mgs2 based on RAN1 agreements

3	A-IoT Msg3: device sends Device ID and/or any other upper layer data (depending on upper layer request)

4  The device considers the contention resolution as successful, if the Msg2 including the same random ID in Msg1 is received. RAN2 assumes the size of random ID in Msg1 should be sufficient for contention resolution purpose.

5  “Msg4” (i.e. the subsequent R2D transmission after D2R transmission) does not need to be always sent in random access. “Msg4” can be considered to handle the Msg3 transmission failure (due to various reasons). “Msg4” usage/presence can be further discussed.

RAN2 will not use “Msg4” term for further discussion of the random access.

Agreements on 2 step CB RA

- A-IoT Msg1: The device sends Device ID and/or any other upper layer data (depending on upper layer request). FFS what device ID is and whether an additional random ID is needed.  This doesn’t preclude any other RAN1 agreed information

- A-IoT Msg2: the reader may echo some information from Msg1.  FFS what some information is.  “Msg2” usage/presence can be further discussed

In this paper, we would like to discuss the random access aspects of Ambient IoT (AIoT) and provide our view.

# Discussion

The contention based AIoT access occasion may be provided by Reader to those AIoT devices who can receive the trigger message. Those AIoT devices may select AIoT access occasion among the resources provided by Reader. The resource selection in the time domain of the AIoT access occasions should be supported. Other schemes of the resource selection of the AIoT occasions can be further studied by RAN1/RAN2.

Proposal 1: The AIoT devices selects the AIoT access occasion among the resources provided by Reader. The resource selection in the time domain of the AIoT access occasion is supported. Other schemes of the resource selection of the AIoT occasions can be further studied by RAN1/RAN2.

The Reader may directly indicate the dedicated AIoT access resource to AIoT devices for performing the random access. That could be similar to the concept of contention free random access which is beneficial for the use case that Reader can identify one particular or a set of target AIoT device to act upon the command.

Proposal 2: Reader can provide the dedicated resource (contention free) to one or multiple AIoT devices separately in the trigger message (e.g., if the Reader has already inventoried and knows the device identifier).

In the inventory or command procedure, the trigger message may optionally include the filter criteria sent by the upper layer to identify the AIoT devices that are supposed to respond to the inventory request or command message. The Reader forwards the filter criteria in the inventory signaling to the AIoT devices. AIoT device is required to check the conditions whether the criteria are satisfied or not before initiating the AIoT access.

For the AIoT access, the AIoT device should check the filter criteria provided by Reader before performing the AIoT access. Only if the criteria are satisfied, the AIoT devices are allowed to perform the AIoT access to the Reader. What information are included in the filter criteria can be up to SA2 for further study.

Proposal 3: If the Reader provides a filter criteria in the initial trigger message, only those AIoT devices that meet the filter criteria can perform AIoT access (i.e., send AIoT Msg1).

In the past RAN2 meetings, the AIoT ‘4-step’ or 2-step random access was preliminary discussed. Since AIoT access is triggered by Reader, the Reader is responsible for providing the access resource considering the different service types, i.e., inventory request or command message. Reader may have preference on which type of AIoT access to be triggered to respond the service request.

For example, if Reader would like to only issue the ‘Write’ command to a small number of AIoT devices, a 2-step AIoT access can be selected for AIoT devices to feed back the simple confirmation after the command execution. In another example, Reader may indicate AIoT devices to perform so called ‘4-step’ AIoT access for the case that a large number of AIoT devices perform AIoT access by using the contention-based resource. Therefore, Reader should be able to indicate the type of AIoT access to AIoT devices in the trigger message.

Proposal 4: Reader may indicate the type of AIoT access (e.g., 2-step or ‘4-step’) to AIoT devices in the initial trigger message.

If Reader does not indicate the type of AIoT access to AIoT devices, AIoT devices may evaluate certain criteria to determine the type of AIoT access.

Proposal 5: If the type of AIoT access is not indicated by Reader in the initial trigger message, AIoT devices may evaluate criteria to determine the type of AIoT access. FFS details of criteria.

RAN2#126 discussed the so-called ‘Msg4’. RAN2 agreed to not use the term ‘msg4’ for further discussion of the AIoT random access. Instead, it could be called subsequent R2D message after D2R transmission. Such message may be a failure indication for Msg3 transmission or could be the acknowledgement like the Msg4 of NR random access.

While some scenarios would need the subsequent R2D message (e.g. for acknowledging the reception of the AIoT device ID and/or AIoT upper layer data sent in the previous D2R message or for triggering the next round of AIoT re-access or AIoT data communication), in other scenarios there would be no such subsequent R2D message (e.g. Reader requesting information from temperature sensors and no intention to ACK/NACK successful reception).

It is beneficial for AIoT device to have correct expectation on potential future steps after transmitting AIoT device ID and/or AIoT upper layer data. Since the information carried in the subsequent R2D message may be different and it is desirable to give Reader some flexibility to determine whether to send the subsequent R2D message and what information should be included in the message, an earlier R2D message can indicate presence and what information carried in the subsequent R2D message to the AIoT device.

Proposal 6: The earlier R2D message (e.g., msg0, msg2) indicates the presence and information carried in the subsequent R2D message.

In some cases, the AIoT access and/or AIoT data reporting procedure may fail. If the AIoT Msg2 fails to echo the random ID sent from AIoT Msg1, or the AIoT devices does not receive AIoT Msg2 at all, the contention resolution is considered unsuccessful. In such cases, the AIoT device won’t be able to transmit the AIoT device ID and/or AIoT upper layer data to the Reader. In legacy NR (4-step) RACH, UE performs re-attempt by sending the Msg1 again after a backoff time if UE fails to receive the RAR within a given time window.

There are also other cases that the device ID and/or AIoT upper layer data sent in AIoT Msg3 was not received successfully at the Reader even if the contention resolution was successful. Even in such cases, AIoT device should be able to perform AIoT re-access. Therefore, RAN2 should study if any other opportunities are provided to AIoT devices for re-access or to handle the different failure cases.

Proposal 7: If the AIoT devices contention resolution is unsuccessful or the AIoT data transmission is failed, the AIoT devices should be able to perform AIoT re-access. FFS details of AIoT re-access.

Due to the nature of backscatter communication of the AIoT device and poor energy storage, the AIoT devices with lower capability may have to first harvest energy before performing AIoT re-access. Thus, one option is that AIoT devices perform re-access only upon reception of next trigger message from Reader. The AIoT device can have chance to charge more energy before the AIoT re-access.

For AIoT device with higher capability, it may have large energy storage, and such device may not need to wait till the next trigger message for energy harvesting and may be able to autonomously perform re-access e.g., even before the next trigger from the Reader. Thus, RAN2 is suggested to study the AIoT devices to perform AIoT re-access with different options.

Proposal 8: RAN2 to study the following options for AIoT devices to perform AIoT re-access.

Option 1: AIoT devices perform re-access only upon reception of next trigger message from Reader.

Option 2: AIoT devices autonomously perform re-access without waiting for next trigger message from Reader.

When AIoT performing re-access, AIoT devices may select the access occasion to transmit the AIoT Msg1 again. The access occasions for AIoT re-access can be configured by the next trigger message and the access occasions may be different from the resource configured by the initial trigger message.

If AIoT device is able to autonomously performs re-access, it also depends on whether the device has been configured with multiple access occasions for re-access by the initial trigger message. It is also possible that Reader may directly indicate the access occasions for AIoT device re-access.

Proposal 9: During AIoT re-access, AIoT devices can transmit the AIoT Msg1 again in a newly selected access occasion or in an indicated access occasion.

Since AIoT device is expected to be ultra-low complexity device with ultra-low power consumption for the very-low end IoT applications, some very low-end AIoT devices, e.g., device type 1, may requires more rounds of energy harvesting to accumulate enough energy for inventory response.

In some cases, the AIoT devices may miss the inventory command due to inability of accurate sync signal tracking. In other cases, the congestion may be heavy in the massive IoT access procedure. In that sense, the multiple rounds of AIoT access should be supported. It will be helpful if Reader could periodically transmit inventory command to discover the nearby AIoT devices.

Proposal 10: RAN2 to study the multiple rounds of AIoT access procedure, e.g., the periodical transmission of inventory, by Reader.

# Conclusion

We have the following observations, and we’d recommend RAN2 to discuss and adopt the following proposals:

Proposal 1: The AIoT devices selects the AIoT access occasion among the resources provided by Reader. The resource selection in the time domain of the AIoT access occasion is supported. Other schemes of the resource selection of the AIoT occasions can be further studied by RAN1/RAN2.

Proposal 2: Reader can provide the dedicated resource (contention free) to one or multiple AIoT devices separately in the trigger message (e.g., if the Reader has already inventoried and knows the device identifier).

Proposal 3: If the Reader provides a filter criteria in the initial trigger message, only those AIoT devices that meet the filter criteria can perform AIoT access (i.e., send AIoT Msg1).

Proposal 4: Reader may indicate the type of AIoT access (e.g., 2-step or ‘4-step’) to AIoT devices in the initial trigger message.

Proposal 5: If the type of AIoT access is not indicated by Reader in the initial trigger message, AIoT devices may evaluate criteria to determine the type of AIoT access. FFS details of criteria.

Proposal 6: The earlier R2D message (e.g., msg0, msg2) indicates the presence and information carried in the subsequent R2D message.

Proposal 7: If the AIoT devices contention resolution is unsuccessful or the AIoT data transmission is failed, the AIoT devices should be able to perform AIoT re-access. FFS details of AIoT re-access.

Proposal 8: RAN2 to study the following options for AIoT devices to perform AIoT re-access.

Option 1: AIoT devices perform re-access only upon reception of next trigger message from Reader.

Option 2: AIoT devices autonomously perform re-access without waiting for next trigger message from Reader.

Proposal 9: During AIoT re-access, AIoT devices can transmit the AIoT Msg1 again in a newly selected access occasion or in an indicated access occasion.

Proposal 10: RAN2 to study the multiple rounds of AIoT access procedure, e.g., the periodical transmission of inventory, by Reader.

# References

- RP-240826, Study on solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA