3GPP TSG-RAN WG2 #127 	R2-2407555

Maastricht, Netherlands, 19th - 23rd Aug 2024 	Revision of R2-2407552

Agenda Item:	8.9.3

Source:	Ericsson

Title:	UL capacity enhancements objectives for IoT NTN

Document for:	Discussion, Decision

# 1	Introduction

In RAN#104, the WID for “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things” was updated [1]. The main objectives related to support of capacity enhancements for uplink are as following:

The aim of this WI is enhancement of IoT-NTN with the following objectives: <<skip the unrelated parts>> - Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2, RAN4] - Multi-tone support for 15 kHz SCS should also be considered - Specify necessary signalling, if needed - Update RF requirements accordingly, if needed Note: Impact of impairment shall be taken into account - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]: - Msg3 transmission without msg1/ Random Access Response (RAR) - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete - Study and specify RRM requirement, if identified [RAN4]

In RAN2#126, the following agreements

1 RAN2 focusses the study on contention-based Msg3 transmission to complete an EDT-like transaction (FFS on the details of Msg3. FFS on the procedural steps, e.g. how much we reuse of EDT and PUR procedures. FFS on allocation of resources). 2. RAN2 can continue the discussion on Diversity Slotted ALOHA (DSA) and Contention Resolution Diversity Slotted Aloha (CRDSA) for Msg3-EDT transmissions without msg1/ RAR, evaluating possible impacts on the specification, in the next RAN2 meeting (RAN2 might send an LS to RAN1 later on this) 3. If an IoT NTN UE in IDLE state is to use the new R19 contention-based procedure, the UE needs to verify/update the uplink synchronization (e.g. get GNSS fix, acquire TA) just before sending msg3.

In this document, we discuss how the preamble-less EDT transmission may work.

# 2	Discussion

## 2.1	Preamble-less EDT

### 2.1.1	Terminology

During RAN2#126, a significant portion of the discussion was centred around the decision of whether the new feature should adopt EDT or PUR as its baseline. Several companies argued that this is merely a matter of nomenclature. However, we think that EDT and PUR are intrinsically distinct, given their respective original purposes. Specifically, PUR is designed for periodic traffic and requires dedicated configuration. Moreover, PUR is associated with a suite of predefined procedures (e.g., Layer 1 Acknowledgement) and UE behaviours (e.g., PUR-responseWindowTimer) that may not be applicable in the NTN context.

As expressed in the WID, RAN2 should take EDT as the baseline for further enhancements, considering the contention of Msg3 due to the absence Msg1 and Msg2, primarily addressing MO traffic. To avoid further confusion and debates over terminology, we suggest that the feature be designated as Preamble-less Early Data Transmission (pl-EDT).

The Rel-19 enhancement of EDT without Msg1 and Msg2 transmission is referred to as preamble-less EDT.

### 2.1.2	Benefits of preamble-less EDT

The justification for the objective is UL capacity enhancements.

The first (RAN1) part of the objective aims at enabling multiplexing multiple UEs on NPUSCH format 1 and NPRACH. This hints at A) improved NPUSCH multiplexing and B) improved NPRACH collisions.

For improved NPRACH collisions to make sense, 1) UEs must be told in broadcasting or dedicated signalling that they can use OCCs, 2) eNB must be able to detect OCCs on the NPRACH, 3) RAR messages must include an option to address different OCC (that is some new RAR format), 4) legacy UEs must not be severely affected. The combined changes seem to result in large spec changes, even though NPRACH collisions is not a limiting issue of RA capacity.

The second part of the objective require RAN2 to “Study and specify, if beneficial” the preamble-less EDT.

Here we do a quick study of the capacity aspects of Contention Resolution Diversity Slotted Aloha (CRDSA), one of the proposed methods, and compare it to the existing 4-step RA from a capacity point of view.

We assume Poisson traffic (for CDRSA and 4-step RA) and a frame N = 100 slots.

Further we assume none of the messages can be received when there is an NPUSCH collision (besides if CRDSA can resolve the collision), that is, no power imbalances nor power capture [4]. We assume that the message is always correctly received if there is no collision.

#### 2.1.2.1	CRDSA and Slotted Aloha

For SA and CRDSA we use similar models as described in R2-2405202 [3].

Each UE sends 2 or 3 replicas randomly selected within the frame.

We ignore overhead from the CRDSA scheme (that is the header info on which other slots in the window where the UE transmitted the same packet, this overhead would be 7 bits with 2 replicas and 14 bits with 3 replicas).

We assume 4-step msg3 and CRDSA use the same packet size and resource size (same amount of frequency and time resources and same number of repetitions, but of course the 4-step has the overhead of the preamble resources that we will discuss later).

We assume CRDSA collisions are resolved if one of the replicas is correctly decoded on a different slot in the frame N.

#### 2.1.2.2	4-step RA

The design idea behind 4-step RA is that there is an overprovisioning of RA resources such that the preamble collision rate is low.

In case there are many collisions, the BS can detect this by counting number of preambles in use each RA occasion and by noticing collisions on MSG3 (if MSG3 SINR is worse than expected). NOTE preamble collisions cannot be detected because the UEs are transmitting the exact same sequence and multiple UEs will only look like multipath to the eNB.

The BS must control the load (that is, increase the back off timer signalled in RAR and if that is not sufficient to increase the level of access class barring, ACB) – ACB is the only way to avoid avalanches of RA attempts in high load situations (stadiums or 911-events).

Thus, in case of high load in 4-step RA, the BS will increase the access delay to decrease the collisions rate.

This has the effect of operating most of the time at the left end of figure from R2-2405202 [3] (marked with yellow):

The 4-step RA is more robust compared to CRDSA as eNB can measure on the preamble and allocate resources for the msg3 transmission (MCS and PRBs) based on the received preamble properties (e.g., received power or delay spread).

For 4-step RA, we assume NB-IoT with 48 subcarriers (sometimes termed preambles) and if preambles collide, then the msg3 will collide and will not be correctly received. With these simple assumptions the number of users attempting access each RA opportunity is Poisson distributed {P[k users access] = (lambda*T)^k*exp(-lambda*T)/k!} and the probability of collision is easily calculated as P[X>0]. NOTE that a Poisson distributed variable that is randomly divided in several other variables are also Poisson distributed with a lower average. However, below we also look at cases with limited number of msg3 transmissions that are easier modelled in a simulator than calculated by hand.

#### 2.1.2.2	Evaluation

With these simple models, collisions and resource usage can be calculated but it is quite complex to do (especially for the limited msg3 tx scheme and for CRDSA frames) while a simulation is easy. In the Appendix we have described how the simulation is done.

We present results for Slotted Aloha and two cases of CRDSA (with 2 and 3 replicas) with frame size N=100.

The Slotted Aloha and CRDSA results correspond exactly to the results shown in R2-2405202 [3].

For 4-step RA resource usage, we assume one preamble transmission occasion uses the same number of resources as one msg3 transmission. The CRDSA always reserves 100 message transmission occasions in one frame N=100, while for 4-step resource usage will depend on how many preambles that are detected and how many msg3 are scheduled in a frame N.

For 4-step RA, we show three cases:

- 48 subcarrier and 15 RA opportunities (see Appendix)

- 48 subcarrier and 10 RA opportunities

- 48 subcarrier and 5 RA opportunities (see Appendix)

We present two versions of each 4-step RA case:

- a limited maximum number of msg3 transmissions within a frame N

  - Limit of 85 (100-15), 90 (100-10), and 95 (100-5) respectively, this scheme uses the same number of resources as CRDSA

- an unlimited number of msg3 transmissions within a frame N

10 RA opportunities means there are 10 RA opportunities within the frame of N=100 slots to transmit preambles, thus the load per RA opportunity will be ten times higher (and thus collision rate is higher but then we add the 48 subcarriers resulting in a much lower load per subcarrier and lower collision rate). This means the resource usage in a frame N will always be 10 preamble transmission occasions plus one message occasion per detected preamble (unless maximum number of msg3 transmissions is reached for the frame N).

In Figure 1, we have the packet loss ratio (PLR) versus the load. The CRDSA has best performance at low load and slotted aloha has the worst performance at low load, however that has no significance as the PLR is so low that it does not matter for low load. For high load, the 4-step RA outperforms the other schemes.

For 4-step RA, we can see that limiting the maximum number of msg3 within a frame N=100 starts taking effect around the average load of 0.9 packets per slot (that is, at 90 slots per frame, as the maximum msg3 tx for this case) while the unlimited case has the similar shape as slotted aloha but with lower PLR.

A well configured 4-step RA scheme has substantially lower packet loss rate than CRDSA schemes at high load.

Figure 1: Packet loss ratio versus load

Figure 2: Throughput versus load

In Figure 2 we have the throughput versus load. Slotted Aloha shows the classic curve with a peak at load = 1. The CRDSA 3 replicas scheme has a sweet spot somewhere between 0.35 and 0.7 packets per slot compared to 4-step RA with 10 RAO and 48 subcarriers. However, the CRDSA schemes breaks down when load increases, which the 4-step RA will do too but at a higher load (for the unlimited case the break down is at a higher load than shown in the figure).

A well configured 4-step RA scheme has substantially higher throughput than CRDSA schemes.

Figure 3: 4-step RA resource usage versus load

In Figure 3 we have plotted the number of msg3 transmissions (resource usage) for the three different 4-step RA configurations. Each detected preamble (collision or not) requires one msg3 transmission (unless the maximum for the frame has been reached). At high load, the limited schemes always use all the allowed msg3 transmissions. See Appendix for more results on the 4-step 15 and 5 RAO configurations.

The CRDSA scheme always uses 100 packet transmissions for a frame of N=100. If we assume that one RA opportunity uses the same number of resources as one msg3, the limited 4-step RA schemes maximum uses 100 resources (msg3 transmissions plus RAOs), which is the same as the resources allocated to CRDSA.

Thus, 4-step RA has higher throughput, uses much lower number of resources at low loads, and is more flexible to adapt to varying traffic and propagation conditions.

A well configured 4-step RA scheme has lower resource usage that CRDSA schemes at low load.

A well configured 4-step RA scheme is more flexible as at each scheduling, the eNB can schedule RA or schedule regular NPUSCH traffic depending on the load and prio of the traffic.

The objective of the preamble-less EDT is to improve capacity but non of the proposed schemes are doing that.

As there are no capacity benefits for the preamble-less EDT enhancements compared to 4-step RA, there is no reason to introduces preamble-less EDT – therefore RAN2 shall focus on efficient delivery of msg4 for EDT.

### 2.1.3	Uplink resource allocation

Preamble-less EDT transmission without Msg1/Msg2 is somewhat similar to NR’s 2-step random access. In this procedure, the cell specific PRACH and PUSCH resource parameters for the transmission of MsgA are configured by MsgA-ConfigCommon included in SIB1, and the network can provide separate PRACH and PUSCH configurations for a specific feature such as small data transmission (SDT, which is similar to EDT in LTE) in FeatureCombinationPreambles. For preamble-less EDT, cell specific common PUSCH resources for Msg3 transmission can be configured via system information. Signalling details such as which SIB to use can be further studied.

In case preamble-less EDT is agreed beneficial, System Information is used to provide preamble-less EDT cell specific PUSCH resources for Msg3 transmission. FFS on signalling details.

### 2.2.4	Configuration of uplink resources

LTE-M and NB-IoT devices support coverage enhancement (CE) to compensate the simplifications in their radio interfaces and improve connectivity in challenging coverage conditions. The most significant CE technique is the support of repetitions.

In EDT, PRACH parameters are configured per CE level in edt-PRACH-ParametersListCE. The UE selects the CE level for the preamble transmission (Msg1) based on network configuration. In addition, transport block size (TBS) and number of repetitions for Msg3 transmission on PUSCH depend on the PRACH CE level.

In legacy EDT, the number of repetitions and the largest TBS for Msg3 PUSCH depend on the CE level.

In legacy EDT, RSRP thresholds are configured for the UE to select the CE level.

For preamble-less EDT, the support of CE is also necessary. Similar to legacy, we think the network should provide cell specific common PUSCH resources for Msg3 EDT transmission per CE level. For each CE level, PUSCH resources can be associated with a certain number of repetitions, largest allowed TBS and a RSRP selection threshold.

In case preamble-less EDT is agreed beneficial, Preamble-less EDT cell specific PUSCH resources for Msg3 transmission are provided per CE level.

In case preamble-less EDT is agreed beneficial, Preamble-less EDT cell specific PUSCH resources are associated with number of repetitions, RSRP selection threshold and largest TBS for Msg3 transmission.

### 2.2.5	UL time alignment for Msg3 transmission

For uplink timing handling, RAN2 already agreed that the UE needs to verify/update the uplink synchronization (e.g., get GNSS fix, precompensate TA) just before sending Msg3. In our view, this is sufficient to ensure the required uplink synchronization for Msg3 transmission and there is no need to use TAT. Note that the UE may not receive any Timing Advance Command (in the current serving cell) before sending Msg3 thus it has no trigger to start TAT and therefore it is meaningless to introduce TAT for Msg3 transmission without Msg1 and Msg2.

The UE will verify uplink synchronization before sending Msg3.

The UE may not receive any Timing Advance Command before sending Msg3.

In case preamble-less EDT is agreed beneficial, Preamble-less EDT does not require a running TAT for Msg3 transmission.

### 2.2.6	Contention mechanism: collision detection and backoff

Msg3 transmission may fail due to collision or bad reception quality at the network side. In NR 2-step random access, when the transmission of MsgA fails, the UE may have another MsgA transmission attempt with a random backoff and an increased MsgA transmission power (i.e., power ramping). The UE would switch to 4-step random access if the number of MsgA transmissions has reached msgA-TransMax, provided that preambleTransMax is greater than msgA-TransMax. A similar mechanism can be introduced for contention based Msg3 EDT transmission to handle Msg3 transmission failures.

In case preamble-less EDT is agreed beneficial, After a failure in Msg3 transmission, the UE may attempt another transmission after waiting for a random back-off and increasing the transmission power.

In case preamble-less EDT is agreed beneficial, After a number of failed Msg 3 transmission attempts, the UE falls back to 4-step random access EDT. FFS on the maximum number of transmission attempts.

### 2.2.7	UE identity for Msg4 transmission

Another open issue for preamble-less EDT is which identity (e.g., RNTI) should be used to schedule Msg4 transmission. In NR’s 2-step random access, MsgB-RNTI is derived based on the time/frequency position of the MsgA PRACH occasion. This principle can naturally be applied to preamble-less EDT, i.e., the RNTI used to schedule Msg4 transmission is derived based on the used Msg3 PUSCH time-frequency resource. The RNTI may be associated with either one or multiple Msg3 PUSCH time-frequency resources, which is up to network implementation.

In case preamble-less EDT is agreed beneficial, The RNTI used to schedule Msg4 transmission is derived based on the PUSCH time-frequency resource(s) used for contention based Msg3 EDT transmission.

### 2.2.8	Efficient delivery of Msg4

The second WI objective of this topic is the efficient delivery (reduced overhead) of Msg4. In last RAN2 meeting, [2] showed how the overhead reduction cannot come from optimizations in the RRCEarlyDataComplete ASN.1 structure. Thus, RAN2 should focus on different alternatives such as the transmission of a shared Msg4.

In 2-step random access, MsgB can consist of one or multiple successRAR and/or fallbackRAR sub-PDUs. Each successRAR consists of UE Contention Resolution Identity, the PDSCH-to-HARQ feedback timing indicator for MSGB HARQ feedback and Timing Advance Command, etc. Hence, a MsgB can be used to respond to multiple UEs which have transmitted MsgA, i.e., a shared MsgB.

NR’s 2-step random access already supports multiplexing multiple UE Contention Resolution Identities in MsgB.

In our view, a similar design can be applied to Msg4, i.e., Msg4 can consist of multiple contention resolution identities for different UEs associated with the PDSCH-to-HARQ feedback timing indicator for Msg4 HARQ feedback, where the contention resolution Identity could be e.g., the TMSI included in RRCEarlyDataRequest in Msg3, or just the first 48 bits of Msg3 (similar to a regular 4-step random access or a successRAR sub-PDU in MsgB). This avoids the transmission of separate Msg4 for each individual UE whose Msg3 transmission is decoded successfully.

Introduce multiplexing of several UE Contention Resolution Identities in Msg4.

## 2.2	CR Diversity Slotted ALOHA

RAN2 agreed to continue discussing Diversity Slotted ALOHA (DSA) and Contention Resolution Diversity Slotted Aloha (CRDSA) for 2-step Msg3-EDT transmissions. As described above, repetition is a crucial feature for LTE-M and NB-IoT. If introducing DSA and CRDSA for 2-step Msg3-EDT transmissions, we shall make sure they can work together with repetition. In our view, RAN2 shall first discuss the feasibility and complexity of integrating DSA and CRDSA with repetition for 2 step Msg3-EDT transmission. DSA and CRDSA shall only be introduced if they can work together with repetition.

In case preamble-less EDT is agreed beneficial, For DSA and CRDSA, RAN2 shall first discuss whether it is feasible to integrate them with repetition.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	A well configured 4-step RA scheme has substantially lower packet loss rate than CRDSA schemes at high load.

Observation 2	A well configured 4-step RA scheme has substantially higher throughput than CRDSA schemes.

Observation 3	A well configured 4-step RA scheme has lower resource usage that CRDSA schemes at low load.

Observation 4	A well configured 4-step RA scheme is more flexible as at each scheduling, the eNB can schedule RA or schedule regular NPUSCH traffic depending on the load and prio of the traffic.

Observation 5	The objective of the preamble-less EDT is to improve capacity but non of the proposed schemes are doing that.

Observation 6	In legacy EDT, the number of repetitions and the largest TBS for Msg3 PUSCH depend on the CE level.

Observation 7	In legacy EDT, RSRP thresholds are configured for the UE to select the CE level.

Observation 8	The UE will verify uplink synchronization before sending Msg3.

Observation 9	The UE may not receive any Timing Advance Command before sending Msg3.

Observation 10	NR’s 2-step random access already supports multiplexing multiple UE Contention Resolution Identities in MsgB.

Based on the discussion in the previous sections we propose the following:

Proposal 1	The Rel-19 enhancement of EDT without Msg1 and Msg2 transmission is referred to as preamble-less EDT.

Proposal 2	As there are no capacity benefits for the preamble-less EDT enhancements compared to 4-step RA, there is no reason to introduces preamble-less EDT – therefore RAN2 shall focus on efficient delivery of msg4 for EDT.

Proposal 3	In case preamble-less EDT is agreed beneficial, System Information is used to provide preamble-less EDT cell specific PUSCH resources for Msg3 transmission. FFS on signalling details.

Proposal 4	In case preamble-less EDT is agreed beneficial, Preamble-less EDT cell specific PUSCH resources for Msg3 transmission are provided per CE level.

Proposal 5	In case preamble-less EDT is agreed beneficial, Preamble-less EDT cell specific PUSCH resources are associated with number of repetitions, RSRP selection threshold and largest TBS for Msg3 transmission.

Proposal 6	In case preamble-less EDT is agreed beneficial, Preamble-less EDT does not require a running TAT for Msg3 transmission.

Proposal 7	In case preamble-less EDT is agreed beneficial, After a failure in Msg3 transmission, the UE may attempt another transmission after waiting for a random back-off and increasing the transmission power.

Proposal 8	In case preamble-less EDT is agreed beneficial, After a number of failed Msg 3 transmission attempts, the UE falls back to 4-step random access EDT. FFS on the maximum number of transmission attempts.

Proposal 9	In case preamble-less EDT is agreed beneficial, The RNTI used to schedule Msg4 transmission is derived based on the PUSCH time-frequency resource(s) used for contention based Msg3 EDT transmission.

Proposal 10	Introduce multiplexing of several UE Contention Resolution Identities in Msg4.

Proposal 11	In case preamble-less EDT is agreed beneficial, For DSA and CRDSA, RAN2 shall first discuss whether it is feasible to integrate them with repetition.

# 4	References

RP-241624, “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things”, MediaTek Inc., RAN#104, June 2024

R2-2405201, “Efficient delivery of RRCEarlyDataComplete message”, TOYOTA Info Technology Center, RAN2#126, May, 2024.

R2-2405202, “Discussion on EDT enhancements”, ESA, RAN2#126 Fukuoka, May 2024.

J. Moberg, M. Löfgren, and R S Karlsson, “Throughput of the WCDMA random access channel”, in IST Mobile Communication Summit, vol. 10. Oct. 2000.

# Appendix	Models for evaluating UL capacity enhancements

## A	CRDSA evaluation

- Given a fixed number of users 1, 2, 3,… , N, …, 3*N users that do pl-EDT transmission in the frame N, find the packet loss ratio (PLR) and save the number for each fixed number of users

  - simplest is to do a short Monte Carlo simulation to get the spread in selection of slots to transmit and collisions etc.

  - model collisions etc.

  - PLR is the number of packets that cannot be correctly decoded within the frame N (all replicas of the packet is colliding with at least one other packet that cannot be correctly decoded)

- Calculate for a Poisson load lambda, what is probability of getting 1, 2, …, 3*N users in the frame N

- Sum the product of the probability of 1, 2, 3, 4, … users with the PLR results from A., this gives the average of packet loss ratio for that lambda

- Repeat from B. with a different load

- Possibly repeat from A. for a different frame N size or different time between pl-EDT occasions etc.

## B	4-step Random access evaluation

- Given a fixed number of users 1, 2, 3,… , N, …, 3*N that do preamble transmission in one RACH occasion in the frame N, find the packet loss ratio (PLR) and save the number for each fixed number of users

  - simplest is to do a short Monte Carlo simulation to model a limit on the maximum number of msg3 with in the frame N and a limit on the number of RA opportunities in a frame N

  - model collisions etc.

  - PLR is the number of packets that cannot be correctly decoded within the frame N (all the packets that are colliding with at least one other packet, or if eNB has used up all the allowed msg3 tx within a frame N)

  - resources used are one resource for each msg3 transmission (colliding or non-colliding)

- Calculate for a Poisson load lambda, what is probability of getting 1, 2, …, 3*N users in the frame N

- Sum the product of the probability of 1, 2, 3, 4, … users with the PLR results from A., this gives the average of packet loss ratio for that lambda

- Repeat from B. with a different load

- Repeat from A. for different number of RAOs in a frame N or with a limit on max msg3 tx

## C	Some of the numerical examples

In the figure below we have numerical results from step A, that is the PLR when the number of users is given.

## D	More results for 4-step RA

Below we show the throughput for some configurations of 4-step RA.

Clearly with 5 RAOs, there are significant collisions of the preambles, but we still reach higher peak throughput than CRDSA.

Below we see the PLR for 4-step RA, clearly for the limited cases when getting to the limit in load, the PLR increases.