3GPP TSG RAN WG3 Meeting #125bis	R3-24xxxx

Hefei, CN, 14th – 18th Oct, 2024

Agenda Item:	10.2

Source:	CATT (moderator)

Title:	SoD of MRO for others

Document for:	Approval

# Introduction

This is a summary of offline discussions on MRO for CHO with candidate SCG(s) and S-CPAC.

# For the Chairman’s Notes

MRO for CHO with Candidate SCG(s)

RLF report is enhanced to including

  - Identifier of candidate PCell(s) which met the configured CHO execution conditions before the RLF is encountered

  - Identifier of candidate PSCell(s) which met the configured CPAC execution conditions before the RLF is encountered

  - The fulfilled execution conditions before the RLF is encountered.

MRO for S-CPAC

Whether capture detection mechanism for S-CPAC in TS37.340? Take R3-245131 as baseline?

Further discuss the forwarding mechanism. The involved SN node in forwarding:

- Target SN (for candidate PScell selection optimization), source SN (for following execution condition optimization) for MN initiated S-CPAC:

- Initiating SN (for candidate PScell selection optimization and initiating execution condition optimization), source SN (for following execution condition optimization), target SN (for candidate PScell selection optimization) for SN initiated inter-SN SCPAC.

Further discuss whether Ping-Pong issue should be solved during S-CPAC procedure, if time allowed.

Further discuss whether to consider suboptimal usage of CPAC or S-CPAC, if time allowed.

# Background

Previous agreements in RAN3

RAN3#123bis

MRO for CHO with candidate SCG failure and near failure cases

Work on the scenarios of failure in S-CPAC. The optimization of non-failure scenarios (e.g., near failure and ping-pong) is not excluded.

RAN3 focuses on NR-DC for MRO for CHO with candidate SCG in R19.

R19 SON/MDT solution discussion is based on R18 work.

RAN3#124

Do not distinguish between fast MCG recovery/no-fast MCG recovery for now (to simplify use cases).

Concurrent error cases (MCG+SCG) is FFS. The definition of these error cases needs to be further clarified.

RAN3 will start with the failure scenarios with UEs configured with CHO with candidate SCGs.

Whether to include failure and near failure scenarios related with configuration of CHO with candidate SCG(s) and CHO only is FFS.

CPAC failure scenarios and detection mechanism captured in stage2 used as baseline.

RAN2 agreement in last meeting

- UE includes following information in RLF report:

- b.	Time information regarding condition fulfilment for CHO with candidate SCGs. Details are FFS. We consider both the case when both CHO condition and associated CPC condition are fulfilled, and the case when CHO (or CPC) is fulfilled but CPC (or CHO) conditions are not fulfilled.

- c.	Measurement results of PCells and PSCells.

RAN3#125

MRO for CHO with Candidate SCG(s)

MRO for Case 1a, 1b, 2a, 2b, 3a, 3b, 7a, 7b, 8a, 8b, 9 will be addressed.

MRO for case 4/5/6 is in the scope.

The RLF report includes:

  - The type of the first fulfilled execution condition e.g. CPAC or CHO, fulfilment means all events of the type are met.

  - Identifier of candidate PCell(s) which met the configured CHO execution conditions before the RLF is encountered

  - Identifier of candidate PSCell(s) which met the configured CPAC execution conditions before the RLF is encountered

  - FFS whether to optimize the association of the PCell and PSCell.

MRO for S-CPAC:

Reusing SCG FAILURE INFORMATION REPORT message over Xn for MN to report SCG failure of SCPAC to the concerned SN.

Based on above agreements, here we provide some analysis on MRO for R18 mobility mechanisms.

RAN2 agreement in this meeting

Agreements 1: Continue the work of MRO for LTM 1st priority. 2: Focus CHO with candidate SCGs (focus on agreed scenarios by RAN3) with 2nd priority 3: For Rel-18 leftovers, regarding SON for SDT, focus on RSRP and data volume in SON reports. Other enhancements (including the additional failure cause) shall be paused unless time permits. 4: Close the stage-2 work on MRO for MR-DC SCG failure. 5: Pause subsequent CPAC and wait if any for RAN3 LS. 6: Pause and wait if any for RAN3 LS for SON/MDT for Slicing and NTN. 7: Send an LS to RAN3 that RAN2 want to prioritize RAN3-based solutions on P5 and P6. 8: Pause and wait for RAN3 LS on MHI Enhancement for SCG Deactivation/Activation

Agreements

1) UE reports the time gap between the first met condition (CHO or CPAC) and the second met condition (CPAC or CHO), and the first met execution condition (as agreed by RAN3), for a failed CHO with candidate SCGs. Details FFS.

2) Include the elapsed time between the point in time of the first fulfilled condition and RLF in RLF report. Details FFS.

# Discussion

## MRO for CHO with Candidate SCG(s)

### Failure use cases

Starting with the use cases collected so far:

| Cases | Figure: CHO with candidate SCG(s) is configured | Brief Description |
| --- | --- | --- |
| Case 1 |  | Failure happens before CHO and/or CPAC execution. 1a) RLF@srcPCell 1b) SCG failure 1c) MCG failure+SCG failure |
| Case 2 |  | CPC/CPA execution condition is met. Failure happens before CHO execution condition is met. 2a) RLF@srcPCell 2b) SCG failure 2c) MCG failure + SCG failure |
| Case 3 |  | CHO execution condition is met. No CHO only or CHO with SCG configuration. Failure happens before CPAC execution condition is met. 3a) RLF@srcPCell 3b) SCG failure 3c) MCG failure + SCG failure |
| Case 4 |  | CHO execution condition is met. There is CHO only or CHO with SCG configuration. CHO execution to a candidate PCell fails. |
| Case 5 |  | CHO execution condition is met. There is CHO only or CHO with SCG configuration. RLF shortly after CHO execution success |
| Case 6 |  | CHO execution condition is met. There is CHO only or CHO with SCG configuration. SCG failure shortly after CHO execution success |
| Case 7 |  | Both CHO and CPAC execution conditions are met. 7a) CHO execution to a candidate PCell failure. 7b) CPAC to a candidate PSCell failure. 7c) Execution to SCG failure and toMCG failure. |
| Case 8 |  | Failure happens shortly after CHO and CPAC execution success 8a) RLF@targetPCell 8b) SCG failure 8c) MCG failure + SCG failure |
| Case 9 |  | Failure due to the execution conditions for candidate PCell and the associated candidate PSCell cannot be met simultaneously. 9a) RLF@srcPCell 9b) SCG failure 9c) MCG failure + SCG failure |

Discussion:

- Q1: For case 4/5/6, whether there is any spec impact?

Postpone.

- Q2: Whether MCG failure + SCG failure is a rare case? Whether case 1c, 2c, 2c, 3c, 7c, 8c, 9c are in the scope of Rel-19?

Postpone.

- Case 9 has been agreed to address in last meeting. It means CHO execution condition and the associated CPAC execution condition cannot be met simultaneously

Companies propose to consider case 10: UE meets the execution conditions of non-associated candidate PCells and PSCells before encountering an RLF or SCG Failure, which may be different from case 9.

Q3: Whether case 10 should be considered or not?

- Q4: The RLF report could be enhanced?

Moderator suggests the first three bullets can be discussed with high priority.

  - Identifier of candidate PCell(s) which met the configured CHO execution conditions before the RLF is encountered

  - Identifier of candidate PSCell(s) which met the configured CPAC execution conditions before the RLF is encountered

  - The fulfilled execution conditions before the RLF is encountered.

  - Time duration between CHO execution conditions being fulfilment and CPAC execution conditions being fulfilment (agreed by RAN2: UE reports the time gap between the first met condition (CHO or CPAC) and the second met condition (CPAC or CHO))

  - Time elapsed between the fulfilled execution condition and RLF (agreed by RAN2: Include the elapsed time between the point in time of the first fulfilled condition and RLF in RLF report)

  - the SCG failure information

  - “Identifier of candidate PCell and PSCell” the ones of the pair selected by UE when both conditions are fulfilled before the RLF is encountered either during the execution phase or shortly after execution completion.

  - PSCell(s) and PCell(s) measurements when CHO/CPAC execution condition of a candidate PCell/PSCell is fulfilled but CPAC/CHO execution condition of the associated candidate PSCell/PCell is not fulfilled but there is other PSCell(s)/PCell(s) from different association(s) with fulfilled execution conditions.

  - An indication of the first fulfilled event /execution condition (CHO or CPAC)

  - Additional information on the state of the execution event that was not triggered

  - Time elapsed between the first fulfilled execution condition and RLF

  - CHO recovery cell ID

  - Add a flag indicating that CHO or CPAC condition was fulfilled

Conclusion:

### Near failure scenarios related with configuration of CHO with candidate SCG(s) and CHO only

Based on current specification, in addition to CHO-CPAC configuration, the network can provide a complementary CHO-only configuration /execution condition to the same UE.  And in case the CHO-CPAC execution conditions are not meet, instead the CHO-only execution condition is met.

Q5: Whether CHO only executed as complementary for configuration of CHO with candidate SCG(s) can be concluded as near failure case?

Conclusion:

## MRO for S-CPAC

### Problem definition and detection mechanism

In last RAN3 meeting, stage-2 description on CPAC problem definition has been agreed to reuse.

As for the Detection mechanism, CPC detection mechanism could also be reuse for S-CPAC.

Q6: Whether any modification for detection mechanism is needed?

Conclusion:

### Forwarding mechanism

- As legacy CPAC, MN is responsible for determining where to forward the information for failed S-CPAC events when receiving SCGfailureInformation message from the UE.

- According to the agreement achieved in last meeting: Reusing SCG FAILURE INFORMATION REPORT message over Xn for MN to report SCG failure of SCPAC to the concerned SN. Here the concerned SN may include:

  - target SN (for candidate PScell selection optimization), source SN (for following execution condition optimization) for MN initiated SCPAC

  - Initiating SN (for candidate PScell selection optimization and initiating execution condition optimization), source SN (for following execution condition optimization), target SN (for candidate PScell selection optimization) for SN initiated inter-SN SCPAC.

- Q7: Whether above description are agreeable?

- Q8: Whether TP for 37.340 is needed for Q6 and/or Q7?

Conclusion:

### Successful use cases

Discussion

- Q9: Ping-Pong issue should be solved immediately or in the next S-CPAC configuration, i.e. whether the updated UHI shall be sent to subsequent target SNs during S-CPAC procedure?

Conclusion:

- Q10: The main criteria to decide between CPAC and S-CPAC configuration are the time between PSCell changes. Whether to consider suboptimal usage of CPAC or S-CPAC?

Conclusion: