SA WG2 Meeting #163	S2-2406813

Jeju island, Korea, May 27 to May 31, 2024

Source:	Xiaomi, Nokia ?, CableLabs ? Charter ? LG ? Oppo ? Vivo ? InterDigital ? Huawei ?, NEC

(Merge of S2-2405932, S2-2406780, S2-2406372, S2-2406018, S2-2406047, S2-2406254, S2-2406546)

Title:	Interim conclusions for KI#4 Identifying non-3GPP Devices Connecting behind a UE or 5G-RG

Document for:	Approval

Agenda Item:	19.8

Work Item / Release:	FS_UIA/ Rel-19

Abstract of the contribution: This contribution proposes an interim conclusion for KI#4 Identifying non-3GPP Devices Connecting behind a UE or 5G-RG.

# 1. 	Discussion

It is proposed to update TR 23.700-32 v0.3.0 as follows:

**** First Change ****

# 8	Conclusions

Editor's note:	This clause will capture conclusions for the study.

### 8.4 	Interim conclusion for KI#4 Identifying non-3GPP Devices Connecting behind a UE or 5G-RG

The following bullets are the interim conclusion principles for KI#4:

- Authentication for non-3GPP device with Device Identifier is performed by UE or 5G-RG, which is not in the scope of 3GPP.

- 5GC is able to identify the traffic from for each individual non-3GPP devices by using the device identifier.

- Device Identity Profile identifier is defined to contain a unique device identifier, IP or MAC address(es)) device specific QoS/Policy Settings.

- The connection between non-3GPP devices and UE/5G-RG is not impacted.

- The UE/5G-RG can support the traffic from non-3GPP devices that are not linked with a device identifier. Devices that do not require differentiated QoS can use the default QoS Flow established using Rel-18 Connectivity Group ID mechanisms.

- Non-3GPP devices can be locally authenticated or via an external AAA server, depending on the device capability.

- NOTE: Whether and how to authenticate/authorize a non-3GPP device is defined by SA3

- UE/5G-RG may indicate feature support for identifying non-3GPP devices for providing differentiated QoS during registration

- For non-3GPP devices requiring QoS differentiation, non-3GPP Device Identifiers and their corresponding QoS/Policies associated with a UE/5G-RG subscription are provisioned into the UDR by the AF. UE/5G-RG will bind the Device Identifier to a non-3GPP device.

- QoS differentiation can be performed within a PDU Session (using different QoS Flows) or between PDU Sessions. In the case of non-3GPP devices sharing a PDU Session, QoS Flows are provisioned in the UE/5G-RG using PDU Session Modification procedures. The URSP Rules and the non-3GPP Device Identifiers can be used to steer raffic from the non 3GPPP device to a suitable PDU Session. Based on the offline mechanisms of the UE with the AF, an AF session with required QoS may be implemented as per clause 4.15.6.6 and/or clause 4.15.6.7a of TS 23.502 for the corresponding non-3GPP device.

NOTE: 	Who (a UE/5G-RG or the AF) decides to request for change of QoS flows shall be discussed and documented during the normative stage.

NOTE: In case of MAC randomization, a change in the MAC address of non-3GPP device will not trigger a new PDU session establishment/modification by the UE/5G-RG.

- The SMF will forward the Device Identifier to the PCF. The PCF will take it into account for policy decisions based on the Device Identifier information retrieved from the UDR.

- The restriction for max number of simultaneously active non-3gpp devices can be handled at the normative stage.

NOTE: UE/5G-RG shall not carry out re-association by initiating PDU Session Modification procedures for a non-3GPP device due to randomization of MAC address.

**** End of Changes ****