SA WG2 Meeting #163	S2-2406251

27 May – 31 May, 2024, Jeju, Korea

Source:	InterDigital Inc.

Title:	Key Issue #1, Conclusion Update

Document for:	Approval

Agenda Item:	19.8

Work Item / Release:	FS_XRM_Ph2/ Rel-19

Abstract of the contribution: This paper proposes a Key Issue #1 conclusion update that is aligned with the pre-SA2 #163 NWM responses.

- 1.	 Discussion

This p-CR proposes to update the Key Issue #1 conclusion based on the pre-SA2 #163 NWM responses which are submitted in S2-2406249.

- 2. Proposals

It is proposed to adopt the following text in TR.23.700-32 v0.3.0.

*** Start of Changes ***

## 8.1	Key Issue #1: Identifying the Human User of a Subscription

### 8.1.1	Conclusions Related to the User Identifier and the User Identity Profile

The following bullets are conclusion principles for KI#1:

The User Identifier format is NAI.

-	The User Identifier can be operator assigned or third party assigned.

-	The User Identity Profile may contain:

-	at least one User Identifier;

NOTE:	Whether more than one User Identifier can be included in the User Identity Profile can be considered during the normative phase.

-	a list of linked subscriptions;

-	QoS Information (e.g. 5QI and ARP);

-	DNN/S-NSSAI combinations that the user is allowed to access.

-	The QoS Information in the User Identity Profile can optionally be stored per DNN/S-NSSAI.

-	The User Identity Profile can be managed by an operator or by a third party.

-	The User Identity Profile is stored in the UDR.

-	The User Identifier is unique within PLMN and can be linked/unlinked with a 5GS subscription identified by a SUPI.

-	NEF APIs can be used by a trusted AF (i.e. operator owned, or 3rd party owned) to manage the User Identity Profile.

-	Managing a User Identity Profile includes creating, modifying, and deleting a User Identity Profile.

-	OAM procedures can also be used to manage the User Identity Profile; however, the details are left to SA WG#5.

-	A trusted AF can use an NEF API to request that a user identifier be linked/unlinked with a subscription.

-	For each user identifier, there is only one AF that is authorized to request that the user identifier be linked to a subscription.

-	For each subscription, there is only one AF that is authorized to request that a user identifier be linked to the subscription.

-	OAM procedures can be used to link/unlink a user identifier with a subscription.

NOTE:	Where user credentials are stored can be decided by SA WG3.

### 8.1.2	Conclusions Related to the User Identifier’s Influence on QoS

The following bullets are conclusion principles for KI#1:

- PCC Rules are adjusted/updated when a user a becomes active with a subscription.

- PCC Rules are updated based on information that the PCF receives from the User Identity Profile.

- PCC Rules can be updated based on invocation of existing PCF/NEF API (e.g., Npcf_PolicyAuthorization_Create).

### 8.1.3	Conclusions Related to Activation of a User

The following bullets are conclusion principles for KI#1:

- A linked user becomes active with a subscription by providing a user identifier in a NAS-MM or a NAS-SM message.

Editor’s note: Whether a NAS-MM or a NAS-SM message is used will be determined at SA2 #163.

### 8.1.4	Conclusions Related to SMS

The following bullets are conclusion principles for KI#1:

- SMS can be used when a user identifier is active with a subscription.

- When a user is active with a subscription, delivery of the SMS service continues based only on information from the subscription.

- When a user is active with a subscription, the SMS service is not impacted.

### 8.1.5	Conclusions Related to IMS

The following bullets are conclusion principles for KI#1:

- IMS can be used when a user identifier is active with a subscription.

- When a user is active with a subscription, delivery of the IMS service continues based only on information from the subscription.

- When a user is active with a subscription, activation of the user does not impact the IMS service.

*** End of Changes ***