SA WG2 Meeting #163	S2-2407225

27 May – 31 May, 2024, Jeju, Korea      (Merge of S2-2406251, 5929, 6012, 6040, 6371, 6480, 6545, 6616, 6816)

Source:	InterDigital Inc., NEC, Nokia, OPPO, CATT, LG Electronics, Lenovo, Huawei, ZTE, Xiaomi, Philips, Deutsche Telekom, Futurewei

Title:	Key Issue #1, Conclusion Update

Document for:	Approval

Agenda Item:	19.8

Work Item / Release:	FS_UIA_ARC/ Rel-19

Abstract of the contribution: This paper proposes a Key Issue #1 conclusion update that is aligned with the pre-SA2 #163 NWM responses.

- 1.	 Discussion

This p-CR proposes to update the Key Issue #1 conclusion based on the pre-SA2 #163 NWM responses which are submitted in S2-2406249.

- 2. Proposals

It is proposed to adopt the following text in TR.23.700-32 v0.3.0.

*** Start of Changes ***

## 8.1	Key Issue #1: Identifying the Human User of a Subscription

### 8.1.1	Conclusions Related to the User Identifier and the User Identity Profile

The following bullets are conclusion principles for KI#1:

The User Identifier format is NAI.

-	The User Identifier can be operator assigned or third party assigned.

-	The User Identity Profile contains:

-	one User Identifier;

-	a list of linked subscriptions;

-	optionally, DNN/S-NSSAI combinations used for URSP Rules

-	optionally, QoS Information (e.g. default 5QI and ARP per DNN/S-NSSAI);

NOTE: 	Subscriber/subscription information will not be moved into a user profile and information from the user profile should not be used to override information in a subscription. For example, the slices and DNNs that are available to the UE do not change based on the user of the UE.

-	The UDM can store which User Identifier is active with a UE Subscription.

-	The User Identity Profile is stored in the UDR and the user identifier can be used as a data key.

-	The User Identity Profile can be managed by an operator. Managing a User Identity Profile includes creating, modifying, and deleting a User Identity Profile.

-	OAM procedures can also be used to manage the User Identity Profile.

-	NEF APIs can be used by an authorized AF (i.e. operator owned, or 3rd party owned) to manage the User Identity Profile.

-	The User Identifier is unique within PLMN and can be linked/unlinked with a 5GS subscription identified by a SUPI.

-	An authorized AF can use an NEF API to request that a user identifier be linked/unlinked with a subscription.

-	OAM procedures can be used to link/unlink a user identifier with a subscription.

-	For each user identifier, there is only one AF that is authorized to request that the user identifier be linked to a subscription.

-	For each subscription, there is only one AF that is authorized to request that a user identifier be linked to the subscription.

NOTE:	Where user credentials are stored can be decided by SA WG3.

-	The UE subscription data is extended to include:

-  	a list of linked User Identifiers, and

The user identifier is an optional feature in 5GS. A user can only become active when the UE is registered in 5G. When the UE moves the 4G, the network will consider the user inactive, and service will be delivered based on information in the UE subscription. When the UE returns to 5G, the UE can initiate the procedure to make the user become active again. Information from the User Profile is not sent to the EPC.

### 8.1.2	Conclusions Related to the User Identifier’s Influence on QoS

The following bullets are conclusion principles for KI#1:

- When a user becomes active, the AMF sends the active user identifier to the SMF(s) that serve the UE’s PDU Session(s) and the SMF sends the user identifier to the PCF.

- During PDU Session Establishment, the AMF sends the active user identifier to the SMF and the SMF sends the user identifier to the PCF.

- PCC Rules may be adjusted/updated when a user becomes active with a subscription.

- PCC Rules may be updated by the PCF based on information received from the User Identity Profile.

- User Identities can be considered by the PCF when generating URSP Rules.

- In addition to the principles listed above, existing Rel-18 functionality can be used to influence PCC Rules, for example PCC Rules can be updated based on invocation of existing PCF/NEF APIs such as Npcf_PolicyAuthorization_Create.

### 8.1.3	Conclusions Related to Activation of a User

The following bullets are conclusion principles for KI#1:

- NAS signalling support initiating the authentication, authorization and activation procedure by sending the user identity information to the network in a NAS-MM message.

- Since he operator that manages the User Identifier and the operator that manages the subscription is assumed to be the same, the AMF knows to for a request for authenticating a user to the home network of the UE. The home network of the UE can discover the AAA Server based on e.g. the realm part of the user identifier.

NOTE 1:	The details of the Authentication mechanisms are in the remit of SA WG3.

### 8.1.4	Conclusions Related to SMS over NAS

There will be no impacts to SMS over NAS in this release.

NOTE 1: 	Depending on input from SA WG1 or SA WG3, SMS over NAS may be disabled or enabled when a user becomes active or inactive.

*** End of Changes ***