SA WG2 Meeting #163	S2-2406251

27 May – 31 May, 2024, Jeju, Korea      (Merge of S2-2406251, 5929, 6012, 6040, 6371, 6480, 6545, 6616, 6816)

Source:	InterDigital Inc., NEC, Nokia, OPPO, CATT, LG, Lenovo?, Huawei, ZTE?, Xiaomi?

Title:	Key Issue #1, Conclusion Update

Document for:	Approval

Agenda Item:	19.8

Work Item / Release:	FS_XRM_Ph2/ Rel-19

Abstract of the contribution: This paper proposes a Key Issue #1 conclusion update that is aligned with the pre-SA2 #163 NWM responses.

- 1.	 Discussion

This p-CR proposes to update the Key Issue #1 conclusion based on the pre-SA2 #163 NWM responses which are submitted in S2-2406249.

- 2. Proposals

It is proposed to adopt the following text in TR.23.700-32 v0.3.0.

*** Start of Changes ***

## 8.1	Key Issue #1: Identifying the Human User of a Subscription

### 8.1.1	Conclusions Related to the User Identifier and the User Identity Profile

The following bullets are conclusion principles for KI#1:

The User Identifier format is NAI.

-	The User Identifier can be operator assigned or third party assigned.

-	The User Identity Profile contains:

-	one User Identifier;

-	a list of linked subscriptions;

-	optionally, DNN/S-NSSAI combinations that the user is allowed to access.

-	optionally, QoS Information (e.g. default 5QI and ARP per DNN/S-NSSAI);

-	optionally, Time duration to limit when a human User is allowed to use the UE.

-	The User Identity Profile may also indicate to which UE Subscription that the User Identity Profile is currently active.

-	The User Identity Profile is stored in the UDR and the user identifier can be used as a data key.

-	The User Identity Profile can be managed by an operator or by a third party. Managing a User Identity Profile includes creating, modifying, and deleting a User Identity Profile.

-	OAM procedures can also be used to manage the User Identity Profile.

-	NEF APIs can be used by an authorized AF (i.e. operator owned, or 3rd party owned) to manage the User Identity Profile.

-	The User Identifier is unique within PLMN and can be linked/unlinked with a 5GS subscription identified by a SUPI.

-	An authorized AF can use an NEF API to request that a user identifier be linked/unlinked with a subscription.

-	OAM procedures can be used to link/unlink a user identifier with a subscription.

-	For each user identifier, there is only one AF that is authorized to request that the user identifier be linked to a subscription.

-	For each subscription, there is only one AF that is authorized to request that a user identifier be linked to the subscription.

NOTE:	Where user credentials are stored can be decided by SA WG3.

-	The UE subscription data is extended to include:

-  	a list of linked User Identifiers, and

-	optionally an activated User Identifier to indicate the User Identifier that is currently active with this UE.

### 8.1.2	Conclusions Related to the User Identifier’s Influence on QoS

The following bullets are conclusion principles for KI#1:

- When the UE with active User Identifier establishes PDU Session, the SMF provides the User Identifier to the PCF.

- PCC Rules may be adjusted/updated when a user becomes active with a subscription.

- PCC Rules may be updated by the PCF based on information received from the User Identity Profile.

- PCC Rules can be updated based on invocation of existing PCF/NEF API (e.g., Npcf_PolicyAuthorization_Create).

- User Identities can be considered by the PCF when generating URSP Rules.

### 8.1.3	Conclusions Related to Activation of a User

The following bullets are conclusion principles for KI#1:

- A linked user becomes active by sending the user identifier to the network in a NAS-MM message.

NOTE 1:	The details of the Authentication and Authorization procedure are in the remit of SA WG3.

### 8.1.4	Conclusions Related to SMS over NAS

There will be no impacts to SMS over NAS in this release.

*** End of Changes ***