3GPP SA WG2 Meeting #164	S2-2407612

Maastricht, NL, 19 - 23 Aug, 2024

Source: 	Intel

Title: 	Update to KI#2 conclusions related to Layer-3 U2U Relay

Document for: 	Agreement

Agenda Item: 	19.3

Work Item / Release:	FS_5G_ProSe_Ph3, 5G_ProSe_Ph3 / Rel-19

Abstract of the contribution: This paper proposes an update of KI#2 conclusion.

# 1	Discussion

SA2#163 meeting agreed that support of Layer-3 multi-hop UE-to-UE Relay is based on solution #3 and #4.

While both solutions #3 and #4 are based on MANET, there are a couple of notable differences as summarized below.

Solution #3 assumes that:

-	The source End UE connects to the nearest U2U Relay(s) without prior discovery of target End UEs that are actually reachable via that U2U Relay.

-	When wishing to send data to a specific target End UE, the source End UE performs a DNS request based on the User Info ID of the remote End UE. It is at this point that the source End UE will actually “discover” whether the target End UE is reachable or not.

-	The DNS request from the source End UE is resolved locally by the nearest U2U Relay (TBC with Sol#3 authors) based on information received from all other U2U Relays connected to the MANET. The DNS-related information (i.e. (User Info ID, IP address/prefix) pairs) is disseminated through the MANET using either extensions to the MANET TC message or using dynamic DNS Updates (RFC 2136???).

In contrast, Solution #4 assumes that:

-	The source End UE performs ProSe Discovery (e.g. Group Member Discovery) before attempting to establish a connection via the nearest U2U Relay(s).

-	The information about target End UEs that are reachable via the MANET network is disseminated via the MANET network to all connected U2U Relays using a dedicated MANET message referred to as MANET Discovery Info message.

-	When wishing to send data to a specific target End UE, the source End UE sends a Direct Connection Request message to the local U2U Relay, which propagates it via the MANET network until it reaches the target End UE. As part of this process the IP address of the target End UE is propagated to the local U2U Relay.

The authors acknowledge that overall the approach in Solution #3 is much simpler in that it basically skips the notion of ProSe Discovery as defined in Rel-18 altogether and avoids the complexity related to propagation of ProSe Discovery-enabling information through the MANET network. It also avoids the complexity of relaying connection management messages (e.g. Direct Connection Request) through the MANET network.

Proposal 1: Update the KI#2 conclusion to clarify that End UEs connect to the local U2U Relay without prior ProSe Discovery.

On the other hand, the authors of this paper are of the opinion that the propagation of End UE’s IP address should better be performed via a dedicated MANET message, such as the MANET Discovery Info proposed in Solution #4, for the following reasons:

-	MANET TC (Topology Control) message is used for topology control and is triggered when there are topology changes in the MANET network. While the addition or removal of an End UE to the MANET network requires updates of the DNS entries in all U2U Relays connected to the MANET, there is no need to trigger the MANET nodes for topology reassessment. In other words, the frequency of topology updates is different and unrelated from the frequency of addition/removal of End UEs.

-	The use of Dynamic DNS requires that each U2U Relay individually contacts all the U2U Relays in the MANET network, instead of relying on the controlled flooding mechanisms that is built in the MANET messages. This may result in significant increase of signalling through the MANET.

-	In contrast, the use of a dedicated MANET message (referred to here as the MANET Discovery Info message) combines the best of both worlds, i.e. it can leverage the built-in controlled flooding mechanism of MANET messages, while being triggered only when an End UE is added or removed.

Proposal 2: Update the KI#2 conclusion to clarify that End UE’s IP addresses and associated User Info IDs are disseminated through the MANET network using a dedicated MANET message (to be defined by CT1 WG).

# 2	Proposal

Proposal 1: Update the KI#2 conclusion to clarify that End UEs connect to the local U2U Relay without prior ProSe Discovery.

Proposal 2: Update the KI#2 conclusion to clarify that End UE’s IP addresses and associated User Info IDs are disseminated through the MANET network using a dedicated MANET message (to be defined by CT1 WG).

It is proposed to agree the conclusion update in TR 23.700-03 as proposed below.

- * * * * First change * * * *

## 8.2	Conclusions for KI#2

The following principles are used for normative work:

For PDU type IP:

Support of Layer-3 multi-hop UE-to-UE Relay to normative phase is based on solution #3 and #4, with the following reasoning:

-	They are based on an existing MANET protocol (IETF RFC 7181 [9]) which is a proven mobile ad-hoc routing protocol.

-	The End UE is aware of other End UEs in the mobile ad-hoc network but the exact paths between the 5G ProSe End UEs is determined by the UE-to-UE Relays at the IP layer. The End UE does not have to manage the routing paths. Routing is performed at the IP layer.

-	Topology changes in the ad hoc network, e.g. due to movement of End UEs, or Relays, is handled by the UE-to-UE Relays at the IP layer based on MANET according to IETF RFC 7181 [9].

-	MANET link state management at IP layer (e.g. IETF RFC 7181 [9]) is reused. Whether further optimization for MANET operation is needed, e.g. reusing some existing PC5-S signalling to reduce MANET signalling, will be decided in the normative phase.

-	The End UE connects to the nearest U2U Relay without prior discovery of End UEs that are actually reachable via that U2U Relay.

-	When wishing to send data to a specific remote End UE, the source End UE performs a DNS request based on the User Info ID of the target End UE.

-	The DNS request is resolved by the local U2U Relay based on information received from all other U2U Relays connected to the MANET. The DNS-related information (i.e. (User Info ID, IP address/prefix) pairs) is disseminated through the MANET network using a dedicated MANET Discovery Info message.

NOTE 1:	The dedicated MANET Discovery Info message is to be defined by CT1 WG.

For PDU type Ethernet and Unstructured:

The support of multi-hop UE-to-UE Relay is based on the extension of Rel-18 methods as described in TS 23.304 [4].

-	For 5G ProSe multi-hop UE-to-UE Relay Discovery & link setup/management, both Model A and Model B are supported.

-	The maximum number of hops could be decided per RSC, or be decided by End UE based on QoS requirements.

-	The UE-to-UE Relay includes its own User Info ID when relaying the discovery message.

-	The 5G ProSe End UE selects the multi-hop UE-to-UE Relay path to another 5G ProSe End UE.

-	To perform link management, the DCR message is unicasted between Relays according to the path information included in the message. The path information is an (ordered) list User Info ID of Relays in the selected path. The 5G ProSe End UE sends the selected path information to the UE-to-UE Relay for communication setup.

-	The End-to-End QoS is handled hop-by-hop, by using Rel-18 QoS handling mechanism, each UE-to-UE Relay decides the QoS of adjacent PC5 hop and the remaining hops.

NOTE 2:	Security aspects will be addressed by SA WG3.

- * * * * End of changes * * * *