SA WG2 Meeting #164	S2-2408975

19 August – 23 August, 2024, Maastricht, Netherlands                        (Revision of S2-2408606)

Source:	InterDigital Inc., Deutsche Telekom (?)

Title:	Key Issue #1, Conclusion Update

Document for:	Approval

Agenda Item:	19.8.1

Work Item / Release:	FS_UIA_ARC/ Rel-19

Abstract of the contribution: This paper proposes a Key Issue #1 conclusion update.

- 1.	 Discussion

This p-CR proposes a Key Issue #1 conclusion update.

The FS_UIA_ARC study has limited scope and includes the following statements:

- The focus is on two use cases. In both use cases one or more user identifiers may be associated with the subscription (i.e., identified by a SUPI) of a UE or RG and the users require different treatment (i.e., service differentiation).

- When the user identifier applies to a human, only a single user identifier is associated with the UE at a given time and it is assumed that the user identifier is associated with all of services that the UE access during the time that the user identifier and UE are associated.

Observation 1: The focus of the study is on service differentiation. Providing the user with user specific access to services such as SMS is not the focus of this study.

Observation 2: If the UE does access services such as SMS while a user is active, there is an assumption in the study that the user identifier is associated with the SMS service.

The incoming LS from SA1 (S1-242347/S1-249216) indicates that SA1 approved a CR with the following requirement:

- The 3GPP network shall be able to protect the privacy of a user when accessing the 3GPP services from a UE which is shared by multiple users.

- Note: Privacy protection mechanisms could be UE implementation dependent, but this is out of scope of 3GPP.

The incoming LS from SA1 also refences the following requirement from TS 22.101:

- The 3GPP system shall enable a user to configure, within the boundaries set by the network operator, which services shall be available on a device where a user logs in. These services include voice, video, and messaging.

The incoming LS from SA1 also states:

- SA1 confirms that MSISDN and IMPUs are still used for SMS and IMS communication. A user identifier is mapped to a corresponding GPSI or IMPU of the subscription that the user identifier is active with.

Observation: In this release, it was not a focus for SA2 to enable to the user identifier to be associated with a user specific MSISDN and/or IMPU.

Proposal: When a user becomes active in, services other than data (i.e. SMS and IMS) can be optionally disabled (i.e. based on operator configuration). In other words, if the user is not associated with the IMPU and MSISDN of the subscription, then the operator disable access to IMS and/or SMS.

- 2. Proposals

It is proposed to adopt the following text in TR.23.700-32 v1.0.0.

*** Start of Changes ***

## 8.1	Key Issue #1: Identifying the Human User of a Subscription

### 8.1.1	Conclusions Related to the User Identifier and the User Identity Profile

The following bullets are conclusion principles for KI#1:

The User Identifier format is NAI.

-	The User Identifier can be operator assigned or third party assigned.

-	The User Identity Profile contains:

-	one User Identifier;

-	a list of linked subscriptions;

-	optionally, DNN/S-NSSAI combinations used for URSP Rules;

-	optionally, QoS Information (e.g. default 5QI and ARP per DNN/S-NSSAI);

NOTE: 	Subscriber/subscription information will not be moved into a user profile and information from the user profile should not be used to override information in a subscription. For example, the slices and DNNs that are available to the UE do not change based on the user of the UE.

-	The UDR can store which User Identifier is active with a UE Subscription.

-	The User Identity Profile data is stored as a new data set in the UDR and the user identifier can be used as a data key.

-	The User Identity Profile can be managed by an operator. Managing a User Identity Profile includes creating, modifying, and deleting a User Identity Profile through UDM service operations.

-	OAM procedures can also be used to manage the User Identity Profile.

-	NEF APIs can be used by an authorized AF (i.e. operator owned, or 3rd party owned) to manage the User Identity Profile.

-	The User Identifier is unique within PLMN and can be linked/unlinked with a 5GS subscription identified by a SUPI.

-	An authorized AF can use an NEF API to request that a user identifier be linked/unlinked with a subscription.

-	OAM procedures can be used to link/unlink a user identifier with a subscription.

-	For each user identifier, there is only one AF that is authorized to request that the user identifier be linked to a subscription.

-	For each subscription, there is only one AF that is authorized to request that a user identifier be linked to the subscription.

NOTE:	Where user credentials are stored can be decided by SA WG3.

-	The UE subscription data is extended to include:

-  	a list of linked User Identifiers, and

The user identifier is an optional feature in 5GS.

### 8.1.2	Conclusions Related to Activation of a User

The following bullets are conclusion principles for KI#1:

- NAS signalling supports initiating activation by sending the user identity information to the network in a NAS-SM message. The UE is configured with a DNN/S-NSSAI of a PDU Session that can be used to send the user identifier to the network. The NAS-SM message only needs to be sent for this one PDU Session to become active.

- Authentication/authorization of the user can be via NAS-SM after the NAS-SM message is sent to activate the user or Authentication/authorization of the user can be via Application layer before the NAS-SM message is sent to activate the user.

- When authentication/authorization is via the Application Layer, the principles of Solution #23 are used for authentication/authorization and the application layer notifies the UDM/UDR that the user has been authenticated/authorized. The principles of Solution #23 are used (i.e. clause 6.23.3.2) for this option.

- When authentication/authorization is via NAS-SM, the SMF initiates the Secondary authorization/authentication by an DN-AAA Server procedure if the UDR does not indicate that the user was already authenticated (i.e. if the user was not already authenticated at the application layer).

- When a user becomes active, the PCF sends the active user identifier to the UDR. The PCF(s) that serve the UE’s other PDU Session(s) are then notified that the user is active.

NOTE 1:	The details of the Authentication/Authorization mechanisms and which approach(es) are used are in the remit of SA WG3.

### 8.1.3	Conclusions Related to the User Identifier’s Influence on QoS

The following bullets are conclusion principles for KI#1:

- During PDU Session Establishment, the SMF sends the user identifier to the PCF through the policy association establishment procedure. The UDR notifies the PCF(s) that serve the UE’s other PDU Session(s) that the user is active.

- PCC Rules may be adjusted/updated when a user becomes active with a subscription.

- PCC Rules may be updated by the PCF based on information received from the User Identity Profile.

- User Identities can be considered by the PCF when generating URSP Rules.

### 8.1.4	Conclusions Related to SMS over NAS

Based on an indication in the user profile, SMS and/or IMS can be disabled when a user becomes active. The indication is based on operator policy.

8.1.5	Conclusions Related to Interworking

Interworking scenarios will be handled as follows.

- When the UE is connected to EPC and user needs to “log-out” or the network wants to “log-out” the user, the principles of Annex H of TS 23.502 [5] can be used; the DN-specific identity can be set to the user identifier.

- When the UE is in EPC, the PCF can initiate the SM Policy Association Modification procedure to send new PCC Rules to the SMF+PGW-C (see TS 23.502 [5], clauses 4.11.0a.2 and 4.16.5.2).

- When the UE is in 5GC, the PCF can initiate the SM Policy Association Modification procedure to send new PCC Rules to the SMF+PGW-C (see TS 23.502 [5], clauses 4.11.0a.2 and 4.16.5.2).

8.1.6	Conclusions Related to Roaming

Activation and deactivation of the user is transparent to the AMF and therefore the feature will work in roaming scenarios.

*** End of Changes ***