SA WG2 Meeting #164	S2-240xxx

19-23 August 2024, Maastricht, Netherlands 	(was S2-2406961)

Source:	Meta USA, Tencent, Vivo, CATT

Title:	KI#9, agreement, rate limitation exposure

Document for:	Approval

Agenda Item:	19.3

Work Item / Release:	FS_XRM Ph2 / Rel-19

Abstract of the contribution: It is proposed to add the Rate limitation as network exposure to agreement for KI#9.

# Intro

At SA2#163, two type of rate limiting information were being discussed: 1) AMBR for PDU session, 2) media rate limiting information for video, to be exposed to AF/AS. Both of these information can be used by the sender (application) to select an initial codec rate that closely matching with the serving network policy. The end-result is less stalling of video frame due to initial rate mismatch.

The exposure path via N6 did not reach consensus. One opponent wanted to study this path in a holistic way to include other possible data which means out of scope of R19.

The other exposure path is via PCF to AF with SBI. AMBR for PDU session is already known in PCF today, and media rate limiting information for video can be locally configured in PCF for Rel-19.

It is proposed to expose both AMBR for PDU session and media rate limiting information for video via PCF-AF interface.

Proposal

It is proposed to add the following interim agreement to the TR 23.700-70

START OF CHANGES

# 8	Conclusions

## 8.X Agreement for KI#9

The following interim agreement is considered for KI#9:

- UPF may provide AMBR for PDU session, and media rate limiting information to the AF

- via SBI with Nnef_EventEexposure service.

- NOTE 1: AMBR for PDU session is only used as an “upper limit” information for an application in the AS. AMBR information is only reported once and does not change even when there is more than one QoS flow in used.

END OF CHANGES