| 3GPP TS 31.121 V17.1.0 (2023-12) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; UICC-terminal interface; Universal Subscriber Identity Module (USIM) application test specification (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	30

Introduction	31

1	Scope	31

2	References	31

3	Definitions and abbreviations	34

3.1	Definitions	34

3.2	Symbols	35

3.3	Abbreviations	35

3.4	Coding Conventions	37

3.5	Generic procedures for E-UTRAN/UTRAN/GERAN/IMS/NB-IoT/5G-NR	37

3.6	Applicability	38

3.6.1	Applicability of the present document	38

3.6.2	Applicability to terminal equipment	38

3.6.3	Applicability of the individual tests	38

3.7	Table of optional features	39

3.8	Applicability table	43

4	Default Values	58

4.1	Definition of default values for USIM-Terminal interface testing (Default UICC)	58

4.1.1	Values of the EF's (Default UICC)	58

4.1.1.1	EFIMSI (IMSI)	58

4.1.1.3	EFLOCI (Location Information)	59

4.1.1.4	EFKeys (Ciphering and Integrity Keys)	59

4.1.1.5	EFKeysPS (Ciphering and Integrity Keys for Packet Switched domain)	59

4.1.1.6	EFACC (Access Control Class)	59

4.1.1.7	EFFPLMN (Forbidden PLMNs)	59

4.1.1.8	EFUST (USIM Service Table)	59

4.1.1.9	EFEST (Enable Service Table)	60

4.1.1.10	EFADN (Abbreviated Dialling Number)	60

4.1.1.11	EFPLMNwACT (User Controlled PLMN Selector with Access Technology)	60

4.1.1.12	EFOPLMNwACT (Operator Controlled PLMN Selector with Access Technology)	61

4.1.1.13	Void	62

4.1.1.14	PIN	62

4.1.1.15	PIN2	62

4.1.1.16	Unblock PIN	62

4.1.1.17	Unblock PIN2	62

4.1.1.18	Other Values of the USIM	62

4.1.1.19	EFPSLOCI (Packet Switch Location Information)	62

4.1.1.20	Universal PIN	63

4.1.1.21	Unblock Universal PIN	63

4.2	Definition of FDN UICC	63

4.2.1	Values of the EF's (FDN UICC)	63

4.2.1.3	EFFDN (Fixed Dialling Numbers)	63

4.2.1.4	EFECC (Emergency Call Codes)	64

4.2.1.5	Other Values of the USIM	64

4.3	Void	65

4.4	Definition of E-UTRAN/EPC UICC	65

4.4.1	EFUST (USIM Service Table)	65

4.4.2	EFEPSLOCI (EPS Information)	65

4.4.3	EFPLMNwACT (User Controlled PLMN Selector with Access Technology)	65

4.4.4	EFOPLMNwACT (Operator Controlled PLMN Selector with Access Technology)	66

4.4.5	EFACSGL (Allowed CSG Lists)	66

4.4.6	EFCSGT (CSG Type)	67

4.4.7	EFHNBN (Home (e)NodeB Name)	69

4.4.8	EFEPSNSC (EPS NAS Security Context)	71

4.5	Definition of E-UTRAN/EPC ISIM-UICC	71

4.5.1	Applications on the E-UTRAN/EPC ISIM-UICC	71

4.5.2	Default USIM values on E-UTRAN/EPC ISIM-UICC	71

4.5.3	Default ISIM values on E-UTRAN/EPC ISIM-UICC	71

4.5.3.1	EFAD (Administrative Data)	71

4.5.3.3	EFIMPI (IMS private user identity)	72

4.5.3.4	EFDOMAIN (Home Network Domain Name)	72

4.5.3.5	EFIMPU (IMS public user identity)	72

4.5.3.6	EFP-CSCF (P-CSCF ADDRESS)	73

4.5.3.7	EFSMS (Short Message Service)	73

4.5.3.8	EFSMSR (Short message status reports)	74

4.5.3.9	EFSMSP (Short message service parameters)	74

4.5.3.10	EFSMSS (SMS Status)	74

4.5.4	Default values at DF_TELECOM	74

4.5.4.1	EFPSISMSC (Public Service Identity of the SM-SC)	74

4.6	Definition of ACSGL/OCSGL E-UTRAN/EPC UICC	75

4.6.1	EFUST (USIM Service Table)	75

4.6.2	EFAD (Administrative Data)	75

4.6.3	EFOCSGL (Operator CSG Lists)	75

4.6.4	EFOCSGT (Operator CSG Type)	76

4.6.5	EFOHNBN (Operator Home (e)NodeB Name)	78

4.7	Definition of Non Access Stratum Configuration UICC	79

4.7.1	Values of the EFs	79

4.7.1.2	EFNASCONFIG (Non Access Stratum Configuration)	79

4.8	Definition of Non Access Stratum Configuration of E-UTRAN/EPC UICC	80

4.8.1	EFUST (USIM Service Table)	80

4.8.2	EFEPSLOCI (EPS Information)	80

4.8.3	EFPLMNwACT (User Controlled PLMN Selector with Access Technology)	80

4.8.4	EFOPLMNwACT (Operator Controlled PLMN Selector with Access Technology)	81

4.8.5	EFACSGL (Allowed CSG Lists)	81

4.8.6	EFCSGT (CSG Type)	82

4.8.7	EFHNBN (Home (e)NodeB Name)	84

4.8.8	EFEPSNSC (EPS NAS Security Context)	85

4.8.9	EFNASCONFIG (Non Access Stratum Configuration)	85

4.9	Definition of 5G-NR UICC	86

4.9.0	Introduction	86

4.9.2	EFIMSI (IMSI)	86

4.9.3	EF5GS3GPPLOCI (5GS 3GPP location information)	86

4.9.4	EFSUCI_Calc_Info (Subscription Concealed Identifier Calculation Information EF)	87

4.9.5	EFRouting_Indicator (Routing Indicator EF)	88

4.9.6	EF5GS3GPPNSC (5GS 3GPP Access NAS Security Context)	88

4.9.7	EF5GSN3GPPNSC (5GS non-3GPP Access NAS Security Context)	88

4.9.8	EFPLMNwACT (User Controlled PLMN Selector with Access Technology)	89

4.10	Definition of 5G-NR UICC – non-IMSI SUPI Type	90

4.10.1	EFUST (USIM Service Table)	90

4.10.2	EFIMSI (IMSI)	90

4.10.3	EFSUPI_NAI (SUPI as Network Access Identifier)	90

4.10.4	EF5GS3GPPLOCI (5GS 3GPP location information)	90

4.10.5	EFSUCI_Calc_Info (Subscription Concealed Identifier Calculation Information EF)	91

4.10.6	EFRouting_Indicator (Routing Indicator EF)	92

4.10.7	EFAD (Administrative Data)	92

4.11	5G-NR UICC – support of Rel-16 features	92

4.11.1	Introduction	92

4.11.2	EFUST (USIM Service Table)	92

4.11.3	EF5GS3GPPNSC (5GS 3GPP Access NAS Security Context)	92

4.11.4	EF5GSN3GPPNSC (5GS non-3GPP Access NAS Security Context)	93

4.12	Definition of CAG 5G-NR UICC	94

4.12.1	EFUST (USIM Service Table)	94

4.12.2	EFCAG (Pre-configured CAG information list)	94

5	Subscription related tests	95

5.1	IMSI / TMSI handling	95

5.1.1	UE identification by short IMSI	95

5.1.1.1	Definition and applicability	95

5.1.1.2	Conformance requirement	95

5.1.1.3	Test purpose	95

5.1.1.4	Method of test	95

5.1.1.4.1	Initial conditions	95

5.1.1.4.2	Procedure	95

5.1.1.5	Acceptance criteria	96

5.1.2	UE identification by short IMSI using a 2 digit MNC	96

5.1.2.1	Definition and applicability	96

5.1.2.2	Conformance requirement	96

5.1.2.3	Test purpose	96

5.1.2.4	Method of test	96

5.1.2.4.1	Initial conditions	96

5.1.2.4.2	Procedure	97

5.1.2.5	Acceptance criteria	97

5.1.3	UE identification by "short" TMSI	98

5.1.3.1	Definition and applicability	98

5.1.3.2	Conformance requirement	98

5.1.3.3	Test purpose	98

5.1.3.4	Method of test	98

5.1.3.4.1	Initial conditions	98

5.1.3.4.2	Procedure	99

5.1.3.5	Acceptance criteria	99

5.1.4	UE identification by "long" TMSI	99

5.1.4.1	Definition and applicability	99

5.1.4.2	Conformance requirement	99

5.1.4.3	Test purpose	99

5.1.4.4	Method of test	100

5.1.4.4.1	Initial conditions	100

5.1.4.4.2	Procedure	100

5.1.4.5	Acceptance criteria	100

5.1.5	UE identification by long IMSI, TMSI updating and key set identifier assignment	101

5.1.5.1	Definition and applicability	101

5.1.5.2	Conformance requirement	101

5.1.5.3	Test purpose	101

5.1.5.4	Method of test	102

5.1.5.4.1	Initial conditions	102

5.1.5.4.2	Procedure	102

5.1.5.5	Acceptance criteria	103

5.1.6	UE identification by short IMSI when accessing E-UTRAN/EPC	104

5.1.6.1	Definition and applicability	104

5.1.6.2	Conformance requirement	104

5.1.6.3	Test purpose	104

5.1.6.4	Method of test	104

5.1.6.4.1	Initial conditions	104

5.1.6.4.2	Procedure	104

5.1.6.5	Acceptance criteria	105

5.1.7	UE identification by short IMSI using a 2 digit MNC when accessing E-UTRAN/EPC	105

5.1.7.1	Definition and applicability	105

5.1.7.2	Conformance requirement	105

5.1.7.3	Test purpose	105

5.1.7.4	Method of test	105

5.1.7.4.1	Initial conditions	105

5.1.7.4.2	Procedure	106

5.1.7.5	Acceptance criteria	106

5.1.8	UE identification after changed IMSI with service "EMM Information" not available	106

5.1.8.1	Definition and applicability	106

5.1.8.2	Conformance requirement	107

5.1.8.3	Test purpose	107

5.1.8.4	Method of test	107

5.1.8.4.1	Initial conditions	107

5.1.8.4.2	Procedure	107

5.1.8.5	Acceptance criteria	108

5.1.9	UE identification by GUTI when using USIM with service "EMM Information" not available	108

5.1.9.2	Conformance requirement	108

5.1.9.3	Test purpose	108

5.1.9.4	Method of test	108

5.1.9.4.1	Initial conditions	108

5.1.9.4.2	Procedure	109

5.1.9.5	Acceptance criteria	109

5.1.10	UE identification by GUTI when using USIM with service "EMM Information" available	109

5.1.10.1	Definition and applicability	109

5.1.10.2	Conformance requirement	109

5.1.10.3	Test purpose	110

5.1.10.4	Method of test	110

5.1.10.4.1	Initial conditions	110

5.1.10.4.2	Procedure	110

5.1.10.5	Acceptance criteria	110

5.2	Access Control handling	111

5.2.1	Access Control information handling	111

5.2.1.1	Definition and applicability	111

5.2.1.2	Conformance requirement	111

5.2.1.3	Test purpose	112

5.2.1.4	Method of test	112

5.2.1.4.1	Initial conditions	112

5.2.1.4.2	Coding details	112

5.2.1.4.3	Procedure	113

5.2.1.5	Acceptance criteria	113

5.2.2	Access Control information handling for E-UTRAN/EPC	125

5.2.2.1	Definition and applicability	125

5.2.2.2	Conformance requirement	125

5.2.2.3	Test purpose	125

5.2.2.4	Method of test	126

5.2.2.4.1	Initial conditions	126

5.2.2.4.2	Coding details	126

5.2.2.4.3	Procedure	126

5.2.2.5	Acceptance criteria	126

5.2.3	Access Control information handling for NB-IoT	137

5.2.3.1	Definition and applicability	137

5.2.3.2	Conformance requirement	137

5.2.3.3	Test purpose	137

5.2.3.4	Method of test	138

5.2.3.4.1	Initial conditions	138

5.2.3.4.2	Coding details	138

5.2.3.4.3	Procedure	138

5.2.3.5	Acceptance criteria	138

5.3	Handling subscription identifier privacy for 5G	150

5.3.1	SUCI calculation by ME using null scheme	150

5.3.1.1	Definition and applicability	150

5.3.1.2	Conformance requirement	150

5.3.1.3	Test purpose	150

5.3.1.4	Method of test	151

5.3.1.4.1	Initial conditions	151

5.3.1.4.2	Procedure	152

5.3.1.5	Acceptance criteria	152

5.3.2	SUCI calculation by ME using Profile B	153

5.3.2.1	Definition and applicability	153

5.3.2.2	Conformance requirement	153

5.3.2.3	Test purpose	153

5.3.2.4	Method of test	153

5.3.2.4.1	Initial conditions	153

5.3.2.4.2	Procedure	154

5.3.2.5	Acceptance criteria	154

5.3.3	UE identification by SUCI during initial registration – SUCI calculation by USIM using profile B	154

5.3.3.1	Definition and applicability	154

5.3.3.2	Conformance requirement	154

5.3.3.3	Test purpose	154

5.3.3.4	Method of test	155

5.3.3.4.1	Initial conditions	155

5.3.3.4.2	Procedure	156

5.3.3.5	Acceptance criteria	156

5.3.4	UE identification by SUCI in response to IDENTITY REQUEST message	156

5.3.4.1	Definition and applicability	156

5.3.4.2	Conformance requirement	157

5.3.4.3	Test purpose	157

5.3.4.4	Method of test	157

5.3.4.4.1	Initial conditions	157

5.3.4.4.2	Procedure	158

5.3.4.5	Acceptance criteria	158

5.3.5	UE identification by SUCI in response to IDENTITY REQUEST message with T3519 timer expiry	159

5.3.5.1	Definition and applicability	159

5.3.5.2	Conformance requirement	159

5.3.5.3	Test purpose	159

5.3.5.4	Method of test	160

5.3.5.4.1	Initial conditions	160

5.3.5.4.2	Procedure	160

5.3.5.5	Acceptance criteria	161

5.3.6	UE identification by SUCI in response to IDENTITY REQUEST message and AUTHENTICATION REJECT	161

5.3.6.1	Definition and applicability	161

5.3.6.2	Conformance requirement	161

5.3.6.3	Test purpose	162

5.3.6.4	Method of test	162

5.3.6.4.1	Initial conditions	162

5.3.6.4.2	Procedure	162

5.3.6.5	Acceptance criteria	163

5.3.7	SUCI calculation by the ME using null scheme – missing parameters for subscription identifier privacy support by the USIM	163

5.3.7.1	Definition and applicability	163

5.3.7.2	Conformance requirement	163

5.3.7.3	Test purpose	164

5.3.7.4	Method of test	164

5.3.7.4.1	Initial conditions	164

5.3.7.4.2	Procedure	164

5.3.7.5	Acceptance criteria	164

5.3.8	UE identification by 5G-GUTI – Last Registered TAI stored on USIM	165

5.3.8.1	Definition and applicability	165

5.3.8.2	Conformance requirement	165

5.3.8.3	Test purpose	165

5.3.8.4	Method of test	165

5.3.8.4.2	Procedure	166

5.3.8.5	Acceptance criteria	166

5.3.9	UE identification by 5G-GUTI – Last Registered TAI stored by ME	166

5.3.9.1	Definition and applicability	166

5.3.9.3	Test purpose	166

5.3.9.4	Method of test	167

5.3.9.4.1	Initial conditions	167

5.3.9.4.2	Procedure	167

5.3.9.5	Acceptance criteria	167

5.3.10	UE identification after SUPI is changed	168

5.3.10.1	Definition and applicability	168

5.3.10.2	Conformance requirement	168

5.3.10.3	Test purpose	168

5.3.10.4	Method of test	168

5.3.10.4.1	Initial conditions	168

5.3.10.4.2	Procedure	168

5.3.10.5	Acceptance criteria	169

5.3.11	SUCI calculation by ME using Profile A	169

5.3.11.1	Definition and applicability	169

5.3.11.2	Conformance requirement	169

5.3.11.3	Test purpose	169

5.3.11.4	Method of test	169

5.3.11.4.1	Initial conditions	170

5.3.11.4.2	Procedure	170

5.3.11.5	Acceptance criteria	171

5.3.12	UE identification by SUCI during initial registration – SUCI calculation by USIM using profile A	171

5.3.12.1	Definition and applicability	171

5.3.12.2	Conformance requirement	171

5.3.12.3	Test purpose	171

5.3.12.4	Method of test	172

5.3.12.4.1	Initial conditions	172

5.3.12.4.2	Procedure	173

5.3.12.5	Acceptance criteria	173

5.3.13	SUCI calculation by ME using null scheme– no Protection Scheme Identifier provisioned in the USIM	173

5.3.13.1	Definition and applicability	173

5.3.13.2	Conformance requirement	173

5.3.13.3	Test purpose	174

5.3.13.4	Method of test	174

5.3.13.4.1	Initial conditions	174

5.3.13.4.2	Procedure	174

5.3.13.5	Acceptance criteria	174

5.3.14	SUCI calculation by ME using null scheme – no Home Network Public Key for supported protection scheme provisioned in the USIM	175

5.3.14.1	Definition and applicability	175

5.3.14.2	Conformance requirement	175

5.3.14.3	Test purpose	175

5.3.14.4	Method of test	175

5.3.14.4.1	Initial conditions	175

5.3.14.4.2	Procedure	176

5.3.14.5	Acceptance criteria	176

5.3.15	SUCI calculation by ME using null scheme with the E-UTRAN/EPC UICC	176

5.3.15.1	Definition and applicability	176

5.3.15.2	Conformance requirement	176

5.3.15.3	Test purpose	176

5.3.15.4	Method of test	177

5.3.15.4.1	Initial conditions	177

5.3.15.4.2	Procedure	177

5.3.15.5	Acceptance criteria	177

5.3.16	SUCI calculation by ME using the lower priority protection scheme when the higher priority protection scheme is not supported by the ME	178

5.3.16.1	Definition and applicability	178

5.3.16.2	Conformance requirement	178

5.3.16.3	Test purpose	178

5.3.16.4	Method of test	178

5.3.16.4.1	Initial conditions	178

5.3.16.4.2	Procedure	179

5.3.16.5	Acceptance criteria	180

5.3.17	SUCI calculation by ME using Profile B with compressed Home Network Public Key	180

5.3.17.1	Definition and applicability	180

5.3.17.2	Conformance requirement	180

5.3.17.3	Test purpose	181

5.3.17.4	Method of test	181

5.3.17.4.1	Initial conditions	181

5.3.17.4.2	Procedure	182

5.3.17.5	Acceptance criteria	182

5.4	Unified Access Control information handling for 5G-NR	183

5.4.1	Unified Access Control – Access identity 0, no access identities indicated by USIM	183

5.4.1.1	Definition and applicability	183

5.4.1.2	Conformance requirement	183

5.4.1.3	Test purpose	183

5.4.1.4	Method of test	184

5.4.1.4.1	Initial conditions	184

5.4.1.4.2	Procedure	184

5.4.1.4.3	Acceptance criteria	185

5.4.1A	Unified Access Control – Access identity 0, no access identities indicated by USIM, Access Category 8	185

5.4.1A.1	Definition and applicability	185

5.4.1A.2	Conformance requirement	186

5.4.1A.3	Test purpose	186

5.4.1A.4	Method of test	186

5.4.1A.4.1	Initial conditions	186

5.4.1.4.2	Procedure	188

5.4.1A.4.3	Acceptance criteria	188

5.4.2	Unified Access Control – Access Identity 1 – MPS indicated by USIM	189

5.4.2.1	Definition and applicability	189

5.4.2.2	Conformance requirement	189

5.4.2.3	Test purpose	189

5.4.2.4	Method of test	190

5.4.2.4.1	Initial conditions	190

5.4.2.4.2	Procedure	190

5.4.2.4.3	Acceptance criteria	191

5.4.2A	Unified Access Control – Access Identity 1 – MPS indicated by USIM, Access Category 8	192

5.4.2A.1	Definition and applicability	192

5.4.2A.2	Conformance requirement	192

5.4.2A.3	Test purpose	193

5.4.2A.4	Method of test	193

5.4.2A.4.1	Initial conditions	193

5.4.2A.4.2	Procedure	194

5.4.2A.4.3	Acceptance criteria	194

5.4.3	Unified Access Control – Access Identity 1 – no MPS indication by USIM and SUPI not changed	195

5.4.3.1	Definition and applicability	195

5.4.3.2	Conformance requirement	195

5.4.3.3	Test purpose	196

5.4.3.4	Method of test	196

5.4.3.4.1	Initial conditions	196

5.4.3.4.2	Procedure	198

5.4.3.4.3	Acceptance criteria	198

5.4.4	Unified Access Control – Access Identity 1 – no MPS indication by USIM and SUPI is changed	199

5.4.4.1	Definition and applicability	199

5.4.4.2	Conformance requirement	199

5.4.4.3	Test purpose	200

5.4.4.4	Method of test	200

5.4.4.4.1	Initial conditions	200

5.4.4.4.2	Procedure	201

5.4.4.4.3	Acceptance criteria	202

5.4.5	Unified Access Control – Access Identity 2 – MCS indicated by USIM	202

5.4.5.1	Definition and applicability	202

5.4.5.2	Conformance requirement	203

5.4.5.3	Test purpose	203

5.4.5.4	Method of test	203

5.4.5.4.1	Initial conditions	203

5.4.5.4.2	Procedure	204

5.4.5.4.3	Acceptance criteria	204

5.4.5A	Unified Access Control – Access Identity 2 – MCS indicated by USIM	205

5.4.5A.1	Definition and applicability	205

5.4.5A.2	Conformance requirement	206

5.4.5A.3	Test purpose	206

5.4.5A.4	Method of test	207

5.4.5A.4.1	Initial conditions	207

5.4.5A.4.2	Procedure	208

5.4.5A.4.3	Acceptance criteria	208

5.4.6	Unified Access Control – Access Identity 2 – no MCS indication by USIM and SUPI is not changed	209

5.4.6.1	Definition and applicability	209

5.4.6.2	Conformance requirement	209

5.4.6.3	Test purpose	210

5.4.6.4	Method of test	210

5.4.6.4.1	Initial conditions	210

5.4.6.4.2	Procedure	211

5.4.6.4.3	Acceptance criteria	212

5.4.7	Unified Access Control – Access Identity 2 – no MCS indication by USIM and SUPI is changed	212

5.4.7.1	Definition and applicability	212

5.4.7.2	Conformance requirement	213

5.4.7.3	Test purpose	214

5.4.7.4	Method of test	214

5.4.7.4.1	Initial conditions	214

5.4.7.4.2	Procedure	215

5.4.7.4.3	Acceptance criteria	216

5.4.8	Unified Access Control – Access Identities 11 and 15 indicated by USIM	216

5.4.8.1	Definition and applicability	216

5.4.8.2	Conformance requirement	217

5.4.8.3	Test purpose	217

5.4.8.4	Method of test	217

5.4.8.4.1	Initial conditions	217

5.4.8.4.2	Procedure	218

5.4.8.4.3	Acceptance criteria	218

5.4.8A	Unified Access Control – Access Identities 11 and 15 indicated by USIM, Access Category 8	219

5.4.8A.1	Definition and applicability	219

5.4.8A.2	Conformance requirement	220

5.4.8A.3	Test purpose	220

5.4.8A.4	Method of test	220

5.4.8A.4.1	Initial conditions	220

5.4.8A.4.2	Procedure	221

5.4.8.4.3	Acceptance criteria	222

5.4.9	Unified Access Control – Access Identities 12, 13 and 14 indicated by USIM	223

5.4.9.1	Definition and applicability	223

5.4.9.2	Conformance requirement	223

5.4.9.3	Test purpose	223

5.4.9.4	Method of test	224

5.4.9.4.1	Initial conditions	224

5.4.9.4.2	Procedure	224

5.4.9.4.3	Acceptance criteria	225

5.4.9A	Unified Access Control – Access Identities 12, 13 and 14 indicated by USIM, Access Category 8	226

5.4.9A.1	Definition and applicability	226

5.4.9A.2	Conformance requirement	226

5.4.9A.3	Test purpose	227

5.4.9A.4	Method of test	227

5.4.9A.4.1	Initial conditions	227

5.4.9A.4.2	Procedure	228

5.4.9A.4.3	Acceptance criteria	229

5.4.10	Unified Access Control – Operator-Defined Access Category	229

5.4.10.1	Definition and applicability	229

5.4.10.2	Conformance requirement	230

5.4.10.3	Test purpose	231

5.4.10.4	Method of test	231

5.4.10.4.1	Initial conditions	231

5.4.10.4.2	Procedure	233

5.4.10.5	Acceptance criteria	233

5.4.11	Unified Access Control – Operator-Defined Access Categories, no change in SUPI	234

5.4.11.1	Definition and applicability	234

5.4.11.2	Conformance requirement	235

5.4.11.3	Test purpose	236

5.4.11.4	Method of test	236

5.4.11.4.1	Initial conditions	236

5.4.11.4.2	Procedure	238

5.4.11.4.3	Acceptance criteria	239

5.4.12	Unified Access Control – Operator-Defined Access Categories, SUPI change	239

5.4.12.1	Definition and applicability	239

5.4.12.2	Conformance requirement	240

5.4.12.3	Test purpose	241

5.4.12.4	Method of test	241

5.4.12.4.1	Initial conditions	241

5.4.12.4.2	Procedure	243

5.4.12.4.3	Acceptance criteria	244

5.5	Handling of operator controlled features	245

5.5.1	Display of registered 5G PLMN name from USIM	245

5.5.1.1	Definition and applicability	245

5.5.1.2	Conformance requirement	245

5.5.1.3	Test purpose	245

5.5.1.4	Method of tests	245

5.5.1.4.1	Initial conditions	245

5.5.1.4.2	Procedure	247

5.5.1.4.3	Acceptance criteria	248

5.5.2	Display of registered 5G PLMN name from ME	248

5.5.2.1	Definition and applicability	248

5.5.2.2	Conformance requirement	248

5.5.2.3	Test purpose	248

5.5.2.4	Method of tests	248

5.5.2.4.1	Initial conditions	248

5.5.2.4.2	Procedure	250

5.5.2.4.3	Acceptance criteria	251

5.6	Handling subscription identifier privacy for 5G - SUPI type in NAI format	251

5.6.1	SUCI calculation by ME using null scheme	251

5.6.1.1	Definition and applicability	251

5.6.1.2	Conformance requirement	251

5.6.1.3	Test purpose	252

5.6.1.4	Method of test	252

5.6.1.4.1	Initial conditions	252

5.6.1.4.2	Procedure	253

5.6.1.5	Acceptance criteria	253

5.6.2	UE identification by SUCI during initial registration – SUCI calculation by USIM using profile A	254

5.6.2.1	Definition and applicability	254

5.6.2.2	Conformance requirement	254

5.6.2.3	Test purpose	254

5.6.2.4	Method of test	255

5.6.2.4.1	Initial conditions	255

5.6.2.4.2	Procedure	256

5.6.2.5	Acceptance criteria	256

5.6.3	UE identification by SUCI during initial registration – SUCI calculation by USIM using profile B	256

5.6.3.1	Definition and applicability	256

5.6.3.2	Conformance requirement	257

5.6.3.3	Test purpose	257

5.6.3.4	Method of test	257

5.6.3.4.1	Initial conditions	257

5.6.3.4.2	Procedure	259

5.6.3.5	Acceptance criteria	259

5.6.4	UE identification after SUPI is changed	259

5.6.4.1	Definition and applicability	259

5.6.4.2	Conformance requirement	260

5.6.4.3	Test purpose	260

5.6.4.4	Method of test	260

5.6.4.4.1	Initial conditions	260

5.6.4.4.2	Procedure	261

5.6.4.5	Acceptance criteria	261

5.6.5	UE identification by SUCI during initial registration – SUCI calculation by ME using profile A	261

5.6.5.1	Definition and applicability	261

5.6.5.2	Conformance requirement	262

5.6.5.3	Test purpose	262

5.6.5.4	Method of test	262

5.6.5.4.1	Initial conditions	262

5.6.5.4.2	Procedure	263

5.6.5.5	Acceptance criteria	264

5.6.6	UE identification by SUCI during initial registration – SUCI calculation by USIM using profile B	264

5.6.6.1	Definition and applicability	264

5.6.6.2	Conformance requirement	264

5.6.6.3	Test purpose	265

5.6.6.4	Method of test	265

5.6.6.4.1	Initial conditions	265

5.6.6.4.2	Procedure	266

5.6.6.5	Acceptance criteria	266

6	Security related Tests	267

6.1	PIN handling	267

6.1.1	Entry of PIN	267

6.1.1.1	Definition and applicability	267

6.1.1.2	Conformance requirement	267

6.1.1.3	Test purpose	267

6.1.1.4	Method of test	267

6.1.1.4.1	Initial conditions	267

6.1.1.4.2	Procedure	267

6.1.1.5	Acceptance criteria	267

6.1.2	Change of PIN	268

6.1.2.1	Definition and applicability	268

6.1.2.2	Conformance requirement	268

6.1.2.3	Test purpose	268

6.1.2.4	Method of test	268

6.1.2.4.1	Initial conditions	268

6.1.2.4.2	Procedure	268

6.1.2.5	Acceptance criteria	268

6.1.3	Unblock PIN	269

6.1.3.1	Definition and applicability	269

6.1.3.2	Conformance requirement	269

6.1.3.3	Test purpose	269

6.1.3.4	Method of test	269

6.1.3.4.1	Initial conditions	269

6.1.3.4.2	Procedure	269

6.1.3.5	Acceptance criteria	270

6.1.4	Entry of PIN2	270

6.1.4.1	Definition and applicability	270

6.1.4.2	Conformance requirement	270

6.1.4.3	Test purpose	270

6.1.4.4	Method of test	270

6.1.4.4.1	Initial conditions	270

6.1.4.4.2	Procedure	271

6.1.4.5	Acceptance criteria	271

6.1.5	Change of PIN2	271

6.1.5.1	Definition and applicability	271

6.1.5.2	Conformance requirement	271

6.1.5.3	Test purpose	271

6.1.5.4	Method of test	271

6.1.5.4.1	Initial conditions	271

6.1.5.4.2	Procedure	271

6.1.5.5	Acceptance criteria	272

6.1.6	Unblock PIN2	272

6.1.6.1	Definition and applicability	272

6.1.6.2	Conformance requirement	272

6.1.6.3	Test purpose	272

6.1.6.4	Method of test	272

6.1.6.4.1	Initial conditions	272

6.1.6.4.2	Procedure	272

6.1.6.5	Acceptance criterias	273

6.1.7	Replacement of PIN	274

6.1.7.1	Definition and applicability	274

6.1.7.2	Conformance requirement	274

6.1.7.3	Test purpose	274

6.1.7.4	Method of test	274

6.1.7.4.1	Initial conditions	274

6.1.7.4.2	Procedure	274

6.1.7.5	Acceptance criteria	274

6.1.8	Change of Universal PIN	275

6.1.8.1	Definition and applicability	275

6.1.8.2	Conformance requirement	275

6.1.8.3	Test purpose	275

6.1.8.4	Method of test	275

6.1.8.4.1	Initial conditions	275

6.1.8.4.2	Procedure	275

6.1.8.5	Acceptance criteria	276

6.1.9	Unblock Universal PIN	276

6.1.9.1	Definition and applicability	276

6.1.9.2	Conformance requirement	276

6.1.9.3	Test purpose	276

6.1.9.4	Method of test	276

6.1.9.4.1	Initial conditions	276

6.1.9.4.2	Procedure	276

6.1.9.5	Acceptance criteria	277

6.1.10	Entry of PIN on multi-verification capable UICCs	277

6.1.10.1	Definition and applicability	277

6.1.10.2	Conformance requirement	277

6.1.10.4	Method of test	277

6.1.10.4.1	Initial conditions	277

6.1.10.4.2	Procedure	278

6.1.10.5	Acceptance criteria	278

6.1.11	Change of PIN on multi-verification capable UICCs	278

6.1.11.1	Definition and applicability	278

6.1.11.2	Conformance requirement	278

6.1.11.3	Test purpose	279

6.1.11.4	Method of test	279

6.1.11.4.1	Initial conditions	279

6.1.11.4.2	Procedure	279

6.1.11.5	Acceptance criteria	280

6.1.12	Unblock PIN on multi-verification capable UICCs	280

6.1.12.1	Definition and applicability	280

6.1.12.2	Conformance requirement	280

6.1.12.3	Test purpose	280

6.1.12.4	Method of test	280

6.1.12.4.1	Initial conditions	280

6.1.12.4.2	Procedure	281

6.1.12.5	Acceptance criteria	282

6.1.13	Entry of PIN2 on multi-verification capable UICCs	282

6.1.13.1	Definition and applicability	282

6.1.13.2	Conformance requirement	282

6.1.13.3	Test purpose	282

6.1.13.4	Method of test	282

6.1.13.4.1	Initial conditions	282

6.1.13.4.2	Procedure	283

6.1.13.5	Acceptance criteria	283

6.1.14	Change of PIN2 on multi-verification capable UICCs	284

6.1.14.1	Definition and applicability	284

6.1.14.2	Conformance requirement	284

6.1.14.3	Test purpose	284

6.1.14.4	Method of test	284

6.1.14.4.1	Initial conditions	284

6.1.14.4.2	Procedure	285

6.1.14.5	Acceptance criteria	285

6.1.15	Unblock PIN2 on multi-verification capable UICCs	285

6.1.15.1	Definition and applicability	285

6.1.15.2	Conformance requirement	285

6.1.15.3	Test purpose	286

6.1.15.4	Method of test	286

6.1.15.4.1	Initial conditions	286

6.1.15.4.2	Procedure	287

6.1.15.5	Acceptance criterias	287

6.1.16	Replacement of PIN with key reference "07"	288

6.1.16.1	Definition and applicability	288

6.1.16.2	Conformance requirement	288

6.1.16.3	Test purpose	288

6.1.16.4	Method of test	288

6.1.16.4.1	Initial conditions	288

6.1.16.4.2	Procedure	289

6.1.16.5	Acceptance criteria	289

6.2	Fixed Dialling Numbers (FDN) handling	290

6.2.1	Terminal and USIM with FDN enabled, EFADN readable and updateable	290

6.2.1.1	Definition and applicability	290

6.2.1.2	Conformance requirement	290

6.2.1.3	Test purpose	290

6.2.1.4	Method of test	291

6.2.1.4.1	Initial conditions	291

6.2.1.4.2	Procedure	291

6.2.1.5	Acceptance criteria	291

6.2.2	Terminal and USIM with FDN disabled	292

6.2.2.1	Definition and applicability	292

6.2.2.2	Conformance requirement	292

6.2.2.3	Test purpose	292

6.2.2.4	Method of test	292

6.2.2.4.1	Initial conditions	292

6.2.2.4.2	Procedure	293

6.2.2.5	Acceptance criteria	293

6.2.3	Enabling, disabling and updating of FDN	293

6.2.3.1	Definition and applicability	293

6.2.3.2	Conformance requirement	293

6.2.3.3	Test purpose	293

6.2.3.4	Method of test	294

6.2.3.4.1	Initial conditions	294

6.2.3.4.2	Procedure	294

6.2.3.5	Acceptance criteria	294

6.2.4	Terminal and USIM with FDN enabled, EFADN readable and updateable (Rel-4 and onwards)	294

6.2.4.1	Definition and applicability	294

6.2.4.2	Conformance requirement	294

6.2.4.3	Test purpose	295

6.2.4.4	Method of test	295

6.2.4.4.1	Initial conditions	295

6.2.4.4.2	Procedure	295

6.2.4.5	Acceptance criteria	296

6.3	Void	296

6.4	Advice of charge (AoC) handling	296

6.4.1	AoC not supported by USIM	296

6.4.1.1	Definition and applicability	296

6.4.1.2	Conformance requirement	296

6.4.1.3	Test purpose	297

6.4.1.4	Method of test	297

6.4.1.4.1	Initial conditions	297

6.4.1.4.2	Procedure	297

6.4.1.5	Acceptance criteria	298

6.4.2	Maximum frequency of ACM updating	298

6.4.2.1	Definition and applicability	298

6.4.2.2	Conformance requirement	298

6.4.2.3	Test purpose	298

6.4.2.4	Method of test	298

6.4.2.4.1	Initial conditions	298

6.4.2.4.2	Procedure	299

6.4.2.5	Acceptance criteria	301

6.4.3	Call terminated when ACM greater than ACMmax	301

6.4.3.1	Definition and applicability	301

6.4.3.2	Conformance requirement	301

6.4.3.3	Test purpose	301

6.4.3.4	Method of test	302

6.4.3.4.1	Initial conditions	302

6.4.3.4.2	Procedure	302

6.4.3.5	Acceptance criteria	304

6.4.4	Response codes of increase command of ACM	304

6.4.4.1	Definition and applicability	304

6.4.4.2	Conformance requirement	305

6.4.4.3	Test purpose	305

6.4.4.4	Method of test	305

6.4.4.4.1	Initial conditions	305

6.4.4.4.2	Procedure	306

6.4.4.5	Acceptance criteria	308

7	PLMN related tests	309

7.1	FPLMN handling	309

7.1.1	Adding FPLMN to the Forbidden PLMN list	309

7.1.1.1	Definition and applicability	309

7.1.1.2	Conformance requirement	309

7.1.1.3	Test purpose	311

7.1.1.4	Method of test	311

7.1.1.4.1	Initial conditions	311

7.1.1.4.2	Procedure	312

7.1.1.5	Acceptance criteria	315

7.1.2	UE updating forbidden PLMNs	317

7.1.2.1	Definition and applicability	317

7.1.2.2	Conformance requirement	318

7.1.2.3	Test purpose	318

7.1.2.4	Method of test	318

7.1.2.4.1	Initial conditions	318

7.1.2.4.2	Procedure	319

7.1.2.5	Acceptance criteria	320

7.1.3	UE deleting forbidden PLMNs	320

7.1.3.1	Definition and applicability	320

7.1.3.2	Conformance requirement	321

7.1.3.3	Test purpose	321

7.1.3.4	Method of test	322

7.1.3.4.1	Initial conditions	322

7.1.3.4.2	Procedure	322

7.1.3.5	Acceptance criteria	323

7.1.4	Adding FPLMN to the forbidden PLMN list when accessing E-UTRAN	324

7.1.4.1	Definition and applicability	324

7.1.4.2	Conformance requirement	325

7.1.4.3	Test purpose	325

7.1.4.4	Method of test	325

7.1.4.4.1	Initial conditions	325

7.1.4.4.2	Procedure	326

7.1.4.5	Acceptance criteria	327

7.1.5	UE updating forbidden PLMNs when accessing E-UTRAN	327

7.1.5.1	Definition and applicability	327

7.1.5.2	Conformance requirement	327

7.1.5.3	Test purpose	328

7.1.5.4	Method of test	328

7.1.5.4.1	Initial conditions	328

7.1.5.4.2	Procedure	328

7.1.5.5	Acceptance criteria	328

7.1.6	UE deleting forbidden PLMNs when accessing E-UTRAN	329

7.1.6.1	Definition and applicability	329

7.1.6.2	Conformance requirement	329

7.1.6.3	Test purpose	329

7.1.6.4	Method of test	330

7.1.6.4.1	Initial conditions	330

7.1.6.4.2	Procedure	330

7.1.6.5	Acceptance criteria	330

7.1.7	Updating the Forbidden PLMN list  after receiving non-integrity protected reject message – UTRAN	331

7.1.7.1	Definition and applicability	331

7.1.7.2	Conformance requirement	331

7.1.7.3	Test purpose	332

7.1.7.4	Method of test	332

7.1.7.4.1	Initial conditions	332

7.1.7.4.2	Procedure	332

7.1.7.5	Acceptance criteria	333

7.1.8	Updating the Forbidden PLMN list after receiving non-integrity protected reject message – E-UTRAN	333

7.1.8.1	Definition and applicability	333

7.1.8.2	Conformance requirement	333

7.1.8.3	Test purpose	334

7.1.8.4	Method of test	334

7.1.8.4.1	Initial conditions	334

7.1.8.4.2	Procedure	334

7.1.8.5	Acceptance criteria	335

7.1.9	Adding FPLMN to the forbidden PLMN list when accessing satellite NG-RAN	335

7.1.9.1	Definition and applicability	335

7.1.9.2	Conformance requirement	336

7.1.9.3	Test purpose	336

7.1.9.4	Method of test	336

7.1.9.4.1	Initial conditions	336

7.1.9.4.2	Procedure	337

7.1.9.5	Acceptance criteria	338

7.1.10	UE updating forbidden PLMNs when accessing satellite NG-RAN	338

7.1.10.1	Definition and applicability	338

7.1.10.2	Conformance requirement	338

7.1.10.3	Test purpose	339

7.1.10.4	Method of test	339

7.1.10.4.1	Initial conditions	339

7.1.10.4.2	Procedure	339

7.1.10.5	Acceptance criteria	339

7.1.11	UE deleting forbidden PLMNs when accessing satellite NG-RAN in manual mode	340

7.1.11.1	Definition and applicability	340

7.1.11.2	Conformance requirement	340

7.1.11.3	Test purpose	340

7.1.11.4	Method of test	340

7.1.11.4.1	Initial conditions	340

7.1.11.4.2	Procedure	341

7.1.11.5	Acceptance criteria	341

7.2	User controlled PLMN selector handling	342

7.2.1	UE updating the User controlled PLMN selector list	342

7.2.1.1	Definition and applicability	342

7.2.1.2	Conformance requirement	342

7.2.1.3	Test purpose	342

7.2.1.4	Method of test	342

7.2.1.4.1	Initial conditions	342

7.2.1.4.2	Procedure	343

7.2.1.5	Acceptance criteria	343

7.2.2	UE recognising the priority order of the User controlled PLMN selector list with the same access technology.	343

7.2.2.1	Definition and applicability	343

7.2.2.2	Conformance requirement	344

7.2.2.3	Test purpose	344

7.2.2.4	Method of test	344

7.2.2.4.1	Initial conditions	344

7.2.2.4.2	Procedure	345

7.2.2.5	Acceptance criteria	346

7.2.3	UE recognising the priority order of the User controlled PLMN selector list using an ACT preference.	347

7.2.3.1	Definition and applicability	347

7.2.3.2	Conformance requirement	347

7.2.3.3	Test purpose	347

7.2.3.4	Method of test	347

7.2.3.4.1	Initial conditions	347

7.2.3.5	Acceptance criteria	348

7.2.4	Void	349

7.2.5	UE updating the User controlled PLMN selector list for E-UTRAN	349

7.2.5.1	Definition and applicability	349

7.2.5.2	Conformance requirement	349

7.2.5.3	Test purpose	349

7.2.5.4	Method of test	349

7.2.5.4.1	Initial conditions	349

7.2.5.4.2	Procedure	349

7.2.5.5	Acceptance criteria	350

7.2.6	UE recognising the priority order of the User controlled PLMN selector list using an ACT preference- UTRAN/E-UTRAN	350

7.2.6.1	Definition and applicability	350

7.2.6.2	Conformance requirement	351

7.2.6.3	Test purpose	351

7.2.6.4	Method of test	351

7.2.6.4.1	Initial conditions	351

7.2.6.4.2	Procedure	351

7.2.6.5	Acceptance criteria	351

7.2.7	UE recognising the priority order of the User controlled PLMN selector list using an ACT preference- GSM/E-UTRAN	352

7.2.7.1	Definition and applicability	352

7.2.7.2	Conformance requirement	352

7.2.7.3	Test purpose	352

7.2.7.4	Method of test	352

7.2.7.4.1	Initial conditions	352

7.2.7.4.2	Procedure	353

7.2.7.5	Acceptance criteria	353

7.2.8	UE recognising the priority order of the User controlled PLMN selector list with the same access technology – E-UTRAN in NB-S1 mode	354

7.2.8.1	Definition and applicability	354

7.2.8.2	Conformance requirement	354

7.2.8.3	Test purpose	354

7.2.8.4	Method of test	354

7.2.8.4.1	Initial conditions	354

7.2.8.2	Procedure	355

7.2.8.5	Acceptance criteria	355

7.2.9	UE recognising the priority order of the User controlled PLMN selector list using the ACT preference – E-UTRAN in WB-S1/E-UTRAN in NB-S1	356

7.2.9.1	Definition and applicability	356

7.2.9.2	Conformance requirement	356

7.2.9.3	Test purpose	356

7.2.9.4	Method of test	356

7.2.9.4.1	Initial conditions	356

7.2.9.4.2	Procedure	357

7.2.9.5	Acceptance criteria	357

7.2.10	UE updating the User controlled PLMN selector list for satellite-NG-RAN	358

7.2.10.1	Definition and applicability	358

7.2.10.2	Conformance requirement	358

7.2.10.3	Test purpose	358

7.2.10.4	Method of test	358

7.2.10.4.1	Initial conditions	358

7.2.10.4.2	Procedure	358

7.2.10.5	Acceptance criteria	358

7.2.11	UE recognising the priority order of the User controlled PLMN selector list using an ACT preference GSM/satellite NG-RAN	359

7.2.11.1	Definition and applicability	359

7.2.11.2	Conformance requirement	359

7.2.11.3	Test purpose	359

7.2.11.4	Method of test	360

7.2.11.4.1	Initial conditions	360

7.2.11.4.2	Procedure	360

7.2.11.5	Acceptance criteria	360

7.2.12	UE recognising the priority order of the User controlled PLMN selector list using an ACT preference UTRAN/satellite NG-RAN	361

7.2.12.1	Definition and applicability	361

7.2.12.2	Conformance requirement	361

7.2.12.3	Test purpose	361

7.2.12.4	Method of test	361

7.2.12.4.1	Initial conditions	361

7.2.12.4.2	Procedure	361

7.2.12.5	Acceptance criteria	362

7.2.13	UE recognising the priority order of the User controlled PLMN selector list using an ACT preference E-UTRAN/satellite NG-RAN	362

7.2.13.1	Definition and applicability	362

7.2.13.2	Conformance requirement	362

7.2.13.3	Test purpose	363

7.2.13.4	Method of test	363

7.2.13.4.1	Initial conditions	363

7.2.13.4.2	Procedure	363

7.2.13.5	Acceptance criteria	363

7.2.14	UE recognising the priority order of the User controlled PLMN selector list using an ACT preference NG-RAN/satellite NG-RAN	364

7.2.14.1	Definition and applicability	364

7.2.14.2	Conformance requirement	364

7.2.14.3	Test purpose	364

7.2.14.4	Method of test	364

7.2.14.4.1	Initial conditions	364

7.2.14.4.2	Procedure	364

7.2.14.5	Acceptance criteria	365

7.3	Operator controlled PLMN selector handling	365

7.3.1	UE recognising the priority order of the Operator controlled PLMN selector list.	365

7.3.1.1	Definition and applicability	365

7.3.1.2	Conformance requirement	366

7.3.1.3	Test purpose	366

7.3.1.4	Method of test	366

7.3.1.4.1	Initial conditions	366

7.3.1.4.2	Procedure	367

7.3.1.5	Acceptance criteria	368

7.3.2	UE recognising the priority order of the User controlled PLMN selector over the Operator controlled PLMN selector list.	369

7.3.2.1	Definition and applicability	369

7.3.2.2	Conformance requirement	369

7.3.2.3	Test purpose	369

7.3.2.4	Method of test	370

7.3.2.4.1	Initial conditions	370

7.3.2.4.2	Procedure	371

7.3.2.5	Acceptance criteria	372

7.3.3	UE recognising the priority order of the Operator controlled PLMN selector list when accessing E-UTRAN	373

7.3.3.1	Definition and applicability	373

7.3.3.2	Conformance requirement	373

7.3.3.3	Test purpose	373

7.3.3.4	Method of test	373

7.3.3.4.1	Initial conditions	373

7.3.3.4.2	Procedure	375

7.3.3.5	Acceptance criteria	375

7.3.4	UE recognising the priority order of the User controlled PLMN selector over the Operator controlled PLMN selector list – E-UTRAN	376

7.3.4.1	Definition and applicability	376

7.3.4.2	Conformance requirement	376

7.3.4.3	Test purpose	376

7.3.4.4	Method of test	376

7.3.4.4.1	Initial conditions	376

7.3.4.4.2	Procedure	377

7.3.4.5	Acceptance criteria	377

7.4	Higher priority PLMN search handling	389

7.4.1	UE recognising the search period of the Higher priority PLMN	389

7.4.1.1	Definition and applicability	389

7.4.1.2	Conformance requirement	389

7.4.1.3	Test purpose	390

7.4.1.4	Method of test	390

7.4.1.4.1	Initial conditions	390

7.4.1.4.2	Procedure	390

7.4.1.5	Acceptance criteria	393

7.4.2	GSM/UmTS dual mode UEs recognising the search period of the Higher priority PLMN	394

7.4.2.1	Definition and applicability	394

7.4.2.2	Conformance requirement	394

7.4.2.3	Test purpose	394

7.4.2.4	Method of test	394

7.4.2.4.1	Initial conditions	394

7.4.2.4.2	Procedure	395

7.4.2.5	Acceptance criteria	397

7.4.3	UE recognising the search period of the Higher priority PLMN – E-UTRAN	397

7.4.3.1	Definition and applicability	397

7.4.3.2	Conformance requirement	398

7.4.3.3	Test purpose	398

7.4.3.4	Method of test	398

7.4.3.4.1	Initial conditions	398

7.4.3.4.2	Procedure	399

7.4.3.5	Acceptance criteria	399

7.4.4	E-UTRAn/EPC capable UEs recognising the search period of the Higher priority PLMN – GSM/E-UTRAN	400

7.4.4.1	Definition and applicability	400

7.4.4.2	Conformance requirement	400

7.4.4.3	Test purpose	400

7.4.4.4	Method of test	400

7.4.4.4.1	Initial conditions	400

7.4.4.4.2	Procedure	401

7.4.4.5	Acceptance criteria	402

7.4.5	E-UTRAn/EPC capable UEs recognising the search period of the Higher priority PLMN – UTRAN/E-UTRAN	402

7.4.5.1	Definition and applicability	402

7.4.5.2	Conformance requirement	402

7.4.5.3	Test purpose	402

7.4.5.4	Method of test	403

7.4.5.4.1	Initial conditions	403

7.4.5.4.2	Procedure	404

7.4.5.5	Acceptance criteria	404

7.5	Void	405

8	Subscription independent tests	405

8.1	Phone book procedures	405

8.1.1	Recognition of a previously changed phonebook	405

8.1.1.1	Definition and applicability	405

8.1.1.2	Conformance requirement	405

8.1.1.3	Test purpose	405

8.1.1.4	Method of test	405

8.1.1.4.1	Initial conditions	405

8.1.1.4.2	Procedure	406

8.1.1.5	Acceptance criteria	406

8.1.2	Update of the Phonebook Synchronisation Counter (PSC)	407

8.1.2.1	Definition and applicability	407

8.1.2.2	Conformance requirement	407

8.1.2.3	Test purpose	407

8.1.2.4	Method of test	407

8.1.2.4.1	Initial conditions	407

8.1.2.4.2	Procedure	408

8.1.2.5	Acceptance criteria	408

8.1.3	Phonebook content handling	409

8.1.3.1	Handling of BCD number/ SSC content extension	409

8.1.3.1.1	Definition and applicability	409

8.1.3.1.2	Conformance requirement	409

8.1.3.1.3	Test purpose	409

8.1.3.1.4	Method of test	409

8.1.3.1.5	Acceptance criteria	412

8.1.4	Phonebook selection	413

8.1.4.1	Definition and applicability	413

8.1.4.2	Conformance requirement	413

8.1.4.3	Test purpose	413

8.1.4.4	Method of test	413

8.1.4.4.1	Initial conditions	413

8.1.4.4.2	Procedure	416

8.1.4.5	Acceptance criteria	416

8.1.5	Local Phonebook handling	417

8.1.5.1	Definition and applicability	417

8.1.5.2	Conformance requirement	417

8.1.5.3	Test purpose	417

8.1.5.4	Method of test	417

8.1.5.4.1	Initial conditions	417

8.1.5.4.2	Procedure	418

8.1.5.5	Acceptance criteria	418

8.2	Short message handling report	419

8.2.1	Correct storage of a SM on the USIM	419

8.2.1.1	Definition and applicability	419

8.2.1.2	Conformance requirement	419

8.2.1.3	Test purpose	419

8.2.1.4	Method of test	419

8.2.1.4.1	Initial conditions	419

8.2.1.4.2	Procedure	420

8.2.1.5	Acceptance criteria	420

8.2.2	Correct reading of a SM on the USIM	420

8.2.2.1	Definition and applicability	420

8.2.2.2	Conformance requirement	420

8.2.2.3	Test purpose	421

8.2.2.4	Method of test	421

8.2.2.4.1	Initial conditions	421

8.2.2.4.2	Procedure	422

8.2.2.5	Acceptance criteria	422

8.2.3	SM memory capacity exceeded handling	422

8.2.3.1	Definition and applicability	422

8.2.3.2	Conformance requirement	422

8.2.3.3	Test purpose	423

8.2.3.4	Method of test	423

8.2.3.4.1	Initial conditions	423

8.2.3.4.2	Procedure	425

8.2.3.5	Acceptance criteria	425

8.2.4	Correct storage of an SM on the UICC	427

8.2.4.1	Definition and applicability	427

8.2.4.2	Conformance requirement	427

8.2.4.3	Test purpose	427

8.2.4.4	Method of test	427

8.2.4.4.1	Initial conditions	427

8.2.4.4.2	Procedure	429

8.2.4.5	Acceptance criteria	430

8.2.5	Correct reading of a SM on the USIM if USIM and ISIM are present	431

8.2.5.1	Definition and applicability	431

8.2.5.2	Conformance requirement	431

8.2.5.3	Test purpose	431

8.2.5.4	Method of test	432

8.2.5.4.1	Initial conditions	432

8.2.5.4.2	Procedure	433

8.2.5.5	Acceptance criteria	433

8.2.6	Correct reading of a SM on the ISIM if USIM and ISIM are present	433

8.2.6.1	Definition and applicability	433

8.2.6.2	Conformance requirement	433

8.2.6.3	Test purpose	434

8.2.6.4	Method of test	434

8.2.6.4.1	Initial conditions	434

8.2.6.4.2	Procedure	435

8.2.6.5	Acceptance criteria	435

8.2.7	Correct storage of an SM on the UICC	435

8.2.7.1	Definition and applicability	435

8.2.7.2	Conformance requirement	436

8.2.7.3	Test purpose	436

8.2.7.4	Method of test	436

8.2.7.4.1	Initial conditions	436

8.2.7.4.2	Procedure	436

8.2.7.5	Acceptance criteria	436

8.3	MMS related tests	436

8.3.1	UE recognising the priority order of MMS Issuer Connectivity Parameters	436

8.3.1.1	Definition and applicability	436

8.3.1.2	Conformance requirement	437

8.3.1.3	Test purpose	437

8.3.1.4	Method of test	437

8.3.1.4.1	Initial conditions	437

8.3.1.4.2	Procedure	442

8.3.1.5	Acceptance criteria	442

8.3.2	UE recognising the priority order of MMS User Connectivity Parameters	442

8.3.2.1	Definition and applicability	442

8.3.2.2	Conformance requirement	442

8.3.2.3	Test purpose	443

8.3.2.4	Method of test	443

8.3.2.4.1	Initial conditions	443

8.3.2.4.2	Procedure	448

8.3.2.5	Acceptance criteria	448

8.3.3	UE recognising the priority order of MMS Issuer Connectivity Parameters over the MMS User Connectivity Parameters	448

8.3.3.1	Definition and applicability	448

8.3.3.2	Conformance requirement	449

8.3.3.3	Test purpose	449

8.3.3.4	Method of test	449

8.3.3.4.1	Initial conditions	449

8.3.3.4.2	Procedure	455

8.3.3.5	Acceptance criteria	455

8.3.4	Usage of MMS notification	455

8.3.4.1	Definition and applicability	455

8.3.4.2	Conformance requirement	455

8.3.4.3	Test purpose	456

8.3.4.4	Method of test	456

8.3.4.4.1	Initial conditions	456

8.3.4.4.2	Procedure	460

8.3.4.5	Acceptance criteria	460

8.4	UICC presence detection	461

8.4.1	Definition and applicability	461

8.4.2	Conformance requirement	461

8.4.3	Test purpose	461

8.4.4	Method of test	462

8.4.4.1	Initial conditions	462

8.4.4.2	Procedure	462

8.4.5	Acceptance criteria	462

8.5	UICC presence detection when connected to E-UTRAN/EPC	462

8.5.1	Definition and applicability	462

8.5.2	Conformance requirement	462

8.5.3	Test purpose	463

8.5.4	Method of test	463

8.5.4.1	Initial conditions	463

8.5.4.2	Procedure	463

8.5.5	Acceptance criteria	463

9	USIM service handling	451

9.1	Access Point Name Control List handling	451

9.1.1	Access Point Name Control List handling for terminals supporting ACL	451

9.1.1.1	Definition and applicability	451

9.1.1.2	Conformance requirement	451

9.1.1.3	Test purpose	451

9.1.1.4	Method of test	451

9.1.1.4.1	Initial conditions	451

9.1.1.4.2	Procedure	452

9.1.1.5	Acceptance criteria	452

9.1.2	Network provided APN handling for terminals supporting ACL	453

9.1.2.1	Definition and applicability	453

9.1.2.2	Conformance requirement	453

9.1.2.3	Test purpose	453

9.1.2.4	Method of test	453

9.1.2.4.1	Initial conditions	453

9.1.2.4.2	Procedure	454

9.1.2.5	Acceptance criteria	454

9.1.3	Access Point Name Control List handling for terminals not supporting ACL	455

9.1.3.1	Definition and applicability	455

9.1.3.2	Conformance requirement	455

9.1.3.3	Test purpose	455

9.1.3.4	Method of test	455

9.1.3.4.1	Initial conditions	455

9.1.3.4.2	Procedure	456

9.1.3.5	Acceptance criteria	456

9.1.4	Access Point Name Control List handling for terminals supporting ACL connected to E-UTRAN/EPC	456

9.1.4.1	Definition and applicability	456

9.1.4.2	Conformance requirement	456

9.1.4.3	Test purpose	456

9.1.4.4	Method of test	456

9.1.4.4.1	Initial conditions	456

9.1.4.4.2	Procedure	457

9.1.4.5	Acceptance criteria	458

9.1.5	Void9.1.6 Void	458

9.2	Service Dialling Numbers handling	458

9.2.1	Definition and applicability	458

9.2.2	Conformance requirement	458

9.2.3	Test purpose	458

9.2.4	Method of test	459

9.2.4.1	Initial conditions	459

9.2.4.2	Procedure	461

9.2.5	Acceptance criteria	461

10	CSG list handling	462

10.1	CSG list handling for E-UTRA	462

10.1.1	Automatic CSG selection in E-UTRA with CSG list on USIM, success	462

10.1.1.1	Definition and applicability	462

10.1.1.2	Conformance requirement	462

10.1.1.3	Test purpose	462

10.1.1.4	Method of test	462

10.1.1.4.1	Initial conditions	462

10.1.1.4.2	Procedure	463

10.1.1.5	Acceptance criteria	463

10.1.2	Automatic CSG selection in E-UTRA with CSG list on USIM, removal of CSG ID from the USIM	464

10.1.2.1	Definition and applicability	464

10.1.2.2	Conformance requirement	464

10.1.2.3	Test purpose	464

10.1.2.4	Method of test	464

10.1.2.4.1	Initial conditions	464

10.1.2.4.2	Procedure	465

10.1.2.5	Acceptance criteria	465

10.1.3	Manual CSG selection in E-UTRA with CSG list on USIM, success	466

10.1.3.1	Definition and applicability	466

10.1.3.2	Conformance requirement	466

10.1.3.3	Test purpose	466

10.1.3.4	Method of test	466

10.1.3.4.1	Initial conditions	466

10.1.3.4.2	Procedure	467

10.1.3.5	Acceptance criteria	467

10.1.4	Manual CSG selection in E-UTRA with CSG list on USIM, rejected	469

10.1.4.1	Definition and applicability	469

10.1.4.2	Conformance requirement	469

10.1.4.3	Test purpose	469

10.1.4.4	Method of test	469

10.1.4.4.1	Initial conditions	469

10.1.4.4.2	Procedure	470

10.1.4.5	Acceptance criteria	470

10.1.5	CSG selection in E-UTRA with no CSG list on USIM, no IMSI change	471

10.1.5.1	Definition and applicability	471

10.1.5.2	Conformance requirement	471

10.1.5.3	Test purpose	471

10.1.5.4	Method of test	472

10.1.5.4.1	Initial conditions	472

10.1.5.4.2	Procedure	472

10.1.5.5	Acceptance criteria	473

10.1.6	CSG selection in E-UTRA with no CSG list on USIM, with IMSI change	474

10.1.6.1	Definition and applicability	474

10.1.6.2	Conformance requirement	474

10.1.6.3	Test purpose	474

10.1.6.4	Method of test	474

10.1.6.4.1	Initial conditions	474

10.1.6.4.2	Procedure	475

10.1.6.5	Acceptance criteria	476

10.1.7	Manual CSG selection without display restrictions in E-UTRA with ACSG list and OCSG list on USIM	476

10.1.7.1	Definition and applicability	476

10.1.7.2	Conformance requirement	476

10.1.7.3	Test purpose	477

10.1.7.4	Method of test	477

10.1.7.4.1	Initial conditions	477

10.1.7.4.2	Procedure	477

10.1.7.5	Acceptance criteria	478

10.1.8	Manual CSG selection with display restrictions in E-UTRA with ACSG list and OCSG list on USIM	479

10.1.8.1	Definition and applicability	479

10.1.8.2	Conformance requirement	479

10.1.8.3	Test purpose	480

10.1.8.4	Method of test	480

10.1.8.4.1	Initial conditions	480

10.1.8.4.2	Procedure	481

10.1.8.5	Acceptance criteria	481

10.2	CSG list handling for UTRA	483

10.2.1	Manual CSG selection without display restrictions in UTRA with ACSG list and OCSG list on USIM	483

10.2.1.1	Definition and applicability	483

10.2.1.2	Conformance requirement	483

10.2.1.3	Test purpose	483

10.2.1.4	Method of test	483

10.2.1.4.1	Initial conditions	483

10.2.1.4.2	Procedure	484

10.2.1.5	Acceptance criteria	484

10.2.2	Manual CSG selection with display restrictions in UTRA with ACSG list and OCSG list on USIM	486

10.2.2.1	Definition and applicability	486

10.2.2.2	Conformance requirement	486

10.2.2.3	Test purpose	486

10.2.2.4	Method of test	486

10.2.2.4.1	Initial conditions	486

10.2.2.4.2	Procedure	487

10.2.2.5	Acceptance criteria	488

10.2.3	Manual CSG selection in UTRA with CSG list on USIM, success	489

10.2.3.1	Definition and applicability	489

10.2.3.2	Conformance requirement	489

10.2.3.3	Test purpose	490

10.2.3.4	Method of test	490

10.2.3.4.1	Initial conditions	490

10.2.3.4.2	Procedure	490

10.2.3.5	Acceptance criteria	491

11	NAS security context parameter handling	492

11.1	NAS security context parameter handling when service "EMM Information" is available	492

11.1.1	Definition and applicability	492

11.1.2	Conformance requirement	492

11.1.3	Test purpose	493

11.1.4	Method of test	493

11.1.4.1	Initial conditions	493

11.1.4.2	Procedure	493

11.1.5	Acceptance criteria	494

11.2	NAS security context parameter handling when service "EMM Information" is not available, no IMSI change	494

11.2.1	Definition and applicability	494

11.2.2	Conformance requirement	494

11.2.3	Test purpose	495

11.2.4	Method of test	495

11.2.4.1	Initial conditions	495

11.2.4.2	Procedure	495

11.2.5	Acceptance criteria	496

11.3	NAS security context parameter handling when service "EMM Information" is not available, IMSI changed	496

11.3.1	Definition and applicability	496

11.3.2	Conformance requirement	496

11.3.3	Test purpose	497

11.3.4	Method of test	497

11.3.4.1	Initial conditions	497

11.3.4.2	Procedure	497

11.3.5	Acceptance criteria	498

11.4	EPS NAS Security Context Storage	498

11.4.1	Definition and applicability	498

11.4.2	Conformance requirement	498

11.4.3	Test purpose	498

11.4.4	Method of test	499

11.4.4.1	Initial conditions	499

11.4.4.2	Procedure	499

11.4.5	Acceptance criteria	500

12	Non Access Stratum (NAS) Configuration parameter handling	500

12.1	EFNASCONFIG – NAS signaling priority handling	500

12.1.1	Definition and applicability	500

12.1.2	Void	500

12.1.3	Void	500

12.1.4	Void	501

12.1.5	Void	501

12.2	EFNASCONFIG – NMO I Network Mode of Operation I handling	501

12.2.1	Definition and applicability	501

12.2.2	Void	501

12.2.3	Void	501

12.2.4	Void	501

12.2.5	Void	501

12.3	EFNASCONFIG – Attach with IMSI handling	501

12.3.1	Definition and applicability	501

12.3.2	Void	501

12.3.3	Void	501

12.3.4	Void	501

12.3.5	Void	501

12.4	EFNASCONFIG – Verifying Minimum Periodic Search Timer	501

12.4.1	Definition and applicability	501

12.4.2	Void	502

12.4.3	Void	502

12.4.4	Void	502

12.4.5	Void	502

12.5	EFNASCONFIG – Extended access barring handling	502

12.5.1	Definition and applicability	502

12.5.2	Void	502

12.5.3	Void	502

12.5.4	Void	502

12.5.5	Void	502

12.6	EFNASCONFIG – Verifying Timer T3245 Behaviour	502

12.6.1	Definition and applicability	502

12.6.2	Conformance requirement	503

12.6.3	Test purpose	503

12.6.4	Method of test	503

12.6.4.1	Initial conditions	503

12.6.4.2	Procedure	504

12.6.5	Acceptance criteria	504

12.7	EFNASCONFIG – Override NAS signalling low priority	505

12.7.1	Definition and applicability	505

12.7.2	Conformance requirement	505

12.7.3	Test purpose	505

12.7.4	Method of test	505

12.7.4.1	Initial conditions	505

12.7.4.2	Procedure	506

12.7.5	Acceptance criteria	506

12.8	EFNASCONFIG – Override Extended access barring	506

12.8.1	Definition and applicability	506

12.8.2	Conformance requirement	506

12.8.3	Test purpose	507

12.8.4	Method of test	507

12.8.4.1	Initial conditions	507

12.8.4.2	Procedure	507

12.8.5	Acceptance criteria	508

12.9	EFNASCONFIG – Fast First Higher Priority PLMN Search	508

12.9.1	Definition and applicability	508

12.9.2	Conformance requirement	508

12.9.3	Test purpose	508

12.9.4	Method of test	508

12.9.4.1	Initial conditions	508

12.9.4.2	Procedure	509

12.9.5	Acceptance criteria	509

12.10	EFNASCONFIG – E-UTRA Disabling Allowed for EMM cause #15	510

12.10.1	Definition and applicability	510

12.10.2	Conformance requirement	510

12.10.3	Test purpose	510

12.10.4	Method of test	510

12.10.4.1	Initial conditions	510

12.10.4.2	Procedure	511

12.10.5	Acceptance criteria	511

12.11	EFNASCONFIG – SM_RetryWaitTime	512

12.11.1	Definition and applicability	512

12.11.2	Conformance requirement	512

12.11.3	Test purpose	512

12.11.4	Method of test	512

12.11.4.1	Initial conditions	512

12.11.4.2	Procedure	513

12.11.5	Acceptance criteria	513

12.12	EFNASCONFIG – SM_RetryAtRATChange	513

12.12.1	Definition and applicability	513

12.12.2	Conformance requirement	513

12.12.3	Test purpose	513

12.12.4	Method of test	514

12.12.5	Acceptance criteria	514

13	UICC interface during PSM	514

13.1	UICC interface in PSM handling for E-UTRAN – No UICC deactivation in PSM	514

13.1.1	Definition and applicability	514

13.1.2	Conformance requirement	514

13.1.3	Test purpose	514

13.1.4	Method of test	514

13.1.4.1	Initial conditions	514

13.1.4.2	Procedure	515

13.1.5	Acceptance criteria	515

13.2	UICC interface in PSM handling for E-UTRAN – PSM not accepted by E-USS/NB-SS	515

13.2.1	Definition and applicability	515

13.2.2	Conformance requirement	515

13.2.3	Test purpose	516

13.2.4	Method of test	516

13.2.4.1	Initial conditions	516

13.2.4.2	Procedure	516

13.2.5	Acceptance criteria	516

13.3	UICC interface in PSM handling for E-UTRAN – UICC deactivation in PSM	517

13.3.1	Definition and applicability	517

13.3.2	Conformance requirement	517

13.3.3	Test purpose	517

13.3.4	Method of test	517

13.3.4.1	Initial conditions	517

13.3.4.2	Procedure	518

13.3.5	Acceptance criteria	518

13.4	UICC interface in PSM for E-UTRAN – SUSPEND UICC	518

13.4.1	Definition and applicability	518

13.4.2	Conformance requirement	518

13.4.4	Method of test	519

13.4.4.1	Initial conditions	519

13.4.4.2	Procedure	519

13.4.5	Acceptance criteria	520

14	UICC interface during eDRX	520

14.1	UICC interface during eDRX for E-UTRAN – eDRX is not supported by the UICC	520

14.1.1	Definition and applicability	520

14.1.2	Conformance requirement	520

14.1.3	Test purpose	520

14.1.4	Method of test	520

14.1.4.1	Initial conditions	520

14.1.4.2	Procedure	521

14.1.5	Acceptance criteria	521

14.2	UICC interface during eDRX for E-UTRAN – eDRX is not accepted by E-USS/NB-SS	522

14.2.1	Definition and applicability	522

14.2.2	Conformance requirement	522

14.2.3	Test purpose	522

14.2.4	Method of test	522

14.2.4.1	Initial conditions	522

14.2.5	Acceptance criteria	523

14.3	UICC interface during eDRX for E-UTRAN – UICC deactivation during eDRX	523

14.3.1	Definition and applicability	523

14.3.2	Conformance requirement	523

14.3.3	Test purpose	524

14.3.4	Method of test	524

14.3.4.1	Initial conditions	524

14.3.4.2	Procedure	525

14.3.5	Acceptance criteria	525

14.4	UICC interface during eDRX for E-UTRAN– SUSPEND UICC	525

14.4.1	Definition and applicability	525

14.4.2	Conformance requirement	525

14.4.3	Test purpose	526

14.4.4	Method of test	526

14.4.4.1	Initial conditions	526

14.4.4.2	Procedure	527

14.4.5	Acceptance criteria	527

15	Authentication procedure and NAS security context handling for 5G	527

15.1	Authentication procedure for EAP-AKA'	527

15.1.1	Authentication procedure for EAP-AKA' - Authentication is successful	527

15.1.1.1	Definition and applicability	527

15.1.1.2	Conformance requirement	528

15.1.1.3	Test purpose	528

15.1.1.4	Method of test	528

15.1.1.4.1	Initial conditions	528

15.1.1.4.2	Procedure	529

15.1.1.5	Acceptance criteria	529

15.1A.1	Authentication procedure for EAP-AKA' - Authentication is Successful	530

15.1A.1.1	Definition and applicability	530

15.1A.1.2	Conformance requirement	531

15.1A.1.3	Test purpose	531

15.1A.1.4	Method of test	531

15.1A.1.4.1	Initial conditions	531

15.1A.1.4.2	Procedure	531

15.1A.1.5	Acceptance criteria	532

15.1.2	Authentication procedure for EAP-AKA' – Authentication is successful - GSM UICC	533

15.1.2.1	Definition and applicability	533

15.1.2.2	Conformance requirement	533

15.1.2.3	Test purpose	533

15.1.2.4	Method of test	534

15.1.2.4.1	Initial conditions	534

15.1.2.4.2	Procedure	534

15.1.2.5	Acceptance criteria	535

15.1.3	Authentication procedure for EAP-AKA' – AUTN fails on the USIM	535

15.1.3.1	Definition and applicability	535

15.1.3.2	Conformance requirement	535

15.1.3.3	Test purpose	535

15.1.3.4	Method of test	536

15.1.3.4.1	Initial conditions	536

15.1.3.4.2	Procedure	536

15.1.3.5	Acceptance criteria	536

15.1.4	Authentication procedure for EAP-AKA' - after SUPI is changed	537

15.1.4.1	Definition and applicability	537

15.1.4.2	Conformance requirement	537

15.1.4.3	Test purpose	537

15.1.4.4	Method of test	537

15.1.4.4.2	Procedure	538

15.2	Authentication procedure for 5G AKA	539

15.2.1.4	Method of test	540

15.2.1.4.1	Initial conditions	540

15.2A.1.4	Method of test	542

15.2A.1.4.1	Initial conditions	542

15.2A.2	Authentication procedure for 5G AKA – Authentication is successful - GSM UICC	544

15.2A.2.1	Definition and applicability	544

15.2A.2.2	Conformance requirement	544

15.2A.2.3	Test purpose	544

15.2A.2.4	Method of test	545

15.2A.2.4.1	Initial conditions	545

15.2A.2.4.2	Procedure	545

15.2A.2.5	Acceptance criteria	546

15.2.2	Authentication procedure for 5G AKA – Authentication is successful - GSM UICC	546

15.2.2.1	Definition and applicability	546

15.2.2.2	Conformance requirement	546

15.2.2.3	Test purpose	547

15.2.2.4	Method of test	547

15.2.2.4.1	Initial conditions	547

15.2.2.4.2	Procedure	547

15.2.2.5	Acceptance criteria	548

15.2.3	Authentication procedure 5G AKA – AUTN fails on the USIM	548

15.2.3.1	Definition and applicability	548

15.2.3.2	Conformance requirement	548

15.2.3.3	Test purpose	549

15.2.3.4	Method of test	549

15.2.3.4.1	Initial conditions	549

15.2.3.4.2	Procedure	549

15.2.3.5	Acceptance criteria	549

15.2.4	Authentication procedure for 5G AKA - after SUPI is changed	550

15.2.4.1	Definition and applicability	550

15.2.4.2	Conformance requirement	550

15.2.4.3	Test purpose	550

15.2.4.4	Method of test	550

15.2.4.4.1	Initial conditions	550

15.2.4.4.2	Procedure	551

15.2.4.5	Acceptance criteria	551

15.2.5.2	Conformance requirement	552

15.2.5.4	Method of test	553

15.2.5.4.1	Initial conditions	553

15.2.5.4.2	Procedure	553

15.2.5.5	Acceptance criteria	554

16	UE Route Selection Policy (URSP) procedure	557

16.1	Pre-configured URSP rules	557

16.1.1	Support for URSP by USIM	557

16.1.1.1	Definition and applicability	557

16.1.1.2	Conformance requirement	557

16.1.1.3	Test purpose	557

16.1.1.4	Method of test	558

16.1.1.4.1	Initial conditions	558

16.1.1.4.2	Procedure	559

16.1.1.5	Acceptance criteria	560

16.1.2	Support for URSP by ME	560

16.1.2.1	Definition and applicability	560

16.1.2.2	Conformance requirement	560

16.1.2.3	Test purpose	560

16.1.2.4	Method of test	560

16.1.2.4.1	Initial conditions	560

16.1.2.4.2	Procedure	561

16.1.2.5	Acceptance criteria	561

16.1.3	Support of Signalled URSP	562

16.1.3.1	Definition and applicability	562

16.1.3.2	Conformance requirement	562

16.1.3.3	Test purpose	562

16.1.3.4	Method of test	562

16.1.3.4.1	Initial conditions	562

16.1.3.4.2	Procedure	564

16.1.3.5	Acceptance criteria	564

17	CAG list handling	565

17.1	CAG list handling for 5G	565

17.1.1	Automatic CAG selection with preconfigured CAG list on USIM	565

17.1.1.1	Definition and applicability	565

17.1.1.2	Conformance requirement	565

17.1.1.3	Test purpose	565

17.1.1.4	Method of tests	565

17.1.1.4.1	Initial conditions	565

17.1.1.4.2	Procedure	566

17.1.1.4.3	Acceptance criteria	566

Annex A (informative):	Structure for uac-BarringInfo and Operator-defined access category	567

Annex B (informative):	Change history	572

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

The present document defines the application test specification.

The aim of the present document is to ensure interoperability between an UICC and a terminal independently of the respective manufacturer, card issuer or operator. The present document does not define any aspects related to the administrative management phase of the UICC. Any internal technical realisation of either the UICC or the Terminal is only specified where these are reflected over the interface.

Application specific details for applications residing on an UICC are specified in the respective application specific documents. The logical and physical Characteristics of the UICC Terminal interface is specified in document ETSI TS 102 221 [5]. The Universal Subscriber Identity Module (USIM)-application for 3G telecommunication networks is specified in document TS 31.102 [4].